3GPP TSG RAN WG2 Meeting #129                                             R2-2500269

Athens, Greece, Feb 17th – 21st, 2025

Agenda Item:	8.12.3

Source:	Xiaomi

Title:	Discussion on modelling of UE-initiated beam report

Document for:	Discussion and Decision

# Introduction

According to the RAN1 discussion on the event-driven beam report, RAN1 agreed two alternatives (i.e. Mode A and Mode B) for reporting the beam measurement result, either via dynamically scheduling UCI by gNB and/or UCI in pre-configured resource(s) for second UL channel). It is further studied that what is the modelling of trigger event determination for UE-initiated beam report.

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

In this contribution, we provide our analysis on how to model the beam measurement report procedure in PHY and MAC.

# Discussion

Figure 1: event-driven beam report procedure

According to the RAN1 agreements, after triggering the beam reporting (i.e. the triggering event determination), the UE uses either Mode A or Mode B to send the UCI including the beam measurement results. The UCI transmission/controlling procedure is captured in PHY, without any RAN2 specification impact. However, regarding how to trigger the measurement report (i.e. trigger event determination), we could have several options to model this function, where some options could have some RAN2 impact.

First of all, based on our understanding, all options for modelling trigger event determination function are technically feasible. None of the above modelling option is like LTM event-based measurement report procedure, as the LTM event-based measurement report procedure does not have the Event-x instance evaluation based on the beam measurement result, and does not count the number of Event-x instance(s) in a time window.

Observation 1: UE-initiated beam report based on Event-x is different from LTM event-based measurement report.

With regards to trigger event determination of UE-initiated beam report, we could split the function into the following steps:

- Step 1 (Reference signal measurement): The UE gets the measurement result by measuring the reference signal of a specific beam.

- Step 2 (Event instance evaluation): The beam measurement result is used to evaluate if an event instance (e.g. Event-2) is fulfilled.

- Step 3 (Event determination): By counting the number of event instances within a time window, the UE determines whether a measurement report is triggered.

There are different options of modelling the reporting procedure between PHY and MAC layer:

- Option 1: All steps in PHY

- Option 2: Event instance evaluation in PHY; Event determination in MAC

- Option 3: both event instance evaluation and event determination in MAC

Figure 2: Options for triggering event determination

Option 1 leaves all steps in PHY, without any RAN2 specification impacts in MAC. On the other hand, both Option 2 and Option 3 would require some interaction between PHY and MAC, which may require lots of coordination (e.g. LSs) between RAN2 and RAN1 for clarifying the details on how to evaluate the event instance and how to count the Event-x instance within a time window.

As the TU(s) for Rel-19 MIMO is quite limited, we would prefer Option 1 to leave all steps of triggering event determination in PHY.

Observation 2: Leaving all reporting procedure defined in PHY can help to reduce cross-WG coordination.

However, since Option 2 is similar to BFR procedure, it is also ok to follow the same design principle as BFR by using the instance indicated by PHY.

Proposal: Trigger event determination procedure of UE-initiated beam reporting is either defined totally in PHY or follow BFR procedure (i.e. MAC uses instance indicated by PHY).

# Conclusion

According to the above analysis, we have the following proposal:

Observation 1: UE-initiated beam report based on Event-x is different from LTM event-based measurement report.

Observation 2: Leaving all reporting procedure defined in PHY can help to reduce cross-WG coordination.

Proposal: Trigger event determination procedure of UE-initiated beam reporting is either defined totally in PHY or follow BFR procedure (i.e. MAC uses instance indicated by PHY).