3GPP TSG-RAN WG2 Meeting #129                                                                    R2-2500272

Athens, Greece, Feb. 17th – 21st, 2025

Source:	CATT

Title:	Discussion on the A-IoT Data Transmission and Other General Aspects

Agenda Item:	8.2.4

Document for:	Discussion

1.	Introduction

In this paper, we give our views on the potential issues for A-IoT data transmission according to the WID in [1]. The following aspects are covered in our discussion,

- Further details on AS ID

- Segmentation for D2R

- Further handling on data transmission

The Section 2 gives the detailed discussion, and followed by Section 3 that summarizes this paper.

2.	Discussion

- Further details on AS ID

The generation of AS ID

In our contribution for A-IoT random access [2], it is proposed that for the purpose of re-sending A-IoT Msg2 to trigger the same A-IoT Msg3 for the “specific” device, the AS ID can be assigned by A-IoT Msg2. Then the candidate solutions to assign this AS ID were captured in the TR 36.839 [3],

- Option 1: a random ID (if used in first D2R message) can be reused; - Option 2: the reader assigns this "AS ID" to the device. It needs to be further discussed via which R2D message. - Option 3: It is up to the reader whether to reuse the random ID (if used in first D2R message) as the "AS ID" or to assign a new "AS ID". It needs to be further discussed via which R2D message.

Option 1 should be excluded, as it does not ensure the uniqueness for the AS ID among the devices. For Option 2 and Option 3, it depends on reader control to ensure the uniqueness for AS ID. The only difference between the two options is whether to allow the reader to reuse the random ID for contention resolution as the AS ID. From our point of view, we prefer using the random ID only for the contention resolution purpose. A new AS ID can be assigned by the reader with a different length than 16 bit random ID.

Proposal 1: Support reader assigns the AS ID in A-IoT Msg2, which is not based on the random ID used in A-IoT Msg1.

On the use of AS ID

Upon receiving the AS ID by A-IoT Msg2, as suggested by our contribution for RACH in [2], it can be used for the re-transmission of A-IoT Msg3 by re-sending the A-IoT Msg2. Then another issue is whether the AS ID can be used for potential scheduling of data transmission after A-IoT Msg3 transmission.

- Case 1 - inventory-only service

For the inventory-only service, the potential D2R data transmission after A-IoT Msg3 can be the segment of device ID, if the UL grant in A-IoT Msg2 does not satisfy the transmission of the complete device ID. In this case, if the A-IoT Msg3 is successfully received by the reader (based on detection of the resource scheduled in A-IoT Msg2), the reader can further schedule this device by the AS ID within the R2D message for the remaining segments of the device ID, i.e., a new R2D message containing the AS ID and the UL grant. Besides, the R2D message should also indicate the initial transmission/re-transmission for this UL grant.

Observation 1: For the inventory-only service, a new R2D message containing the AS ID and UL grant is needed to schedule the potential initial transmission/re-transmission after A-IoT Msg3 transmission.

- Case 2 - command-only service

For the command-only service, now the baseline procedure is to firstly perform inventory to the device and then send the command to the device, i.e., the command signalling is sent to the device via the Step C1 captured in the TR [3],

In the R2D message of Step C1, besides the upper layer command, it should also include the corresponding UL grant for the command response. So the AS ID assigned by A-IoT Msg2 can be used to associate the UL grant for the corresponding device for the potential initial transmission/re-transmission of the response. That means, a new R2D message is needed to contain the upper layer command and include the UL grant associated the AS ID for the command response.

Observation 2: For the command-only service, a new R2D message including a container for upper layer command, the AS ID and the associated UL grant is needed to schedule the potential initial transmission/re-transmission after A-IoT Msg3 transmission.

- Case 3 - inventory and command service

For the inventory and command service, the only difference between Case 3 and Case 2 is that the reader firstly receives the inventory request from CN, then followed by receiving the command request. From the reader implementation perspective, it is also by the Step A, Step B, Step C1 and Step C2, which is in general of the same procedure as the command-only service (i.e. implemented with firstly inventory and then command).

Observation 3: For the inventory and command service, a new R2D message including a container for upper layer command, the AS ID and the associated UL grant is needed to schedule the potential initial transmission/re-transmission after A-IoT Msg3 transmission.

In summary, no matter which use case is initiated from CN, a new R2D message is needed to include the AS ID and the associated UL grant for the potential initial transmission/re-transmission after A-IoT Msg3. Note that the UL grant information may be provided in L1 control signalling or MAC signalling. For the Case 2 and Case 3, the R2D message should be able to optionally provide the container to convey the upper layer command to the device.

Therefore, how AS ID is used along with the corresponding R2D message is proposed that,

Proposal 2: Introduce a new R2D message for data transmission of the initial transmission/re-transmission after A-IoT Msg3. The R2D message includes at least the following information,

- AS ID assigned by A-IoT Msg2;

- UL grant associated with the AS ID (FFS provided in L1 control signalling or in A-IoT MAC layer signalling);

- Initial transmission/re-transmission indication associated with this UL grant;

- (Optional) container for upper layer command.

On the maintenance of AS ID

As discussed above, upon receiving the AS ID by A-IoT Msg2, the reader may further use this AS ID for potential data transmission after A-IoT Msg3. So the device should store the AS ID in its memory.

Proposal 3a: The device should store the AS ID upon receiving the AS ID by A-IoT Msg2.

If the device confirms that it had completed the service triggered by the reader, the device should release the AS ID. In last meeting discussion, we agreed that,

- The "R2D failure/success feedback indication" also applies to "following D2R data" after the random access procedure for re-access purpose.

So this optional feedback indication can be used by the device to determine the successful reception of the last data transmission by the reader, i.e., the service has been completed by the device.

Proposal 3b: The device should release the AS ID, if it determines the service is completed.

- Segmentation for D2R

For the AS layer segmentation functionality, RAN2 aims to design a simple solution as much as possible with the following baseline,

- Do not support sequence number for A-IoT segmentation functionalities. - Do not support segment number and the number of segments for A-IoT segmentation functionalities. - Capture the following option in TR: An indication is used to indicate to reader whether the data is segmented and whether it is last segment. FFS the details on how this is designed and number of bits (one or two). - RAN2 assumes that for the device we will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s).

It is still unclear how the segmentation works at the A-IoT MAC layer, specifically whether 1-bit to indicate the last segmentation is enough or not.

The detailed solution may have dependency on whether the reader can know the expected D2R message size from the CN indication. As per the SA2 latest progress [4],

The following assistance information may be provided: - AIoT service type (e.g. Inventory, Command); - Optionally, approximate number of AIoT devices based on AF request; - Optionally, approximate D2R message size based on AF request.

The CN can optionally provide the D2R message size to the reader.

Potential case A: the CN does not provide the expected D2R message size to the reader

Take the NR RLC-based segmentation functionality in UM as a reference, where the following information is needed,

Table I: NR RLC-based segmentation for UM mode RLC entity

| Control information | Intention of the control information |
| --- | --- |
| SN number for segmented RLC SDU | Only the segmented RLC SDU has SN number; If the RLC PDU contains a segment that maps to the last byte of an RLC SDU, then increment the SN state variable by one, i.e., all segments share same SN number to let the receiver know whether the RLC segments belong to the same original RLC SDU; |
| SI indication | Indicate whether an RLC PDU contains a complete RLC SDU or the first, middle, last segment of an RLC SDU; |
| SO indication | Indicate the position of the RLC SDU segment in bytes within the original RLC SDU; |

Now it was agreed to have no indications like SI/SO for A-IoT. Then, in order to indicate the receiver how to reassemble the segments if the device performs segmentation for a certain upper layer data, it needs to send the first segment, the middle segment(s) and the last segment in order. For example, after sending the first segment, the device waits for the scheduling for next transmission (implicitly confirm the successful reception of the previous transmission by the reader). Upon receiving the next scheduling, the device sends the following segments. Upon each segment transmission, the device uses 1-bit to indicate whether it is the end/last segment of the upper layer data. If the upper layer data is not segmented, it will indicate this is the last segment, which means no segmentation for data transmission.

Proposal 4: For the case of CN not providing the expected D2R message size to the reader:

- Without the segment number and the number of segments, the device should send the segments in order. The device does not send the subsequent segment until it confirms the successful reception of the previous transmission by the reader.

- One-bit indication is enough for A-IoT device to indicate the reader whether the data is segmented and whether it is last segment. Each segment uses this 1-bit indication to indicate whether it is the last segment. If the upper layer data is not segmented by the device, it will indicate this is the last segment.

Potential case B: the CN provides the expected D2R message size to the reader

With the expected D2R message size from the CN, the reader knows the whole D2R message size that the device needs to send. Then based on the expected D2R message size indication and the scheduled TBS, the reader can know whether there is remaining data in the device size. The only requirement is that the device should send the segment in order.

To pursue a uniform A-IoT MAC PDU design, the setting of one-bit indication as in above Proposal 4 for Case A can still be followed. It’s up to reader implementation to recognize the last segment based on this one-bit indication or the expected D2R message size.

Proposal 5: For the case of CN providing the expected D2R message size to the reader:

- Without the segment number and the number of segments, the device should send the segments in order. The device does not send the subsequent segment until it confirms the successful reception of the previous transmission by the reader.

- The one-bit indication as in Proposal 4 is followed (for a uniform A-IoT MAC design).

- Failure handling on data transmission

For the potential failure of data transmission, the baseline is that the device re-sends the same upper layer data based on the UL grant associated with the initial transmission or re-transmission. That means at each time the device receives an UL grant, it needs to store the start/end location for the upper layer data transmitted using this UL grant. If the next UL grant is for initial transmission, the device sends the data following the end location; else, the device should send the data from the start location again.

Proposal 6: To support the potential initial transmission or re-transmission of the failed D2R	 data, the device should store the start/end location for the upper layer data transmitted using the UL grant.

3. Conclusion

Based on the analysis in the Section 2, the conclusions of this paper are summarized as follows:

Observation 1: For the inventory-only service, a new R2D message containing the AS ID and UL grant is needed to schedule the potential initial transmission/re-transmission after A-IoT Msg3 transmission.

Observation 2: For the command-only service, a new R2D message including a container for upper layer command, the AS ID and the associated UL grant is needed to schedule the potential initial transmission/re-transmission after A-IoT Msg3 transmission.

Observation 3: For the inventory and command service, a new R2D message including a container for upper layer command, the AS ID and the associated UL grant is needed to schedule the potential initial transmission/re-transmission after A-IoT Msg3 transmission.

Further details on AS ID

Proposal 1: Support reader assigns the AS ID in A-IoT Msg2, which is not based on the random ID used in A-IoT Msg1.

Proposal 2: Introduce a new R2D message for data transmission of the initial transmission/re-transmission after A-IoT Msg3. The R2D message includes at least the following information,

- AS ID assigned by A-IoT Msg2;

- UL grant associated with the AS ID (FFS provided in L1 control signalling or in A-IoT MAC layer signalling);

- Initial transmission/re-transmission indication associated with this UL grant;

- (Optional) container for upper layer command.

Proposal 3a: The device should store the AS ID upon receiving the AS ID by A-IoT Msg2.

Proposal 3b: The device should release the AS ID, if it determines the service is completed.

Segmentation for D2R

Proposal 4: For the case of CN not providing the expected D2R message size to the reader:

- Without the segment number and the number of segments, the device should send the segments in order. The device does not send the subsequent segment until it confirms the successful reception of the previous transmission by the reader.

- One-bit indication is enough for A-IoT device to indicate the reader whether the data is segmented and whether it is last segment. Each segment uses this 1-bit indication to indicate whether it is the last segment. If the upper layer data is not segmented by the device, it will indicate this is the last segment.

Proposal 5: For the case of CN providing the expected D2R message size to the reader:

- Without the segment number and the number of segments, the device should send the segments in order. The device does not send the subsequent segment until it confirms the successful reception of the previous transmission by the reader.

- The one-bit indication as in Proposal 4 is followed (for a uniform A-IoT MAC design).

Failure handling on data transmission

Proposal 6: To support the potential initial transmission or re-transmission of the failed D2R	 data, the device should store the start/end location for the upper layer data transmitted using the UL grant.

4. References

[1] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[2] R2-2500271 “Discussion on the Random Access for Ambient IoT” CATT

[3] 3GPP TR 38.769 “Study on solutions for ambient IoT (Internet of Things)” V2.0.1

[4] 3GPP TS 23.xyz “Architecture support for Ambient power-enabled Internet of Things” V0.1.0