3GPP TSG-RAN WG2 Meeting #129                                                        R2-2500274

Athens, Greece, 17th – 21st Feb, 2025

Source:              CATT

Title:	             Discussion on Random Access in SBFD Symbols

Agenda Item:	8.11.2

Document for:	Discussion and Decision

Introduction

In the past three RAN2 meetings (RAN2#127, RAN2#127bis and RAN2#128), the random access in SBFD symbols was discussed. But there are still some leftover issues. In this contribution, the following leftover issues will be further discussed:

- Whether the working assumption on supported RACH triggers can be confirmed?

- Whether early identification in Msg3 needs to be supported when legacy RO is selected?

- For CFRA, which signaling is used to indicate the RO type？

- How to determine the RO type in case of CFRA fallbacks to CBRA?

- For CBRA, how to determine the RO type for initial RACH transmission?

- For CBRA, whether it is possible that UE first selects legacy RO and then performs RO type fallback after a number of RACH attempt?

- Whether RA-RNTI collision needs to be considered?

- Whether 2-step RA on SBFD symbols needs to be supported?

Discussion

Whether the working assumption on supported RACH triggers can be confirmed?

In RAN2#127 meeting, regarding to the supported RACH triggers, the following working assumption was reached:

- Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

In[1], all the RACH triggers are listed as below:

### 9.2.6 Random Access Procedure The random access procedure is triggered by a number of events: - Initial access from RRC_IDLE; - RRC Connection Re-establishment procedure; - DL or UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure (see clause 18.0) is ongoing, when UL synchronisation status is "non-synchronised"; - UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available; - Handover, except for when RACH-less HO is configured; - SR failure; - Explicit request by RRC upon synchronous reconfiguration; - RRC Connection Resume procedure from RRC_INACTIVE; - To establish time alignment for a primary or a secondary TAG; - Request for Other SI (see clause 7.3); - Beam failure recovery; - Consistent UL LBT failure on SpCell; - SDT in RRC_INACTIVE (see clause 18); - Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning; - Early UL synchronization with an LTM candidate cell; - RACH-based LTM cell switch.

From RAN2 perspective, allowing SBFD-aware to use RACH resources in SBFD symbol can reduce the RACH latency and improve RACH capacity. There is no motivation to prohibit SBFD-aware UE to use RACH resource on SBFD symbol for any of the above RACH trigger events. In RAN2#127bis and RAN2#128 meeting, some companies suggested excluding some RACH triggers which are not delay-sensitive, such as Msg1-based SI request and etc., but the motivation is not persuasive. Hence, the above working assumption reached in RAN2#127 meeting can be confirmed.

Proposal 1: Confirm the working assumption that random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Whether early identification is needed when legacy RO is selected?

In RAN2#127bis meeting, the following agreements were reached:

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

Based on the above agreements, it is obvious that if additional RO is selected, early identification via Msg1 is possible. If legacy RO is selected, since SBFD is not introduced as a feature combination, early identification via Msg1 is not supported.

Based on the above background, what needs to be discussed next is whether early identification via Msg3 needs to be supported in case of legacy RO is selected. According to [1], the RRC connection setup procedure is as below:

Figure-1 UE triggered transition from RRC_IDLE to RRC_CONNECTED

If early identification via Msg3 (step 1 in the Figure-1) is supported, the NW can allocate UL resource on SBFD symbols for RRCSetupComplete(step2a), ULInformationTransfer(step5) and securityModeComplete(step7a). It can reduce the UE access latency. On the contrary, if early identification via Msg3 is not supported, the NW can only aware whether the UE supports SBFD via UE capability enquiry procedure which is performed after AS security activation (e.g., after step 7a in Figure-1). It means step 2a/step 5/step 7a can only on transmitted on UL subframes. In order to reduce the access latency, it is slightly preferred to support early identification via Msg3 in case of legacy RO is selected.

Proposal 2: If legacy RO is selected, early identification via Msg3 can be introduced.

Furthermore, early identification via Msg3 has already been supported for other features, such as (e)RedCap UE and PUCCH repetition of Msg4 HARQ-ACK. In order to support early identification via Msg3 for these features, new LCIDs for CCCH were introduced. For SBFD, if early identification via Msg3 needs to be supported, the same early identification method can be reused, that is to introduce new LCIDs for CCCH of SBFD-aware UE.

Proposal 3: If legacy RO is selected and early identification in Msg3 is supported for SBFD-aware UE, new LCIDs for CCCH SDU specific for SBFD-aware UE can be introduced.

Signaling for the RO type indication for CFRA

In RAN2#128 meeting, regarding to initial RO type selection of CFRA, the following agreement was reached:

The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed).

Based on the above agreements, it is obvious that the RO type is indicated by NW for CFRA. But the detailed signaling is FFS.

In NR, CFRA can be triggered by the following cases:

- Case 1: PDCCH order;

- Case 2: Msg1-based SI request;

- Case 3: BFR;

- Case 4: RRCReconfigurationWithSync;

- Case 5: LTM.

For Case 1, the dedicated CFRA resources are indicated in PDCCH order. In RAN1#119 meeting, RAN1 already agreed that:

Agreement For CFRA triggered by PDCCH order for SBFD aware-UEs, SBFD-aware UE is explicitly indicated to use additional-RO or legacy-RO via a 1-bit field in the DCI of PDCCH order

Based on the above RAN1 agreement, it is obvious that RAN1 already agreed that the RO type can be indicated in the DCI of PDCCH order.

Proposal 4: For CFRA triggered by PDCCH order, follow RAN1 agreement, the RO type is indicated in DCI of the PDCCH order.

For Case 2, the CFRA resources will be indicated in SIB1. Hence, it is obvious that the RO type should also be indicated in SIB1, e.g., indicated in SI-RequestResources of SIB1.

Proposal 5: For CFRA triggered by Msg1-based SI request, the RO type is indicated in SI-RequestResources of SIB1.

For Case 3, the beam failure recovery configuration is delivered by dedicated RRC signalling, e.g., BeamFailureRecoveryConfig in RRCReconfiguration. Hence, the RO type should also be indicated in the BeamFailureRecoveryConfig.

Proposal 6: For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig.

For Case 4, the dedicated RACH resource is configured via the RACH-ConfigDedicated in ReconfigurationWithSync. Hence, it is obvious that the RO type should be indicated in RACH-ConfigDedicated.

Proposal 7: For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated.

For Case 5, for LTM, the dedicated RACH resource is configured via LTM Cell Switch Command MAC CE as below:

Figure-2 LTM Cell Switch Command MAC CE

Considering there is still R bit in the LTE Cell Switch Command MAC CE, one R bit can be used to indicate RO type.

Proposal 8: For CFRA triggered by LTM, one reserved bit in the LTM Cell Switch Command MAC CE can be used to indicate the RO type.

RO type selection in case of CFRA/CBRA fallback

According to [2], the following two kinds of RA type fallback are supported for CFRA:

- Case 1: CFRA fallbacks to CBRA.

- Case 2: CFRA fallbacks to CBRA, and CBRA fallbacks to CFRA again…….

For Case 1, CFRA fallbacks to CBRA include the following two sub-cases:

- Sub-case 1: CFRA is triggered but there is no suitable SSB/CSI-RS (i.e. no SSB/CSI-RS satisfy the RSRP requirement) or the preamble index indicated in PDCCH order is 0b000000;

- Sub-case 2: CFRA is performed for N(N≥ 1) times and when performs RA re-attempt, no SSB/CSI-RS satisfy the RSRP requirement.

For Case 2, after CFRA fallback to CBRA, for RACH re-attempt, once there is suitable SSB/CSI-RS, it can fallback to CFRA again. And during CFRA, if no SSB/CSI-RS satisfy the RSRP threshold, CFRA can fallback to CBRA again…….

According to RAN2 agreement, for CFRA, the RO type can be indicated by NW. In case of CFRA fallback to CBRA, there are two kinds of RO type selection methods:

- Alternative 1:  Always use the RO type indicated by NW for CFRA;

- Alternative 2:  When performing CFRA, uses the RO type indicated by NW; when performs CBRA, it follows the RO type selection rule used for CBRA.

For Alternative 1, the RO type can be same for CFRA and CBRA. While for Alternative 2, it is possible that the RO type can be different for consecutive two RACH procedure and it needs to discuss how to handle the power ramping. Hence, Alternative 1 is slightly preferred.

Proposal 9: If CFRA fallbacks to CBRA, the UE always use the RO type indicated by NW for CFRA.

Furthermore, for CBRA, it was agreed that RO type fallback can be performed after a number of RA attempts. It should further discuss whether the combination of CBRA/CFRA fallback and RO type fallback needs to be considered. Considering for CFRA, the RO type is indicated by NW. It means the RACH performance using the indicated RO type is better than another RO type. Hence, RO type fallback is unnecessary.

Proposal 10: For CFRA fallbacks to CBRA, RO type fallback is not considered during CBRA re-attempts.

Initial RO type selection for CBRA

In RAN2#128 meeting, regarding to the initial RO type selection of CBRA, the following agreement was reached:

FFS on the following options Option 1 Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one.

Based on the discussion in section 2.4, for CFRA fallback to CBRA, the RO type follows the NW indication. In order to make a uniform solution, Option 1 is slightly preferred.

Proposal 11: For CBRA, if RO type is indicated by NW, UE follows the RO type indicated by NW.

Furthermore, if NW does not indicate the RO type, which RO type can be selected by the UE should also be discussed. There are three potential options:

- Option 1: Based on RSRP;

- Option 2: Always selects legacy RO;

- Option 3: Always selects the SBFD RO.

According to the contributions in RAN2#128 meeting, most companies supports to use RSRP. That is if the RSRP of the DL pathloss reference is above the RSRP threshold (e.g., in the cell center), SBFD RO will be selected; otherwise, legacy RO can be selected in order to reduce the CLI.

Proposal 12:  For CBRA, if RO type is not indicated by NW, UE can determine the RO type based on the RSRP of DL pathloss reference. The detailed criterion is as below:

- If the RSRP of the DL pathloss reference is above the RSRP threshold, SBFD RO will be selected;

- Otherwise, legacy RO will be selected.

In addition, in RAN2#128 meeting, it also mentioned the case that how to determine the RO type when the RSRP threshold is not configured. The similar issue was also existed in the Rel-16 2/4-step RA type selection. And in TS38.331, it restricted that when both 2-step RA and 4-step RA are configured, this RSRP threshold is mandatory present. The similar rule can be used in RO type selection. That is to say when both SBFD RO and legacy RO are configured, the RSRP threshold should be mandatory present. There is no need to consider the case that RSRP threshold is not configured.

Proposal 13: If both SBFD RO and legacy RO are configured, the RSRP threshold for RO type selection should be mandatory present.

RO type fallback from legacy RO to additional RO for CBRA

In RAN2#127bis, regarding to the RO type fallback, the following agreement was reached:

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

Based on the discussion in section 2.5, it is possible that for CBRA, UE first select the legacy RO for initial RACH transmission. In our understanding, if UE selects legacy RO for initial RACH transmission, after certain (configured) number of RACH attempt, it is not reasonable for UE to fallback to SBFD RO. The reason is that if UE selects legacy RO for initial RACH transmission, it may follow the NW indication or based on RSRP. If it follows the NW indication, fallback is not wanted by NW. If it selects legacy RO based on RSRP, it means the UE is in the cell edge since legacy RO is selected, fallback to SBFD RO will increase the CLI.

Proposal 14: For CBRA, if legacy RO is selected for initial RACH transmission, it cannot fallback to SBFD RO after a number of times of RACH attempts.

RA-RANTI collision

In the past RAN2 meeting, some companies suggested discussing the RA-RNTI collision in case of the additional RO and legacy RO is overlapped for RACH configuration Option 2.

But in RAN1#119 meeting, the following agreements were agreed:

Agreement For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, down-select from the following alternatives: - Alt-1: do not optimize this case - Alt-2: the additional-ROs are considered invalid when overlapping with legacy-ROs

It is obvious RAN1 is discussing this issue. Hence, we can wait for RAN1 progress.

Proposal 15: For RA-RANTI collision issue, RAN2 can wait for RAN1 progress.

Whether 2-step RA on SBFD symbols needs to be supported?

In Rel-16, 2-step RA was introduced to reduce RA latency. When SBFD is introduced in Rel-19, it should further discuss whether 2-step RA on SBFD symbols needs be supported. Regarding to this issue, in RAN1#116 meeting and in RAN2#127 meeting, the following agreements were reached:

Agreements in RAN1#116 meeting: For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols. - FFS Type-2 random access procedure (2-step RACH) Agreements in RAN2#127 meeting: RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

The main benefit of introducing 2-step RA is to reduce the latency, which is same as one of the motivation of introducing SBFD. Since both of the two RACH features can achieve the same effect, whether the combination of them needs justification.

Proposal 16: Whether the combination of SBFD and 2-step RA should be supported needs to be justified.

Furthermore, during the RA procedure, when the order of RA type selection (2/4 step RA) and the RO type selection is tightly related with whether 2-step RA can be supported. Before RAN2 discussing the order of the RA type selection and the RO type selection, RAN2 should decide whether 2-step RA can be supported firstly.

Proposal 17: Before discussing the order of RA type (2/4-step RA) selection and the RO type selection, RAN2 should first make decision on whether 2-step RA on SBFD symbol is supported or not.

Conclusion

According to the analysis in section 2, it is proposed:

RACH triggers

Proposal 1: Confirm the working assumption that random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Early identification in Msg3

Proposal 2: If legacy RO is selected, early identification via Msg3 can be introduced.

Proposal 3: If legacy RO is selected and early identification in Msg3 is supported for SBFD-aware UE, new LCIDs for CCCH SDU specific for SBFD-aware UE can be introduced.

Signaling for the RO type indication for CFRA

Proposal 4: For CFRA triggered by PDCCH order, follow RAN1 agreement, the RO type is indicated in DCI of the PDCCH order.

Proposal 5: For CFRA triggered by Msg1-based SI request, the RO type is indicated in SI-RequestResources of SIB1.

Proposal 6: For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig.

Proposal 7: For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated.

Proposal 8: For CFRA triggered by LTM, one reserved bit in the LTM Cell Switch Command MAC CE can be used to indicate the RO type.

RO type selection for CFRA/CBRA fallback

Proposal 9: If CFRA fallbacks to CBRA, the UE always use the RO type indicated by NW for CFRA.

Proposal 10: For CFRA fallbacks to CBRA, RO type fallback is not considered during CBRA re-attempts.

RO type selection for initial RACH attempt (CBRA)

Proposal 11: For CBRA, if RO type is indicated by NW, UE follows the RO type indicated by NW.

Proposal 12:  For CBRA, if RO type is not indicated by NW, UE can determine the RO type based on the RSRP of DL pathloss reference

- If the RSRP of the DL pathloss reference is above the threshold, SBFD RO will be selected;

- Otherwise, legacy RO will be selected.

Proposal 13: If both SBFD RO and legacy RO are configured, the RSRP threshold for RO type selection should be mandatory present.

RO type fallback from legacy RO to SBFD RO (CBRA)

Proposal 14: For CBRA, if legacy RO is selected for initial RACH transmission, it cannot fallback to SBFD RO after a number of times of RACH attempts.

RA-RNTI collision

Proposal 15: For RA-RANTI collision issue, RAN2 can wait for RAN1 progress.

Support of 2-step RA

Proposal 16: Whether the combination of SBFD and 2-step RA should be supported needs to be justified.

Proposal 17: Before discussing the order of RA type (2/4-step RA) selection and the RO type selection, RAN2 should first make decision on whether 2-step RA on SBFD symbol is supported or not.

Reference

- TS38.300  NR; NR and NG-RAN Overall Description; Stage 2; (Release-18)

- TS38.321  NR; Medium Access Control (MAC) protocol specification (Release-18)