3GPP TSG-RAN WG2 Meeting #129	 R2-2500282

Athens, Greece, Feb 17-21, 2025

Agenda Item:	8.4.2

Source: 	NEC

Title:  	Discussion on LP-WUS in RRC_IDLE/INACTIVE

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#128 meeting, the following agreements are made:

FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session. FFS on the following options - - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping. - - Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed. - - Option 3: The formula for UE_ID based PEI subgrouping is reused.

In this contribution we further discuss the remaining issues for LP-WUS in RRC_IDLE/INACTIVE including configuration aspect, subgrouping and MR/LR band.

- 2 Discussion

- 2.1 configuration aspect

In the past few meetings, RAN2 has discussed dedicated LP-WUS signalling for UE-specific LP-WUS configuration and enabler/disabler of LP-WUS monitoring. In this section we further discuss this issue.

In our understanding, UE-specific LP-WUS configuration is useful especially for entry/exit condition of LP-WUS monitoring as different UE has different LR capability, one unified entry/exit condition broadcasted by network may not be suitable for all UEs to determine whether the UE need to monitor LP-WUS or not, such dedicated LP-WUS configuration can provide more precise evaluation. However, it seems like majority think this is not needed, then we are fine to not continue proposing this.

While for enabler/disabler of LP-WUS monitoring, it means the NW can indicate LP-WUS capable UE whether need to monitor LP-WUS or not when the UE is related to RRC_IDLE/INACTIVE. If without this enable/disable indication, normally all LP-WUS supporting UEs need to apply LP-WUS feature (i.e., monitoring LP-WUS when entry condition is fulfilled) as long as the NW is broadcasting LP-WUS configuration, this is not ideal and there are following reasons to support enable/disable indication:

1. Paging false alarm

When large amount of UEs are monitoring LP-WUS, once one UE is paged, unnecessary LP-WUS reception to other UEs is increased. Therefore if NW can control part of UEs (instead of all LP-WUS supporting UEs) monitor LP-WUS and other part of UEs do not monitor LP-WUS, it can provide flexibility and avoid serious paging false alarm issue.

2. Latency requirement

For LP-WUS monitoring UE, the paging delay is increased compared with normal paging monitoring UE when the paging is initiated after LP-WUS monitoring occasion and before paging monitoring occasion. If NW want one specific LP-WUS capable UE not to enter LP-WUS monitoring mode (e.g., considering latency), the NW can send this dedicated enable/disable indication to this UE, which provide flexibility and avoid latency issue.

3. Power consumption

The purpose of introducing LP-WUS is for power saving, but if the transition between MR and LR is frequent for one specific UE, the power saving could be overkilled. For example, if the paging probability is relatively high, the UE better not to monitor LP-WUS even though this UE support LP-WUS feature, otherwise, every time when this UE is monitoring LP-WUS, it will wake up its MR soon, which cannot be accepted. Therefore, NW can, based on implementation (e.g., information from paging probability), to determine whether such kind of UEs need to enter LP-WUS monitoring mode or not.

Based on the above reasons, we propose:

Proposal-1: support dedicated enable/disable indication in RRCRelease to indicate whether the UE need to perform LP-WUS monitoring or not during RRC_IDLE/INACTIVE.

Currently, entry/exit condition of LP-WUS monitoring is introduced to control whether the LP-WUS capable UE need to monitor LP-WUS or not when released to RRC_IDLE/INACTIVE. As mentioned above, the UE-specific entry/exit condition is not promising to be introduced, which means there is only one cell-specific entry/exit condition, if any cases, a UE fulfilling entry condition of LP-WUS monitoring may not be suitable to apply LP-WUS feature, e.g., the LP-WUS detection performance is still not ideal but LP-SS measurement is showing that the UE can apply LP-WUS feature. In such case, it is expected that the UE can fall back to legacy RRC_IDLE/INACTIVE after a long time to check whether everything is OK.

Furthermore, even though entry/exit condition of LP-WUS monitoring can work well, based on RAN1 discussion, LP-WUS is sent in codepoint way, if no subgroup in LP-WUS is paged, the NW will not send LP-WUS (i.e., implicit way), which means the LP-WUS monitoring UE is not aware that whether it is missed detection or just because NW does not send anything. It is better to have some mechanism to let UE fall back to legacy RRC_IDLE/INACTIVE if LP-WUS is not detected for a long time, to improve the robust.

Therefore some LP-WUS fallback mechanism can be considered, for example, one solution is timer-based LP-WUS fallback, e.g., a timer can be configured, if the UE has not received LP-WUS during the timer running, the UE should exit from LP-WUS monitoring. Or it could be up to UE implementation that the UE could exit from LP-WUS monitoring after a long time when LP-WUS is not detected.

Proposal-2: the following options can be considered for LP-WUS fallback:

Option-1: timer-based LP-WUS fallback, i.e., the UE exit from LP-WUS monitoring if LP-WUS is not detected during timer running

Option-2: up to UE implementation, i.e., the UE could exit from LP-WUS monitoring after a long time when LP-WUS is not detected

There could be some scenarios to also restrict UE to perform LP-WUS monitoring besides entry/exit condition of LP-WUS monitoring, e.g., other feature (e.g., SDT/MBS) is ongoing during RRC_IDLE/INACTIVE, or latency requirement (e.g., emergency session is registered), etc. If these exceptional cases are being operated by the UE, the UE should be allowed to not perform LP-WUS monitoring. In a word, entry/exit condition of LP-WUS monitoring could be the general one, but more exceptional cases to impact on LP-WUS monitoring can be FFS. Having said this, specific condition seems not needed to be specified in the spec, instead a NOTE may be sufficient to clarify UE behaviour.

Proposal-3: In some cases (e.g., MBS, SDT, etc.), the UE is allowed not to perform LP-WUS monitoring operation (i.e., exit from LP-WUS monitoring or not to enter LP-WUS monitoring) even if entry/exit condition of LP-WUS monitoring is (being) met.

Proposal-3a: If the P3 is agreeable, a NOTE can be added to clarify this.

- 2.2 Subgrouping

In this meeting, RAN2 need to down select UE-ID based subgrouping solution from the following options:

- Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping.

SubgroupID = (floor(UE_ID/(N*Ns*K)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID),

Where K = the subgrouping number for UE_ID based PEI subgrouping.

Where UE-ID = 5G-S-TMSI mod X

- Option 2: The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed.

SubgroupID = (floor(UE_ID/(N*Ns*K)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID),

Where K = fixed or configured value (e.g., 8).

Where UE-ID = 5G-S-TMSI mod X

- Option 3: The formula for UE_ID based PEI subgrouping is reused.

When the UE choose to monitor both LP-WUS and PEI (based on UE implementation), for option 1 and 2, by adding a new parameter related to PEI subgroup number (i.e., K), the UE belonging to the same PEI subgroup could be separately assigned to different LP-WUS subgroup, which could further reduce the paging false alarm. In our understanding, this is a way forward for false alarm issue and can be supported. More detail is that this is also related to RAN3 design on UE identify index value, follow-up actions in RAN3 will be proceed once we go for this solution. As for the difference between option 1 and 2, we didn’t see too much difference, both can separate the UE belonging to the same PEI subgroup, since this value K is anyway related to PEI subgroup number, we slightly prefer to go for option 1 which is clearer.

As for option 3, it is true that paging false alarm can be further reduced by NW configuring different subgroup number between LP-WUS and PEI, but once the LP-WUS subgroup number is an integer multiple of the PEI subgroup number, further reducing false alarm cannot be achieved, therefore the efficiency is not good.

Proposal-4: for subgrouping, adopt option-1, i.e., the subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping.

- 2.3 MR/LR band issue

Based on RAN1 discussion, the UE may not support LP-WUS reception on all the bands supported by the UE. In RAN#106, agreement is made that for idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656[1]). For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered.

Here are the solutions for intra-frequency case:

Solution-1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band).

Solution-2: MR is prioritized to camp on the same band as LR (System information, Paging on LR band; and RACH on another MR band based on reselection criteria).

Solution-3: MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation)

The main issue is that for one UE in RRC_IDLE/INACTIVE, if UE LR is camping on one band/cell different from the band/cell camped by UE MR, the MR and LR measurement result can not be used for each other, which causes RRM relaxation/offloading may not be achieved and some impact on entry/exit condition of LP-WUS monitoring can be foreseen. Therefore, in Rel-19, we only support intra-frequency case. But furthermore, companies think always restricting MR to camp on the same band/cell as LR could cause overload issue, i.e., all LP-WUS capable UEs need to camp on some specific band/cell for service, which is not promising.

Based on the above analysis, option-2 can be way forward, it seems like a combination between intra-frequency and inter-frequency, first we still restrict UE MR and LR to camp on the same band/cell to avoid inter-frequency scenario, meanwhile, before the UE back to RRC_CONNECTED after LP-WUS/paging reception, the UE can select a better cell for service (e.g., LR camped band is not the best cell for RRC_CONNECTED). Once solution-2 is agreed, more details should be considered, e.g., how the UE determine which cell is LR camped cell (e.g., based on frequency priorities or LP-WUS supporting information in SI) and which cell should be selected as the cell for RACH after LP-WUS/paging reception.

Proposal-5: in Rel-19, MR and LR should camp on the same band/cell for LP-WUS/paging monitoring, and MR may select another cell/band for random access after LP-WUS/paging reception, FFS how the UE determine which band/cell is for LP-WUS/paging monitoring and which band/cell is for random access.

- 3 Conclusions

Proposal-1: support dedicated enable/disable indication in RRCRelease to indicate whether the UE need to perform LP-WUS monitoring or not during RRC_IDLE/INACTIVE.

Proposal-2: the following options can be considered for LP-WUS fallback:

Option-1: timer-based LP-WUS fallback, i.e., the UE exit from LP-WUS monitoring if LP-WUS is not detected during timer running

Option-2: up to UE implementation, i.e., the UE could exit from LP-WUS monitoring after a long time when LP-WUS is not detected

Proposal-3: In some cases (e.g., MBS, SDT, etc.), the UE is allowed not to perform LP-WUS monitoring operation (i.e., exit from LP-WUS monitoring or not to enter LP-WUS monitoring) even if entry/exit condition of LP-WUS monitoring is (being) met.

Proposal-3a: If the P3 is agreeable, a NOTE can be added to clarify this.

Proposal-4: for subgrouping, adopt option-1, i.e., the subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping.

Proposal-5: in Rel-19, MR and LR should camp on the same band/cell for LP-WUS/paging monitoring, and MR may select another cell/band for random access after LP-WUS/paging reception, FFS how the UE determine which band/cell is for LP-WUS/paging monitoring and which band/cell is for random access.

- 4 References

[1] RP-242656 	RAN#106 	vodafone