3GPP TSG-RAN WG2 Meeting #129	 R2-2500284

Athens, Greece, Feb 17-21, 2025

Agenda Item:	8.4.4

Source: 	NEC

Title:  	Discussion on LP-WUS in RRC_CONNECTED

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#128 meeting, the following agreements are made:

The drx-onDurationTimer is not started with Option 1-2 LP-WUS.

For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS.

Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

In this contribution we further discuss the remaining issue for LP-WUS in RRC_CONNCTED including short C-DRX, CSI/SRS reporting and DCCA.

- 2 Discussion

- 2.1 Short C-DRX for option 1-2

In this section we discuss whether the short C-DRX can be configured with LP-WUS simultaneously in the case of option 1-2 is configured for LP-WUS monitoring.

In option 1-2, we already agreed that drx-onDurationTimer will not be started for PDCCH monitoring, instead, the UE monitor PDCCH during a new active timer triggered by LP-WUS or during other potential active timer (except for drx-onDurationTimer) is running. When it comes to short C-DRX cycle, in legacy behavior, if drx-InactivityTimer is expired or DRX Command MAC CE is received, short C-DRX cycle will be used, and drx-onDurationTimer will be started more frequently for PDCCH monitoring based on short C-DRX cycle. However things are different that in option 1-2, drx-onDurationTimer will not be started for PDCCH monitoring and it is only for CSI reporting during the congifured window. In this sense it is questionable that whether we still need to consider short C-DRX mechanism in option 1-2. Therefore it is suggested that RAN2 should make down selection for whether to use short C-DRX or not.

Proposal-1: for option 1-2, RAN2 to down select the following options for short C-DRX:

Opt-1: LP-WUS cannot be configured with short C-DRX simultaneously

Opt-2: The drx-onDurationTimer is not started with Option 1-2, including drx-onDurationTimer triggered by short C-DRX.

- 2.2 CSI reporting for option 1-2

In last meeting, we agreed that For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time. This is referred to DCP mechanism, but it is not complete in our understanding.

Fig.1 UE report periodic CSI during the configured drx-onDurationTimer

Based on the agreement, it can be seen from Fig.1 that UE can report periodic CSI/L1-RSRP during the configured drx-onDurationTimer even though this timer will never be started. In legacy, why we need this control of CSI reporting is that when the drx-onDurationTimer is not running due to DCP is not detected, the UE still can report periodic CSI/L1-RSRP. In fact, the general principle should be as long as the UE is within the active time window, CSI reporting can be enabled. When it comes to LP-WUS, not only the legacy drx-onDurationTimer but also the new introduced active timer triggered by LP-WUS should be considered, if the new active timer is not started due to LP-WUS, the network can configure (with a flag) whether the UE can report periodic CSI/L1-RSRP or not when this new active timer is not running, this is exactly to refer DCP mechanism. The diagram can be seen from Fig.2.

Fig.2 network can configure whether the UE can report CSI or not during new active timer

Proposal-2: for option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the new active timer triggered by LP-WUS when this timer is not running.

- 2.3 DCCA

No matter it is DCP or LP-WUS, both are wake-up signal for RRC_CONNECTED UE to trigger C-DRX active timer for PDCCH monitoring. In this sense, similar approach can be referred for both signal regarding DCCA.

First we discuss DC case, there is no restriction that which CG can be configured with DCP, which means DCP can be configured per cell group, and each cell group is independently operating DCP. Since we already have NR-DC framework, it is no problem to let each cell group to operate its own LP-WUS. Furthermore it is also noted that DCP can only be configured in SpCell within the same cell group. In this sense, the LP-WUS is also recommended to be configured in SpCell and can trigger PDCCH monitoring of all activated serving cell within the same cell group.

Proposal-3: LP-WUS can be configured independently in MCG and SCG, and LP-WUS configured in SpCell can trigger the PDCCH monitoring of all activated serving cells within the same cell group.

Then we discuss CA case, it is noted that when CA is configured, DCP is only configured on the PCell and the network does not configure secondary DRX group with DCP simultaneously, we prefer to use the same approach. Reason is that for secondary DRX group, when two DRX groups are configured, each serving cell is uniquely assigned to either of the two DRX groups. Normally the secondary DRX group mechanism is used to differentiate serving cell characteristic, e.g., serving cells belonging to FR1 is assigned to one DRX group and other serving cells belonging to FR2 is assigned to another DRX group. When it comes to LP-WUS, it is not clear that whether the LP-WUS deployment for FR2 cell is practical, therefore, to keep it simple, we can just follow DCP principle, i.e., the network does not configure secondary DRX group with LP-WUS simultaneously.

Proposal-4: the network cannot configure secondary DRX group with LP-WUS simultaneously.

One more thing for DCCA case is that whether there is some indication in LP-WUS content to indicate SCell dormancy for CA case and SCG deactivation for CG case. In our understanding, to save bit in LP-WUS content, nothing but subgroup indication should be introduced in LP-WUS content. RAN1 agreed that SCell dormancy will not be considered, furthermore, SCG deactivation is also not needed, RAN2 can confirm the following proposal.

Proposal-5: RAN2 confirm that LP-WUS is not used to indicate SCG deactivation and SCell dormancy.

- 2.4 others

It is recommended to give the new active timer triggered by LP-WUS a name, so that unnecessary terminology misunderstanding can be avoided and discussion can be facilitated. From our perspective, since both LP-WUS and DCP are used to trigger a basic active timer for PDCCH monitoring, it is better to call it as drx-onDurationTimerLP-WUS.

Proposal-6: name the new active timer triggered by LP-WUS as drx-onDurationTimerLP-WUS.

- 3 Conclusions

Proposal-1: for option 1-2, RAN2 to down select the following options for short C-DRX:

Opt-1: LP-WUS cannot be configured with short C-DRX simultaneously

Opt-2: The drx-onDurationTimer is not started with Option 1-2, including drx-onDurationTimer triggered by short C-DRX.

Proposal-2: for option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the new active timer triggered by LP-WUS when this timer is not running.

Proposal-3: LP-WUS can be configured independently in MCG and SCG, and LP-WUS configured in SpCell can trigger the PDCCH monitoring of all activated serving cells within the same cell group.

Proposal-4: the network cannot configure secondary DRX group with LP-WUS simultaneously.

Proposal-5: RAN2 confirm that LP-WUS is not used to indicate SCG deactivation and SCell dormancy.

Proposal-6: name the new active timer triggered by LP-WUS as drx-onDurationTimerLP-WUS.

- 4 References