3GPP TSG-RAN WG2 Meeting #129	R2-2500287

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.3.1

Source:	OPPO

Title:	Discussion on text proposal

Document for:	Discussion, Decision

Discussion

There is one agreement @RAN2#127bis meeting missed in TR[1] proposal and now it is captured in this version.

The Simulation assumption of RRM measurement prediction can be reused unless otherwise specified.

For agreements @128 meeting, the part marked with grey is to be either reflected in the template or under post email discussion [Post128][018][AI Mob] generalization (Apple). So they are expected to be resolved at this meeting. The rest agreements marked with cyan are captured in this version.

One thing to clarify is that rapporteur found out that the listed GC#1 of generalization study in table 1 on UE speed is not aligned with agreed definition. Here is agreed definition:

•	Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B.

Taking baseline 30km/h (FR1) as example. The definition suggests following combinations:

|  | Training @Dataset: 30km/h | Training @Dataset: 60km/h | Training @Dataset: 90km/h | Inference @30km/h | Inference @60km/h | Inference @90km/h |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | Yes |  |  | Yes |  |  |
| GC#1 |  | Yes | Yes | Yes |  |  |
| GC#2 | Yes | Yes | Yes | Yes |  |  |

Table 1

While current agreed combinations are:

|  | Training @Dataset: 30km/h | Training @Dataset: 60km/h | Training @Dataset: 90km/h | Inference @30km/h | Inference @60km/h | Inference @90km/h |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | Yes |  |  | Yes |  |  |
| GC#1 | Yes |  |  |  | Yes | Yes |
| GC#2 | Yes | Yes | Yes | Yes |  |  |

Table 2

The common aspect between these two approaches is that one model in one configuration will be tested in another configuration to verify the generalization characteristics. For GC#1 cases in Table 2, the performance of the model is impacted by both UE speed itself and the generalization operation. While the GC#1 cases in Table 1 are impacted by generalization operation only because the inference is done with same UE speed. From evaluation perspective to align GC#1 with agreed definition is a better approach, which is reflected in Table 5.2.1-5 of current version of text proposal.

Proposal 1: RAN2 confirm that GC#1 of UE speed for both FR1 and FR2 is aligned with agreed definition.

Annex Agreements @RAN2#128 meeting

Agreements/Observations to be captured in TR

- The prediction accuracy for intra-frequency temporal domain case B reduces as MRRT increases.

- For temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance.  Companies may report the adopted skipping pattern when providing simulation results.   Companies are not required to run new simulations but can clarify in the spreadsheet.    We can capture in the TR how the skipping patter affects the performance.

- For the inter-frequency prediction, the evaluation results show that the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy.   FFS on observations on low correlations.

- For inter-frequency, cluster as input (i.e. measurements from different cells as inputs )can improve the prediction accuracy than single cell as input.    For temporal domain, the gains are unclear.   RAN2 will focus on frequency domain for cluster based approach.

- In cluster approach the model takes measurements from more than one cell as inputs.

Agreements/observations

- Companies can compare results with non-AI approaches.   Temporal domain, sample and hold, and frequency domain, pathloss offset.   Companies can consider other simple models (e.g. ARIMA).

- For FR1 intra-frequency temporal domain case B, when PW is short, the performance between AI and sample-and-hold is not significant. However, when PW becomes larger, AI outperforms sample-and-hold.

- In frequency domain prediction cases (2GHz and 4GHz), cell-based approaches achieve limited gain compared to pathloss offset without the help of neighbor cell measurement. The cluster-based shows better performance compared to pathloss offset.

- For intra-frequency temporal domain case A, AI can provide gains (in terms of L3 cell RSRP difference) compared to sample and hold. The gain improves with UE speed.

Agreements

- Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following cases,

- Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B.

- Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B.

- Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B.

2	Companies can choose which case they compare with and should report it with simulation results.

3	Generalization issues on RRM measurement prediction are prioritized.

4     Start the study with generalization issue with RRM measurement prediction in temporal domain.   Companies can chose to study frequency domain prediction cases and report what they have simulated.

Study generalization over UE speeds

The simulation assumption of FR1 temporal domain case B is reused for generalization study with 3 UE speeds i.e. 30Km/h, 60Km/h and 90Km/h.  FFS on combinations

The simulation assumption of FR2 temporal domain case A is reused for generalization study with 3 UE speeds i.e. 60Km/h, 90Km/h and 120Km/h.  FFS on combinations

Companies that would like to study inter-frequency generalization can start with input 2GHz and output 4GHz, and 4GHz to 2GHz.   FFS if we introduce a third frequency.

Study model generalization across different cell configurations (e.g. ISD, gNB height, power, beam pattern, etc).   FFS which parameters we prioritize.

Agreements for generalization

1	Between GC#1 and GC#2, RAN2 focus on GC#2

2	simulation combination for FR1 generalization study:

|  | Training @Dataset: 30km/h | Training @Dataset: 60km/h | Training @Dataset: 90km/h | Inference @30km/h | Inference @60km/h | Inference @90km/h |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | Yes |  |  | Yes |  |  |
| GC#1 | Yes |  |  |  | Yes | Yes |
| GC#2 | Yes | Yes | Yes | Yes |  |  |
| Baseline |  | Yes |  |  | Yes |  |
| GC#1 |  | Yes |  | Yes |  | Yes |
| GC#2 | Yes | Yes | Yes |  | Yes |  |
| Baseline |  |  | Yes |  |  | Yes |
| GC#1 |  |  | Yes | Yes | Yes |  |
| GC#2 | Yes | Yes | Yes |  |  | Yes |

2a simulation combination for FR2 generalization study:

|  | Training @Dataset: 60km/h | Training @Dataset: 90km/h | Training @Dataset: 120km/h | Inference @60km/h | Inference @90km/h | Inference @120km/h |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | Yes |  |  | Yes |  |  |
| GC#1 | Yes |  |  |  | Yes | Yes |
| GC#2 | Yes | Yes | Yes | Yes |  |  |
| Baseline |  | Yes |  |  | Yes |  |
| GC#1 |  | Yes |  | Yes |  | Yes |
| GC#2 | Yes | Yes | Yes |  | Yes |  |
| Baseline |  |  | Yes |  |  | Yes |
| GC#1 |  |  | Yes | Yes | Yes |  |
| GC#2 | Yes | Yes | Yes |  |  | Yes |

Agreements on measurements events

1 Agree to listed 4 definitions of indirect measurement event prediction

Indirect measurement event prediction for temporal domain case A:

In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model.

Indirect measurement event prediction for temporal domain case B:

In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model  for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model.

Indirect measurement event prediction for frequency domain:

In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model.

Indirect measurement event prediction for spatial domain:

In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model

2	The input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Additional input(s) is also allowed.

3	For intra-frequency temporal domain case B indirect measurement event prediction is taken as baseline. Direct prediction is optional.

4	To agree following definition for true event prediction, false event detection and missed event detection for indirect measurement event prediction

Counter n3(true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa

Counter n1(false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, maximum ETD]

Counter n2(missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, maximum ETD]

5	To agree following definition for true event prediction, false event detection and missed event detection for direct measurement event prediction

Counter n3’ (true event prediction): it increases by 1 when a real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold

Counter n1’ (false event detection): it increases by 1 when no real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold

Counter n2’ (missed event detection): it increases by 1 when a real event occurs, but it doesn’t fall in the occurrence window of any predicted event whose possibility is higher than a predefined threshold

6	Agree following definition of F1 score:

F1 score = 2*Precision*Recall/(Precision + Recall)			Formula_3 (13/14)

7	In addition to F1 score, precision and recall in following formula are optional metrics to report. (12/14)

Precision	= n3/(n1+n3) 		Formula_1

Recall 	=n3/(n2+n3)			Formula_2

Note: for direct prediction, the counter should be n1’,n2’,n3’

1	Agreed values for Case A.  NOTE1 indirect prediction only

| Parameters | baseline value | Note |
| --- | --- | --- |
| A3 event offset (db) | 2 | Open for 3db |
| TTT (ms) | 320 | Open for one shorter value |
| UE speed (km/h) | 90 | Open for 60 and 120km/h |
| OW length (ms, note1) | N/A | Up to implementation |
| PW length (ms, note1) | 400 | Open for more values |
| Max ETD (ms, note1) | 80 | Open for more values |

2	Agreed values for Case B. NOTE1 indirect prediction only

| Parameters | baseline value | Note |
| --- | --- | --- |
| A3 event offset (db) | 2 | Open for 3db |
| TTT (ms) | 320 | Open for one shorter value |
| UE speed (km/h) | 30 | Open for 60 and 90km/h |
| OW length (ms,note1) | N/A | Up to implementation |
| PW length (ms,note1) | 200 (non-sliding) 40ms (sliding) | Open for more values |
| Max ETD (ms, note1) | 80 | Open for more values |
| MRRT | 50% | Open for more values |

3 Company can report which filtering options is being used for the input L3 RSRP of sub-use case 2: option 1, option 2, option 3.

4	to capture the 3 options into TR

Agreements on RLF prediction:

1	 To agree following indirect RLF prediction definition:

The future L1 SINR results are predicted based on actual L1 SINR results of the serving cell by following same way of intra-frequency temporal domain case A. Afterwards, RLF event at one time instance is determined based on predicted and optionally actual L1 SINR results within T310 duration, without further AI/ML models. As baseline L1 SINR refers to raw L1 SINR without L1 filtering.

2	Reuse agreed metrics for measurement event prediction for RLF prediction for both indirect and direct prediction respectively

3	Table 5.1-1 in TR 38.744 is taken as baseline simulation assumption for RLF prediction for both FR1 and FR2

4	It is assumed that all cells are fully loaded for interference modelling and no resource scheduler is needed

5	Interference in simulation comes from co-site cells and surrounding 6 sites of serving cell, i.e., interference comes from 20 cells

6	The beam with highest L1 RSRP of the serving cell is taken as serving beam, which is taken as the serving signal of RLM

1	Agree to following values (NOTE1 indirect only)

| Parameter | Value |
| --- | --- |
| Qin threshold | -6db |
| Qout threshold | -8db |
| Sample rate (TIndication_interval) | 20ms (FR2)/40ms(FR1) |
| Qin evaluation period | 100ms |
| Qout evaluation period | 200ms |
| T310 | 1000ms |
| N310 | 1 |
| N311 | 1 |
| Max ETD (ms, note1) | 80ms |
| PW length (ms,note1) | 400 |
| OW length (ms, note1) | Up to implementation |

2	The beam transmission pattern is synchronized across the site/cells i.e., at any given time the transmitted beam index is the same across the site/cells.

Agreements on direct prediction for RLF and event predictions

1	The time window for direct prediction goes for interpretation 2

2	For direct prediction, following values are agreed and company can report probability threshold for corresponding case (other values are allowed)

|  | Measurement event prediction | RLF prediction |
| --- | --- | --- |
| Time window length (Interpretation 2) | PW length as indirect case (FR1:200ms; FR2:400ms) | PW length as indirect case FR1/FR2: 400ms |

For System level simulation

1 	For measurement event prediction for temporal domain case B:

UE reports when A3 event is satisfied with actual measurements and predicted results.   And handover command will be received after handover preparation.

Remove T0 from the picture

2	For measurement event prediction for temporal domain case A, company focus on option 2 or option 3.

Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1))

Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT.

3	Handover Preparation time is 40ms and handover execution time: 40ms

4	RAN2 focus on indirect prediction methodology.

Agreements on SLS :

1	As for handover model it is proposed to agree for both temporal domain case A and case B:

Network start with handover preparation once a predicted measurement event is received. A handover command will be transmitted at least after preparation is completed. After that one fixed execution time is assumed.

3	To reuse HO failure model and corresponding metrics i.e. HO failure rate, total number of HO attempts per UE per second from 36.839

4	The agreed parameters in section 2.1.3 for measurement event prediction can be reused for SLS

5	The agreed parameters in table 2.2.2-1 i.e. all but last 3 parameters can be reused for both FR2 temporal domain case A and FR1 temporal domain case B in SLS

6	Interference model in section 2.2.3 is reused for SLS

As for simulation based on temporal domain case B, agree following approach:

If a predicted A3 event at t1 is reported at t0 (t0<=t1) then HO command is transmitted at t3, where t3=t0+max(HO prep time, t1-t0).

-	Samsung and Xiaomi thinks we should be able to report at t1.  Interdigital thinks that it should be when the command is assumed to be received (rather than when it is transmitted)

Agreements on inter-frequency prediction :

To use Pearson correlation coefficient for correlation coefficient calculation (14/14)

Reference

[1] R2-2410186 Discussion on Text proposal.docx