3GPP TSG-RAN WG2 Meeting #129	R2-2500290

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.3.2.1

Source:	OPPO

Title:	Analysis on simulation results of RRM measurement prediction

Document for:	Discussion, Decision

Introduction

In RAN2#126, the following cases are identified for RRM measurement prediction with 3 cases of high priority for evaluation.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In RAN2#128, companies have uploaded their RRM simulation results to an FTP server, with the files located under the directory path:

Email_Discussion/[Misc]/R19 AI MOB Simulation Results/RRM Prediction.

This dataset provides a comprehensive collection of simulations that various companies have conducted, offering valuable insights into the complexity and performance of AI/ML models used in different scenarios. In this paper, we will delve into the intricacies of these models by analyzing the data provided. The analysis of these models is not only pertinent to understanding their computational demands but also to gauging their potential impact on network performance and user experience.

Discussion

Complexity of model

Understanding AI/ML model complexity is pivotal. Figure 1, derived from the results of RAN2#128, offers a comprehensive visualization of the interplay between model parameters (K) and computational complexity measured in FLOPs for high-priority scenarios 2, 3, and 4.

Figure 1: Complexity of AI/ML models from RAN2#128 results

After applying a logarithmic transformation to the data, a clear linear relationship emerges between the number of model parameters (K) and computational complexity in FLOPs (M). This log-linear relationship indicates that as the number of parameters increases exponentially, the computational complexity grows at a proportional rate. This finding is critical for understanding the scalability and resource demands of deploying AI/ML models in practical applications

Observation 1: Computational complexity in FLOPs (M) exhibits a log-linear relationship with the number of model parameters (K).

The figure underscores that the size of the model, which is directly proportional to the number of parameters, dictates the amount of required computational resources. Larger models necessitate more processing power and memory, emphasizing the importance of balancing model size with available hardware capabilities when deploying AI/ML solutions in real-world environments.

For all scenarios examined, the largest model used does not exceed 10 million parameters. Given the current capability of UEs to handle models with billions of parameters, it is feasible for devices to deploy several specialized models tailored to different scenarios. This flexibility allows for optimized performance across various use cases without overwhelming device resources. By leveraging advanced hardware and software optimizations, future UE deployments could dynamically switch between models based on real-time needs, enhancing both efficiency and effectiveness.

Observation 2: The largest model being used has fewer than 10 million parameters. Compared to current UE capabilities, it is feasible for a UE to deploy multiple specialized models for different scenarios.

The examination of AI/ML model complexity in RAN2#128 simulations provides a foundational understanding of the computational and practical considerations involved in deploying these models. The model complexity analysis could also have implications for the latency of getting prediction results, which are critical performance metrics in RAN4. Therefore, it is proposed that

Proposal 1: Capture Figure 1 (complexity of AI/ML models in terms of FLOPs and number of parameters for 3 high-priority scenarios) into TR

- Note: the figure can be revised accordingly if the dataset changes.

The metrics to capture in TR

During post email discussion [POST128][021][AI Mob] Templates for simulations, one issue is raised for Q13 i.e. what kind of prediction accuracy is to be captured when RAN2 tries to capture statistics information into TR.

Now in the template there are 3 types prediction accuracy for temporal domain case A and case B:

- Average L3 cell RSRP difference (dB)

- Last predicted point L3 cell RSRP difference (dB)

- Multiple predicted point L3 cell RSRP differences (dB)

It will be difficult to capture the 3rd option because the number of the measurement results depends on detail simulation assumptions. For temporal domain case B, it is related to detail skipping pattern. For temporal domain case A, it is related to PW length, sliding or non-sliding filtering approach and measurement period for non-sliding filtering approach.

The distance between two measurement results in PW is one sampling period and one measurement period for sliding and non-sliding filtering approach respectively. And RAN2 agreed that longer PW length correlates with decreased prediction accuracy. So, the prediction accuracy of individual measurement results in PW could be quite different between these two approaches. Taking FR2 temporal domain case A as example:

Figure 2: Comparison between two filtering approach

In Figure 2, the 1st measurement result of non-sliding filtering approach is aligned with 5th measurement result of sliding filtering approach. So, most likely the 1st measurement results of non-sliding filtering approach is not comparable with 1st one of sliding filtering approach because it could be similar to 5th one. And it will be even more difficult to compare among companies.

Observation 3: The prediction accuracy of measurement results is not comparable between different filtering approaches

Because the PW is usual small for temporal domain case B, there is not much different among companies assuming same filtering is adopted. And for temporal domain case A, per post email discussion [POST128][021][AI Mob] the statistics could be performed within same PW length. So, the prediction accuracy of measurement results within PW is comparable among companies.

Observation 4: For same filtering approach, the prediction accuracy of measurement results within PW is comparable among companies.

Proposal 2: Average L3 cell RSRP difference and worst (last) predicted point L3 cell RSRP difference of measurement results within PW is captured in TR

Proposal 3: For both temporal domain case A and case B, simulation result with different filtering approach is listed separately.

Conclusion

In this contribution, we have shared our analysis on simulation results on RRM measurement prediction and have the following observations:

Observation 1: Computational complexity in FLOPs (M) exhibits a log-linear relationship with the number of model parameters (K).

Observation 2: The largest model being used has fewer than 10 million parameters. Compared to current UE capabilities, it is feasible for a UE to deploy multiple specialized models for different scenarios.

Observation 3: The prediction accuracy of measurement results is not comparable between different filtering approach

Observation 4: For same filtering approach, the prediction accuracy of measurement results within PW is comparable among companies.

It is proposed that

Proposal 1: Capture Figure 1 (complexity of AI/ML models in terms of FLOPs and number of parameters for 3 high-priority scenarios) into TR

- Note: the figure can be revised accordingly if the dataset changes.

Proposal 2: Average L3 cell RSRP difference and worst (last) predicted point L3 cell RSRP difference of measurement results within PW is captured in TR

Proposal 3: For both temporal domain case A and case B, simulation result with different filtering approach is listed separately.