3GPP TSG-RAN WG2 Meeting #129	R2-2500292

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.7.5

Source: 	Canon Research Centre France

Title:	Discussion on RLC AM Enhancements

Document for:	Discussion

# Introduction

During the RAN2#128 meeting at Orlando ([1]), RAN2 discussed RLC AM and reached the following agreements:

Agreements on unnecessary RLC retransmissions avoidance - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced

Special handling to avoid PDCP control PDU discard is not needed. A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

- Timely RLC retransmissions – autonomous versus polling Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

In this contribution we discuss both how to avoid unnecessary retransmission (2.1), and how to ensure timely RLC retransmission (2.2).

# Discussion

## Avoid unnecessary retransmission

The agreements reached at Orlando meeting concerns the RX side of the combined approach. In this document we will give our thoughts on some details on the TX side of the combined approach.

At TX side, retransmissions that occur after the PSDB shall be avoided because they will likely be discarded (e.g., even if successfully received by the network) and yet they occupy valuable transmission time that could otherwise be allocated to another PDU Set (e.g., transmission of the next PDU Set). Such unnecessary retransmissions can delay the subsequent transmission of data.

Observation 1: Any retransmission that occurs after the PSDB shall be avoided.

Proposal 1: Some elapsed PSDB indication shall be provided to RLC transmitter as trigger to stop retransmission of a RLC PDU

The RLC AM ARQ mechanism is based on perpetual retransmission of an RLC PDU, which may belong to a PDU Set. A radio link failure condition (RLF) occurs if the retransmission is stopped (e.g., if the number of retransmissions exceeds the maxRetxThreshold [2]). This can lead to the current session being reset, at which point the UE may reconnect to another gNB.

Proposal 2: Stopping the retransmission of a RLC PDU due to PSDB condition shall not trigger a RLF

Stopping the retransmission after PSDB shall be done in dependence on a number of retransmissions of the PDU exceeding a predetermined discard threshold value that is less than the threshold value associated with a radio link failure of the network.

Alternatively, the RLC transmitter can rely on upper layer discard timer to stop the retransmission of the PDU linked to a PDU Set.

Proposal 3: Study two alternatives to stop the retransmission of RLC PDUs linked to PDU Sets after PSDB and before RLF

•	Based on a second threshold

•	Based on discard timer

## Timely RLC retransmission

- how to  ensure timely RLC retransmissions for XR, e.g. details of the solution covering both autonomous retransmission and polling enhancements

Each retransmission solution presents unique advantages.

With two retransmission solutions allowed it is clear that UEs with different capabilities will coexist.

Observation 2: UEs can be sorted into 5 categories:

- UEs with no retransmission enhancement capacity

- UEs with polling enhancement capability

- UEs with autonomous retransmission capability

- UEs with both polling and autonomous capabilities, but configurable with only one capability active at a time

- UEs with both polling and autonomous capabilities able to be simultaneously configured with both capabilities active.

Proposal 4: RAN2 to specify UE configuration by the network according to the 5 UE categories.

For UEs of the fourth and fifth categories it could be useful to let the network control the activation and deactivation of each of the retransmission solutions.

Proposal 5: The network activates and deactivates each retransmission solution.

During the last meetings, the polling enhancement solution has not been detailed.

We think that polling can be triggered by the transmitter based on remaining time information. However, if the polling bit triggers a status report based on PDB, but the t-Reassembly timer is still running, there will be no NACK information sent back to the transmitter. Also, the prohibit timer might prevent the receiver to send back a status report.

Observation 3: There is a need for certain polling bits to be interpreted differently at the receiving side.

For example, a polling bit sent based on remaining time information is enhanced with dedicated signalling that allows the receiving entity to handle differently the reassembly and prohibit timers.

Proposal 6: Enhance the polling bit signalling to indicate a low remaining time condition.

We also propose some details about the autonomous retransmission triggering.

For example, the autonomous retransmission of PDUs having an unknown status may occur following the first transmission of all SDUs of a PDU Set.

Proposal 7: Trigger autonomous retransmission of an SDU with unknown status if all SDUs of the PDU Set are sent or have unknown status.

The autonomous retransmission of PDUs having an unknown status may occur when the transmit and retransmit buffers are empty.

Proposal 8: Trigger autonomous retransmission of an SDU with unknown status if there are no other SDUs to send or retransmit.

To decrease even more the needed resources, the autonomous retransmission trigger can be further extended to a remaining time threshold applied to the transmit window. If the transmit window is stalled up to the remaining time threshold of a PDU Set, then autonomous retransmission may be triggered.

Proposal 9: Trigger autonomous retransmission if the transmit window is stalled below a time threshold.

# Conclusion

In this contribution, we conclude the following observations and proposals:

Observation 1: Any retransmission that occurs after the PSDB shall be avoided.

Proposal 1: Some elapsed PSDB indication shall be provided to RLC transmitter as trigger to stop retransmission of a RLC PDU

Proposal 2: Stopping the retransmission of a RLC PDU due to PSDB condition shall not trigger a RLF

Proposal 3: Study two alternatives to stop the retransmission of RLC PDUs linked to PDU Sets after PSDB and before RLF

•	Based on a second threshold

•	Based on discard timer

Observation 2: UEs can be sorted into 5 categories:

- UEs with no retransmission enhancement capacity

- UEs with polling enhancement capability

- UEs with autonomous retransmission capability

- UEs with both polling and autonomous capabilities, but configurable with only one capability active at a time

- UEs with both polling and autonomous capabilities able to be simultaneously configured with both capabilities active.

Proposal 4: RAN2 to specify UE configuration by the network according to the 5 UE categories.

Proposal 5: The network activates and deactivates each retransmission solution.

Observation 3: There is a need for certain polling bits to be interpreted differently at the receiving side.

Proposal 6: Enhance the polling bit signalling to indicate a low remaining time condition.

Proposal 7: Trigger autonomous retransmission of an SDU with unknown status if all SDUs of the PDU Set are sent or have unknown status.

Proposal 8: Trigger autonomous retransmission of an SDU with unknown status if there are no other SDUs to send or retransmit.

Proposal 9: Trigger autonomous retransmission if the transmit window is stalled below a time threshold.

# References

- R2-2410915, Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei)

- TS 38.322, Radio Link Control (RLC) protocol specification