3GPP TSG-RAN WG2 Meeting #129	R2-2500295

Athens, Greece, 17-21 Feb 2025

Agenda Item:	8.1.4

Source:	NEC

Title:	Analysis of solutions for UE side model data collection

Document for:	Discussion, Decision

# Introduction

Extension discussions were taken during RAN#106 on the way forward for UE side model data collection. A way forward was endorsed in RP-243307.

RAN2 will focus on the discuss on the following aspects, according to the endorsed way forward:

o	Study RAN aspects related to data transfer over UP

o	Discuss level of NG-RAN involvement in the control and configuration of UE side data collection.

o	Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion).

o	Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain.

o	Discuss on the scalability aspects of CP

A tunnel can be established

This paper provides further analysis on the solutions identified for UE side model data collection

# Discussion

For CP based data collection, we see there is a scalability issue for the signalling based data transfer mechanism. For example, for one RRC message sent from the gNB to UE, it need to fits one single PDCP SDU( with the size up to 9000 bytes). Taking account of the possible RRC message segmentation, the maximum signalling transferred from gNB to the UE should be less than 144K in general, since the current maximum segments allowed is 16 over DL DCCH. In addition, a large amount of segments for a single RRC message may be not preferred due to its potential delay introduced for its transmission over the air. Data collection can also be based on continuous small RRC message but it may overwhelm the other RRC messages carried by SRB. Hence a pure CP based data collection for UE side model is not desirable.

Proposal-1: Data transfer over CP for UE side model data collection is not pursued.

According to the analysis of the different options identified for UE side model data collection, the UP-based data transfer can apply to all of the options (including option 1a/1b, option 2 and option 3). As supported by legacy approach, for the data transfer over UP, it is the AMF/SMF in 5GC that control the establishment of the PDU session and the corresponding tunnel between UE and UPF. On top of the current system architecture in 5G CN, the existing NWDAF (Network Data Analytics Function) entity in the current 5G CN domain may be used for Data Collection Function purpose for AIML model training for option-2. As such the UP-based data transfer may be the better option.

Proposal-2: Data transfer over UP for UE side model data collection is prioritized for Option-2.

Proposal-3: The existing NWDAF (Network Data Analytics Function) entity in the current 5G CN domain may be used for Data Collection Function purpose for AIML model training for Option-2.

In case of UP based data transfer, it is not clear, how the UE is configured to collect and report the data collected at the UE to the 5G CN via UP tunnel. One possibility is to allow the AMF to configure the UE via NAS layer signalling the data collection configuration and data report criteria. However, it should be noted that traditionally, the UE is configured by RRC signalling for any type of data report. In addition, the 5G CN may be not able to know which type of data the UE should collect and report. It is not clear if the NWDAF can be assumed to hold these type of information.

Proposal-4: To support Data transfer over UP for UE side model data collection, NAS layer may be employed to configure the data collection configuration and data report criteria to the UE for Option-2.

From the data collection procedure perspective, for Option-2, it would be essential if it is UE initiated procedure or network initiated procedure. If it is UE initiated procedure, it is like the traditional procedure for UE triggered uplink data transfer to the network. The signalling procedure should be governed by NAS layer between the UE and AMF/SMF. If it is network initiated procedure, a paging alike procedure may be used by 5GC to request the NG-RAN node to page the UE to trigger the report for its collected data. Then a special type of paging may be sent from NG-RAN node to the UE. The same principle may apply to Option-3 also.

Proposal-5: RAN2 to discuss if it is UE initiated procedure or network initiated procedure to trigger the data report from the UE to the network.

Proposal-6: NG-RAN may be involved in the paging procedure to page the UE to trigger the report for its collected data in case of network initiated procedure.

# Conclusion and Proposal

We have the following observations and proposals:

Proposal-1: Data transfer over CP for UE side model data collection is not pursued.

Proposal-2: Data transfer over UP for UE side model data collection is prioritized for Option-2.

Proposal-3: The existing NWDAF (Network Data Analytics Function) entity in the current 5G CN domain may be used for Data Collection Function purpose for AIML model training for Option-2.

Proposal-4: To support Data transfer over UP for UE side model data collection, NAS layer may be employed to configure the data collection configuration and data report criteria to the UE for Option-2.

Proposal-5: RAN2 to discuss if it is UE initiated procedure or network initiated procedure to trigger the data report from the UE to the network.

Proposal-6: NG-RAN may be involved in the paging procedure to page the UE to trigger the report for its collected data in case of network initiated procedure.

# Reference

- RP-243307

- RP-242850

# Annex-1: RP-243307 Way forward on AIML UE sided data collection

For SA

From RAN perspective, transfer of data over UP for Solution 2 in SA2 should be studied and provide feedback on fulfilment of visibility and controllability requirements.  Transfer of data over UP for Solution 3 can be studied by SA5

RAN has already concluded that transfer of data over CP for Solution 2 and 3 can meet the visibility and controllability RAN requirements, even though some scalability concerns have been raised for high amount of data transfer.  RAN is not expecting SA2 and SA5 to study the transfer of data over CP for now.

For RAN2

- During Rel-19 RAN2 continues studies on AIML UE sided data collection (excluding 1b)

o Study RAN aspects related to data transfer over UP

o Discuss level of NG-RAN involvement in the control and configuration of UE side data collection.

o Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion).

o Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain.

o Discuss on the scalability aspects of CP

o RAN2 can Liaise with other WGs if/when needed

General

RAN will make a final decision on way forward taking into account SA inputs when available.  RAN will have a checkpoint in Q2, 2025

# Annex-2: Analysis of different data collection options for UE-side model

| option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |  |  |  |