3GPP TSG-RAN WG2 Meeting #129	R2-2500298

Athens, Greece, 17-21 Feb 2025

Agenda Item:	8.11.2

Source:	NEC

Title:	Random Access for SBFD Operation

Document for:	Discussion, Decision

# Introduction

In RAN2#128 meeting, the following agreements were reached for the random access procedure for SBFD operation:

Agreements on RACH with SBFD

CFRA =>The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed). CBRA =>FFS on the following options Option 1: Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2: UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one. RACH configuration =>Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

This paper intends to further discuss the RAN2 impact of the random access procedures for SBFD operation.

# Discussion

## CFRA

In the last RAN2 meeting, it was agreed that RO type is indicated by the network for CFRA based random access for SBFD. However it is FFS for the detailed signaling and the SI request case. In our understanding one bit based indication may be used to indicate the RO type (i.e., legacy RO or additional RO) to be used for CFRA within the corresponding RACH configuration for the additional ROs by SIB1.

Proposal-1: Within the corresponding RACH configuration for the additional ROs, one bit is used to indicate the RO type for CFRA.

## Initial RACH Occasion selection for CBRA

When a random access procedure is initiated, the UE performs the following operations in sequence: select the carrier for performing RA procedure, perform the BWP operation, select the set of RA resources applicable to the current random access procedure, determine the RA type, perform the initialization of variables specific to RA type and perform the corresponding RA resource selection (including preamble, SSB and RO, etc.) procedure. Considering the additional RO for SBFD is introduced, the RO type selection between the legacy RO and additional RO should be performed during the whole RA procedure. We think the order of RO type selection should be performed after the RA type selection, because it may reduce the access delay on top of the selected RA type. And after the RO type selection, the UE performs the initialization of the variables specific to RA type and RO type.

Proposal-2: The RO type selection is performed after the RA type selection.

In the last RAN2 meeting, the following options was identified for the initial RACH Occasion selection for CBRA case.

Option 1: Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO.

If there is no such indication from the NW, FFS on the following mechanism

- UE select legacy RO or SBFD RO based on SSB RSRP, or

- UE select the legacy RO, or

- UE select the SBFD RO, or

- Other metrics than SSB RSRP.

Option 2: UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one.

In our option, the discussion here are mainly two issues and we should treat them one by one.

One issue is whether the network can provide a RO indication to direct the UE on the prioritization of the legacy ROs and additional ROs, if yes, the UE can always select the type of ROs based on this network indication. We think it is beneficial to have such RO indication for CBRA case, since the network should be allowed to balance the load in different ROs. This will improve the flexibility for the network.

Proposal-3: The network can indicate the RO types (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA.

The second issue is whether abovementioned network indication is always present. If it is always present, no other solutions are needed. However, if it is optional present, additional solution should be considered for the case that there is no network indication. We assume the abovementioned network indication is optional present. Then the UE needs to know a rule to choose between legacy ROs and additional ROs for its CBRA based access.

Proposal-4: The network indication of RO type for the case of CBRA is optional present.

During the random access procedure, the UE may sweep the beams and measure their SSBs/CSI RSs in order to determine the best-associated beam with the best RSRP performance. In the meantime, in order to avoid random access delay, it may be desirable for the SBFD-aware UEs to select the first available RO among all of the RACH resources valid to the UE (including both legacy RACH resources and additional RACH resources) for its initial access. In this manner, the UE is expected to select the best suitable RO set or RO type (legacy RO or additional RO) to initiate random access.

Proposal-5: UE selects the first available RO type(legacy RO or additional RO) among all of the RACH resources valid to the UE for its CBRA based access, when the indication of RO type from the network is absent.

In order to support random access in SBFD symbols by UEs in RRC_IDLE/INACTIVE mode, RACH resource selection for SI request and RA-SDT should also be discussed. The same discussion for proposal 2 can be applied for RACH resource selection for SI request and RA-SDT. Whether the network can configure separated criteria for SI request and RA-SDT is FFS.

Proposal-6: For SI request and RA-SDT, the UE always selects the first available RO type(legacy RO or additional RO) among all of the RACH resources valid to the UE in case of CBRA, when the indication of RO type from the network is absent.

## RACH occasions switch

In the previous RAN2 meetings, it was agreed that for the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. Meanwhile there is an FFS about the case when UE selects legacy ROs first.

In our opinion, if the UE selects legacy ROs first, the UE should be allowed to also switch to additional SBFD RACH occasions.

Proposal-7: the UE should be allowed to switch to additional SBFD RACH occasions after certain (configured) number of times of RACH attempt in legacy RACH occasions, if the UE selects legacy ROs first.

In addition, how to determine the preamble received target power for preamble re-attempt after the RO type switch should be discussed. One possible way is to reset the PREAMBLE_POWER_RAMPING_COUNTER, i.e., no power ramping is needed during the RO type switch. Another way is to compensate the power difference between the SBFD RO and legacy RO type. This can be achieved by introducing the new power offset by considering the power ramping count and power ramping step, e.g., new offset = (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREAMBLE_POWER_RAMPING_STEP_SBFD – PREAMBLE_POWER_RAMPING_STEP).

Proposal-8: RAN2 to select one of the options to determine the received target power for preamble transmission when the RO type is switched:

- Option 1: reset the PREAMBLE_POWER_RAMPING_COUNTER;

- Option 2: continue the power ramping during this RA procedure, and compensate the power difference between the SBFD RO and legacy RO type, e.g., a new power offset is determined by (PREAMBLE_POWER_RAMPING_COUNTER–1) × (PREAMBLE_POWER_RAMPING_STEP_SBFD – PREAMBLE_POWER_RAMPING_STEP).

Another issue is whether the backoff mechanism can be used when the RO type switch is occurred. In current random access procedure, we observed that if the RA/preamble re-attempt is performed for the same RA type, the UE may take the backoff time into consideration for RA resource selection; if the RA re-attempt is performed for the different RA type (i.e., 2-step RA switch to 4-step RA), the UE performs the random access resource selection without consideration of the backoff time. For RO type switch scenario, it is necessary to discuss whether a backoff mechanism should be used.

Proposal-9: RAN2 to discuss whether the backoff mechanism should be used when the RO type switch occurs during one RACH procedure.

## RSRP threshold for SSB/CSI-RS selection

There is one more issue for the RACH resource decision. In general, the RSRP threshold for SSB/CSI-RS selection can be the same for both legacy RACH resources and additional RACH resources. However, in practice, the network may configure the UE with a different threshold for SSB/CSI-RS signal strength (e.g. RSRP) or threshold for the path loss that UE experiences for SSB/CSI-RS for RACH resources occurring during SBFD symbols.

Proposal-10: RAN2 to discuss if RSRP threshold for SSB/CSI-RS selection can be the same or different for legacy RACH resources and additional RACH resources

## Activation and deactivation of Additional ROs

In addition, when there is additional RACH configuration for SBFD-aware UEs, it would be beneficial to allow the deactivation and activation of all or a subset of such ROs. Usually less number of ROs configured in the system will improve the system throughput, since the additional resources can be used schedule the data transmission for the connected UEs. RAN2 can discuss the method on how to deactivate and activate additional RACH Occasions for SBFD-aware UEs after it is configured. RRC message or MAC CE based approach may be discussed in this context. Such message may also indicate the time duration for which the deactivation is applicable for. For example, this can be in form of DRX cycle configured by gNB where within the DRX-on duration (when gNB is in active state), gNB enables all ROs and for DRX-off duration (when gNB is in low power state), gNB can configure a set of ROs to be deactivated.

Proposal-11: RAN2 to discuss the method on how to deactivate and activate additional RACH Occasions for SBFD-aware UEs after it is configured.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: Within the corresponding RACH configuration for the additional ROs, one bit is used to indicate the RO type for CFRA.

Proposal-2: The RO type selection is performed after the RA type selection.

Proposal-3: The network can indicate the RO types (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA.

Proposal-4: The network indication of RO type for the case of CBRA is optional present.

Proposal-5: UE selects the first available RO type(legacy RO or additional RO) among all of the RACH resources valid to the UE for its CBRA based access, when the indication of RO type from the network is absent.

Proposal-6: For SI request and RA-SDT, the UE always selects the first available RO type(legacy RO or additional RO) among all of the RACH resources valid to the UE in case of CBRA, when the indication of RO type from the network is absent.

Proposal-7: the UE should be allowed to switch to additional SBFD RACH occasions after certain (configured) number of times of RACH attempt in legacy RACH occasions, if the UE selects legacy ROs first.

Proposal-8: RAN2 to select one of the options to determine the received target power for preamble transmission when the RO type is switched:

- Option 1: reset the PREAMBLE_POWER_RAMPING_COUNTER;

- Option 2: continue the power ramping during this RA procedure, and compensate the power difference between the SBFD RO and legacy RO type, e.g., a new power offset is determined by (PREAMBLE_POWER_RAMPING_COUNTER–1) × (PREAMBLE_POWER_RAMPING_STEP_SBFD – PREAMBLE_POWER_RAMPING_STEP).

Proposal-9: RAN2 to discuss whether the backoff mechanism should be used when the RO type switch occurs during one RACH procedure.

Proposal-10: RAN2 to discuss if RSRP threshold for SSB/CSI-RS selection can be the same or different for legacy RACH resources and additional RACH resources

Proposal-11: RAN2 to discuss the method on how to deactivate and activate additional RACH Occasions for SBFD-aware UEs after it is configured.

# Reference

- RAN1#118bis meeting minutes

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)