3GPP TSG-RAN WG2 Meeting #129	R2-2500307

Athens, Greece, Feb. 17th- 21st, 2025

Agenda item:	8.13.2

Source:	Samsung

Title:	Considerations on relay discovery and (re)selection

WID/SID:	NR_SL_relay_enh2

Document for:	Discussion and Decision

Introduction

In last RAN2 meeting, the relay discovery and (re)selection was extensively discussed for multi-hop SL relay, and reached the following agreement:

Agreements:

A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE.

If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible).  FFS if operation as an intermediate relay UE towards a different cell would be supported.

The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.  If the upper bound is not configured, there is no threshold, but this does not override the previous agreement.  FFS if there would be additional restrictions in the case of no upper bound.

Agreements:

The following cases are supported for L2 multihop relay:

- One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE).

- Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE.

FFS if the last relay UE can use the same L2ID for both of the connections in either case.

Cases with two indirect paths to the gNB for the same remote UE are not supported.

An Intermediate Relay UE can serve multiple multi-hop indirect paths of different Remote UEs.

If the intermediate Relay UE also is acting as a Remote UE, it cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links.

In this contribution, we will share our further thinking on the discovery and relay selection/re-selection.

Discussion

Issue 1: UE type

In multi-hop SL relay, four types of UE are defined, i.e., remote UE, first relay UE, intermediate relay UE, and last relay UE. The discovery among those UEs can be performed. However, it is not clear whether each UE can be acted any type of UEs or only one type of UEs. In our understanding, depending on UE capability, an UE can be acted as remote UE only, first relay UE only, or intermediate relay UE only, or last relay UE only, or any type combination among those three types.

Proposal 1: RAN2 is kindly asked to discuss whether the UE can act as one type or multiple types among four types, i.e., remote UE, first relay UE, intermediate relay UE, and last relay UE.

If an UE can be acted as one or some types of UE, the connection establishment between two UEs depends on the UE type. For example, the remote UE cannot allow the access of other UEs, the first relay UE can only allow the access of remote UE. Thus, during the discovery procedure, each UE can indicate its capability to neighbouring UEs. Two options can be considered to indicate the UE capability:

- Option 1: use the UE type indication

Such indication can indicate the supported capability of each UE, e.g., remote UE, first relay UE, intermediate relay UE, last relay UE

- Option 2: use the supported number of descendant hops

In this option, the remote UE can indicate the supported number of descendant hop as zero, while the first relay UE can indicate the supported number of descendant hop as one. For the intermediate/last relay UE, the supported number of descendant hops can be determined based on the processing capability or the load status.

Proposal 2: if UE is differentiated via different types, RAN2 is kindly asked to discuss the following options to indicate the UE type during discovery procedure:

- Option 1: use the UE type indication

- Option 2: use the supported number of descendant hops

In last meeting, the following FFS is left for the intermediate relay UE:

If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible).  FFS if operation as an intermediate relay UE towards a different cell would be supported.

Once an UE is camped/connected one cell, it has to operate based on the system information from this cell. Moreover, the CA operation is not supported for intermediate relay UE/first relay UE/remote UE. Thus, the last relay UE cannot be operated as an intermediate relay UE towards a different cell.

Proposal 3: RAN2 is kindly asked to agree that the last relay UE over one cell cannot be operated as an intermediate relay UE towards a different cell.

Issue 2: hop number exposure

R19 supports multiple hops between remote UE and the gNB. However, the total number of hops are limited. Thus, the remote UE/relay UE should not connect with its parent node when the total number of hops exceeds the supported number of hops. To avoid this, the discovery message can contain hop count related information.

Proposal 4: RAN2 is kindly asked to discuss whether the hop count information is included in the discovery message.

Issue 3： Multi-hop access control

With the increase of number of hops, the E2E QoS guarantee may become more difficult. In other words, the number of hops in the network cannot be increased forever. Thus, how to limit the number of hops become an issue. In our understanding, this should be under the control of gNB.

Proposal 5: RAN2 is kindly asked to discuss whether there is need to limit the number of hops.

Issue 4: L2ID of last relay UE

As agreed in the last meeting, one last relay UE can have different indirect paths to two different remote UEs, or one remote UE and one intermediate relay UE. One FFS is:

The following cases are supported for L2 multihop relay:

- One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE).

- Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE.

FFS if the last relay UE can use the same L2ID for both of the connections in either case.

Since it is the same last relay UE, the same L2ID can be applied regardless of the number of indirect paths towards remote UE(s)/intermediate relay UEs.

Proposal 6: RAN2 is kindly asked to agree that the same L2ID can be applied to the last relay UE regardless of the number indirect paths under such last relay UE.

Issue 5: Protocol stack for discovery between intermediate relay UEs

In current stage 2, the protocol stack for discovery is between the remote UE and relay UE. To extend such protocol stack, the protocol stack should be between the remote UE and the first relay UE, between two intermediate relay UEs, and between intermediate relay UE and the last relay UE.

Proposal 7: RAN2 is kindly asked to agree the protocol stack of discovery should be drawn between  the remote UE and the first relay UE, between two intermediate relay UEs, and between intermediate relay UE and the last relay UE.

Conclusion

In this contribution, we address some issues for the relay discovery and (re)selection, and have the following proposals:

Proposal 1: RAN2 is kindly asked to discuss whether the UE can act as one type or multiple types among four types, i.e., remote UE, first relay UE, intermediate relay UE, and last relay UE.

Proposal 2: if UE is differentiated via different types, RAN2 is kindly asked to discuss the following options to indicate the UE type during discovery procedure:

- Option 1: use the UE type indication

- Option 2: use the supported number of descendant hops

Proposal 3: RAN2 is kindly asked to agree that the last relay UE over one cell cannot be operated as an intermediate relay UE towards a different cell.

Proposal 4: RAN2 is kindly asked to discuss whether the hop count information is included in the discovery message.

Proposal 5: RAN2 is kindly asked to discuss whether there is need to limit the number of hops.

Proposal 6: RAN2 is kindly asked to agree that the same L2ID can be applied to the last relay UE regardless of the number indirect paths under such last relay UE.

Proposal 7: RAN2 is kindly asked to agree the protocol stack of discovery should be drawn between  the remote UE and the first relay UE, between two intermediate relay UEs, and between intermediate relay UE and the last relay UE.

References