3GPP TSG-RAN WG2 Meeting #129	R2-2500309

Athens, Greece, 17th – 21st Feb, 2025

Agenda Item:	8.9.2

Source: 	OPPO

Title:  	Discussion on Store & Forward satellite operation

Document for:	Discussion and Decision

# Introduction

In Rel-19 IoT NTN WID [1], Store & Forward satellite operation is one of the features to be supported.

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

This contribution would like to provide our considerations on this objective of Rel-19 IoT NTN.

# Discussion on Store & Forward satellite operation

Below are the agreements made in the previous RAN2 meetings.

RAN2#127bis Agreements:

1.	The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

2.	RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

3.	An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

RAN2#128 Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

3.	“S&F operation” indication is provided in SIB1

4.	RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

5.	RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

6.	RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

7.	RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

In the last meeting, on top of the “S&F operation” indication, it was further agreed to introduce the time information to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. Here we further discuss the leftover issues highlighted above.

For the time information, it is still FFS which SIB is used, i.e., SIB1 or SIB31. In our understanding, if no significant benefit is observed, the time information should be provided in SIB31 to avoid occupying SIB1 capacity.

- Time information on transition from S&F operation mode to normal mode is provided in SIB31.

Besides, it is also FFS if absolute or relative time is used for the time information. We understand that an absolute time is better for both UE and NW sides. Since the time information in manner of absolute time will not change until the time of transition, NW does not have to change the broadcasted time information frequently, and RRC_IDLE UE can rely on system information change notifications to follow the update. Note that we can mimic the way already used for t-Service in Rel-17 IoT-NTN, i.e., indicate a time in multiples of 10 ms in a UTC time manner.

- Use absolute UTC time to broadcast the time information, and changes of it should result in system information change notifications.

Another FFS issue is the time information for the opposite transition, i.e., whether to indicate when the “S&F operation” will start. In our understanding, using the same field of time information to indicate both directions of transition could be supported without additional effort. UE can know the transition direction based on the “S&F operation” indication broadcasted in SIB1. If time information and “S&F operation” indication are both present, UE can realize the time information is for the transition from “S&F operation” mode to real-time/normal mode. And if time information is present but “S&F operation” indication is not present, UE can realize the time information is for the transition from real-time/normal mode to “S&F operation” mode. Thus, it is no harm to further support indicating the time information for the opposite transition. Besides, the time information for the opposite transition is beneficial for Rel-19 UEs not supporting S&F operation, which can allow these UEs to start neighbour cell measurement for reselection before transiting to “S&F operation” mode.

- The time information can also be broadcasted to indicate when the current satellite operation mode will transit from normal mode to S&F operation mode.

For “S&F operation” indication broadcasted in SIB1, there are three states, i.e., “1”, “0”, and “not present”. One issue is whether the change of “S&F operation” indication leads to system information change notifications or not. There are three cases for the changes.

- Case-1) In case that the current satellite operation mode transits from “S&F operation” mode to real-time/normal mode, “S&F operation” indication in SIB1 changes to be not present.

- Case-2) In case that the current satellite operation mode transits from real-time/normal mode to “S&F operation” mode, “S&F operation” indication in SIB1 changes to be present.

- Case-3) “S&F operation” indication in SIB1 changes from 0 to 1, or vice versa.

For Case-1, thanks to the time information agreed in the last meeting, RRC_IDLE UE does not have to rely on system information change notifications to determine the changes. UE can be aware of the changes according to time information.

For Case-2, since it is still FFS on whether the time information on transition from real-time/normal mode to “S&F operation” mode is introduced or not, it is questionable whether UE can know the changes if not relying on system information change notifications.

For Case-3, system information change notifications are essential, since no other information is provided to UE.

Considering that system information change notifications are required at least for Case-3, but it is weird to only apply system information change notifications to Case-3, we think for all cases above it is necessary to still rely on system information change notifications to estimate the change of “S&F operation” indication as legacy, i.e., changes of “S&F operation” indication should result in system information change notifications.

- Changes of S&F operation indication should result in system information change notifications.

Furthermore, it is agreed that the “S&F operation” indication setting as “0” means the cell is operating in “S&F operation” mode for UE in RRC_CONNECTED mode, while NB-IoT/eMTC UEs in RRC_CONNECTED mode are not required to detect system information changes. Thus, it is questionable how UE in RRC_CONNECTED mode behaves in the following scenarios:

- Scenario 1: the cell is switching from “S&F operation” mode to normal mode

- Scenario 2: the cell is switching from normal mode to “S&F operation” mode

Due to the difference in latency between “S&F operation” mode and normal mode, for both Scenario 1 and 2, Rel-19 UEs supporting S&F operation may be updated with some NAS parameters, such as QoS parameters, in NAS layer, and then some AS configurations may need to be updated accordingly. Especially for UEs in Scenario 2, i.e., the cell that is switching from normal mode to “S&F operation” mode, the latency requirement of ongoing services may not be fulfilled due to lake of the feeder link connection.

Besides, for a cell in normal mode, except for Rel-19 UEs supporting S&F operation, it is possible to also allow legacy UEs and Rel-19 UEs not supporting S&F operation to access. Therefore, if this cell is switching from normal node to “S&F operation” mode, these legacy UEs should be considered during NW handling.

In general, from RAN2 perspective, there are three ways of NW handling in AS layer for the two scenarios above:

- Release UE to RRC_IDLE mode,

- Trigger handover to switch the UE to a correct cell,

- Update the radio bearer configuration to adapt the operation mode of the current cell, for Rel-19 UEs supporting S&F operation.

We understand all these three ways are fully up to NW implementation, and no special UE behaviour is needed in AS layer from RAN2 perspective. But considering for the two scenarios above maybe some NAS procedures are necessary which is up to SA2, it is not sure whether there is any AS impact caused by NAS layer. It would be safer to further check it with SA2.

- For UE in RRC_CONNECTED mode, from RAN2 perspective, rely on NW implementation for UE handling if NW switches between S&F operation mode and normal mode, and no special AS impact is foreseen. FFS whether there is any AS impact caused by NAS layer.

Another leftover issue is whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC_IDLE mode because the feeder link becomes unavailable. We understand that no need to introduce the new release cause. If UE is released to RRC_IDLE mode, based on “S&F operation” indication and the time information for two-direction transition provided in system information, UE can anyway know the current situation of feeder link.

- No need to introduce a new release cause for feeder link becoming unavailable.

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- Time information on transition from S&F operation mode to normal mode is provided in SIB31.

- Use absolute UTC time to broadcast the time information, and changes of it should result in system information change notifications.

- The time information can also be broadcasted to indicate when the current satellite operation mode will transit from normal mode to S&F operation mode.

- Changes of S&F operation indication should result in system information change notifications.

- For UE in RRC_CONNECTED mode, from RAN2 perspective, rely on NW implementation for UE handling if NW switches between S&F operation mode and normal mode, and no special AS impact is foreseen. FFS whether there is any AS impact caused by NAS layer.

- No need to introduce a new release cause for feeder link becoming unavailable.

# 4. Reference

[1] RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek