3GPP TSG-RAN WG2 Meeting #129	R2-2500310

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.9.3

Source: 	OPPO

Title:  	Discussion on CB-msg3 EDT and msg4 enhancement

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to provide our considerations on open issues for CB-msg3 EDT and msg4 enhancement.

Discussion

Configuration

In last RAN2 meeting, how to provide CB-msg3 EDT related configurations was discussed and the following agreement was made.

- Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B)

In our understanding, whether the TA validation parameters are needed relies on RAN1/RAN4 discussion, i.e., in their previous reply LS, it is noted that the pre-compensated TA is sufficient for CB-msg3 transmission in case of NB-IoT with 3.75kHz SCS and eMTC CE mode A, further discussion is needed for NB-IoT with 15kHz SCS and eMTC CE mode B.

1. Overall Description: RAN1 would like to thank RAN2 for the LS in R1-2405786/R2-2405769 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2. Regarding RAN2’s questions, following is the RAN1’s response: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? RAN1’s response: From RAN1 perspective, assuming Note 1 and Note 2 are satisfied, an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy requirement for Msg3 transmission/reception without Msg1/Msg2 at least in the following cases: - NB-IoT with 3.75kHz SCS. - eMTC CE mode A. Note 1: RAN1 assumes NTA=0 for Msg3 transmission without Msg1/Msg2 Note 2: RAN1 assumes that the RAN4 UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN, respectively. Note 3: RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B

Since RAN2 may not be able to evaluate the case, it is proposed to wait for RAN1/RAN4 progress, after which RAN2 decides whether dedicated signalling is needed for provision of TA validation parameters.

Wait for RAN1/RAN4 progress on the need of TA validation parameters for the case of 15kHz NB-IoT and eMTC CE mode B.

According to the following agreements, the CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level and RSRP threshold-based CE level selection procedure is reused. Furthermore, it was also agreed that there will be another RSRP threshold used to determine whether the CB-msg3 EDT can be selected.

- Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels

- There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC)

For the RSRP thresholds, it is unclear whether the same RSRP thresholds as those used EDT can be applied and whether the RSRP threshold for CB-msg3 EDT selection needs to be configured per CE level. We think the two types of RSRP threshold, though serving different purposes, are highly related and there would be two alternatives to configure these RSRP thresholds.

- Alt1: Reuse the same RSRP thresholds as EDT for CE level selection and configure per CE-level RSRP thresholds for EDT/CB-msg3 EDT selection.

- Alt2: Configure CB-msg3 EDT specific RSRP thresholds for CE level selection and one RSRP threshold for EDT/CB-msg3 EDT selection.

The UE selection orders differ between the two alternatives. In Alt1, the UE first selects the CE level and then determines whether to trigger CB-msg3 EDT. Conversely, in Alt2, the UE first determines whether CB-msg3 EDT can be used and then further selects among the CE levels associated with CB-msg3 EDT. Both alternatives can ensure that the radio link quality is adequate for CB-msg3 EDT. Given the difficulty for RAN2 to determine the necessity of specific CE level selection thresholds for CB-msg3 EDT, Alt1 is preferred to make the procedure simpler.

Reuse the same RSRP thresholds as EDT for CE level selection. For each CE level, an RSRP threshold to determine whether to use CB-msg3 EDT is configured.

Another FFS regards to whether there is a need to configure a separate set of RSRP thresholds in case CB-msg3 EDT is combined with OCC. According to the LS from RAN1, it is mentioned that one of conditions to apply OCC solution is when the power imbalance is small between UEs selecting the PUSCH with OCC. In our understanding, RSRP threshold might be helpful to avoid large power imbalance, e.g., CB-msg3 resources combined with OCC can be only selected by the UEs within the same RSRP range, correspondingly, the CB-msg3 resources with OCC need to be configured per RSRP range. While whether/how to configure the RSRP threshold should be up to RAN1 evaluation, thus it is suggested to send an LS to RAN1 for further clarification.

# 1. Overall Description: RAN1 thanks RAN2 for their question on OCC for CB-Msg3 NPUSCH and would like to provide a response to the question. Question: RAN2 is working on CB-msg3 EDT in IDLE state and would like to ask RAN1 whether the OCC solution for NPUSCH on which RAN1 is working can be applied to CB-msg3 NPUSCH or not.. Response: The OCC solution that RAN1 is working on may be applied to CB-Msg3 NPUSCH format 1 single tone at least for OCC length 2, for the subcarrier spacing(s) for which CB-Msg3 is supported, when the power imbalance is small. The design details for the OCC solution are ongoing.

RAN2 asks RAN1 on the need of RSRP threshold to determine whether to select OCC resources.

For EDT, the UE needs to check the TBS-threshold associated with the selected level when determining whether to trigger the procedure. For CB-msg3 EDT, it was also agreed that the TBS needs to be considered as one of the trigger conditions.

- The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

Regarding whether the maximum TBS is the same or different for different CE level, it is preferred to follow the same way as EDT for flexibility.

TBS-threshold is configured per CE level, whether to configure the same value is up to the NW.

For NB-IoT, the EDT resources can be configured on anchor carrier and non-anchor carriers for each CE level as specified in TS36.321. Also for load balance purpose, the same way of configuration can be followed for CB-msg3 EDT.

- for EDT, each enhanced coverage level has zero or one anchor carrier PRACH resource present in nprach-ParametersList-EDT and zero or one PRACH resource for each non-anchor carrier signalled in ul-ConfigList.

For NB-IoT, the CB-msg3 EDT resources can be configured on anchor and/or non-anchor carriers for a CE level.

For NB-IoT UEs, the SCS of the UL grant in RAR can be configured as 3.75kHz or 15KHz and the NW selects the numerology to be scheduled based on the radio link quality. To accommodate UEs operating under different radio link conditions, it is necessary to configure CB-msg3 EDT resource with multiple numerologies.

For NB-IoT, the CB-msg3 EDT resources associated with a CE level can be configured with different numerologies (e.g. 3.75kHz SCS and 15kHz SCS).

DL response reception

In last RAN2 meeting, the following agreements were made for DL response reception after CB-msg3 transmission. In this section, we would like to provide our considerations on remaining issue regarding DL response reception for CB-msg3 EDT.

7.	For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

8.	For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

9.	The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

10.	Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

11.	Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

12.	RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

For PUR, 4 subframes processing delay was considered when starting the DL response reception window, i.e., pur-ResponseWindowTimer.

After transmission using PUR, the MAC entity shall monitor PDCCH identified by PUR-RNTI in the PUR response window using timer pur-ResponseWindowTimer: - if PUR was transmitted in a non-terrestrial network and UE supports delaying the start of the pur-ResponseWindowTimer: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes plus UE-eNB RTT. - else: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes.

For saving UE power consumption, processing delay should also be considered when starting the PDCCH monitoring window in CB-msg3 EDT. Different from PUR, the eNB may face greater challenges in processing the CB-msg3 because of contention, thus it is questionable whether 4 subframes is enough and we think the proper value should be decided by RAN1.

The processing time is considered for the start of PDCCH monitoring window. Up to RAN1 to determine the value.

As agreed, the Contention Resolution Identity should be included in CB-msg4 to declare the contention resolution. If msg4 enhancement is not considered, CB-msg3 EDT is also terminated by receiving DL RRC message as in EDT. Therefore, if there is no RRC message together with the CB-msg4, the UE needs to keep monitoring the RRC message after contention resolution. Since there is no RAR with TC-RNTI in CB-msg3 EDT, the RNTI used for monitoring the RRC message needs to be provided in CB-msg4.

C-RNTI needs to be included in DL response if RRC message is not delivered together with CB-msg4.

Another issue is whether to support CB-msg3 retransmission scheduled by the NW. In EDT, msg3 re-transmission scheduled by TC-RNTI is supported. NW-scheduled re-transmission aids in faster contention resolution. Unlike EDT, the RNTI for DL reception in CB-msg3 EDT is associated with the selected PUSCH resources, i.e., the same RNTI can be calculated by the UEs selecting different PUSCH occasions. If NW-scheduled CB-msg3 retransmission is to be supported, further discussion is required on how to schedule the UL grant for re-transmission. Before diving the details, it is proposed that RAN2 first reach a conclusion on whether to support NW-scheduled CB-msg3 retransmission.

RAN2 to discuss whether to support NW-scheduled CB-msg3 re-transmission.

For DSA, it is questionable whether we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. It has been agreed that the RNTI used for DL monitoring is derived based on the resource associated to CB-msg3 PUSCH occasion. For DSA, different RNTIs can be calculated for each replica. If only one window is used, the UE would need to keep monitoring the DL responses addressed to all these RNTI during the time window, leading to increased power consumption. Therefore, it is preferred to have multiple PDCCH monitoring windows. When the contention is considered as resolved, the UE stops all the windows.

For DSA, separate PDCCH monitoring window is maintained for each msg3 replica

When the contention is resolved, the UE stops all the PDCCH monitoring windows.

Efficient delivery of msg4

As per WID, the other aspect on EDT enhancement is to support efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete. In legacy EDT, after UE transmit UL data together with RRCEarlyDataRequest or RRCConnectionResumeRequest message in Msg3, the UE will not consider the EDT as successfully completed until it receives RRCEarlyDataComplete or RRCConnectionRelease message, i.e. only RRC message can be used to indicate the successful reception of the EDT transmission by the eNB.

For PUR, DL response optimization is specified for CP solution, i.e., eNB may terminate the PUR procedure by sending a L1 ACK optionally containing a Time Advance Command, a MAC Time advance Command or RRC response message with no data. By introducing L1/L2 ACK, the performance of resource efficiency, UE power consumption as well as MO latency can be further improved compared to EDT.

For CP-solution EDT, if there is no follow-up DL data, L1/L2 signalling can be considered for improvement of msg4 efficiency. The following solutions can be considered for further discussion:

- Solution1: If the NW is able to determine there is no follow-up DL data when responding msg4, the EDT termination indication can be delivered to the UE together with the contention resolution ID MAC CE.

- Solution2: If there is no follow-up DL data after contention resolution, L1 signalling scheduling by UE-dedicated RNTI  can be used to indicate EDT termination.

In case there is no follow-up DL data for CP-solution EDT/CB-msg3 EDT, the following solutions can be considered for msg4 enhancement:

  - L1 ACK scheduled by UE-dedicated RNTI.

  - A new MAC CE including contention resolution ID and EDT termination indication.

Conclusion

Based on the discussion in section2, we have following observation proposals:

Proposal 1	Wait for RAN1/RAN4 progress on the need of TA validation parameters for the case of 15kHz NB-IoT and eMTC CE mode B.

Proposal 2	Reuse the same RSRP thresholds as EDT for CE level selection. For each CE level, an RSRP threshold to determine whether to use CB-msg3 EDT is configured.

Proposal 3	RAN2 asks RAN1 on the need of RSRP threshold to determine whether to select OCC resources.

Proposal 4	TBS-threshold is configured per CE level, whether to configure the same value is up to the NW.

Proposal 5	For NB-IoT, the CB-msg3 EDT resources can be configured on anchor and/or non-anchor carriers for a CE level.

Proposal 6	For NB-IoT, the CB-msg3 EDT resources associated with a CE level can be configured with different numerologies (e.g. 3.75kHz SCS and 15kHz SCS).

Proposal 7	The processing time is considered for the start of PDCCH monitoring window. Up to RAN1 to determine the value.

Proposal 8	C-RNTI needs to be included in DL response if RRC message is not delivered together with CB-msg4.

Proposal 9	RAN2 to discuss whether to support NW-scheduled CB-msg3 re-transmission.

Proposal 10	For DSA, separate PDCCH monitoring window is maintained for each msg3 replica

Proposal 11	When the contention is resolved, the UE stops all the PDCCH monitoring windows.

Proposal 12	In case there is no follow-up DL data for CP-solution EDT/CB-msg3 EDT, the following solutions can be considered for msg4 enhancement:

a.	L1 ACK scheduled by UE-dedicated RNTI.

b.	A new MAC CE including contention resolution ID and EDT termination indication.