3GPP TSG-RAN WG2 Meeting #129	R2-2500313

Athens, Greece, Feb. 17th – 21st, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussions on simulation results of the RRM measurement prediction

Agenda Item:	8.3.2.1

Document for: 	Discussion and Decision

- Introduction

At the RAN#102 meeting, the SID on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The SID describes the study scope of the AI/ML-aided mobility enhancements, as below,

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

At RAN2 #126 meetings, six cases and their priorities were agreed, and the detailed simulation assumptions were listed [4].

The following is agreed

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this paper, we submit our evaluation results about the 3 high-priority cases and discuss some remaining issues on the simulations.

- Remaining issues for simulation result processing

For the AI/ML model performance of RRM measurement predictions, RAN2 has collected the results from sources for the second round. One remaining issue regarding the collected results is processing and drawing conclusions from them. The following processing can be considered when referring to the methods used by Rel-18 and Rel-19 studies on AI/ML for air interface.

In TR38.834 and following Rel-19 studies on AI/ML for other use cases, the following observations are drawn based on the evaluation results collected.

- Detailed observations that capture all the results collected for each case and subcase, which can be itemized to capture all the contributions.

- Summarized observations that capture an overview of the results for the conclusion, where a summarized range of the performance gain is included. The summarized range can be calculated based on the mean, median, or quartiles of the results from sources.

Following the same principle, we suggest the same methodology to process the collected results as the following proposal,

Proposal 1

- Draw observations from the collected results following the same principle of TR38.843, which includes

  - Draw detailed observations for each case, subcase, and important configuration to capture all submitted results from different sources.

  - For each case and important configuration, draw a summarized observation for the conclusion, which can be based on the following options,

    - Opt. 1: Average [and standard deviation] of the performance gain.

    - Opt. 2: Median [and/or quartiles] of the performance gain.

- Evaluation results

Following the email discussions about the results collection templates, we resubmitted the detailed simulation assumptions and results in the following files with the updated templates,

- RRM_Scen2_128_DCM_2410020_V2.xlsx for Case 2 (FR1-to-FR1 intra-frequency temporal domain Case B).

- RRM_Scen3_128_DCM_2410020_V2.xlsx for Case 2 (FR1-to-FR1 inter-frequency).

- RRM_Scen4_128_DCM_2410020_V2.xlsx for Case 2 (FR2-to-FR2 intra-frequency temporal domain Case A).

The corresponding simulation assumptions are listed in the Appendix. The following sections show some observations from the simulation results.

  - Case 2, FR1-to-FR1 intra-frequency temporal domain case B

We evaluated this case with MRRT = 1/2 and 3/4, respectively. The sample-and-hold (SaH) approach, which uses the latest measured results as predictions, is also evaluated as the comparison baseline. The results are also plotted in Figure 1 to show the increasing tendency of the RSRP gap with the prediction distance.

Figure 1 Evaluation results for Case 2.

For Case 2, we have the following observations,

Observation 1

- For Case 2 (FR1-to-FR1 intra-frequency temporal domain case B)

  - Compared the results with MRRT = 0.5 and MRRT = 0.75, when more temporal domain measurements are reduced, the prediction performance is degraded even with the same prediction distance (e.g., 200ms).

  - With the TF-based predictor, the performance of Sub-Case 3 is better.

  - The AI/ML predictor outperforms the SaH for all sub-cases.

  - Case 3, FR1-to-FR1 inter-frequency (frequency domain)

As discussed during the RAN2 #127 meeting, we evaluate the performance of Case 3 considering both the prediction for 4 GHz based on the measurements on the 2 GHz band and the prediction for 2 GHz based on the measurements on the 4 GHz band.

For comparison, the performance of the RSRP offset scheme is also evaluated, where a 6 dB offset is compensated to the RSRP according to the frequency differences between the two bands, and the compensated RSRP is used as the prediction of another band.

Figure 2 Average RSRP difference of Case 3

For Case 3, we have the following observations,

Observation 2

- For Case 3 (FR1-to-FR1 inter-frequency)

  - The AI/ML-based predictor outperforms the RSRP offset scheme.

  - With the TF-based predictor, the performance of Sub-Case 3 is better.

  - Case 4, FR2-to-FR2 intra-frequency temporal domain case A

Figure 3 Evaluation results for Case 4.

For Case 4, 5 measurement samples with 400 ms intervals are predicted. We report the average L3 cell-level RSRP for these 5 predictions in Table 7 and plot them in Figure 3. From the results, we have the following observations,

Observation 3

- For Case 4 (FR2-to-FR2 intra-frequency temporal domain case A)

  - The prediction performance is degraded with the UE velocity.

  - The AI/ML-based predictor outperforms the SaH.

  - With the TF-based predictor, the performance of Sub-Case 3 is better.

  - Performance trends on 3 sub-use cases

At the RAN2 #127bis meeting, the majority view about 3 sub-use cases is the following [6],

Majority of companies observe that among sub cases 1, 2, and 3, at least with shorter prediction window sub case 2 demonstrates the highest prediction accuracy.

To analyze the conditions of this observation further, we take Case 4 as an example case and check the performance of 3 sub-cases under different conditions. After analyzing the results submitted to the RAN2 #127bis meeting, we observed that different AI/ML backbones are used among companies, such as CNN, LSTM, and Transformer (TF). Among them, many companies considered an LSTM-based model backbone for the prediction, and we use the TF. Therefore, we also evaluate the performance with an LSTM-based model besides the TF we used for all cases.

Figure 4 demonstrates the performance of TF and LSTM for 3 sub-cases, where the RSRP difference is the averaged value over a 2s prediction window. When using LSTM, Sub-Case 2 is superior to the others, consistent with the majority view. However, if the TF is used, it performs best with Sub-Case 3, and the performance is also the best among all results.

Figure 4 Average RSRP difference between LSTM and TF for an example prediction.

The different characteristics between the LSTM and TF lead to this performance trend. The LSTM has a memory unit that explores and records the correlations in a series. The LSTM has memory and gates to explore the correlations in the input series. However, when the input dimension or length is high, the gradient-vanishing issue may occur and then cause performance degradation. Therefore, LSTM is more potent for Sub-Case 2, where the input dimension is more minor. The TF has different characteristics in that it explores the global correlations in the input series regardless of length. It can abstract more hidden information and features when the L1-RSRPs are input to the TF, which can be used to produce better predictions.

In summary, we have the following observations based on our analysis and evaluations,

Observation 4

- The performance of 3 sub-cases depends on the characteristics of the AI/ML model backbones used for the evaluations.

  - The TF-based model can extract more information in the raw observations (L1-RSRP), resulting in better prediction accuracy for Sub-Case 3.

  - The LSTM-based one performs better for Sub-Case 2, where the input dimension is more minor.

  - Overall, the TF-based model performs best (w/ Sub-Case 3).

The performance trends of 3 sub-cases depend on implementation issues, where the model backbone may only be one of them. Therefore, we have no substantial evidence to support the down-selection among sub-cases, and we have the following proposal.

Proposal 2

- Continue the study with sub-cases up to the companies’ preferences.

- Conclusion

In this contribution, we have the following observations and proposals,

Observation 1

- For Case 2 (FR1-to-FR1 intra-frequency temporal domain case B)

  - Compared the results with MRRT = 0.5 and MRRT = 0.75, when more temporal domain measurements are reduced, the prediction performance is degraded even with the same prediction distance (e.g., 200ms).

  - With the TF-based predictor, the performance of Sub-Case 3 is better.

  - The AI/ML predictor outperforms the SaH for all sub-cases.

Observation 2

- For Case 3 (FR1-to-FR1 inter-frequency)

  - The AI/ML-based predictor outperforms the RSRP offset scheme.

  - With the TF-based predictor, the performance of Sub-Case 3 is better.

Observation 3

- For Case 4 (FR2-to-FR2 intra-frequency temporal domain case A)

  - The prediction performance is degraded with the UE velocity.

  - The AI/ML-based predictor outperforms the SaH.

  - With the TF-based predictor, the performance of Sub-Case 3 is better.

Observation 4

- The performance of 3 sub-cases depends on the characteristics of the AI/ML model backbones used for the evaluations.

  - The TF-based model can extract more information in the raw observations (L1-RSRP), resulting in better prediction accuracy for Sub-Case 3.

  - The LSTM-based one performs better for Sub-Case 2, where the input dimension is more minor.

  - Overall, the TF-based model performs best (w/ Sub-Case 3).

Proposal 1

- Draw observations from the collected results following the same principle of TR38.843, which includes

  - Draw detailed observations for each case, subcase, and important configuration to capture all submitted results from different sources.

  - For each case and important configuration, draw a summarized observation for the conclusion, which can be based on the following options,

    - Opt. 1: Average [and standard deviation] of the performance gain.

    - Opt. 2: Median [and/or quartiles] of the performance gain.

Proposal 2

- Continue the study with sub-cases up to the companies’ preferences.

Appendix: Simulation assumptions

Table  A1 Common simulation assumptions

| Parameter | FR1 Band #1 (common) | FR1 Band #2 (inter-frequency) | FR2 |
| --- | --- | --- | --- |
| Carrier frequency | 4 GHz (TDD) | 2 GHz (FDD) | 30 GHz |
| SCS | 30 kHz | 15 kHz | 120 kHz |
| Deployment | 2-tier with wrap around (7 sites, 3 sectors per site) |  | 2-tier with wrap around (7 sites, 3 sectors per site) |
| ISD | 500 m |  | 200 m |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. Inter-frequency correlation model: Section 7.6.5 in TR38.901. |  | UMi w/ distance-dependent LoS probability function defined in Table 7.4.2-1 of TR38.901. |
| System bandwidth | 20 MHz |  | 80 MHz |
| Sample period | 40 ms |  | 80 ms |
| UE speed | 90 kmph |  | 60, 90, 120 kmph |
| UE distribution/dropping | 100% outdoor |  | 100% outdoor |
| BS antenna configuration | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ Number of BS beams: 4 |  | (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Number of BS beams: 32 |
| BS antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi |  | TR 38.802 Table A.2.1-6 |
| UE antenna configuration | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |  | (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Number of UE beams: 4 |
| UE antenna radiation pattern | Omni-direction |  | TR 38.802 Table A.2.1-8 |
| BS Tx power | 44 dBm for 20 MHz bandwidth |  | 40 dBm |
| Maximum UE Tx power | 23 dBm |  | 23 dBm |
| BS receiver noise figure | 5 dB |  | 7 dB |
| UE receiver noise figure | 9 dB |  | 10 dB |
| BS antenna height | 25 m |  | 10 m |
| UE antenna height | 1.5 m |  | 1.5 m |
| Spatial consistency | Procedure A in TR38.901 |  | Procedure A in TR38.901 |
| UE trajectory model | Option 3 |  | Option 3 |
| UE boundary model | Option 3 |  | Option 3 |

The RRC parameters are configured following the post-RAN2 #126 email agreements in the following tables.

Table A2 RRC parameters for the Layer 3 filtering

| L3 filtering parameter | value |
| --- | --- |
| FR1 Filter Coefficient | 4 |
| FR2 Filter Coefficient | 4 |

Table  A3 RRC parameters for the measurement period

| Measurement period | value |
| --- | --- |
| FR1 to FR1 intra-frequency w.o. gap | 200 ms |
| FR1 to FR1 inter-frequency with gap | 200 ms |
| FR2 to FR2 intra-frequency w.o. gap | 400 ms |

Table  A4 RRC parameters for beam consolidation/selection

| Consolidation parameter | value |
| --- | --- |
| nrofSS-BlocksToAverage for FR1 | 1 |
| nrofSS-BlocksToAverage for FR2 | 3 |
| absThreshSS-BlocksConsolidation for FR1 | -110 dBm |
| absThreshSS-BlocksConsolidation for FR2 | -110 dBm |

References

- Moderator (CMCC), RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR”, Dec. 2023.

- RAN2 Chair (InterDigital), “RAN2 #125bis Chair Notes”, Apr., 2024.

- 3GPP TR 38.901 v17.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”, Mar., 2022.

- RAN2 Chair (InterDigital), “RAN2 #126 Chair Notes”, May, 2024.

- RAN2 Chair (InterDigital), “RAN2 #127 Chair Notes”, Aug., 2024.

- RAN2 Chair (InterDigital), “RAN2 #127bis Chair Notes”, Oct., 2024.