3GPP TSG-RAN WG2 Meeting #129	R2-2500315

Athens, Greece, Feb. 17th – 21st, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussions on measurement event prediction

Agenda Item:	8.3.3

Document for: 	Discussion and Decisions

- Introduction

During the RAN#102 meeting, the SID on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The SID describes the study scope of the AI/ML-aided mobility enhancements, as below,

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

This paper will discuss the detailed evaluation methodology and results for the measurement event predictions.

- Discussions on the detailed HO modeling for prediction Case A

At the RAN2 #128 meeting, the following options for SLS HO modeling of measurement event prediction Case A were agreed as the following[2],

For System level simulation

For measurement event prediction for temporal domain case A, company focus on option 2 or option 3.

Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1))

Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT.

The detailed definition of the HO failure should be further clarified to allow a fair comparison with the legacy HO model and align the simulation results among sources.  We will discuss this issue in the following.

Figure 1 Example of HO procedure with legacy and different options.

Figure 1 shows the example procedures of the HO with legacy, Option 2, and Option 3 models. For Option 2, the predicted measurement events can trigger the HO preparation, and then the whole HO procedure can be executed earlier than the legacy model. For Option 3, the HO command can also be triggered earlier as soon as the entering of Event A3 in case Event A3 is predicted to happen.

Due to the revision of the HO procedures, the definition of HoF should be discussed further. The following three states are defined in [3] for the legacy HO model, as shown in Figure 2.

- State 1: Before the event A3 entering condition is satisfied;

- State 2: After the event A3 entering condition is satisfied but before the handover command is successfully received by the UE;

- State 3: After the handover command is received by the UE, but before the handover complete is successfully sent by the UE.

Figure 2 Definition of three states for legacy HO model in TR36.839

Based on the definition of the states, the HoF is then defined in [3],

- In State 2

  - T310 has been triggered or is running when HO_CMD is received by the UE, or

  - RLF is declared.

- In State 3

  - Target cell downlink filtered average (the filtering/averaging here is the same as that used for starting T310) wideband CQI is less than the threshold Qout at the end of the handover execution time in state 3.

Following the same principle of these definitions in TR36.839, we propose definitions about the states and HoF for Option 2 and Option 3, respectively.

  - Definitions of states and HoF for Option 2

For Option 2, the HO command is also triggered by the actual measured Event A3, like the legacy HO model. The accuracy of the predicted Event A3 only decides whether the HO preparation can be executed before the HO command. It does not impact the actual execution of the HO procedure. Therefore, the definitions of 3 states and HoF in TR36.839 can be reused with some minor wording revisions as the following,

Proposal 1

- Reuse the definition of 3 states in TR36.839 for Option 2 with the following wording revision,

  - State 1: Before the actual event A3 entering condition is satisfied;

  - State 2: After the actual event A3 entering condition is satisfied but before the handover command is successfully received by the UE;

  - State 3: After the handover command is received by the UE, but before the handover complete is successfully sent by the UE.

- Reuse the definition of HoF in TR36.839 for Option 2 (i.e., the definition of HoF in State 2 and State 3).

An example of the state definition is shown in Figure 3.

Figure 3 Definition of 3 states for Option 2

  - Definitions of states and HoF for Option 3

For Option 3, the HO command can be triggered by the entering of Event A3, which is different from the Option 2. It is equivalent to a case to set the TTT 0 based on the measurement event predictions. Therefore, State 2 is also shortened to 0 length and becomes a time instance in the simulation view. Although State 2 does not exist and it is the time instance at the end of State 1 or the beginning of State 3, we suggest keeping it to align the definitions with other options and reusing the HoF definitions, as shown in Figure 4. Therefore, we recommend the following definitions, which are revised from the ones in [3].

Proposal 2

- Following the principle and text in TR36.839, the definition of 3 states for Option 3 is,

  - State 1: Before the actual event A3 entering condition is satisfied;

  - State(Instance) 2: The instance when event A3 entering condition is satisfied and the HO command is received by the UE;

  - State 3: After the handover command is received by the UE, but before the handover complete is successfully sent by the UE.

- Reuse the definition of HoF in TR36.839 for Option 3 (i.e., definition of HoF in State(Instance) 2 and State 3).

An example of the state definition is shown in Figure 4.

Figure 4 Definition of 3 states/instance for Option 3

- Initial evaluation results on measurement event predictions

  - Simulation setup and assumptions

In this paper, we evaluate the intermediate and system-level performance of two scenarios for overhead reduction, which is Case 2 (FR1-to-FR1 temporal domain prediction Case B) and Case 3 (FR1-to-FR1 inter-frequency predictions). Following the agreements at RAN2 #126 and RAN2 #127, the common simulation assumptions are listed below in Appendix, and the simulation assumptions dedicated to the measurement event prediction are shown in Table 1.

The target for both Case 2 and Case 3 is measurement overhead reduction. Therefore, the HO procedures are not impacted. We reuse the legacy HO model and the HO definition in TR36.839 [3] to evaluate the system-level KPIs. The only difference is that the predictions replace some of the Layer 3 measurement results.

Table 1 Simulation assumptions for measurement event prediction

| Parameter | Value |
| --- | --- |
| A3 event offset (dB) | 2 |
| TTT (ms) | 80 |
| UE speed (km/h) | 30, 60, 90 |
| OW length (ms) | N/A |
| PW length (ms) | 40ms (sliding) |
| Max ETD (ms) | 80 |
| MRRT (Case 2) | 50% |

The details of the following evaluation results have been submitted in the file:

- ME_Case B_129_DCM_2500315_V1.xlsx

  - Case 3 (FR1-to-FR1 inter-frequency prediction)

The evaluation results are shown in Table 2. In this study, we consider the case that the UE is initially on the 2 GHz band and measure/predict the measurement results of cells on the 4 GHz band. Both intermediate and SLS performance metrics are shown in the table below.

Table 2 Evaluation results for FR1-to-FR1 inter-frequency prediction

|  | UE speed | Intermediate KPI |  |  |  | SLS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | F1 score | Precision | Recall | Average RSRP difference [dB] | HoF rate | HO attempt rate [per second per UE |
| Baseline (w/o AI/ML) | 30 kmph | N/A | N/A | N/A | N/A | 12.2% | 0.11 |
|  | 90 kmph | N/A | N/A | N/A | N/A | 23.9% | 0.30 |
| With AI/ML predictions | 30 kmph | 0.95 | 0.92 | 0.98 | 0.18 | 12.1% | 0.11 |
|  | 90 kmph | 0.95 | 0.91 | 0.98 | 0.18 | 23.5% | 0.30 |

Because we considered a heavy-loaded scenario as the agreement, which introduces severe inter-cell interference and high UE speed in the simulations, we observed a relatively higher HoF ratio for the baseline case. When the inter-frequency prediction is used for the cells on the 4 GHz band, we did not observe a non-negligible performance difference compared with the baseline. Therefore, we have the following observations and proposals.

Observation 1

For Case 3 (FR1-to-FR1 inter-frequency prediction)

- The F1 score of the inter-frequency measurement event prediction can reach 0.95.

- The HO performance is not impacted by the inter-frequency predicted measurement event regarding HoF rate and HO attempt rate.

Proposal 3

- Conclude that based on the SLS simulations, the AI/ML-based FR1-to-FR1 inter-frequency prediction can reduce the measurement overhead (measurement gap).

  - Case 2 (FR1-to-FR1 temporal domain prediction Case B)

The intermediate and SLS evaluation results are shown in Table 3.

Table 3 Evaluation results for FR1-to-FR1 temporal domain prediction Case B

|  | UE speed | Intermediate KPI |  |  |  | SLS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | F1 score | Precision | Recall | Average RSRP difference [dB] | HoF rate | HO attempt rate [per second per UE] |
| Baseline (w/o AI/ML) | 30 kmph | N/A | N/A | N/A | N/A | 12% | 0.11 |
|  | 60 kmph | N/A | N/A | N/A | N/A | 19% | 0.21 |
|  | 90 kmph | N/A | N/A | N/A | N/A | 24% | 0.31 |
| With AI/ML predictions | 30 kmph | 0.72 | 0.58 | 0.96 | 0.09 | 8% | 0.12 |
|  | 60 kmph | 0.72 | 0.59 | 0.93 | 0.12 | 14% | 0.23 |
|  | 90 kmph | 0.69 | 0.56 | 0.90 | 0.13 | 18% | 0.32 |

Unlike the inter-frequency prediction, prediction's influence on the measurement event prediction and system-level performance is significant for temporal domain prediction Case B. The HO and related measurement events all involve temporal domain procedures. For the temporal domain prediction Case B, the Layer 3 measurements are the mixed results of predictions and actual measurements with different statistical characteristics. Although the RSRP difference is minor according to the intermediate results, their impacts on the triggering of Event A3 can be significant.

Our initial simulation results show that the AI/ML prediction model can be trained to ensure a high recall score with some sacrifices on the precision score. During the HO procedures, such a model tends to trigger Event A3 more sensitively and early, which reduces the HoF rate in a heavy-loaded scenario with some increases in the HO attempts.

If the AI/ML model is trained with a different strategy, it may behave differently during the HO procedures. A temporal domain model's behavior during HO cannot be projected based only on the average RSRP differences.

Therefore, the system-level simulations are essential for the temporal-domain prediction cases. We have the following suggestions,

Proposal 4

- For temporal domain prediction cases (Case A and Case B), use the SLS results to conclude the study due to the complex relations between the temporal domain predictions and procedures.

- Conclusion

In this contribution, we have the following observations and proposals,

Observation 1

For Case 3 (FR1-to-FR1 inter-frequency prediction)

- The F1 score of the inter-frequency measurement event prediction can reach 0.95.

- The HO performance is not impacted by the inter-frequency predicted measurement event regarding HoF rate and HO attempt rate.

Proposal 1

- Reuse the definition of 3 states in TR36.839 for Option 2 with the following wording revision,

  - State 1: Before the actual event A3 entering condition is satisfied;

  - State 2: After the actual event A3 entering condition is satisfied but before the handover command is successfully received by the UE;

  - State 3: After the handover command is received by the UE, but before the handover complete is successfully sent by the UE.

- Reuse the definition of HoF in TR36.839 for Option 2 (i.e., the definition of HoF in State 2 and State 3).

Proposal 2

- Following the principle and text in TR36.839, the definition of 3 states for Option 3 is,

  - State 1: Before the actual event A3 entering condition is satisfied;

  - State(Instance) 2: The instance when event A3 entering condition is satisfied and the HO command is received by the UE;

  - State 3: After the handover command is received by the UE, but before the handover complete is successfully sent by the UE.

- Reuse the definition of HoF in TR36.839 for Option 3 (i.e., definition of HoF in State(Instance) 2 and State 3).

Proposal 3

- Conclude that based on the SLS simulations, the AI/ML-based FR1-to-FR1 inter-frequency prediction can reduce the measurement overhead (measurement gap).

Proposal 4

- For temporal domain prediction cases (Case A and Case B), use the SLS results to conclude the study due to the complex relations between the temporal domain predictions and procedures.

Appendix: Simulation assumptions

Table  A1 Common simulation assumptions

| Parameter | FR1 Band #1 (common) | FR1 Band #2 (inter-frequency) | FR2 |
| --- | --- | --- | --- |
| Carrier frequency | 4 GHz (TDD) | 2 GHz (FDD) | 30 GHz |
| SCS | 30 kHz | 15 kHz | 120 kHz |
| Deployment | 2-tier with wrap around (7 sites, 3 sectors per site) |  | 2-tier with wrap around (7 sites, 3 sectors per site) |
| ISD | 500 m |  | 200 m |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. Inter-frequency correlation model: Section 7.6.5 in TR38.901. |  | UMi w/ distance-dependent LoS probability function defined in Table 7.4.2-1 of TR38.901. |
| System bandwidth | 20 MHz |  | 80 MHz |
| Sample period | 40 ms |  | 80 ms |
| UE speed | 90 kmph |  | 60, 90, 120 kmph |
| UE distribution/dropping | 100% outdoor |  | 100% outdoor |
| BS antenna configuration | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ Number of BS beams: 4 |  | (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Number of BS beams: 32 |
| BS antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi |  | TR 38.802 Table A.2.1-6 |
| UE antenna configuration | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |  | (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Number of UE beams: 4 |
| UE antenna radiation pattern | Omni-direction |  | TR 38.802 Table A.2.1-8 |
| BS Tx power | 44 dBm for 20 MHz bandwidth |  | 40 dBm |
| Maximum UE Tx power | 23 dBm |  | 23 dBm |
| BS receiver noise figure | 5 dB |  | 7 dB |
| UE receiver noise figure | 9 dB |  | 10 dB |
| BS antenna height | 25 m |  | 10 m |
| UE antenna height | 1.5 m |  | 1.5 m |
| Spatial consistency | Procedure A in TR38.901 |  | Procedure A in TR38.901 |
| UE trajectory model | Option 3 |  | Option 3 |
| UE boundary model | Option 3 |  | Option 3 |

The RRC parameters are configured following the post-RAN2 #126 email agreements in the following tables.

Table  A2 RRC parameters for the Layer 3 filtering

| L3 filtering parameter | value |
| --- | --- |
| FR1 Filter Coefficient | 4 |
| FR2 Filter Coefficient | 4 |

Table  A3 RRC parameters for the measurement period

| Measurement period | value |
| --- | --- |
| FR1 to FR1 intra-frequency w.o. gap | 200 ms |
| FR1 to FR1 inter-frequency with gap | 200 ms |
| FR2 to FR2 intra-frequency w.o. gap | 400 ms |

Table  A4 RRC parameters for beam consolidation/selection

| Consolidation parameter | value |
| --- | --- |
| nrofSS-BlocksToAverage for FR1 | 1 |
| nrofSS-BlocksToAverage for FR2 | 3 |
| absThreshSS-BlocksConsolidation for FR1 | -110 dBm |
| absThreshSS-BlocksConsolidation for FR2 | -110 dBm |

References

- Moderator (CMCC), RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR,” Dec. 2023.

- RAN2 Chair (InterDigital), “RAN2 #128 Chair Notes”, Nov., 2024.

- 3GPP TR36.839 v11.1.0, Mobility enhancements in heterogeneous networks (Release 11), Dec., 2012.

- 3GPP TR 38.843 v2.0.0, Study on Artificial Intelligence (AI)/Machine Learning(ML) for NR air interface (Release 18), Dec., 2023.