3GPP TSG-RAN WG2 Meeting #129                                           R2-2500324

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.3.2.1

Source:	Mediatek Inc.

Title:	Simulation Results for RRM Prediction and System Level Simulation

Document for:	Discussion, Decision

Introduction

For System level simulation 1 For measurement event prediction for temporal domain case B: UE reports when A3 event is satisfied with actual measurements and predicted results. And handover command will be received after handover preparation. Remove T0 from the picture

In RAN2 #128 meeting, RAN2 agree to consider system-level simulation for temporal domain case B and the assumption of the HO procedure is given as follows.

In this contribution, we provide the system-level simulation results for temporal domain case B and discuss the correlation between system-level metrics and intermediate metrics, e.g., F1 score. We notice the F1 score value highly depends on the method of deriving the ground-truth label, and further clarification is needed to obtain a valid F1 score. Additionally, we discuss the expression of the summary of simulation results. Given the numerous use cases, options, and parameters, we need a clear method to conclude all the companies’ results that can be captured in the TR.

Discussion

Simulation Results of Temporal Domain Case B

We have already examined the AI performance of RRM temporal domain case B via the intermediate KPI, e.g., L3 RSRP difference. Our previous results [1] show that AI can provide a high-accuracy prediction for the short prediction distance, e.g., 40ms, in this case. However, we still need system-level simulation to verify the impact of HO performance when we apply the AI prediction to replace the real measurement. Our system-level simulation settings are given in the following table and other general settings can be found in the Appendix.

| Setting/Parameters | Value |
| --- | --- |
| TTT | 320ms |
| A3 offset | 2dB |
| # of UE | 8 |
| Running time for each UE | 400s |
| Max ETD | 80ms |

Table 1: Parameters of system level simulation

In system-level simulation (SLS), we compare three different approaches (1) legacy HO: this is the normal HO without any measurement reduction, which provides a baseline to compare the performance loss (2) sample and hold: the non-AI approach considers the measurement reduction based on MRRT, replacing the absent measurement with the last real measurement result. (3) AI: Using AI to predict the absent measurement. We consider two different MRRTs, e.g., 0.5 and 0.8, and the corresponding results are given in the following two tables.

| FR1 Temporal domain case B, UE speed = 30 km/hr, MRRT = 0.5 | Intermediate KPI | System level KPI |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | L3 RSRP diff | HOF rate | HO/s/# of UE | HOF/s/# of UE | Total # of HOF (all UE) |
| Legacy HO | NA | 0.094 | 0.059 | 0.0056 | 18 |
| Sample and hold | 0.124 | 0.089 | 0.062 | 0.0056 | 18 |
| AI | 0.059 | 0.094 | 0.063 | 0.0059 | 19 |

Table 2: System level result of temporal domain case B, MRRT=0.5

| FR1 Temporal domain case B, UE speed = 30 km/hr, MRRT = 0.8 | Intermediate KPI | System level KPI |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | L3 RSRP diff | HOF rate | HO/s/# of UE | HOF/s/# of UE | Total # of HOF (all UE) |
| Legacy HO | NA | 0.094 | 0.059 | 0.0056 | 18 |
| Sample and hold | 0.293 | 0.087 | 0.071 | 0.0062 | 20 |
| AI | 0.241 | 0.068 | 0.072 | 0.005 | 16 |

Table 3: System level result of temporal domain case B, MRRT=0.8

We observe that the HOF rate is quite low (less than 1%) in the FR1(UE speed=30km/hr) case. For MRRT=0.5, the system-level performances of the three different approaches are similar. It should be noted that the HOF rate of sample and hold is slightly lower than others, however, this reduction does not come from the reduction of HOF but from the increase of HO attempts. This is because the sample and hold approach will repeat the same measurement multiple times, making the A3 event condition will be triggered more easily. Since the HOF rate does not completely reflect HO performance, we propose adding HOF/s/# of UE (total number of handover failure per second per UE) as an SLS metric that makes it easier to realize the variation of the number of HOF. Compared to sample and hold, the AI approach has similar HO attempts and a slightly worse HOF rate which is under the acceptable margin of error. Since the total number of HOF in the entire experiment is quite low, e.g., 18 times for 8UE with 400s running time, the difference between HOF rates comes from the increase/decrease of 1 HOF.

For MRRT=0.8, similar results are observed. Compared to the sample and hold, the AI approach has a similar # of HO attempts, but the total # of HOF reduces from 20 to 16. This is because, for MRRT=0.8, the sample and hold approach will repeat each real measurement 4 times which may lead to a wrong HO decision. However, in such low-speed FR1 scenario, the overall HO performance is still quite good, and the margin for AI improvement is limited.

Observation 1: The HOF happens with low probability for the setting given in temporal domain case B.

Observation 2: The HOF rate does not completely represent the HO performance, as different approaches have different numbers of HO attempts.

Proposal 1: Add HOF/s/# of UE (total number of handover failures per second per UE) as a metric for system-level simulation.

Observation 3: The AI approach can reduce the measurement overhead without significant system-level performance loss. The system performance is comparable to non-AI approach, e.g., sample and hold. The overall HO performance is still quite good in the low UE speed FR1 scenario even if we skip part of the measurements directly.

Discussion on the Intermediate KPI and Ground Truth Label

In addition to the L3 RSRP difference, RAN2 also consider the F1 score of event prediction accuracy as one of the intermediate KPIs in the indirect measurement even prediction temporal domain case B. However, we notice that the F1 score highly depends on the method of deriving the ground-truth label. An unsuitable method will lead to a meaningless F1 score value that does not reflect the HO performance. This can be observed in the following table, where we compare the SLS result and the F1 score. Here, the ground-true label is derived based on the legacy HO procedure, more specifically, we record all the events and their triggering time as the ground-truth label.

We can see that the F1 score is quite low, especially for the case with MRRT = 0.8, even though the system performance is actually good. This is because the F1 score is derived based on the ground-truth label which is pre-derived based on the legacy HO procedure and does not follow the current status, i.e., the current serving cell based on the AI approach. Thus, once a prediction error occurs such that UE decides to HO to a cell that is different from the ground-truth (derive from legacy HO), it may affect the subsequent decisions in the entire experiment and result in very low prediction accuracy and F1 score. The error propagation is illustrated in the following Figure 1.

| FR1, Temporal domain case B, UE speed = 30 km/hr, | Intermediate KPI |  | System level KPI |  |  |
| --- | --- | --- | --- | --- | --- |
|  | L3 RSRP diff | F1 Score | HOF rate | HO/s/# of UE | HOF/s/# of UE |
| Legacy HO | NA | NA | 0.094 | 0.059 | 0.0056 |
| Sample and hold (MRRT = 0.5) | 0.124 | 0.680 | 0.089 | 0.062 | 0.0056 |
| AI (MRRT = 0.5) | 0.059 | 0.678 | 0.094 | 0.063 | 0.0059 |
| Sample and hold (MRRT = 0.8) | 0.293 | 0.320 | 0.087 | 0.071 | 0.0062 |
| AI (MRRT = 0.8) | 0.241 | 0.236 | 0.068 | 0.072 | 0.005 |

Table 4: F1 score of indirect measurement event prediction (temporal domain case B)

Figure 1: Error propagation when considering pre-derived ground-truth label.

Since there is still no agreement on how to derive the ground-truth label, based on the above observation, to obtain a meaningful F1 score that can match the SLS performance, we propose deriving the ground-truth label using the following method

- Method of deriving ground-truth label: The ground-truth label of real events is derived every time an HO is successful or a HOF/RLF occurs. Based on the current serving cell according to the AI approach, assume there is a parallel legacy HO procedure without measurement reduction under the same status, the ground truth label of the next real event is the next event triggered by the legacy HO procedure.

Observation 4: The intermediate KPI, i.e., F1 score, highly depends on the method of deriving the ground-truth label. Directly deriving the ground-truth label from a legacy HO procedure will result in a very low F1 score, which is meaningless and mismatches the SLS results.

Proposal 2: For measurement event prediction, RAN2 align the method of deriving ground-truth labels of real events:

- The ground-truth label of real events is derived every time an HO is successful or a HOF/RLF occurs. Based on the current serving cell according to the AI approach, assume there is a parallel legacy HO procedure without measurement reduction under the same status, the ground truth label of the next real event is the next event triggered by the legacy HO procedure.

Discussion on the expression of the summary of the simulation results

Since RAN2 has already generated part of the simulation result, we would like to discuss how to conclude and summery all the results from companies. Since for each use case, we have different settings and options, it is challenging to compare the results from each company. Take the RRM Temporal domain case B as an example, we are allowed to provide the result with different sub-use cases, options, and parameters which are listed in the following tables.

| Group A: Scenario settings group | Group B: Implemented options group |
| --- | --- |
| - MRRT - UE speed | - Sub-use case (1,2,3) - Filtering option - Sliding/non-sliding - OW - PW - Cluster/cell-specific - AI Model |

Table 5: Example of the sub-use cases, options, and parameters in RRM temporal domain case B.

In the above table, we categorize the variables into two groups (1) scenario settings group: this group contains variables that correspond to a specific scenario. (2) implemented option group: this group contains variables that can be chosen and implemented by device. Based on the category, we can make comparisons under each specific scenario setting, for example, in this case, we have a total of 8 (2MRRT *4UE speed) different scenarios. Our comparison does not need under the same setting listed in the implemented options group since in practice companies can select what they think is the best as their solution.

The intention of separating these two groups is to allow us to compare all the results under the same scenario setting to ensure a fair comparison and to limit the total number of comparisons (we do not need to consider all combinations of all implemented options). But of course, if we would like to compare the result of specific implemented options, for example, to examine the impact of OW or PW, we can compare them separately.

Based on the methodology, we can provide statistical results among all companies’ results. Here we take the case (MRRT = 0.5, UE speed = 30km/hr, Sliding) as an example. Each company may have multiple results since they may consider different implemented options. Thus, we propose the following options to derive the statistical result.

- Option 1 (Take optimal): Take the optimal result, e.g., the one with the lowest RSRP, from each company.

- Option 2 (List all): Take all results from each company.

And the statistical result can be presented by

- Option a: CDF-like curve, where the y-axis is the ratio of all results and the x-axis is the metric value, e.g., L3 RSRP difference value.

- Option b: Bar chart figure, where the y-axis is the number of results and the value under the bar is the partition of metric, e.g., L3 RSRP difference value. The partition threshold can be FFS.

The following figures are examples based on the Excel table submitted by all companies for the temporal domain case B use case. We provide the example figure for Options 1-a, 1-b, 2-a, and 2-b.

Figure 2: Example of options 1-a and 1-b, where only one optimal result of each company is taken into account.

Figure 3: Example of options 2-a and 2-b, where all results of each company are taken into account.

For option 1-b, the total number is 6 + 3 = 9 which is equal to the number of companies that provide the result, since we only take one result from each company. For option 2-b, the total number is larger than 9, since multiple results from a company can be taken into account.

Proposal 3: RAN2 discuss the parameters/settings to compare the performance. One approach is to categorize all results based on the parameters in Group A. The results with the same parameter listed in Group A are comparable and used to derive the statistical result, while the other parameters listed in Group B do not affect the categorization and can be compared together.

Proposal 4: The definition of Groups A and B can be the scenario setting group and implemented option group given in Table 5, other options are FFS.

Proposal 5: RAN2 discuss the way to handle the statistics from the results with different parameters in Group B. Other options are not precluded.

- Option 1 (Take optimal): Take the optimal result, e.g., the one with the lowest RSRP, from each company.

- Option 2 (List all): Take all results from each company.

Proposal 6: RAN2 discuss the method to demonstrate the statistics. Other options are not precluded.

- Option a: CDF-like curve, where the y-axis is the ratio of all results, x-axis is the metric, e.g., L3 RSRP difference value.

- Option b: bar chart figure, where the y-axis is the number of results, the value under the bar is the partition of metric, e.g., L3 RSRP difference value. The partition threshold can be FFS.

Conclusion

Simulation results: SLS result of intra-frequency temporal domain case B

Observation 1: The HOF happens with low probability for the setting given in temporal domain case B.

Observation 2: The HOF rate does not completely represent the HO behavior, as different approaches have different numbers of HO attempts.

Proposal 1: Add HOF/s/# of UE as a metric for system-level simulation.

Observation 3: The AI approach can reduce the measurement overhead without significant system-level performance loss. However, the improvement compared to the non-AI approach, e.g., sample and hold, are limited, since the overall HO performance is still quite good in the low UE speed FR1 scenario even if we skip part of the measurements directly.

Discussion on intermediate KPI and ground-truth label

Observation 4: The intermediate KPI, i.e., F1 score, highly depends on the method of deriving the ground-truth label. Directly deriving the ground-truth label from a legacy HO procedure will result in a very low F1 score, which is meaningless and mismatches the SLS results.

Proposal 2: For measurement event prediction, RAN2 align the method of deriving ground-truth labels of real events:

- The ground-truth label of real events is derived every time an HO is successful or a HOF/RLF occurs. Based on the current serving cell according to the AI approach, assume there is a parallel legacy HO procedure without measurement reduction under the same status, the ground truth label of the next real event is the next event triggered by the legacy HO procedure.

Discussion on the expression of the summary of simulation results

Proposal 3: RAN2 discuss the parameters/settings to compare the performance. One approach is to categorize all results based on the parameters in Group A. The results with the same parameter listed in Group A are comparable and used to derive the statistical result, while the other parameters listed in Group B do not affect the categorization and can be compared together.

Proposal 4: The definition of Groups A and B can be the scenario setting group and implemented option group given in Table 5, other options are FFS.

Proposal 5: RAN2 discuss the way to handle the statistics from the results with different parameters in Group B. Other options are not precluded.

- Option 1 (Take optimal): Take the optimal result, e.g., the one with the lowest RSRP, from each company.

- Option 2 (List all): Take all results from each company.

Proposal 6: RAN2 discuss the method to demonstrate the statistics. Other options are not precluded.

- Option a: CDF-like curve, where the y-axis is the ratio of all results, x-axis is the metric, e.g., L3 RSRP difference value.

- Option b: bar chart figure, where the y-axis is the number of results, the value under the bar is the partition of metric, e.g., L3 RSRP difference value. The partition threshold can be FFS.

Reference

[1] 	R2-2409868 Simulation Results for AIML RRM Prediction and Remaining Issues

Appendix: Simulation Setting

| parameters | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz,30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model (7 sites, 3 sectors/cells per site) | 2-tier model (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and LOSsoft; Inter-frequency correlation model is optional. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and LOSsoft; |
| System BW | 20MHz | 80MHz |
| UE speed | 30,90 km/h for study targeting measurement reduction 60,120 km/h for study targeting HO performance improvement | 30,,90 km/h for study targeting measurement reduction 60,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ | (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) | Antenna setup and port layouts at UE: (1, 4, 2, 1, 1, 1, 1) |
| UE Antenna pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| Tx/Rx beam | Tx: 1, Rx:1 | Tx:32, Rx:4 |
| BS Tx Power | 44dBm | 40 dBm |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | Procedure A in TR38.901 with coarse update | Procedure A in TR38.901 with coarse update |
| UE trajectory model | Option 1 | Option 1 |
| UE trajectory boundary processing model | Option 2: circle-bouncing model, | Option 2: circle-bouncing model, |
| Sampling period | 40ms | 20ms *4 = 80ms |