3GPP TSG-RAN WG2 Meeting #129	R2-2500329

Athens, Greece, 17th – 21st of February 2025

Agenda item:	7.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On SSBs from Source and Target Satellite in Soft Switching with Resynchronization

WID/SID:	NR_NTN_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

At RAN2#128 (November 2024) it was fiercely discussed how to avoid the SSB overlapping in time for soft-satellite switching with resynchronization. In this paper we present our view how this issue might be addressed without major impact on the legacy specification.

# 2	Discussion

## 2.1	Background

The issue has been discussed via offline thread at RAN2#128 (summary in [2]) and three options have been listed there:

- Option 1: shifted cell timing

- Option 2: shifted SSB pattern

- Option 3: temporarily shifted SSB pattern

Companies in RAN2 were not in favour of Option 1 and preferred to further consider Option 2 or Option 3. However, there were also concerns regarding Option 3, as it may potentially result in the UE having to reacquire the DL synchronization at the t-Service. Thus, the LS to RAN1 and RAN4 has been agreed in [3] to confirm whether indeed Option 3 will result in the necessity to obtain DL synchronization once again.

Eventually the following was agreed at RAN2#128 [1]:

- Send a LS to RAN1/RAN4 on the feasibility of option 3. RAN2 will work on solution 2b or solution 3 depending on the LS response

Solution 2b is a subset of Option 2, wherein the SMTC is adjusted autonomously by the UE, as explained in [2].

While awaiting the LS response from RAN1 and RAN4, we analyse these three options in the next subsection of this paper.

## 2.2 	Analysis of the Options listed in R2-2410971

During the discussion at RAN2#128 companies have rejected Option 1 [1][2]. There was a strong reluctance to change the entire cell’s timing. Thus other two options were supported/further considered (as explained briefly in Introduction). The problem with Option 2 and 3 is such that they both assume individual SSBs may be shifted within the radio frame. However, according to the 3GPP RAN1 specification [4], SSBs within the radio frame have a pre-defined and hard-coded position on the time axis.

Observation 1: SSBs within the radio frame have a fixed position, as per TS 38.213 definition.

As a result, Options 2 and 3 are currently not doable when considering the existing 3GPP framework.

Observation 2: Option 2 and Option 3 listed in R2-2410971 are not feasible.

Thus, it seems none of the options outlined in [2] can be directly used to address the SSB overlapping issue for soft satellite switching with resynchronization.

## 2.3	Use of NCD-SSBs during t-ServiceStart to t-Service period

If the issue identified in [2] for soft-satellite switching is to be resolved without significantly impacting the legacy specification, then none of the Options considered can be implemented, as indicated in section 2.2. We propose an alternative approach which takes advantage of the Non-Cell Defining (NCD) SSBs. As a brief reminder, NCD-SSBs are capable of providing DL synchronization but these are not associated with RMSI (Remaining Minimum System Information, including SIB1). The main purpose of NCD-SSBs was to serve in bandwidth parts (BWPs) which do no comprise regular SSBs and help e.g. the RedCap UEs to obtain/maintain connectivity.

Observation 3: NCD-SSBs are capable of providing DL synchronization but are not associated with RMSI.

It is worth noting that NCD-SSBs are configurable by the NW in the orthogonal resources to those resources that are used for the transmission of regular SSBs. The position of NCD-SSBs within the radio frame does not have to be fixed.

Observation 4: NCD-SSBs are configurable by the NW in the orthogonal resources to those resources that are used for the transmission of regular SSBs.

Observation 5: The position of NCD-SSBs within the radio frame does not have to be fixed.

As the result of the abovementioned benefits of NCD-SSBs, we suggest to resolve the SSB timing issue for soft-satellite switching with resynchronization by using NCD-SSBs during the time period (t-ServiceStart, t-Service). The high-level sequence of actions could be as follows:

- After t-ServiceStart the target satellite will start broadcasting NCD-SSBs. There is no transmission of the regular SSBs from the target satellite during that period. Source satellite continues transmitting regular SSBs between the time period (t-ServiceStart, t-Service).

- During the time window (t-ServiceStart, t-Service) the UE acquires synchronization with the target satellite using NCD-SSBs.

- At time t-Service the source satellite stops broadcasting SSBs and covering that area. Target satellite stops the transmission of NCD-SSBs and switches to regular (cell-defining) SSB transmission. UE does not have to reacquire the DL synchronization with the target satellite.

Proposal 1: RAN2 to consider relying on NCD-SSBs to mitigate the SSB overlapping issue for soft-satellite switching with resynchronization.

If the scheme described in Proposal 1 is to be operational then System Information Block (SIB) should convey the details regarding the temporary configuration of NCD-SSBs. Most logically this shall be included in SatSwitchWithReSync IE within SIB19.

Proposal 2: RAN2 considers extending SatSwitchWithReSync in SIB19 by including NCD-SSB configuration which would be valid from t-ServiceStart to t-Service.

Obviously, the final decision taken by RAN2 should be also subject to RAN1 and RAN4 feedback. Thus, we are fine to postpone the ultimate resolution to the next meeting (i.e. RAN2#129bis) when the response LSs from RAN1 and RAN4 are expected.

Proposal 3: RAN2 may take the ultimate decision once the responses from RAN1 and RAN4 are received (i.e. at RAN2#129bis).

# 3	Conclusion

This document has made the following observations:

Observation 1: SSBs within the radio frame have a fixed position, as per TS 38.213 definition.

Observation 2: Option 2 and Option 3 listed in R2-2410971 are not feasible.

Observation 3: NCD-SSBs are capable of providing DL synchronization but are not associated with RMSI.

Observation 4: NCD-SSBs are configurable by the NW in the orthogonal resources to those resources that are used for the transmission of regular SSBs.

Observation 5: The position of NCD-SSBs within the radio frame does not have to be fixed.

And proposed the following:

Proposal 1: RAN2 to consider relying on NCD-SSBs to mitigate the SSB overlapping issue for soft-satellite switching with resynchronization.

Proposal 2: RAN2 considers extending SatSwitchWithReSync in SIB19 by including NCD-SSB configuration which would be valid from t-ServiceStart to t-Service.

Proposal 3: RAN2 may take the ultimate decision once the responses from RAN1 and RAN4 are received (i.e. at RAN2#129bis).

# References

- R2-2410913 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #128 Orlando, USA, Nov 18th – 22nd, 2024

- R2-2410971 Report of [AT128][301][NR NTN] SMTC issues 3GPP TSG-RAN WG2 Meeting #128 Orlando, USA, Nov 18th – 22nd, 2024

- R2-2410978 LS soft satellite switch with resync 3GPP TSG-RAN WG2 Meeting #128 Orlando, USA, Nov 18th – 22nd, 2024

- 3GPP TS 38.213 NR; Physical layer procedures for control, v18.5.0, January 2025

- R2-2409213 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #127bis Changsha, China, Apr 15th – 19th, 2024