3GPP TSG-RAN WG2 Meeting #129	R2-2500330

Athens, Greece, 17th – 21st of February 2025

Agenda item:	8.8.6

Source:	Nokia, Nokia Shanghai Bell

Title:	On E-UTRAN TN to NR NTN enhancements

WID/SID:	LTE_TN_NR_NTN_mob/NR_NTN_enh-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

At RAN#106 the following (highlighted) changes to LTE TN to NR NTN objective have been approved [1]:

### 4.1 Objective of SI or Core part WI or Testing part WI The work item aims at specifying IDLE and CONNECTED mode mobility from E-UTRA TN to NR NTN with the following assumptions: - GSO (Geo Synchronous Orbit) and NGSO (Non-Geo Synchronous Orbit). - NGSO includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO). - Earth fixed tracking area. Quasi-Earth fixed & Earth moving cells for NGSO - UEs with GNSS (Global Navigation Satellite Systems) capability - FDD mode for NR-NTN - NTN-FR1 band is considered in priority - Implicit compatibility to support HAPS (High Altitude Platform Station) and ATG (Air To Ground) scenarios, where relevant The work item aims at specifying support for the following objectives: - idle mode mobility based on cell reselection from E-UTRA TN to NR NTN, where E-UTRA TN provides satellite information for NR NTN neighbor cells in a System Information Block. [RAN2] - connected mode mobility based on handover from E-UTRA TN to NR NTN, where E-UTRA TN provides necessary satellite information for NR NTN neighbor cell measurements (e.g. polarization information, ephemeris data, etc.) in NR specific measurement configurations. [RAN2] Note: - For idle mode mobility, the solutions shall be reusing the existing signaling introduced for IoT-NTN and mobility procedures between NR TN and NR NTN to the maximum extent. - For connected mode mobility, the solutions shall be reusing the existing signaling for inter-RAT measurements to NR in TS 36.331 and the related satellite assistance information specified in TS 38.331 for NR NTN. Support of multiple SMTC is not within the scope.

In this paper we provide our view regarding how to address this new objective. In addition, we follow up on the redirection from LTE TN to NR NTN which has been discussed at RAN2#128 [2] without final conclusions.

# 2	Discussion

## 2.1	Mobility from E-UTRA TN to NR NTN in CONNECTED

As RAN2 did great job in Release 19 in defining the mechanisms to enable LTE TN to NR NTN reselections, RAN#106 (in December 2024) has assigned a new task to RAN2 – define a solution for Connected mode handover from E-UTRA TN to NR NTN. As a part of this work, RAN2 needs to decide how the UE acquires the information about NR NTN cell’s (e.g. polarization, ephemeris, etc.) while it is being served by the E-UTRAN TN cell.

Observation 1: To address the new objective in the WID (RP-243115) RAN2 needs to decide how the UE acquires the information about NR NTN (e.g. cell’s polarization, ephemeris, etc.) while it is being served by the E-UTRA TN cell.

As can be noticed in [1], the work does not involve multiple NR SMTCs the UE connected to LTE TN cell should measure. It is a similar restriction as agreed for the mechanism that applies to IDLE/INACTIVE.

Observation 2: The solution RAN2 needs to define for CONNECTED mode does not include multiple SMTCs.

In order to execute a handover to NR NTN while being served by E-UTRA TN the UE needs to possess the information regarding the ephemeris and polarization. Similar need was identified in Release 18, when Event D2 has been defined. It was concluded then that the UE should obtain all necessary components directly from RRC Reconfiguration (i.e. dedicated message) and it should not try to match some information from SIB19 with the corresponding information from the dedicated message.

Observation 3: When designing Event D2 in Release 18 it was concluded that the UE should obtain all necessary components directly from RRC Reconfiguration (i.e. dedicated message) and it should not try to match some information from SIB19 with the corresponding information in dedicated message.

To implement what has been summarized in Observation 3, NTN-NeighbourCellInfo is given as a part of MeasObjectNR [3] if measurement object is configured by the serving cell for measuring Earth-moving NTN neighbour cell associated with the reporting configuration with EventD2 or condEventD2.

Observation 4: As per NR RRC, NTN-NeighbourCellInfo can be given as a part of MeasObjectNR if measurement object is configured by the serving cell for Earth-moving NTN neighbour measurements associated with the reporting configuration with EventD2 or condEventD2.

Similar IE (e.g. NTN-NeighbourCellInfo or ntn-Config) should be available in RRC Connection Reconfiguration provided in E-UTRA cell. The UE has to obtain this information for measurements and subsequent access (handover) to an NR NTN cell. It may be suboptimal, though, to provide the information which has an excessive size and may overlap with the content of LTE system information blocks (e.g. SIB33 in [4]).

Observation 5: It is not optimal to provide the information elements of excessive size (such as an ephemeris) in dedicated signalling, especially when these overlap with the content of LTE system information block (e.g. NeighSatelliteInfoNR in SIB33, as per R2-2407617).

Nevertheless, if RAN2 is convinced the UE should conveniently receive all necessary parameters/information in dedicated LTE signalling then RAN2 should decide to insert NeighSatelliteInfoNR somewhere within MeasConfig [5]. One option is to do it similarly to what was defined for EventD2 in Release 18 NR (as discussed above). In this case MeasObjectNR might be extended with the NeighSatelliteInfoNR (which is to be introduced in Release 19 as per [4]).

Proposal 1: RAN2 considers introducing the NeighSatelliteInfoNR into MeasObjectNR in LTE RRC.

A separate question that has to be answered is whether the UE should be configured with Event B1 or Event B2 to measure and subsequently report the results regarding to the NR NTN neighbour. In our view that would be a natural step, considering Events B1 and B2 were defined for the purpose of inter-RAT measurement and reporting.

Proposal 2: UE in E-UTRA TN cell measures and reports the results on NR NTN neighbours based on Event B1 or B2 configuration.

## 2.2	Redirection from LTE TN to NR NTN

At RAN2#128 the redirection from LTE TN to NR NTN has been discussed and the following has been concluded [2]:

- RAN2 understands that the UE behaviour is currently not clear if the network includes a NR NTN frequency in redirectedCarrierInfo in the RRCConnectionRelease message in a TN cell. We can further discuss whether we want to clarify the UE behaviour and in case whether we need to introduce a new UE capability.

As can be seen in the excerpt provided above, the topic is still considered to be open with the possibility to introduce a separate UE capability to describe such behaviour. In our view it would be beneficial if LTE’s redirectedCarrierInfo can also indicate NR NTN frequency and the corresponding UE’s behaviour is predictable (i.e. once the UE supports associated functionality/indicates the capability). However, it is not necessary to indicate the NeighSatelliteInfoNR directly in the RRCConnectionRelease. Instead, if the NW wants to redirect the UE to NR NTN frequency, then the satellite assistance information in SIB33 should be valid for the carrier frequency indicated in redirectedCarrierInfo. Thus, we make the following proposals:

Proposal 3: LTE’s redirectedCarrierInfo in RRCConnectionRelease can indicate NR NTN frequency and the UE supporting this functionality performs associated measurements for inter-RAT reselection.

Proposal 4: The NW ensures the satellite assistance information in SIB33 is also valid for the carrier frequency indicated in redirectedCarrierInfo.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: To address the new objective in the WID (RP-243115) RAN2 needs to decide how the UE acquires the information about NR NTN (e.g. cell’s polarization, ephemeris, etc.) while it is being served by the E-UTRA TN cell.

Observation 2: The solution RAN2 needs to define for CONNECTED mode does not include multiple SMTCs.

Observation 3: When designing Event D2 in Release 18 it was concluded that the UE should obtain all necessary components directly from RRC Reconfiguration (i.e. dedicated message) and it should not try to match some information from SIB19 with the corresponding information in dedicated message.

Observation 4: As per NR RRC, NTN-NeighbourCellInfo can be given as a part of MeasObjectNR if measurement object is configured by the serving cell for Earth-moving NTN neighbour measurements associated with the reporting configuration with EventD2 or condEventD2.

Observation 5: It is not optimal to provide the information elements of excessive size (such as an ephemeris) in dedicated signalling, especially when these overlap with the content of LTE system information block (e.g. NeighSatelliteInfoNR in SIB33, as per R2-2407617).

Proposal 1: RAN2 considers introducing the NeighSatelliteInfoNR into MeasObjectNR in LTE RRC.

Proposal 2: UE in E-UTRA TN cell measures and reports the results on NR NTN neighbours based on Event B1 or B2 configuration.

Proposal 3: LTE’s redirectedCarrierInfo in RRCConnectionRelease can indicate NR NTN frequency and the UE supporting this functionality performs associated measurements for inter-RAT reselection.

Proposal 4: The NW ensures the satellite assistance information in SIB33 is also valid for the carrier frequency indicated in redirectedCarrierInfo.

# References

- RP-243115	Revised WID: Inter RAT mobility support from E-UTRAN TN to NR-NTN 3GPP TSG RAN Meeting #106 Madrid, Spain 9 - 12, 2024

- R2-2410913 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #128 Orlando, USA, Nov 18th – 22nd, 2024

- 3GPP TS 38.331 NR; Radio Resource Control (RRC); Protocol specification, V18.4.0 (2024-12)

- R2-2407617 Introduction of LTE TN to NR NTN IDLE mode mobility 3GPP TSG-RAN2 Meeting #127 Maastricht, Netherlands, 19th - 23rd Aug 2024

- 3GPP TS 36.331 Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification V18.4.0 (2024-12)