3GPP TSG-RAN WG2 Meeting #129 	R2-2500332

Athens, Greece, 17 – 21 February 2025	revision of R2-2410441

Agenda item:	8.6.3

Source:	Nokia

Title:	On L1 Measurement Reporting Enhancements for Rel-19 LTM

WID/SID:	NR_Mob_Ph4 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The phase 4 mobility enhancements work item description captures the measurement enhancements in the [1] as follows:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

    - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

Following agreements have been made in the previous RAN2 meetings [2-4]:

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12.	NW can configure a dedicated SR configuration for MR MAC CE transmission.

Agreements made at the most recent RAN2 meeting (RAN2#128 [5]):

Agreements on L1 event-driven MR:

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

- TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

- TTT is applied to the leaving condition.

- Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

- Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

- For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

- A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

- Support the truncated measurement report MAC CE.

In this paper we present our views on the remaining open issues for event-triggered L1 measurement reporting framework for LTM.

# 2	Event-triggered reporting for LTM

## 2.1 	Resource Indicator in MR MAC CE

The following was agreed in one of the preceding meetings:

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

LTM-CSI resource configuration is defined as follows in NR RRC:

Figure 1. LTM-CSI-Resource config -R18

Rel-18 L1 reporting format for LTM includes RS index value that is determined based on the configuration order of the RS in the resource set i.e., in the LTM-CSI-SSB-ResourceList shown in Figure 1. The bit length of the RS index field in Release 18 L1 reporting format is determined by the number of configured RSs in the resource set.

In the resource set (ltm-CSI-SSB-ResourceList):

- For each candidate cell, up to 64 RS indices can be included and

- RSs for up to 8 candidate cells can be configured.

This results in up to 512 RS indices which means that up to 9 bits per RS may be needed. This causes the resource indicator to consume the space that overflows the single octet i.e. the reporting of an RSRI requires 2 octets in MAC CE to be included.

Observation 1: To support reporting of measurements on 512 candidate RS indices in the CSI resource configuration, the resource indicator may need 9 bits.

The 9 bit value of the RSRI and the 7 bit absolute RSRP value result in 2 full octets. If the truncation is applied, both RSRP and RSRI need to be omitted. Thus the 9-bit value does not allow efficient truncation of MAC CE.

Observation 2: The 9-bit value of RSRI does not enable efficient truncation of the MAC CE i.e. both the RSRP value and RSRI value need to be truncated.

Instead of 9-bit RSRI, a 6-bit RSRI could easily be used by using the  RSRI index space per candidate cell,. The candidate are cells for which the RS listed can be indicated by a candidate cell bitmap. The 6-bit RSRI fits to one octet with additional R bits available.

Proposal 1: Support MR MAC CE format that includes RSRI field of 6 bits (to reflect up to 64 RS indices per LTM candidate cell).

If lower number of resource indicators (i.e. lower than 6 bits) is needed for reporting for a candidate cell, the RS index field can only encode resource indicators on required bit positions while maintaining the size allocated per RS index.

To differentiate reported RSs of different candidate cells, the RS index field encodes the resource indicator field that points to the reference signals, in the LTM CSI-ResourceConfig per PCI. The resource indicator ID space (of 64) is reused for each candidate cell in the CSI-resource config. The detailed structure is shown in Figure 2.

Observation 3: The resource indicator ID space can be reused for each candidate cell in the LTM-CSI-configuration associated with reporting event.

Proposal 2: The LTM Report MAC CE includes Resource Indicator field that encodes indicator pointing to the reference signals of the candidate cell in the LTM-CSI-ResourceConfig associated with the reporting configuration.

Proposal 3:  The resource indicator SSBRI/CRI fields are associated with indicated candidate identifier.

Figure 2. Example MAC CE format with 6 bit RSRI field.

## 2.2 	Event Evaluation at MAC Layer

The RAN plenary has revised the Mobility enhancement WI objectives [1] and it is not required anymore to manage potential overlaps between the MIMO WI and Mobility WI.

To prevent too frequent event triggered reporting, a mechanism to perform at least time domain filtering would be needed. It has been agreed that for LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific offset can be applied [4].

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

For the LTM event evaluation, the TTT concept, similar to L3 reporting, was adopted. In the TTT approach, the UE is configured to observe whether the evaluated criteria (e.g. LTM3 event) is fulfilled for the duration of the TTT. In a certain case, the TTT duration could be configured to be ‘0’ which means that if the event criteria is fulfilled, the event and the associated reporting is triggered immediately. As the MAC layer is used for event evaluation, the TTT related procedure (start /stop/expiry) is performed at MAC layer where the event evaluation procedure is defined.

In the agreed operation, depicted in Figure 3, the L1 is providing the measurement results to MAC layer (e.g. on RS1… RSk) that includes the candidate RS measurements and serving cell RS measurements, depending on the LTM event type. MAC layer determines based on the configured event criteria when the event criteria is fulfilled and starts the TTT timer. MAC layer also observes that whether the criteria is fulfilled for the duration of the TTT. Once the TTT expires, the event triggered report is generated, reporting the RS or RSs fulfilling the criteria.

Figure 3. Event evaluation by MAC layer processing the L1 measurements.

Modelling of the TTT timer handling may have the following options:

- either the TTT timer start/stop/expiry is modelled explicitly or

- only the reporting procedure is modelled as result of the evaluation.

Observation 4: Modelling of the TTT timer handling may have the following options: either the TTT timer start/stop/expiry is modelled explicitly or only the reporting procedure is modelled as result of the evaluation.

Proposal 4: Model the TTT timer handling explicitly, i.e. TTT timer start/stop/expiry is modelled at MAC.

Proposal 5: The value of TTT is configurable by NW and one TTT value is configured per LTM reporting configuration.

Proposal 6: The TTT range is to be decided in the later phase of Stage-3 discussions.

## 2.3 	Report on Leave

The measurement framework for NR has numerous components which were defined over several releases, starting from NR Rel-15. One such configurable aspect is reportOnLeave. It is a flag indicating if the UE should report also when one of the cells that previously met a reporting criteria has now left the cellsTriggeredList. A similar mechanism was agreed to be defined for LTM L1 measurements events where the report on leave could report on beam level that the reporting criteria is not fulfilled anymore:

MR can be sent when the leaving condition is met, based on NW configuration.

Nevertheless, RAN2 needs to discuss how report on leave in MAC layer should look like. Among the questions to be answered one can list at least the following:

- Report on leave is RS/beam-specific or is candidate cell also considered?

- Does report on leave use the same MAC CE format as the report when entry condition is met?

- Is the report on leave type of event indicated explicitly or implicitly?

- Is the leaving condition an equivalent of the entry condition or can it be configured separately and in a different manner?

Proposal 7: RAN2 considers at least the following list of open issues when designing the report on leave for L1 measurement reporting:

- Report on leave is RS/beam-specific or is candidate cell also considered?

- Does report on leave use the same MAC CE format as the report when entry condition is met?

- Is the report on leave type of event indicated explicitly or implicitly?

- Is the leaving condition an equivalent of the entry condition or can it be configured separately and in a different manner?

## 2.4 Periodic Transmission of the MAC CE Report

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

RAN2 agreed in one of the previous meetings to support the periodic event triggered reporting for LTM. At least the periodicity i.e. one-time report/periodic and the number of repetitions should be the configurable parameters. The configuration should be per event triggered reporting configuration.

Proposal 8: The periodic reporting of the LTM event triggered report can be configured per reporting configuration with configurable periodicity and/or number of repetitions.

One further discussion point for periodic reporting is the resource allocation. RAN2 should discuss how to support periodic resource allocation for the report. One option is to provide periodic resources when the UE sends an SR. In such cases, the details of UL grant resource allocation via PDCCH can be left to RAN1. Also, if the network does not configure periodic resource allocation, the UE needs to request the resources separately for each reporting. The measurement report should be able to be provisioned on any available UL grant.

Proposal 9: RAN2 agree to support the configuration of periodic resource allocation for event triggered periodic MR MAC CE.

## 2.5 Prohibit Timer For triggering the MAC CE Report

To avoid too frequent measurement reporting by the UE, a prohibit timer could be applied for the event triggered reporting. The prohibit timer could be applied per reporting configuration i.e. the UE would not be allowed to trigger another MR MAC CE for the same reporting configuration while the associated prohibit timer is running.

Observation 5: A prohibit timer could be applied for the LTM event triggered reporting to avoid too frequent measurement reporting.

Proposal 10: RAN2 to support a prohibit timer for event triggered reporting. The prohibit timer is applied per reporting configuration.

## 2.6 Number of reported beams in MR MAC CE

The following has been agreed so far regarding the number of beams and what kind of beams could be reported:

7. For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

The definition of Rel-18 LTM report content in TS 38.331 is as follows:

LTM-ReportContent-r18 ::=     SEQUENCE {

nrOfReportedCells-r18                          ENUMERATED {n1,n2,n3,n4},

nrOfReportedRS-PerCell-r18                     ENUMERATED {n1,n2,n3,n4},

spCellInclusion-r18                            ENUMERATED {true}                                          OPTIONAL -- Need R

}

The maximum number of reported beams in R18 LTM was specified to be 16. We see no strong reason to deviate from this and thus the number of reported beams can be up to N=16 (4 cells x 4 beams per cell) and the value is network configurable.

Observation 6: There is no reason to deviate from the R18 reporting parameters, where reporting of up to 16 beams can configured (4 cells x 4 beams per cell)

Proposal 11: RAN2 assumes the maximum number for N as in R18, i.e, 16 (up to 4 cells, and up to 4 beams per cell). RAN2 can ask RAN1 to confirm.

In some case, the number of reported beams not fulfilling the criteria should be limited to prevent the UE from including information that may not be useful for the network. For example, when the maximum number of beams which can be reported is relatively large, but only a few beams fulfil the reporting criteria, it is not beneficial to always include up to N beams in the MR MAC CE. This could unnecessarily  consume UL resource that could otherwise be available for other information.

Also, when truncating the MR MAC CE, any beam not fulfilling the reporting criteria should be omitted.

Observation 7: The number of reported candidate beams that do not fulfil the reporting criteria should be limited.

Proposal 12: A maximum limit should be set for the number of candidate beams in the MR MAC CE that do not fulfil the reporting criteria.

## 2.7 Beam/cell specific offset

Regarding the beam-specific offset to be used in LTM measurement event evaluation the only relevant RAN2 agreement made as of now was formulated as follows:

6. For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

This aspect was also briefly discussed in post-meeting e-mail discussion [109], devoted to MAC CR drafting. In the parallel thread “[POST128][108][MOB] RRC running CR” it was also proposed to capture the offset in NR RRC in the following way:

LTM-CSI-ReportConfig information element

-- ASN1START

-- TAG-LTM-CSI-REPORTCONFIG-START

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

...,

eventTriggered-r19                        SEQUENCE {

ltm-EventTriggeredMeasurement-r19             LTM-EventTriggeredResourceConfigId-r19,

eventId-r19                                     CHOICE {

eventLTM2-r19                                     SEQUENCE {

ltm2-Threshold-r19                                MeasTriggerQuantity,

hysteresis-r19                                    Hysteresis,

timeToTrigger-r19                                 TimeToTrigger

},

eventLTM3-r19                                     SEQUENCE {

ltm3-Offset-r19                                   MeasTriggerQuantityOffset,

ltm3-Threshold-r19                                MeasTriggerQuantity,

hysteresis-r19                                    Hysteresis,

timeToTrigger-r19                                 TimeToTrigger

},

eventLTM4-r19                                     SEQUENCE {

ltm4-Threshold-r19                                MeasTriggerQuantity,

hysteresis-r19                                    Hysteresis,

timeToTrigger-r19                                 TimeToTrigger

},

eventLTM5-r19                                     SEQUENCE {

ltm5-Threshold1-r19                               MeasTriggerQuantity,

ltm5-Threshold2-r19                               MeasTriggerQuantity,

hysteresis-r19                                   Hysteresis,

timeToTrigger-r19                                TimeToTrigger

},

...

},

…

}

#### - MeasTriggerQuantityOffset Offsets defined in measurement report configurations (e.g., events defined in ReportConfig-NR, or LTM-CSI-ReportConfig) for triggering measurement reports (e.g., by RRC message MeasurementReport or by MAC CE in TS 38.321 [3]). Values in the unit of 'dB'. -- ASN1START -- TAG-MEASTRIGGERQUANTITYOFFSET-START MeasTriggerQuantityOffset ::= CHOICE { rsrp INTEGER (-30..30), rsrq INTEGER (-30..30), sinr INTEGER (-30..30) } -- TAG-MEASTRIGGERQUANTITYOFFSET-STOP -- ASN1STOP

Based on that, for a report configuration, the offset is configured for each event and should be applied to all RSs/beams in the resource set associated with the report configuration. This could be a simple way to implement beam level offset. The other possible options include inserting the offset to CSI resource configuration or even linking it to a TCI state of the serving beam.

For simplicity we propose the following:

Proposal 13: Apply the event evaluation offset for LTM3 on LTM CSI resource configuration level. Each beam in the CSI resource configuration is applied with same offset value.

## 2.8 Event/Reporting Identifier

Basic information included in MR MAC CE:

- Triggered event information (e.g., ReportConfigID)

Since the event triggered reporting supports multiple event types, at least one field in MAC CE should indicate:

- the event for which the reporting is triggered and

- the reporting configuration

For example, a field in the MAC CE should point to the reporting configuration associated with the event and resource configuration for which the reporting is performed.

Proposal 14: The LTM Report MAC CE includes an identifier that identifies the LTM reporting configuration. The LTM reporting configuration is associated with the reporting event type (e.g. LTM3) and the measurement resource configuration for which the reporting is performed.

## 2.9 On the Differential RSRP

In the Rel-18 LTM L1 report, the RSRP is reported in differential format. In differential RSRP reporting format the first listed RSRP value is absolute (and consumes 7 bits). The other listed values are in differential format (consume 4 bits) relative to the absolute RSRP value.

The differential RSRP reporting has potential to bring overhead savings but requires design to allow that. If the RSRI is 9 bits and with 7 bit RSRP field the pair needs full two octets and leaves no room for reserved bits. If the differential RSRP is used, as shown in Figure 4, the RSRI+RSRP pair still reserves 2 octets and use of differential RSRP allows the octet including the RSRP value to have  R bits but no overhead reduction for the MAC CE size.

Observation 8: If the RSRI indicator is 9 bits, it does not allow overhead reduction for the MAC CE size using differential RSRP.

Figure 4. RSRI+diffRSRP occupying 2 octets.

In order to save in MAC CE size the RSRI value should be less than 9 bits. In our proposal (in section 2.1) the candidate ID is signaled separately from RSRI which could reduce the size of RSRI to 6 bits. That enables the efficient design based on the differential RSRP.

Figure 5. Two differential RSRP values in a single octet (when RSRI is trimmed to 6 bits).

Observation 9: If the RSRI indicator is less than 9 bits, it allows the octet having the RSRP field to have up to 2 RSRP differential RSRP values.

Proposal 15: Support MAC CE design that allows reporting of 2 differential RSRP values in the same octet.

Whether to use absolute RSRP values in each report or rely on differential RSRP values could be also specific to certain events.

## 2.10 Reporting Serving Cell Information

10. Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

It makes sense, especially when the LTMx event compares against the serving beam.

Proposal 16: Include serving cell RSRP measurement for events for which the current serving beam RS is used for event evaluation i.e. LTM 2/3/5.

In order to consider to inter-working with the beam management procedure, it might be beneficial if the UE includes RS associated with indicated (and activated TCI states) in the event based measurement report. Network can use this to delay LTM cell switch if another serving cell beam is strong, e.g. based on beam management procedure.

Proposal 17: UE can be configured to include RS associated with serving cell’s activated TCI states in the event based measurement reporting.

The MAC CE format could include indication whether the serving cell information is included (e.g. when the MAC CE is truncated).

## 2.11 Scheduling Request to indicate a reporting event

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12. NW can configure a dedicated SR configuration for MR MAC CE transmission.

If the SR ID is associated with LTM event triggered reporting, it should be possible to share the same SR ID with SR for other purposes. If the SR is not configured, the MAC CE reporting the event is provisioned by triggering CBRA. The dedicated SR should not be associated with specific event but shared between configured events. Further information on the reported event is provided by the MR MAC CE.

Proposal 18: An SR ID can be associated with event triggered reporting. The SR can be used to indicate a reporting event/request for UL resource for provision of the MAC CE.

Proposal 19: The dedicated SR ID is not associated with a particular event, further information on the event is provisioned by MR MAC CE.

Proposal 20: The SR ID for event triggered reporting can be shared with SR for other purposes.

Proposal 21: If SR is not configured, CBRA procedure (for BSR) is triggered for providing the MAC CE reporting the measurements.

## 2.12 Transmitting the MAC CE for LTM Reporting

RAN2 agreed MAC CE is used as a reporting container. It should be possible to provide the LTM report MAC CE on any UL grant.

Proposal 22: The LTM Report MAC CE can be multiplexed in any available UL grant.

Proposal 23: If the SR for indicating reporting event is configured and UE has available UL grant, the UE is not required to transmit the SR and it can multiplex the MAC CE on any available UL grant.

## 2.13 Measurement Quantity for reporting

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

Providing the RSRP value associated with reported RS identifier gives the network essential information on the quality of the RS that triggered the event. In the previous RAN1 meeting there was no conclusion to support L1-SINR measurement based on CSI-RS:

Conclusion There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells

Proposal 24: The LTM Report MAC CE includes RSRP value associated with each reported RS identifier.

Proposal 25: Reuse the RSRP values table for L1 RSRP in 38.133 the RSRP values in MR MAC CE.

# 3	Conclusion

This paper discussed RAN2 aspects of LTM L1 measurement framework which shall be enhanced as a part of Release 19.

The following observations have been made:

Observation 1: To support reporting of measurements on 512 candidate RS indices in the CSI resource configuration, the resource indicator may need 9 bits.

Observation 2: The 9-bit value of RSRI does not enable efficient truncation of the MAC CE i.e. both the RSRP value and RSRI value need to be truncated.

Observation 3: The resource indicator ID space can be reused for each candidate cell in the LTM-CSI-configuration associated with reporting event.

Observation 4: Modelling of the TTT timer handling may have the following options: either the TTT timer start/stop/expiry is modelled explicitly or only the reporting procedure is modelled as result of the evaluation.

Observation 5: A prohibit timer could be applied for the LTM event triggered reporting to avoid too frequent measurement reporting.

Observation 6: There is no reason to deviate from the R18 reporting parameters, where reporting of up to 16 beams can configured (4 cells x 4 beams per cell).

Observation 7: The number of reported candidate beams that do not fulfil the reporting criteria should be limited.

Observation 8: If the RSRI indicator is 9 bits, it does not allow overhead reduction for the MAC CE size using differential RSRP.

Observation 9: If the RSRI indicator is less than 9 bits, it allows the octet having the RSRP field to have up to 2 RSRP differential RSRP values.

And the following has been proposed:

Proposal 1: Support MR MAC CE format that includes RSRI field of 6 bits (to reflect up to 64 RS indices per LTM candidate cell).

Proposal 2: The LTM Report MAC CE includes Resource Indicator field that encodes indicator pointing to the reference signals of the candidate cell in the LTM-CSI-ResourceConfig associated with the reporting configuration.

Proposal 3:  The resource indicator SSBRI/CRI fields are associated with indicated candidate identifier.

Proposal 4: Model the TTT timer handling explicitly, i.e. TTT timer start/stop/expiry is modelled at MAC.

Proposal 5: The value of TTT is configurable by NW and one TTT value is configured per LTM reporting configuration.

Proposal 6: The TTT range is to be decided in the later phase of Stage-3 discussions.

Proposal 7: RAN2 considers at least the following list of open issues when designing the report on leave for L1 measurement reporting:

- Report on leave is RS/beam-specific or is candidate cell also considered?

- Does report on leave use the same MAC CE format as the report when entry condition is met?

- Is the report on leave type of event indicated explicitly or implicitly?

- Is the leaving condition an equivalent of the entry condition or can it be configured separately and in a different manner?

Proposal 8: The periodic reporting of the LTM event triggered report can be configured per reporting configuration with configurable periodicity and/or number of repetitions.

Proposal 9: RAN2 agree to support the configuration of periodic resource allocation for event triggered periodic MR MAC CE.

Proposal 10: RAN2 to support a prohibit timer for event triggered reporting. The prohibit timer is applied per reporting configuration.

Proposal 11: RAN2 assumes the maximum number for N as in R18, i.e, 16 (up to 4 cells, and up to 4 beams per cell). RAN2 can ask RAN1 to confirm.

Proposal 12: A maximum limit should be set for the number of candidate beams in the MR MAC CE that do not fulfil the reporting criteria.

Proposal 13: Apply the event evaluation offset for LTM3 on LTM CSI resource configuration level. Each beam in the CSI resource configuration is applied with same offset value.

Proposal 14: The LTM Report MAC CE includes an identifier that identifies the LTM reporting configuration. The LTM reporting configuration is associated with the reporting event type (e.g. LTM3) and the measurement resource configuration for which the reporting is performed.

Proposal 15: Support MAC CE design that allows reporting of 2 differential RSRP values in the same octet.

Proposal 16: Include serving cell RSRP measurement for events for which the current serving beam RS is used for event evaluation i.e. LTM 2/3/5.

Proposal 17: UE can be configured to include RS associated with serving cell’s activated TCI states in the event based measurement reporting.

Proposal 18: An SR ID can be associated with event triggered reporting. The SR can be used to indicate a reporting event/request for UL resource for provision of the MAC CE.

Proposal 19: The dedicated SR ID is not associated with a particular event, further information on the event is provisioned by MR MAC CE.

Proposal 20: The SR ID for event triggered reporting can be shared with SR for other purposes.

Proposal 21: If SR is not configured, CBRA procedure (for BSR) is triggered for providing the MAC CE reporting the measurements.

Proposal 22: The LTM Report MAC CE can be multiplexed in any available UL grant.

Proposal 23: If the SR for indicating reporting event is configured and UE has available UL grant, the UE is not required to transmit the SR and it can multiplex the MAC CE on any available UL grant.

Proposal 24: The LTM Report MAC CE includes RSRP value associated with each reported RS identifier.

Proposal 25: Reuse the RSRP values table for L1 RSRP in 38.133 the RSRP values in MR MAC CE.

# References

[1]	RP-242356 Revised Work Item: NR mobility enhancements Phase 4 3GPP TSG RAN Meeting #105, Sep 9-12, 2024

[2] R2-2403731 	Report from session on V2X/SL, R19 NES and MOB 3GPP TSG-RAN WG2 Meeting #125bis Changsha, China, April 15 – 19, 2024

[3] R2-2405701	Report from session on V2X/SL, R19 NES and MOB 3GPP TSG-RAN WG2 Meeting #126	 Fukuoka, Japan, May 20 – 24, 2024

[4] R2-2409211 Report from session on V2X/SL, R18/19 MOB, and R19 NES 3GPP TSG-RAN WG2 Meeting #127bis Hefei, China, October 14th – 18th, 2024

[5] R2-2410911 Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES 3GPP TSG-RAN WG2 Meeting #128	, Orlando, USA, November 18th – 22nd, 2024