3GPP TSG-RAN WG2 Meeting #129	R2-2500337

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.11.2

Source:	vivo

Title:	Discussion on random access procedure in SBFD

Document for:	Discussion and Decision

Introduction

In this contribution, we will further discuss the remaining issues on random access procedure in SBFD.

Discussion

SBFD RACH configuration

During last meeting, the following agreement was made [1]:

RACH configuration Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

Before that, RAN1 also agreed on the following principles from UE point of view [2]:

- Enabling both options at the same time for a UE is not supported;

- UE is not required to support both options;

Although RAN1 agreed that UE is not required to support both options, we think there is no harm for RAN2 to review this conclusion. Till this day, we think these two options only differ in the form of configuration, and it is really weird to say the SBFD-aware UE only supports one of these two options while not supporting the other option simply due to signaling difference. Therefore, we propose RAN2 to discuss how the SBFD UE support the RACH configuration options. The following options can be a start for discussion:

- Alt.1: Define a new UE capability to indicate which SBFD RACH configuration the UE supports;

- Alt.2: Since SBFD-aware UE is not required to support both SBFD RACH configuration options and the cell only supports one option, then only the UEs supporting the configured option in the cell can apply SBFD operation.

Proposal 1: RAN2 to discuss how the SBFD UE support the RACH configuration options:

- Alt.1: Define a new UE capability to indicate which SBFD RACH configuration the UE supports;

- Alt.2: Since SBFD-aware UE is not required to support both SBFD RACH configuration options and the cell only supports one option, then only the UEs supporting the configured option in the cell can apply SBFD operation.

RACH triggering events

During R2#127 meeting, RAN2 agreed on the following working assumption [3]:

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

The major comments from the companies include:

- No strong concern is observed to exclude any of the events;

- Since the main benefit of SBFD is to reduce latency, we may focus on the latency sensitive events and connected state.

RAN1 agreed during last meeting to extend the previous RAN1 agreements for UEs in RRC_CONNECTED mode to UEs in RRC_IDLE/INACTIVE mode. This implies not only the benefit from SBFD operation applies to both RRC_CONNECTED and RRC_IDLE/INACTIVE, but also we strive for a common design for these modes. Since we do not observe any strong concern to exclude any of the events, and it is anyway up to the network to determine whether to configure SBFD resources for certain purpose, we propose RAN2 to confirm the working assumption. Otherwise, we need to spend much time to case by case discuss which cases are allowed in SBFD while others are not.

Proposal 2: RAN2 to confirm the following working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Msg3-based early identification

During last meeting, RAN2 discussed with no consensus on Msg-3 based method. From our point of view, when the UE chooses additional RO for preamble transmission, the NW is able to identify SBFD UE and thus can schedule SBFD Msg3 resources so that the SBFD UE may have higher possibility to successfully complete the random access procedure. However, if the UE chooses legacy RO for preamble transmission, this may reflect that the current situation for the UE is not suitable for further using SBFD resources for the coming Msg3 transmission. Besides, the NW will anyway know the UE supports SBFD when the RRC establishment procedure is completed. Therefore, we do not observe the benefit for Msg-3 based early identification if the UE chooses legacy RO with either option which RAN2 will further determine as agreed during last meeting [1]:

CBRA FFS on the following options Option 1 - Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one.

Observation 1: If the SBFD-aware UE chooses legacy RO with the RO selection criteria which RAN2 will determine, it reflects the fact that SBFD-based transmission may not be a good choice from scheduling point of view within some upcoming duration. The NW will anyway know the UE supports SBFD when the RRC establishment procedure is completed.

Proposal 3: Do not support Msg3-based early identification for SBFD-aware UE.

Random access procedure

RO selection

RAN2 had agreed at last meeting that [1]:

CBRA FFS on the following options Option 1 - Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one.

From our perspective, regardless whether the CBRA is UE-initiated or NW-initiated RACH procedure, the NW cannot know whether the channel state for a certain UE is good or not. The UE has better knowledge of the channel state by means of RSRP or CLI measurement. Therefore, we think option 1 does not work well and thus prefer option 2.

Proposal 4: Adopt Option 2 for CBRA RO selection, i.e. UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs.

For option 2, we need to further discuss how the SSB RSRP threshold is configured and used. There are two main aspects the NW may consider whether to configure/schedule SBFD resources to SBFD-aware UE. One is configuring SBFD for coverage enhancement, and the other is to avoid severe UE-to-UE CLI.

Different configuration strategy determines how an RSRP threshold is used for RO selection in SBFD. For example, if the main concern of NW configuring SBFD is for coverage enhancement, the NW can configure the UE to select additional ROs when the RSRP threshold of the selected SSB/CSI-RS is below a configured RSRP threshold. Another example is that if the NW configures additional ROs only for larger RACH capacity and does not mean to have severe UE-to-UE CLI across different SBFD-capable cells, the NW can configure the UE to select additional ROs when the RSRP threshold of the selected SSB/CSI-RS is above a configured RSRP threshold. From the given examples, we can see how the UE prioritizes selection of additional ROs with the configured RSRP threshold depends on the target the NW wants to achieve.

Observation 2: How the UE selects SBFD RO with SSB RSRP condition should depend on the use case, e.g. for minimizing CLI across cells/UEs, or for coverage enhancement.

Therefore, we propose that the NW can configure two kinds of SSB RSRP thresholds for different usages, e.g. Threshold1 or Threshold2. When the NW configures additional RO for coverage enhancement, the NW configures Threshold1. The UE selects additional RO if the measured SSB RSRP is below Threshold1. When the NW configures additional RO for RACH capacity enhancement and aims to minimize inter-cell UE-to-UE CLI, the NW configures Threshold2. The UE selects additional RO if the measured SSB RSRP is above Threshold2. However, the NW should only configures either Threshold1 or Threshold2. If none of the thresholds is configured, the UE can prioritize to select additional RO.

Proposal 5: For Option 2, the NW configures either Threshold1 or Threshold2, and the UE:

- Selects additional RO when the measured SSB RSRP is above Threshold1, if Threshold1 is configured;

- Selects additional RO when the measured SSB RSRP is below Threshold2, if Threshold2 is configured;

- If none of the thresholds is configured, the UE can prioritize to select additional RO;

PRACH transmission re-attempt

During R2#127bis meeting, RAN2 agreed as follows on PRACH transmission re-attempt [4]:

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

RAN2 discussed at last meeting on the FFS part, but did not reach a consensus. We think if the UE fails in accessing the network with legacy ROs, it may imply that the load on legacy ROs is congested, or the UE is at the cell edge. From this understanding, it is beneficial for the SBFD UE to switch to additional SBFD ROs for further re-attempt. Thus, we propose the UE switch to additional SBFD ROs after a configured number of times of RACH attempt in legacy RACH occasions, if the UE selects legacy ROs first.

Proposal 6: If the UE selects legacy ROs first for a RACH procedure, it can switch to additional SBFD ROs after a configured number of times of RACH attempt in legacy ROs.

Carrier selection

A serving cell can be configured with supplementary uplink (SUL) for the purpose of coverage enhancement. In the Rel-18 WI Coverage Enhancement (CovEnh), it was discussed and determined that:

- Msg1 repetition can be configured on SUL;

- It is up to the NW whether to configure both NUL with Msg1/3 repetition and SUL for the purpose of CovEnh;

In SBFD, we propose that similar principle can also be agreeable.

Proposal 7: The NW can configure both NUL with SBFD operation and SUL for the purpose of coverage enhancement (no spec impact).

The next issue is whether SBFD operation has an impact on carrier selection for random access procedure. In legacy procedure, the UE directly determines the UL carrier if it is explicitly indicated by the NW. Otherwise, the UE compares the DL RSRP with the NW configured SUL threshold to determine it [5].

In SBFD, the NW can enable Msg1 repetition in SBFD resources on TDD-NUL according to RAN1 agreement [6].

Agreement For SBFD aware UEs, at least support the following: - PRACH transmission with preamble repetitions within additional-ROs (not across additional-Ros and legacy ROs) Note: SBFD aware UEs support PRACH transmission with preamble repetitions only within legacy-ROs (not across additional-ROs and legacy ROs) as in legacy releases.

The UE compares the DL RSRP with Msg1 repetition threshold to determine the number of repetition. Msg1 repetition on SBFD resources enhance the NUL coverage to some extent, which weakens the motivation for the UE to select SUL. Therefore, we propose RAN2 to discuss whether the NW configuring SBFD on NUL has an impact on NUL/SUL selection. For example, if the UE selects to perform SBFD operation on NUL, the UE can rely on Msg1 repetition with SBFD on NUL instead of choosing SUL.

Proposal 8: For NUL/SUL carrier selection when NUL is configured with SBFD, RAN2 to discuss whether an SBFD aware UE selects based on SBFD configuration or relies on legacy NUL/SUL carrier selection method.

SSB/CSI-RS selection

SSB/CSI-RS selection is a step in random access procedure. If not indicated by the NW, the UE compares the RSRP of the SSB/CSI-RS with the threshold and select an SSB/CSI-RS with SS-RSRP/CSI-RSRP above rsrp-ThresholdSSB/rsrp-ThresholdCSI-RS. The UE then determines RO based on the selected SSB/CSI-RS [5].

In SBFD, legacy ROs and additional ROs can be both mapped to the same SSB/CSI-RS. Considering the NW can evaluate the CLI in SBFD resources based on the UE reports and adjust the SBFD configuration accordingly, the NW may configure different threshold for the UE to select additional ROs or legacy ROs to minimize the CLI in SBFD resources. However, we may just rely on the legacy SSB/CSI-RS selection method. Therefore, we propose RAN2 to confirm that SBFD operation has no impact on SSB/CSI-RS selection for random access procedure.

Proposal 9: Legacy SSB/CSI-RS selection procedure is reused for random access procedure with SBFD operation.

Conclusion

Based on the discussion, we have the following observation and proposals:

SBFD RACH configuration

Proposal 1: RAN2 to discuss how the SBFD UE support the RACH configuration options:

- Alt.1: Define a new UE capability to indicate which SBFD RACH configuration the UE supports;

- Alt.2: Since SBFD-aware UE is not required to support both SBFD RACH configuration options and the cell only supports one option, then only the UEs supporting the configured option in the cell can apply SBFD operation.

RACH triggering events

Proposal 2: RAN2 to confirm the following working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Observation 1: If the SBFD-aware UE chooses legacy RO with the RO selection criteria which RAN2 will determine, it reflects the fact that SBFD-based transmission may not be a good choice from scheduling point of view within some upcoming duration. The NW will anyway know the UE supports SBFD when the RRC establishment procedure is completed.

Msg-3 based early identification

Proposal 3: Do not support Msg3-based early identification for SBFD-aware UE.

Random access procedure

RO selection

Proposal 4: Adopt Option 2 for CBRA RO selection, i.e. UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs.

Observation 2: How the UE selects SBFD RO with SSB RSRP condition should depend on the use case, e.g. for minimizing CLI across cells/UEs, or for coverage enhancement.

Proposal 5: For Option 2, the NW configures either Threshold1 or Threshold2, and the UE:

- Selects additional RO when the measured SSB RSRP is above Threshold1, if Threshold1 is configured;

- Selects additional RO when the measured SSB RSRP is below Threshold2, if Threshold2 is configured;

- If none of the thresholds is configured, the UE can prioritize to select additional RO;

PRACH transmission re-attempt

Proposal 6: If the UE selects legacy ROs first for a RACH procedure, it can switch to additional SBFD ROs after a configured number of times of RACH attempt in legacy ROs.

Carrier selection

Proposal 7: The NW can configure both NUL with SBFD operation and SUL for the purpose of coverage enhancement (no spec impact).

Proposal 8: For NUL/SUL carrier selection when NUL is configured with SBFD, RAN2 to discuss whether an SBFD aware UE selects based on SBFD configuration or relies on legacy NUL/SUL carrier selection method.

SSB/CSI-RS selection

Proposal 9: Legacy SSB/CSI-RS selection procedure is reused for random access procedure with SBFD operation.

References

- R2-2410912	Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD

- R1#117 meeting minutes

- R2-2407572	Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD

- R2-2409212	Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD

- TS 38.321, V18.4.0

- R1#118 meeting minutes