3GPP TSG-RAN WG2 Meeting #129	R2-2500338

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.11.3

Source:	vivo

Title:	SBFD other aspects

Document for:	Discussion and Decision

Introduction

According to R2#128 meeting, the following agreements were made [1]:

Prioritization of SBFD cells / frequencies during cell reselection is not considered. RAN2 wait for input from the other WGs regarding whether for inter-cell CSI-RS measurements, UE needs to be provided with information of the SBFD configuration of neighbouring cells.

In this contribution, we will discuss about SBFD other aspects.

Discussion

UE-specific configuration

During R2#127 meeting, RAN2 agreed that [2]:

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

From our understanding, it is possible that under certain circumstances the NW reserves some dedicated SBFD resources for temporary usage when the channel state of certain UE is unstable. In other words, not all the SBFD resources within a cell should be cell-specific. However, RAN1 has not progressed further whether to support UE-specific configuration and thus we propose to wait.

Proposal 1: Wait for RAN1 further input for discussion on UE-specific configuration.

CLI handling

We understand that CLI measurement and reporting is of certain importance for SBFD operation. For every past RAN2 meeting, CLI related proposals are raised by companies. Yet it is still hard to start any meaningful discussion in RAN2 now according to the RAN1 progress so far. We observe that RAN1 continues to make progress on RAN2 related issues step by step every meeting, e.g. [3].

Agreement For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of periodic CLI measurement resources, the TCI state(s) with QCL typeD for the CLI measurement resources in the periodic CLI measurement resource set are configured per CLI measurement resource (either for all resources in the set or none) by higher-layer parameter - For each periodic CLI measurement resource, this higher-layer parameter provides a reference to one TCI-State in TCI-States for providing the QCL source and QCL typeD. - If the TCI state is not configured, the UE QCL assumptions are the default rules agreed in RAN1#118bis For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of semi-persistent CLI measurement resources, the gNB may activate and deactivate the configured semi-persistent CLI measurement resource sets by sending a new SP CLI Measurement Resource Set Activation/Deactivation MAC CE - TCI state IDi field is included in the new MAC CE, which is used as typeD QCL source for the CLI measurement resource within the Semi Persistent CLI measurement resource set. Either all TCI state ID fields are present or none. - If the TCI state ID field(s) are absent, the UE QCL assumptions are the default rules agreed in RAN1#118bis Conclusion There is no RAN1 consensus to support aperiodic CLI reporting on PUCCH in Rel-19

Since there is not any CLI handling related topic that can be observed to make conclusion in RAN2, we propose RAN2 to wait for RAN1 LS if any for starting discussion on CLI handling.

Proposal 2: Wait for RAN1 LS if any on CLI (along with related parameter list) for discussion on CLI handling.

Conclusion

Based on the discussion, we have the following observation and proposals:

Proposal 1: Wait for RAN1 further input for discussion on UE-specific configuration.

Proposal 2: Wait for RAN1 LS if any on CLI (along with related parameter list) for discussion on CLI handling.

References

- R2-2410912	Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD

- R2-2407572	Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

- R1#119 meeting minutes