3GPP TSG-RAN WG2 Meeting #129	R2-2500341

Athens, Greece, 17th Feb 2025 – 21st Feb 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on on-demand SIB1 operation for NES

Agenda Item:	8.5.3

Document for:	Discussion and decision

# Introduction

In the RAN#105 meeting, the revised Work Item for network energy saving enhancement has been approved in RP-242354 to specify the Case 2 based procedure and signaling for on-demand SIB1 operation [1]:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In the last meeting, RAN2 continued the discussion on on-demand SIB1 operation and several agreements have been achieved [2]:

Agreements on OD-SIB1

1.  NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

2.	New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

3.	Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

4.	Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

5.	The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

6.	The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

7.	If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

8.	A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

9.	UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

10.	Working assumption: UL-WUS configuration in RRC release is not supported.

In the RAN#106 meeting, a way forward on NES was agreed [4]:

- RAN confirms that Case 2 also includes the case where a particular Cell A can switch to become a NES Cell.    This means that Cell A can provide UL-WUS configuration for itself before becoming a NES cell.

  - No further optimizations will be considered for NES UEs for this scenario.

  - No further optimizations to handle non-NES capable UEs will be considered for this scenario. It is assumed that these UEs can be handled by careful deployment.

  - No WID update is required

- On RRC Connected mode

  - The scope of WID will not be updated to include RRC Connected.

  - However, RAN2 will address the specific case of RLF, to use the same procedure as idle/inactive to request OD-SIB1 transmission. No need to change the scope of WI to indicate this.

In this contribution, we will discuss the remaining issues of on-demand SIB1 operation.

# Discussion

## 2.1 The RAN Plenary agreement understanding

In the RAN#106 discussion, it was clarified that case 2 also includes the case where a particular Cell A can switch to become a NES Cell. That means that Cell A can provide UL-WUS configuration for itself before becoming a NES Cell. For this case, Cell A provides UL-WUS configuration for itself and then stops broadcasting SIB1 becoming a NES cell.

It is worth recalling that during RAN2#127bis, it was agreed that:

NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

In our view, the note that it includes its own WUS configuration, already supports the scenario agreed at RAN#106. There is no mechanism to forbid broadcasting SIBX before Cell A becomes a NES cell, considering it also includes neighbouring cell’s WUS configs in the same SI message. The WUS config validity is controlled by the gNB together with the validity of SIBX, as in other OD-SIB1 scenarios.

Observation 1: Based on previous RAN2 agreements the RAN WF is already supported. No further optimizations are needed.

The second RAN WF refers to RRC_CONNECTED mode support for OD-SIB1 request. RAN2 should address the specific case of RLF, to use the same procedure as idle/inactive to request OD-SIB1 transmission in a NES Cell. Since a UE in RLF stays in Connected mode and the NW cannot predict when a RLF might happen, a clarification is needed that allows this UE to request for OD-SIB1 in this state. To perform the request the UE needs to know the WUS config of the current NES cell, assuming the acquisition of SIBX also in Connected mode.

Proposal 1: SIBX can be acquired in RRC_CONNECTED by the NES UE (by reading SIBX or NW providing it through dedicated signalling to the UE, e.g. if the UE has an active BWP with no common search space configured to monitor SI or paging).

Proposal 2: If a UE has a valid WUS config for a cell, it can request OD-SIB1 transmission in RRC_CONNECTED mode in the case of radio link failure (RLF).

## 2.2 How to identify the NES Cell and bar legacy UEs

In RAN1 and RAN2 discussions, the identification of NES Cell and cell barring of legacy UEs was discussed with the following agreements [3]:

Agreement@RAN1#118bis For further work of on-demand SIB1 in idle/inactive mode, it is assumed that: - SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster - UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A. Agreement@RAN1#119 On how to use PDCCH-ConfigSIB1 for R19 NES-capable UE in MIB of NES Cell when K_SSB = 30 in FR1 or K_SSB = 14 in FR2 on NES cell if SSB of NES cell is on the sync raster, down-select from the following options: - Option 1: Use it to indicate frequency assistance information to search SSB for Cell A - FFS: Details on the range/granularity of the frequency assistance information - FFS: Potential impact to RAN3 - Option 2: Use it to indicate searchSpaceZero and controlResourceSetZero of OD-SIB1 - Option 3: Above feature is not supported. Agreement @ RAN2#126 Legacy UE impact NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism) Agreement @ RAN2#127bis Cell barring Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

Whether the NES Cell can be identified by a UE when it performs its initial search on the NES Cell is being discussed in RAN1. The discussion is focused on the usage of PDCCH-ConfigSIB1. From RAN1’s agreement, the value of K_SSB =14 and K_SSB = 30 may be used in the NES case. But the real usage of PDCCH-ConfigSIB1 is left for RAN1’s discussion. RAN1 agrees that when NES cell’s SSB is transmitted on the sync raster, the value of K_SSB is restricted to a range value larger than 23 for FR1 and larger than 11 for FR2. Such SSB is called non cell defining SSB (NCD-SSB) in RAN2 specifications.

If the UE detects an SSB with K_ssb>= 24 in FR1 or 12 in FR2, it cannot determine whether it is a NES cell or a legacy cell. Therefore, the UE cannot confirm if the detected cell is indeed a NES cell. To address this, the RAN1 agreement mentioned above requires the UE to have the WUS config to determine if a cell is a NES cell or not. RAN2 also agreed that when a Cell’s SSB contains K_SSB < 24 for FR1 and K_SSB < 12 for FR2, the UE will acquire SIB1 as in legacy. RAN2 will not change the meaning of CD-SSB and the usage of PDCCH-ConfigSIB1.

For 23 < K_SSB < 30 for FR1 and 11 < K_SSB < 14 for FR2, the SSB is NCD-SSB. UE could use the PDCCH-ConfigSIB1 information to find the CD-SSB frequency position, which means that UE could find the CD-SSB on another frequency.

For K_SSB = 31 for FR1 and K_SSB = 15 for FR2, the SSB is NCD-SSB. The PDCCH-ConfigSIB1 provides the information of a frequency range where UE could not find CD-SSB.

For K_SSB = 30 for FR1 and K_SSB = 14 for FR2, the values of PDCCH-ConfigSIB1 are assumed reserved with no usage by the UE. RAN1 is discussing how to use them to extend the pointing capability towards Cell A, to indicate search space 0 and CORESET0, or keep them as reserved.

In TS 38.300, the PCell is always associated to a CD-SSB located on the sync raster. Since NES Cell is a PCell for R19 idle/inactive UEs, determining if the cell is a NES Cell by PBCH is unlikely to be supported based on RAN1’s discussion. If the solution from RAN1 is feasible, we can follow the solution and change the corresponding UE behaviour in RAN2.

Another option would be to send NES Cell’s SSB off sync raster. In this case, legacy UEs will not detect the NES cell and will not camp on it since the SSB is off sync raster, resulting that these legacy UEs are barred from the NES cell. Similarly, Rel-19 NES UEs will also not be able to detect NES cell and will not camp on it during their initial cell search procedure. However, once a cell A is detected, R19 UE will have the possibility of acquiring the WUS configuration. After that, they can identify the NES cell, and be aware of its existence, and its availability for camping. Hence, a R19 UE can camp on it, and/or access it, if suitable. In this case, the NES Cell can be functionally the PCell for R19 UEs.

Observation 2: From RAN2 perspective sending NES UEs off sync raster can achieve legacy UE barring. The legacy UEs will not see the cell at all, contrary to barring based on K_SSB.

In the previous meeting, RAN2 also agreed that a UE needs to check the SIB1 broadcast status before triggering OD-SIB1 transmission.

There are three options to realize it:

- Option 0: It was already agreed that a NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

- Option 1: UE could try to acquire the SIB1 at the corresponding time window of OD-SIB1 broadcasting.

- Option 2: UE could receive some information provided by the NES Cell to indicate whether the SIB1 is broadcasted or not.

For option 0, the K_SSB value is in the range of CD-SSB. UE will think the SIB1 is being broadcast periodically as in legacy.

For option 1, UE needs to know the time information of SIB1 broadcasting. Since RAN1 has agreed that SIB1 is triggered to be broadcasted in a time window, whose starting time is calculated based on the reference time of receiving RAR and offset to the reference time, it is difficult for other UE which did not send WUS to know the receiving time of RAR. In this option the UE would know the time window from the WUS config, but would not know the window starting time.

For option 2, before sending WUS to trigger OD-SIB1 transmission, a UE could only know the information provided by SSB in the NES Cell. However, it is undesirable to use the spare bit in MIB to represent the broadcast status of OD-SIB1. Whether other information could be used to indicate the broadcast could be discussed after RAN1 has determined the design of K_SSB.

Proposal 3: Wait for RAN1’s further process on the SSB transmission in NES Cell whether NES cell state can be derived from PBCH, how to bar legacy UEs, and whether SIB1 broadcast status can be derived from PBCH during the window of OD-SIB1.

## 2.3 The transmission behavior of OD-SIB1

RAN1 has discussed the contents of RAR in response to UL WUS and achieved the following agreements:

Agreement@RAN1#118bis RAN1 to discuss the contents of RAR in response to UL WUS using the legacy RAR for OSI as a starting point Agreement@RAN1#119 At least the contents of RAR in response to SI request are included in RAR in response to UL WUS The duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration. - Unit of the duration is in slot - FFS: Starting time

As highlighted above, RAN1 considers the legacy RAR for OSI as a starting point. The duration of the time window for on-demand SIB1 was decided to be indicated in the UL WUS configuration. If the NW does not intend to send OD-SIB1 in a specific timeframe, it should be also possible to indicate that SIB1 will not be broadcasted, so that the UE will not repeat the OD-SIB1 request. For that purpose, the RAR should include status indication of SIB1 transmission. If the NES cell receives the UE request and decides to respond negatively, i.e., not ready to send OD-SIB1 at least not in the near future shown in Figure 2, then the UE will benefit from this response to stop its WUS request procedure (power ramp up with retransmissions) creating less interference in the uplink and saving the UE power. If the cell response is positive, i.e., the OD-SIB1 provision will start as shown in Figure 1, then the UE could start monitoring the type 0 PDCCH monitoring occasions during the time window indicated in the WUS config.

Proposal 4: The RAR responding to OD-SIB1 request should contain OD-SIB1 broadcast status.

Figure 1: Illustration of a WUS response with sib1-BroadcastStatus = broadcast, meaning that SIB1 will be send during a time window for type 0 PDCCH monitoring occasions.

Figure 2: Illustration of a WUS response with sib1-BroadcastStatus = not broadcast and no OD-SIB1 reception nor WUS request (re)transmission.

## 2.4 Details of the UL WUS configuration

RAN1 has discussed UL WUS configuration contents during RAN1#118bis and RAN1#119 and reached the following agreements.

Agreement

For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration.

-	rsrp-ThresholdSSB

-	prach-RootSequenceIndex

-	msg1-SubcarrierSpacing

-	restrictedSetConfig

Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon

The parameter “n-TimingAdvanceOffset” is included in the UL-WUS configuration.

Confirm the working assumption below from RAN1 #118-bis to include it in the UL-WUS configuration.

-	(working assumption) ULSubCarrierSpacing

At least for RO validation determination for WUS transmission, the parameter “ssb-PeriodicityServingCell” is included in the UL-WUS configuration at least for TDD system.

-	FFS: FDD system

| Purpose | Parameters |  |
| --- | --- | --- |
| To which cell does the configuration applies | NES-CellId | PhysCellId |
|  |  | ARFCN-ValueNR |

Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell.

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| WUS transmission | frequencyBandList |  |  |
|  | absoluteFrequencyPointA |  |  |
|  | offsetToCarrier |  |  |
|  | p-Max |  |  |
|  | ULSubCarrierSpacing |  |  |
|  | n-TimingAdvanceOffset |  |  |
|  | ssb-PeriodicityServingCell |  |  |
| Purpose | Parameters |  |  |
| WUS transmission | SIB1-RequestConfig | ss-PBCH-BlockPower |  |
|  |  | SSB-positionInBurst |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResources | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |

RAN1 has agreed to include the above parameters in the WUS configuration. The CORESET#0 and the SS#0 can be included in the WUS configuration when SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2. Furthermore, whether the above parameters are mandatory or optional is also under RAN1’s discussion. RAN2 can consider this as the baseline and wait for more progress.

Proposal 5: Take RAN1’s agreement on UL WUS configuration contents as baseline and wait for more progress.

Due to the fact that the WUS config is provided by Cell A, if the Cell A bars a specific UE type (e.g., NTN or RedCap) from accessing or camping on it, that UE type would not be able to obtain the WUS config for neighbouring NES cells. In the current spec, if a UE bars the Cell, there is no need for it to acquire its OSI. Since these system information blocks are useless for a UE that is barred. But in the OD-SIB1 case, the WUS config in new SIBX provided by Cell A is useful for other NES Cells which might not be barred for this UE type. Therefore, we propose that NES UEs could acquire Cell A’s SIB1 to know whether Cell A is an anchor cell or has the broadcast information (i.e., the SchedulingInfo) of the new SIBX with WUS configs. If yes, the NES UE could acquire Cell A’s new SIBX to be able to trigger OD-SIB1 in neighbouring NES Cells.

Proposal 6: Once a NES UE acquires Cell A’s SIB1 and knows that it provides the new SIBX, it can acquire Cell A’s SIBX to obtain WUS configurations of neighbouring cells, even if the UE type is barred in SIB1.

Another issue that has not been discussed before is whether to transmit SIB1 barring bits to the UE before it requests OD-SIB1. In previous releases, several new barring bits were added in SIB1 for access control of new UE types, such as RedCap, NTN, R18 NES, etc. It would be beneficial to include those bits in the WUS config so that relevant UEs can know in advance whether they are allowed on an OD-SIB1 cell or not. If these bits are not provided beforehand, the UE would only find out if it is barred after acquiring OD-SIB1, which would waste UE energy and cause a delay in network access because the UE would need to go back to Cell A or reselect to another cell if it is barred.

Proposal 7: Feature specific barring bits from SIB1 are included in the WUS config.

## 2.5 WUS configuration validity

In the last meeting, there was an FFS on SI validity based on area rather than cell level. The question raised was whether the WUS config could be considered as area common information. In the previous RAN2 meeting, it was agreed that the UE should only consider the WUS config valid in a cell directly succeeding reselection from a cell where the UE acquired the given WUS configuration. Additionally, in the previous meetings it was agreed that the WUS config of the neighbouring cells can be broadcasted by multiple serving cells. The question now is whether the SIBX and/or WUS config can be area specific. Based on the previous agreement, it is evident that a single WUS config provided to multiple serving cells is the same. If the same WUS config can be reused when the UE moves back to a serving cell or moves from one serving cell to another, the UE will not need to acquire the broadcasted WUS config or trigger the serving cell to broadcast the WUS config, thus saving energy.

In the current specification, the area specific system information is already supported with the field systemInformationAreaID. The system information area ID is located at the top of all system information, indicating that all system information belongs to a single area. Whether a specific system information block belongs to the area is marked by the araeScope field.

Observation 3: The systemInformationAreaID is common for all SI in a given cell.

Cited from 38.331 v18.4.0:

SI-SchedulingInfo ::= SEQUENCE { schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo, si-WindowLength ENUMERATED {s5, s10, s20, s40, s80, s160, s320, s640, s1280, s2560-v1710, s5120-v1710 }, si-RequestConfig SI-RequestConfig OPTIONAL, -- Cond MSG-1 si-RequestConfigSUL SI-RequestConfig OPTIONAL, -- Cond SUL-MSG-1 systemInformationAreaID BIT STRING (SIZE (24)) OPTIONAL, -- Need R ... } SIB-TypeInfo ::= SEQUENCE { type ENUMERATED {sibType2, sibType3, sibType4, sibType5, sibType6, sibType7, sibType8, sibType9, sibType10-v1610, sibType11-v1610, sibType12-v1610, sibType13-v1610, sibType14-v1610, spare3, spare2, spare1,... }, valueTag INTEGER (0..31) OPTIONAL, -- Cond SIB-TYPE areaScope ENUMERATED {true} OPTIONAL -- Need S }

In the OD-SIB1 WUS config case, there may be multiple WUS configurations in the new SIBX. The legacy mechanism of areaScope and valueTag fields can only represent the whole content of the new SIB, not the specific WUS config. In order to support per WUS config validity, a WUS config specific valueTag and areaScope would be needed. These parameters can be generated by the NES Cell. Cell A can broadcast the WUS config identification and WUS specific valueTag and areaScope in its SIB1. This would allow the UE to check the specific WUS validity information and determine whether the stored WUS config is valid. This solution has significant overhead though, as both SIBX and SIB1 would need to carry this information for each WUS config. The other drawback of this scenario is that even if only one WUS config is not valid, the UE needs to obtain the whole SIBX.

Observation 4: The SI valueTag and areaScope are defined per SI type.

Given the current design of SI validity RAN2 needs to decide whether valueTag and areaScope is defined for SIBX and whether more granularity (i.e., per WUS config) is needed.

We can identify the following options:

- Option 1: Do not support valueTag and areaScope for SIBX, i.e. SIBX is per cell.

- Option 2: Support valueTag and areaScope for SIBX on SI level (as legacy). Since the SIBX will most probably be different in different cells (contain different WUS configs), this option has limited applicability.

- Option 3: Support valueTag and areaScope per WUS config. This is the most complex solution having significant overhead in SIBX and SIB1. In this scenario even if only one WUS config is not valid, the UE needs to obtain the whole SIBX.

Proposal 8: RAN2 to discuss whether valueTag and areaScope are defined for SIBX and with what granularity.

# Conclusion

In this contribution, we discussed the issues on on-demand SIB1 operation, and made the following observations and proposals:

Observation 1: Based on previous RAN2 agreements the RAN WF is already supported. No further optimizations are needed.

Proposal 1: SIBX can be acquired in RRC_CONNECTED by the NES UE (by reading SIBX or NW providing it through dedicated signalling to the UE, e.g. if the UE has an active BWP with no common search space configured to monitor SI or paging).

Proposal 2: If a UE has a valid WUS config for a cell, it can request OD-SIB1 transmission in RRC_CONNECTED mode in the case of radio link failure (RLF).

Observation 2: From RAN2 perspective sending NES UEs off sync raster can achieve legacy UE barring. The legacy UEs will not see the cell at all, contrary to barring based on K_SSB.

Proposal 3: Wait for RAN1’s further process on the SSB transmission in NES Cell whether NES cell state can be derived from PBCH, how to bar legacy UEs, and whether SIB1 broadcast status can be derived from PBCH during the window of OD-SIB1.

Proposal 4: The RAR responding to OD-SIB1 request should contain OD-SIB1 broadcast status.

Proposal 5: Take RAN1’s agreement on UL WUS configuration contents as baseline and wait for more progress.

Proposal 6: Once a NES UE acquires Cell A’s SIB1 and knows that it provides the new SIBX, it can acquire Cell A’s SIBX to obtain WUS configurations of neighbouring cells, even if the UE type is barred in SIB1.

Proposal 7: Feature specific barring bits from SIB1 are included in the WUS config.

Observation 3: The systemInformationAreaID is common for all SI in a given cell.

Observation 4: The SI valueTag and areaScope are defined per SI type.

Proposal 8: RAN2 to discuss whether valueTag and areaScope are defined for SIBX and with what granularity.

# References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

- R2-2410911 Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES

- Chair notes RAN1#119 eom0.doc

- PR-243297 Way forward on NES, RAN2 chair (InterDigital)