3GPP TSG-RAN WG2 Meeting #129		R2-R2-2500343

Athens, Greece, 17th – 21st Feb. 2025

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_IDLE/INACTIVE

Agenda Item:	8.4.2

Document for:	Discussion and Decision

- Introduction

In previous meetings, some agreements for LP-WUS/WUR in RRC_IDLE/INACTIVE were made [1] [2] [3] [4] [5] as below.

RAN2#125bis: - The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups. RAN2#126: - The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - LP-SS configuration - LP-WUS configuration - Entry/exit condition for LP-WUS monitoring (FFS if it always configured) RAN2#127: - Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. RAN2#127bis: - Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. - If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). - For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion. - For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping. RAN2#128: - FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session. - FFS on the following options - - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping. - - Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed. - - Option 3: The formula for UE_ID based PEI subgrouping is reused.

In this contribution, we will further discuss the remaining issues for RRC_IDLE/ INACTIVE procedures with LP-WUR.

- Discussion

  - LP-WUS configuration

In RAN2#126 meeting, it was agreed that the entry/exit condition for LP-WUS monitoring is included in LP-WUS configuration. However, it is still FFS whether the entry/exit condition for LP-WUS monitoring is always configured in SIB. Besides, RAN1 concluded that it is up to RAN2 discussion [6].

It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

Some companies thought if the MR and LR have the same coverage, the entry/exit condition for LP-WUS monitoring seems not always need to be configured in RAN2#125bis meeting.

According to the WID [7], the target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for Msg3. However, it depends on the real deployment on whether the coverage of LP-WUS and LP-SS is full coverage or partial coverage. For example, if the size of a cell is small, the coverage of LP-WUS and LP-SS can be full coverage.  On the contrary, if the size of a cell is large, the coverage of LP-WUS and LP-SS can be partial coverage. In our understanding, the entry/exit condition should be mandatorily configured to make UE correctly check whether the entry/exit condition for LP-WUS monitoring is met or not.

Some companies may argue it should be optional to leave the flexibility to network configuration. But our understanding is network could either provide a very small value in case the network always allows UE to perform LP-WUS monitoring or provide the same value as the threshold of condition for RRM relaxation/offloading, which provides enough flexibility for network configuration.

Proposal 1: The entry/exit condition for LP-WUS monitoring is mandatorily configured if the network is intended to support the LP-WUS feature.

In RAN2#125bis and RAN2#126 meeting, it was agreed that the LP-WUS related configuration includes LP-SS configuration, LP-WUS configuration, and the entry/exit condition for LP-WUS monitoring. Besides, in RAN2#127 meeting, RAN2 agreed that separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. If the network supports the OOK-based WUR, the LP-SS configuration should be configured in the LP-WUS related configuration. Otherwise, after UE with OOK-based WUR enters LP-WUS monitoring, OOK-based WUR cannot receive LP-WUS without the synchronization from LP-SS.

Proposal 2: If the network supports the OOK-based WUR, the LP-SS configuration should be always provided.

  - Entry/exit condition of using LP-WUS for OFDM-based LR

In RAN2#127 meeting, RAN2 agreed that separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Besides, RAN1 agreed that overlaid OFDM sequence(s) is supported for LP-SS as below in RAN1#118bis meeting [8].

Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences.

Then, the entry/exit thresholds of using LP-WUS for OFDM-based WUR monitoring the overlaid OFDM sequence(s) needs to be discussed and decided, i.e., the entry/exit thresholds could be the same as either the entry/exit thresholds for OFDM-based WUR or entry/exit thresholds for OOK-based WUR.

In our understanding, RAN1 agreed that the design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. Thus, the entry/exit thresholds and requirements for OFDM-based WUR monitoring the overlaid OFDM sequence(s) should be the same as the entry/exit thresholds and requirements for OOK-based WUR. Considering the corresponding requirements should be decided in RAN4, while RAN2 needs to determine the entry/exit threshold of using LP-WUS.

Proposal 3: The entry/exit thresholds of using LP-WUS for OFDM-based WUR monitoring the overlaid OFDM sequence(s) are the same as that for OOK-based WUR.

  - Other wake up conditions

As legacy PEI design, the MR in ultra-deep sleep/monitoring LP-WUS should also wake up in case there is UL data/signalling arrival.

Proposal 4: MR should wake up for UE-initiated UL transmission, e.g. RACH/MO-SDT, due to UL data and signaling arrival when monitoring LP-WUS.

  - LP-WUS Subgrouping

Network indication to indicate whether UE to monitor both LP-WUS and PEI

Regarding the subgrouping method for LP-WUS, RAN2 agreed that the PEI subgrouping method is taken as baseline, i.e. CN assigned and UE_ID based subgrouping. In the following part, we will analyse the specific detail for subgrouping methods.

The first issue is whether a network indication should be introduced to indicate the UE to monitor LP-WUS and PEI or not.

At the network side, PEI could be configured in the case that the coverage of LP-SS/LP-WUS is the partial coverage. According to legacy design, if there is PEI configuration, all UE supporting PEI could monitor PEI, which means that network needs to always provide PEI if it indicates supporting PEI. Similarly, if network supports LP-WUS and provides the LP-WUS configuration, all UE supporting LP-WUS could monitor LP-WUS, which means that network needs to always provide LP-WUS if it indicates support LP-WUS.

At the UE side, there is a case that PEI and LP-WUS are applicable at one UE when the entry condition of LP-WUS monitoring is fulfilled. In RAN1#116 meeting [9], RAN1 made the following agreement:

Agreement For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

It could be observed that, when UE supports PEI and PEI is configured by the gNB, it is up to UE implementation whether to monitor PEI or not after the UE receives LP-WUS indicating wake-up.

Observation 1: According to the RAN1 agreement, when UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

Based on the above design at network side, regardless of whether the UE monitors PEI or not after receiving LP-WUS indicating wake-up, the network would provide PEI and LP-WUS in case it provides the PEI and LP-WUS configuration. Thus, there is no additional effort for network to provide both PEI and LP-WUS. In this way, there is no need for network to provide an indication to indicate the UE to monitor both PEI and LP-WUS or not.

Proposal 5: No need to introduce a network indication to indicate UE to monitor both LP-WUS and PEI or not.

Subgrouping design for LP-WUS

With the above analysis, the UE may monitor PEI and LP-WUS together. But the network does not know whether the UE monitors LP-WUS and PEI or one of them. Thus, in case LP-WUS and PEI are used together for one UE, it is expected that UE could achieve more power saving gain via two levels of subgrouping. According to the following Figure 1 and Figure 2, assuming there are 1024 UEs supporting both LP-WUS and PEI subgrouping sharing the same PO and the subgrouping number of LP-WUS and PEI are both 4, in case UE_ID#0 is paged:

- In Figure 1, LP-WUS subgrouping and PEI subgrouping are totally same, 256 UEs will be woken up by LP-WUS to monitor PEI, and all of them will be indicated by PEI to monitor the following PO.

- In Figure 2, LP-WUS subgrouping and PEI subgrouping are orthogonal, 256 UEs will be woken up by LP-WUS to monitor PEI, while only 64 UEs will be indicated by PEI to monitor the corresponding PO.

Figure 1: LP-WUS and PEI subgrouping are same

Figure 2: LP-WUS and PEI subgrouping are diverse/ orthogonal

It could be observed that if the subgroup ID of LP-WUS and PEI are the same, there is no benefit for PEI monitoring, instead it leads to more power consumption as UE has to monitor PEI after LP-WUS. However, if the subgroup ID of LP-WUS and PEI are independent, the false alarm rate could be further reduced with PEI.

Observation 2: False alarm rate could be further reduced if the subgrouping ID of LP-WUS and PEI are independent.

Proposal 6: In order to allow UE to use LP-WUS together with PEI to reduce paging false alarm rate, it is expected for one UE independent subgroup ID is applied for LP-WUS and PEI.

To achieve additional power saving gain as analyzed above and considering UE_ID based subgrouping and CN assigned subgrouping are applied to both PEI and LP-WUS, the following scenarios could be considered to achieve more power saving gain:

- Case#1: For CN assigned subgrouping, it is expected that the CN could assign different subgrouping ID for LP-WUS and PEI for the same UE as much as possible. However, it is up to the SA2/CT1 design and up to network implementation to assign same or different subgrouping ID for the same UE.

- Case#2: For UE_ID based subgrouping, a different formula could be used as UE_ID for LP-WUS and PEI subgrouping.

- Case#3: For case CN assigned subgrouping is used for one feature (i.e. LP-WUS or PEI), while UE_ID based subgrouping is used for another feature, considering different subgrouping methods are used, it is expected that independent subgrouping ID is assigned/calculated for LP-WUS and PEI naturally. It could be achieved by the restriction that the number of CN assigned subgrouping is the same within the registration area, which is up to network implementation.

Thus, RAN2 should strive for the Case#2. RAN2 agreed that the similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping. The formula for PEI is as follows.

SubgroupID = (floor (UE_ID/(N*Ns)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID), where: - N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state as specified in clause 7.1 - Ns: number of paging occasions for a PF - UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192 - subgroupsNumForUEID: number of subgroups for UE_ID based subgrouping in a PO, which is broadcasted in system information

Besides, the following FFS is made in RAN2#128 meeting.

FFS on the following options - - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping. - - Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed. - - Option 3: The formula for UE_ID based PEI subgrouping is reused.

As the analysis and Figure 1 above, the Option 3 will lead to dependent subgrouping ID for PEI and LP-WUS for a UE, and UEs waking up by LP-WUS may always wake up in one subgroup for PEI. Then, UE could not obtain additional power saving gain.

Option 1 is to use the actual subgroup number for UE_ID based PEI subgrouping, while Option 2 is to use a fixed number, i.e. the maximum number of subgroup for PEI. In our understanding, there is no difference between these two options. And either option is fine for us. Following the same logic as PEI, we prefer the actual subgroup number for PEI, i.e. Option 1.

Proposal 7: For UE_ID based subgrouping, the formula is defined as below for LP-WUS subgrouping (i.e. Option 1 is supported),

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod wusSubgroupsNumForUEID) + (wusSubgroupsNumPerPO – wusSubgroupsNumForUEID), where

- UE_ID is related to 5G-S-TMSI,

- N is the number of total paging frames in one DRX cycle,

- Ns is the number of the PO for a PF,

- Np is the number of subgroupsNumForUEID for PEI, if configured; otherwise, Np is 1,

- wusSubgroupsNumForUEID and wusSubgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

Regarding UE_ID, similar as PEI design, it could be derived from 5G-S-TMSI. What is more, RAN1 agreed that the maximum number of LP-WUS subgroups per PO is 32 in RAN1#118bis meeting [8] as follows. Therefore, if eDRX is not configured, UE_ID=5G-S-TMSI mode (8192* 32).

Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 32.

Proposal 8: For LP-WUS UE_ID based subgrouping, UE_ID = 5G-S-TMSI mode (8192* 32).

Subgrouping method selection

Considering UE's LP-WUS subgroup can be either assigned by CN or derived based on UE_ID, the UE needs to know how to obtain its subgroup when camping on a cell. In Rel-17 PEI, a cell can indicate its capability of CN assigned subgrouping and UE_ID based subgrouping to UE by the configuration. Specifically, the detail is as follows.

- If  PEI is configured and both subgroupsNumPerPO and subgroupsNumForUEID are equal to 1, the cell supports PEI without subgrouping, i.e., there is only one subgroup and all UEs supporting PEI belong to the same subgroup.

- If PEI is configured and subgroupsNumForUEID has the same value (except 1) as subgroupsNumPerPO, the cell only supports the UE_ID based subgrouping.

- If PEI is configured and subgroupsNumForUEID is absent, the cell only supports the CN assigned subgrouping.

- If PEI is configured and subgroupsNumForUEID < subgroupsNumPerPO, the cell supports both CN assigned subgrouping and UE_ID based subgrouping.

Correspondingly, according to TS 38.304 [10], the UE behaviors for the PEI subgroup are captured as follows.

- If UE only supports UE_ID based/CN assigned subgrouping and the cell supports both CN assigned subgrouping and UE_ID based subgrouping, UE uses the UE_ID based/CN assigned subgrouping.

- If the cell only supports UE_ID based/CN assigned subgrouping and the UE supports both CN assigned subgrouping and UE_ID based subgrouping, UE uses the UE_ID based/CN assigned subgrouping.

- If both UE and cell support CN assigned subgrouping and UE_ID based subgrouping, UE uses the CN assigned subgrouping.

- If both UE and cell only support UE_ID based/CN assigned subgrouping, UE uses the UE_ID based/CN assigned subgrouping.

- If both UE and cell only support CN assigned subgrouping and UE has no CN assigned subgroup ID, UE monitors the legacy PO.

- If the cell supports PEI without subgrouping, UE uses the UE_ID based subgrouping.

- If UE only supports one kind of subgrouping and the cell only supports another kind of subgrouping, UE monitors the legacy PO.

For LP-WUS subgrouping, we think it is straightforward to reuse the PEI mechanism. That is, the network provides the capability on supporting LP-WUS subgrouping method(s) based on the configuration. For UE supporting both UE_ID based and CN assigned subgrouping, it is up to network configuration to use UE_ID based or CN assigned subgrouping, similar to the PEI mechanism.

Proposal 9: The network provides the capability on supporting LP-WUS subgrouping method(s) based on the configuration, similar as PEI design.

Proposal 10: For UE supporting both UE_ID based and CN assigned subgrouping, it is up to network configuration to use UE_ID based or CN assigned subgrouping, similar as PEI design.

  - Emergency PDU session

In Rel-17 PEI, RAN2 discussed whether UE monitors PEI when it has an emergence PDU session [11]. In details, for UE supports CN assigned subgrouping, if there is an active emergency PDU session, the UE won’t inform the CN that it has the corresponding UE capability on CN assigned subgrouping.

When the UE has an active emergency PDU Session: - The UE shall not signal Paging Subgrouping Support Indication in the Registration Request message.

In this way, the CN won’t assign the subgroup ID for this UE [12]. Hence there won’t be paging latency for the CN assigned subgrouping. We think the similar method could be used for LP-WUS to avoid the paging latency during the emergency PDU session. However, it should be up to the SA2/CT1 decision.

Proposal 11: For CN assigned subgrouping, RAN2 assumes the mechanism for PEI when there is an active emergency PDU Session could be re-used for LP-WUS as below, and send an LS to SA2/CT1 on the decision.

- When the UE has an active emergency PDU Session, the UE shall not signal Paging Subgrouping Support Indication in the Registration Request message.

For UE_ID based subgrouping, in the case of CN triggered paging for a UE with an emergency PDU session, the RAN doesn’t know if there is an emergency PDU session, thus the UE_ID based subgrouping for PEI is not able to be restricted which will introduce the paging latency. After the discussion within a few meetings in RAN2, RAN2 concluded that disabling UE_ID based PEI is not supported. It means that UE still monitors the PEI with UE_ID based subgrouping when it has an emergency PDU session. Considering the time offset between PEI and its corresponding PO is small, e.g., 50ms, the probability that the emergency PDU session arrives within the time offset is small. Besides, SA2 confirmed that the service impact is limited even without any enhancement on UE_ID based subgrouping since it only has impacts on the emergency call back scenario. Besides, other features in addition to the PEI may also cause the MT call set up delay, thus it is not a big deal.

Observation 3: No enhancement is introduced for the use of PEI with UE_ID based subgrouping during an emergency PDU session.

However, since the time offset between LP-WUS and its corresponding PO may be much larger than the PEI scenario, the paging latency may not be acceptable when UE has an emergency PDU session. We need to discuss and collect the companies’ views on whether to resolve this paging latency issue during an emergency PDU session.

If the latency issue on the use of UE_ID based subgrouping for LP-WUS during the PDU session needs to be addressed, we think the following options could be considered:

- Option 1: Introducing an indication in NG paging, Xn paging (if needed) and F1 paging to disable the LP-WUS function.

- Option 2: Leaving it to UE implementation, i.e., if a UE supporting UE_ID based subgrouping indicates its capability to gNB, when there is an active emergency PDU session, the UE should update its UE capability via indicating the gNB that it doesn’t support the UE_ID based subgrouping to gNB by UE implementation. After the emergency PDU session is released, the UE could update its UE capability again and indicate it supports the UE_ID based subgrouping to gNB by UE implementation. Similar to the legacy, when UE reports the capability, the RAN will indicate UE RADIO CAPABILITY INFO INDICATION to the CN, and CN will indicate this capability in CN paging for RRC_IDLE UEs, or in NGAP core Networking assistance information for RRC_INACTIVE UEs.

Proposal 12: RAN2 to discuss whether and how to address the latency issue for LP-WUS with UE_ID based subgrouping during an emergency PDU session. And the following solutions could be considered if it needs to be addressed:

- Option 1: Introducing an indication to disable the LP-WUS function in NG Paging, RAN paging and F1 Paging during an emergency PDU session.

- Option 2: Leaving it to UE implementation, e.g. UE updates its capability of UE_ID based subgrouping for LP-WUS.

  - UE capability

For CN/RAN paging, the network needs to know the UE capability for paging to decide which kind of paging mechanism is used, e.g. PEI/LP-WUS or legacy paging. In Rel-17 PEI, separate  UE capabilities of CN assigned  subgrouping and UE_ID based subgrouping is introduced. Besides, the UE capability to indicate supporting PEI and UE_ID based subgrouping are same, i.e., if UE supporting PEI it will also support the UE_ID based subgrouping, and this combined UE capability is reported to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message. While UE’s capability of supporting the CN assigned subgrouping is reported to CN by NAS signalling during the registration request or TAU procedure.

Observation 4: For UE capability design in Rel-17 PEI, the UE capability supporting PEI is also used to indicate the capability supporting UE_ID based subgrouping.

Observation 5: For UE capability design in Rel-17 PEI, the UE capabilities for UE_ID based subgrouping and CN assigned subgrouping are separate.

Considering LP-WUS is also used for CN/RAN paging, UE capability for LP-WUS should also be reported to the network like PEI. Different from PEI, besides two kinds of subgrouping methods, there are two kinds of LR receivers for LP-WUS, i.e. OOK-based WUR and OFDM-based WUR. Since RAN2 has agreed that separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs,  correspondingly, it is reasonable to introduce separate UE capabilities for OFDM-based WUR and OOK-based WUR. But the final decision should be determined in RAN1.

Proposal 13: RAN2 assumes separate UE capabilities for different LR types, i.e. OFDM-based WUR and OOK-based WUR, should be introduced and reported to the network. Final decision is up to RAN1.

Regarding the UE capability for CN assigned subgrouping and UE_ID based subgrouping for LP-WUS, it is straightforward to follow the capability design method for PEI. That is, the UE capabilities on supporting CN assigned subgrouping and UE_ID based subgrouping are separately indicated. And no need to introduce additional UE capability for UE_ID based subgrouping, while using the UE capability supporting the LP-WUS to indicate whether supporting the UE_ID based subgrouping. Though there are separate UE capabilities for different types of LR, i.e., the UE capability supporting OFDM-based WUR and OOK-based WUR, we think no matter UE indicates which LR type among these two capabilities, it will also support the UE_ID based subgrouping.

Proposal 14: Introduce separate UE capabilities for CN assigned subgrouping and UE_ID based subgrouping for LP-WUS, similar as PEI design.

Proposal 15: UE capability for supporting LP-WUS (assuming capability for OFDM-based WUR or OOK-based WUR), indicates supporting of UE_ID based subgrouping, i.e. capability supporting LP-WUS and UE_ID based subgrouping is one capability, similar as PEI design.

Regarding the capability reporting for LP-WUS, we think the similar reporting mechanism for PEI could be reused directly. In detail, UE’s capability of supporting the CN assigned  subgrouping is reported to CN by NAS signalling during the registration request or TAU procedure. And for the combined capability to indicate supporting LP-WUS and UE_ID based subgrouping is reported to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message, and gNB will report this UE capability to AMF.

Proposal 16: The capability reporting design for PEI is reused for LP-WUS, i.e. UE reports capability of supporting the CN assigned subgrouping to CN during the registration request or TAU procedure; UE reports the combined capability to indicate supporting LP-WUS and UE_ID based subgrouping to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message.

After the AMF acquired the UE capabilities for LP-WUS, for RRC_IDLE UEs, the CN will include these UE capabilities in NG paging message to inform the gNB. For RRC_INACTIVE UEs, the CN will inform the gNB about these UE capabilities via NGAP core Networking assistance information IE, then the last serving gNB could store this UE capabilities. After the gNB acquire the UE capabilities for LP-WUS, it could determine whether to send LP-WUS with CN assigned subgrouping ID or UE_ID based subgrouping ID and which kind of WUR (i.e. OFDM-based WUR or OOK-based WUR) is used based on the included UE capability information.

Proposal 17: The CN paging message, includes UE’s OFDM/OOK-based WUR capability.

  - Inter-band operation

For the separate band issue, RAN made the following WF in RAN#106 meeting [13] as follows.

For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656). For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered. Detailed discussions and final solution selection is left up to WGs.

Specifically, Solution 1-4 in [14] are as below.

Intra-Frequency - Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band). - Solution 2: MR is prioritized to camp on the same band as LR (System information, Paging on LR band; and RACH on another MR band based on reselection criteria). - Solution 3: MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation). Inter-Frequency - Solution 4: MR (SI/Paging/RACH) is camped on a different band than LR. LR can wake up MR on the band it is camped on.

For Solution 1, LP-WUS capable UE will camp on the cell in LR band by performing cell reselection based on the higher frequency priority for LR band. In this case, the UE monitors system information and paging, and performs RACH on the LR band. It can be noted that the RACH load is high in Solution 1 due to large number of UEs supporting LP-WUS, which may cause the overload issue. The specification impact for this solution is to configure separate priority for LR band in SI in RAN2, while no other impact on RAN4.

For Solution 2, similar to Solution 1, LP-WUS capable UE will camp on the cell within LR band by prioritizing the frequency of LR band (i.e. solution 1) or by redirection from gNB (i.e. solution 3). In this solution, the UE only performs system information reception and paging monitoring on this LR band, while performs RACH to another MR band by cell reselection after receiving LP-WUS and paging. In this way, the potential RACH overhead issue (if any) can be effectively solved. However, the latency is higher than other solutions due to the additional latency for UE to perform cell (re-)selection and SIB acquisition on MR band. The specification impact on this solution is higher than other solutions, as we need to define new behaviour for the UE to perform cell (re-)selection after receiving LP-WUS and paging in RAN2, at the same time, solution 1/solution 3 is still needed for this solution to let UE can camp on the LR band when monitoring LP-WUS and paging. Besides, considering the latency after receiving LP-WUS and paging to RACH, we may need to define new requirements in RAN4 to restrict the latency, as the current requirements defined in RAN4 for cell reselection are unacceptable. Anyway, it is RAN4 business for this part. But it could be discussed later after we agree this solution in RAN2.

For Solution 3, the network configures the dedicated frequency priority for LR band to release some LP-WUS capable UEs to LR band via the dedicated signalling. In this solution, RACH overeload issue could be effectively avoided by network implementation, e.g. releasing UEs to the LR band only when the load is not high. Due to the same reason, this solution will restrict the usage of LP-WUS as it depends on the dedicated signaling, and may also cause more signaling overhead. This solution is already supported in current specification, so there is no spec impact for this solution.

Based on the analysis above, the pros and cons of different solutions are summarized in the following table.

Table 1: The pros and cons of Solution 1-3

|  | RACH overload issue | Latency | Limited usage of LP-WUS | Specification impact |
| --- | --- | --- | --- | --- |
| Solution 1 | Yes | As normal | No | RAN2: Medium RAN4: No |
| Solution 2 | No | Long | No | RAN2: Higher than solution 1/3 RAN4: Potential new requirements may be needed |
| Solution 3 | No (by NW implementation) | As normal | Yes when LR band’s load is high | RAN2: Already supported, and resolve potential issue, if any RAN4: No |

In our understanding, different solutions have different advantages and use cases. For Solution 2, it is the best way to solve the RACH overload and the usage restriction for LP-WUS, especially when the number of UE supporting LP-WUS is very large. For the case in which the number of UE supporting LP-WUS is not too large, the RACH load in Solution 1 could be accepted. Even though the number of UE supporting LP-WUS is large, Solution 1 and Solution 3 can be combined to relieve the RACH overhead. In this case, with Solution 1, LP-WUS capable UEs camp on the LR band, and with Solution 3, UE could reselect to the other band in case the load of LR band is high by the method that network configures the other band than LR band with higher priority via dedicated signalling. For the case in which LR band’s load is not high, Solution 3-only can also work well.

Therefore, we think Solution 2 should be supported, at the same time Solution 1/3 is also needed to support Solution 2, i.e. Solution1/3 is part of Solution 2. At the same time, considering Solution 1 and Solution 3 have different use cases in the real deployment. Thus, we could also support Solution 1 and Solution 3 to work alone or together by network implementation. In this way, different solution(s) could work based on different UE capability and network configuration.

Proposal 18: RAN2 supports Solution 2 for inter-band operation. At the same time Solution, Solution 1/3 is part of Solution 2.

Proposal 19: RAN2 supports Solution 1 and Solution 3, which could work alone or together.

  - SA2 LS

In SA2#166 meeting, SA2 sent an LS to RAN2 and RAN3 [15]. Firstly, SA2 asks RAN2 to consider whether the terminology of “LP-WUS” is appropriate since SA2 understands that the Wake Up Signal is transmitted with high power to enable a large coverage area, and low power refers to a low power wake up receiver used in the mobile.

In our understanding, there is no conclusion in RAN WGs whether the WUS is transmitted with high power. On the other hand, the LP-WUS could only be used in the area which may be smaller than a cell due to the entry/exit condition introduced for whether UE could use LP-WUS. Besides, the terminology has been used for a long time in RAN WGs, including R18 study item and R19 work item, and the meaning is clear in RAN side without any misunderstanding. Thus, we suggest to keep the original terminology, i.e. “LP-WUS” without any change.

Proposal 20: RAN2 will still use the term of “LP-WUS” for discussion and how to capture it in the specification(s) will be further synced between different WGs in RAN.

Regarding the second issue in the LS about how UE to report the UE capability on LP-WUS supporting in case the initial Registration is performed on a pre-Rel 19 gNB (or a later gNB that does not support LP-WUS). In this case, the legacy gNB does not understand the LP-WUS capability and thus cannot include this capability in the UERadioPagingInformation inter-node message to CN or other gNBs for paging, i.e. the LP-WUS capability reported by the UE is not maintained by the network, which results in inconsistent paging operations between the UE and the network and causes paging issues.

The same issue on PEI capability maintenance had been discussed in RAN2#120 meeting as follows [16].

RAN2#120 meeting R2-2211603 Discussion on the PEI capability maintenance Huawei, HiSilicon discussion Rel-17 NR_UE_pow_sav_enh-Core - Xiaomi think that the network deployment fixes this. The network is required to support homogenously, OPPO agrees, VDF agrees as well. HW think we didn’t agree homogenous requirement for this case. - Ericsson think there is some truth to the HW observation. Legacy gNB us required to support the latest ASN.1 Noted, not agreeable (no support)

To resolve the misalignment between the gNB and the UE on the UE capability on PEI supporting which will result the paging lost as above, R2-2211603 proposed the following 2 solutions:

- Solution#1: Introduce a capability filter for the PEI capability reporting where the UE only reports the PEI capability

- Solution#2: Introduce an additional bit to indicate whether the gNB has been upgraded to support the PEI feature, UE decides whether to report the PEI capability based on this indication.

However, during the online discussion, other companies thought the issue does not exist considering the network is required to support homogenously, or legacy gNB may require to support the latest ASN.1. Finally, no conclusion on the solutions was agreed for this issue.

In our understanding, it could be up to the network implementation to avoid this issue, e.g. the network support on LP-WUS is homogenously in the TA/RNA or all legacy gNB is upgraded to support the latest ASN.1 for the UE capability on LP-WUS.

Proposal 21: Reply LS to SA2 that it is up to network implementation to address the misalignment issue between the gNB and the UE on the UE capability of supporting LP-WUS in case the Initial Registration is performed on a pre-Rel-19 gNB, e.g., all legacy gNB is upgraded to support the latest ASN.1 for the UE capability on LP-WUS. No specification impact is foreseen.

- Conclusion

In this contribution, we discuss the remaining issues for RRC_IDLE/ INACTIVE procedures with LP-WUR. Based on the discussion, we have the following observations and proposals.

LP-WUS configuration

Proposal 1: The entry/exit condition for LP-WUS monitoring is mandatorily configured if the network is intended to support the LP-WUS feature.

Proposal 2: If the network supports the OOK-based WUR, the LP-SS configuration should be always provided.

Entry/exit condition of using LP-WUS for OFDM-based LR

Proposal 3: The entry/exit thresholds of using LP-WUS for OFDM-based WUR monitoring the overlaid OFDM sequence(s) are the same as that for OOK-based WUR.

Other wake up conditions

Proposal 4: MR should wake up for UE-initiated UL transmission, e.g. RACH/MO-SDT, due to UL data and signaling arrival when monitoring LP-WUS.

LP-WUS Subgrouping

Observation 1: According to the RAN1 agreement, when UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

Observation 2: False alarm rate could be further reduced if the subgrouping ID of LP-WUS and PEI are independent.

Proposal 5: No need to introduce a network indication to indicate UE to monitor both LP-WUS and PEI or not.

Proposal 6: In order to allow UE to use LP-WUS together with PEI to reduce paging false alarm rate, it is expected for one UE independent subgroup ID is applied for LP-WUS and PEI.

Proposal 7: For UE_ID based subgrouping, the formula is defined as below for LP-WUS subgrouping (i.e. Option 1 is supported),

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod wusSubgroupsNumForUEID) + (wusSubgroupsNumPerPO – wusSubgroupsNumForUEID), where

- UE_ID is related to 5G-S-TMSI,

- N is the number of total paging frames in one DRX cycle,

- Ns is the number of the PO for a PF,

- Np is the number of subgroupsNumForUEID for PEI, if configured; otherwise, Np is 1,

- wusSubgroupsNumForUEID and wusSubgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

Proposal 8: For LP-WUS UE_ID based subgrouping, UE_ID = 5G-S-TMSI mode (8192* 32).

Proposal 9: The network provides the capability on supporting LP-WUS subgrouping method(s) based on the configuration, similar as PEI design.

Proposal 10: For UE supporting both UE_ID based and CN assigned subgrouping, it is up to network configuration to use UE_ID based or CN assigned subgrouping, similar as PEI design.

Emergency PDU session

Observation 3: No enhancement is introduced for the use of PEI with UE_ID based subgrouping during an emergency PDU session.

Proposal 11: For CN assigned subgrouping, RAN2 assumes the mechanism for PEI when there is an active emergency PDU Session could be re-used for LP-WUS as below, and send an LS to SA2/CT1 on the decision.

- When the UE has an active emergency PDU Session, the UE shall not signal Paging Subgrouping Support Indication in the Registration Request message.

Proposal 12: RAN2 to discuss whether and how to address the latency issue for LP-WUS with UE_ID based subgrouping during an emergency PDU session. And the following solutions could be considered if it needs to be addressed:

- Option 1: Introducing an indication to disable the LP-WUS function in NG Paging, RAN paging and F1 Paging during an emergency PDU session.

- Option 2: Leaving it to UE implementation, e.g. UE updates its capability of UE_ID based subgrouping for LP-WUS.

UE capability

Observation 4: For UE capability design in Rel-17 PEI, the UE capability supporting PEI is also used to indicate the capability supporting UE_ID based subgrouping.

Observation 5: For UE capability design in Rel-17 PEI, the UE capabilities for UE_ID based subgrouping and CN assigned subgrouping are separate.

Proposal 13: RAN2 assumes separate UE capabilities for different LR types, i.e. OFDM-based WUR and OOK-based WUR, should be introduced and reported to the network. Final decision is up to RAN1.

Proposal 14: Introduce separate UE capabilities for CN assigned subgrouping and UE_ID based subgrouping for LP-WUS, similar as PEI design.

Proposal 15: UE capability for supporting LP-WUS (assuming capability for OFDM-based WUR or OOK-based WUR), indicates supporting of UE_ID based subgrouping, i.e. capability supporting LP-WUS and UE_ID based subgrouping is one capability, similar as PEI design.

Proposal 16: The capability reporting design for PEI is reused for LP-WUS, i.e. UE reports capability of supporting the CN assigned subgrouping to CN during the registration request or TAU procedure; UE reports the combined capability to indicate supporting LP-WUS and UE_ID based subgrouping to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message.

Proposal 17: The CN paging message, includes UE’s OFDM/OOK-based WUR capability.

Inter-band operation

Proposal 18: RAN2 supports Solution 2 for inter-band operation. At the same time Solution, Solution 1/3 is part of Solution 2.

Proposal 19: RAN2 supports Solution 1 and Solution 3, which could work alone or together.

SA2 LS

Proposal 20: RAN2 will still use the term of “LP-WUS” for discussion and how to capture it in the specification(s) will be further synced between different WGs in RAN.

Proposal 21: Reply LS to SA2 that it is up to network implementation to address the misalignment issue between the gNB and the UE on the UE capability of supporting LP-WUS in case the Initial Registration is performed on a pre-Rel-19 gNB, e.g., all legacy gNB is upgraded to support the latest ASN.1 for the UE capability on LP-WUS. No specification impact is foreseen.

- References

- R2-2403732, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

- R2-2405702, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

- R2-2407572, Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD

- R2-2409212, Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD

- R2-2410912, Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD

- R1-2403821, Final Report of 3GPP TSG RAN WG1 #116b v1.0.0

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO

- R1-2409341, Final Report of 3GPP TSG RAN WG1 #118b v1.0.0

- R1-2401937, Final Report of 3GPP TSG RAN WG1 #116 v1.0.0

- TS38.304 v18.3.0

- R2-2209018 Report of [AT119-e][004] [ePowSav] Subgrouping/PEI (MediaTek)

- TS24.501 v19.1.1

- draft_MeetingReport_RAN_106_241212_eom

- RP-242656, On LR and MR operating in different bands in release 19

- R2-2500050/S2-2412876, LS Reply on LP-WUS subgrouping

- R2-2300002, Report of 3GPP TSG RAN WG2 meeting #120, Toulouse, France

Annex – [draft] reply LS to SA2 on LP-WUS operation

Title:	[Draft] reply LS to SA2 on LP-WUS operation

Response to:	R2-240xxxx/ S2-2412876

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	TBD [to be RAN2]

To:	SA2

Cc: 	RAN3

Contact Person:

Name:	              TBD

E-mail Address:	 TBD

1. Overall Description:

RAN2 would like to thank SA2 for the LS on LS Reply on LP-WUS subgroupingin R2-2500050/ S2-2412876].

For the terminology and name, RAN WG(s) will still use the term of “LP-WUS” for discussion. How to capture it in the specification(s) will be further synced between different WGs in RAN. Thus, it is up to SA2 whether to introduce another term in SA2 specification, but RAN2 recommends SA2 to use the same term as RAN side.

For updating UE Radio Capability for Paging Information in the mentioned scenario, it is up to network implementation to address the misalignment issue between the gNB and the UE on the UE capability of supporting LP-WUS in case the Initial Registration is performed on a pre-Rel-19 gNB, e.g, the legacy gNB is upgraded to support the latest ASN.1 to guarantee that all capabilities (e.g. LP-WUS, PEI, (e)RedCap) of UE is not lost. No specification impact is forseen in RAN3.

2. Actions:

To RAN SA2:

RAN2 kindly request SA2 to take the above information into account and provide feedback, if any.

3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #129bis		7th Apr. – 11th Apr. 2025		             Wuhan, China

TSG RAN WG2 Meeting #130		19th May. – 23rd May. 2025		St Julian’s Malta