3GPP TSG-RAN WG2 Meeting #129		R2-2500345

Athens, Greece, 17th – 21st Feb. 2025

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_CONNECTED

Agenda Item:	8.4.4

Document for:	Discussion and Decision

- Introduction

In this contribution, we will further discuss the remaining issues on LP-WUS/WUR in RRC_CONNECTED from RAN2 perspective.

Besides, RAN1 send an LS to RAN2 in [1], with the below questions:

- Overall Description: In RAN#119, RAN1 made following conclusion and agreement to use existing L2 signalling to stop C-DRX related timers and resuming LP-WUS monitoring: Conclusion - For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909. Agreement - For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909.. Regarding the above conclusion on Option 1-1, RAN1 would like to ask RAN2 whether RAN2 has same understanding or not. Regarding the above agreement on Option 1-2, RAN1 would like to ask RAN2 the feasibility of the above potential solution.

In this contribution, we will also provide our views on the how to reply RAN1 LS.

- Discussion

  - RAN1 LS on DRX Command MAC CE and long DRX Command MAC CE

Regarding the LS from RAN1 [1], firstly, we want to clarify that, current TS 38.321 has captured the existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group as below [2]:

1> if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission or a Long DRX Command MAC CE is received: 2> stop drx-onDurationTimer for each DRX group; 2> stop drx-InactivityTimer for each DRX group.

Regarding the first issue on Option 1-1 in the LS, since RAN2 has the agreement:

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

Considering DCP is monitored when UE is not in C-DRX Active Time, thus it is also reasonable that LP-WUS in Option 1-1 is monitored when UE is not in C-DRX Active Time.

Besides, DCP is only applied when UE uses Long DRX cycle, then whether Option 1-1 is only applied to Long DRX cycle should be confirmed. If this is a common understanding in RAN2, when DRX Command MAC CE and Long DRX Command MAC CE is received to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group, the UE behaviour should be:

- If UE receives DRX Command MAC CE, UE will use Short DRX cycle (if configured) according to current specification, since Option 1-1 is only applied to Long DRX cycle, Option 1-1 won’t be used in this case.

- If UE receives Long DRX Command MAC CE or DRX Command MAC CE in case Short DRX cycle is not configured, UE will use Long DRX cycle according to current specification, since UE will leave the C-DRX Active time when Long DRX Command MAC CE is received, the LP-WUS will be monitored in this case.

Proposal 1: RAN2 confirms in Option 1-1, LP-WUS is only applied when UE using Long DRX cycle.

Proposal 2: If P1 is agreed, reply RAN1 that for Option 1-1:

- If UE receives DRX Command MAC CE, UE will use Short DRX cycle (if configured) according to current specification, since Option 1-1 is only applied to Long DRX cycle, Option 1-1 won’t be used in this case.

- If UE receives Long DRX Command MAC CE or DRX Command MAC CE in case Short DRX cycle is not configured, UE will use Long DRX cycle according to current specification. When Long DRX Command MAC CE is received, the UE will leave the C-DRX Active Time, and will monitor LP-WUS.

Regarding the second issue on Option 1-2 in the LS, RAN2 has the following agreements [3]:

The drx-onDurationTimer is not started with Option 1-2 LP-WUS.

It could be observed that no distinction is made between Long DRX cycle and Short DRX cycle for Option 1-2 with the agreement.  Considering drx-onDurationTimer is not started when LP-WUS is used, no matter Long DRX cycle or Short DRX cycle is used, LP-WUS could be monitored outside the active time. In this way, the typical network configuration will only configure one DRX cycle with LP-WUS. Thus, there is no restriction on Long DRX cycle and Short DRX cycle for the use of LP-WUS, i.e. Option 1-2 LP-WUS could be used with either Short DRX cycle or Long DRX cycle.

Observation 1: Option 1-2 LP-WUS could be used not matter Short DRX cycle or Long DRX cycle is used.

Besides, RAN2 also has the following agreements on LP-WUS monitoring and termination method of PDCCH monitoring triggered by LP-WUS [3][4]:

In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS.

Thus, for Option 1-2 LP-WUS, when DRX command MAC CE or Long DRX command MAC CE is received, UE will stop the new timer and drx-InactivityTimer. In this case, the UE will leave C-DRX Active Time and go back to LPWUS monitoring.

Proposal 3: Reply RAN1 that for Option 1-2, if DRX command MAC CE or Long DRX command MAC CE is received, UE will stop the new timer and drx-InactivityTimer. In this case, the UE will leave C-DRX Active Time and go back to LP-WUS monitoring.

  - UE behaviour within the Active time

CSI report behaviour within the Active time triggered by LP-WUS

For LP-WUS Option 1-2, in RAN2#128 meeting, we agreed that the drx-ondurationTimer is not started if Option 1-2 LP-WUS is configured and in this case, during the non-Active time configured by drx-onDurationTimer UE doesn’t report the semi-periodic CSI and report the periodic CSI/L1-RSRP based on network configuration [3], for example:

- If wus-TransmitOtherPeriodicCSI is configured with value true, UE reports periodic CSI;

- If wus-TransmitPeriodicL1-RSRP is configured with value true, UE reports periodic L1-RSRP.

- The drx-onDurationTimer is not started with Option 1-2 LP-WUS. - For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

Observation 2: For Option 1-2, within legacy drx-onDurationTimer duration and the MAC entity is not in Active Time, it is up to network configuration to determine whether to report the periodic CSI/L1-RSRP, i.e. same as in Rel-16 DCP.

However, we have not discussed the UE behaviour on CSI report during the Active Time triggered by LP-WUS Option 1-2. Since when LP-WUS Option 1-2 is configured, the drx-onDurationTimer is not started, then the Active Time triggered by LP-WUS is most frequently compared with Active Time caused by other triggers, thus we think the LP-WUS triggered C-DRX Active Time is more similar to the legacy drx-onduraionTimer rather than C-DRX Active Time caused by other triggers. Accordingly, the CSI/L1-RSRP reporting behaviour in LP-WUS triggered C-DRX Active Time should be same as the legacy drx-ondurationTimer. Specifically, UE reports the periodic CSI on PUCCH, Semi-periodic CSI report on PUCCH and Semi-periodic CSI report on PUSCH during this timer triggered by LP-WUS is running. Otherwise, insufficient CSI reporting may cause the failure of data transmission during Active Time when CSI-mask is set to true.

Proposal 4: For LP-WUS Option 1-2, during the timer triggered by LP-WUS is running, periodic CSI/L1-RSRP report and SP CSI report behaviour are the same as legacy drx-onDurationTimer duration without DCP configuration, i.e. always report.

Coexistence with PDCCH skipping

Besides, since UE is in C-DRX Active Time when the new timer triggered by LP-WUS is running and UE will monitor the PDCCH within this duration, PDCCH skipping could also used together to stop the PDCCH monitoring.

When UE receives the PDCCH skipping indication, UE will not perform PDCCH monitoring. However the PDCCH skipping indication will not impact the C-DRX Active Timer switch. In this case, UE may be still within the C-DRX Active Time (but just stops the PDCCH monitoring), in this way, UE won’t switch back to the LP-WUS monitoring since we only agreed UE goes back to LP-WUS monitoring when UE is not in C-DRX active time as below [4].

In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring.

However, the network may want to trigger UE to perform the data transmission again later after it indicates UE to skip the PDCCH monitoring within this C-DRX Active Time in some cases, e.g. the drx-InactivityTimer is set to a long value, and considering the LP-WUS monitoring only consume little power, we think UE could also go back to LP-WUS monitoring to reduce the data transmission latency in this case.

Proposal 5: For Option 1-2, when LP-WUS is used together with PDCCH skipping, the UE stops the PDCCH monitoring upon receiving the PDCCH skipping indication, and starts the LP-WUS monitoring.

  - LP-WUS MO configuration and collision issue

UAI report time offset between LP-WUS and PDCCH monitoring

In RAN1#118bis meeting, RAN1 agreed that UE reports the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting, besides, UE could indicate via UAI its preference for the configured time offset to the network [5].

Agreement For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported candidate value - UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional. - FFS: A single time offset is configured for a UE at a given time

In our understanding, if the UE reports the time offset via the UAI, the network will configure the time offset between LP-WUS and PDCCH monitoring according to the UAI, and the time when MR is waked up is determined by the UE capability. The figure1 shows the following 3 cases:

- Case#1: MR wakes up earlier than the configured time offset, e.g., t1 in figure1. In this case, the value of time offset reported via UAI is longer than the value reported by UE capability.

- Case#2: MR wakes up just as the configured time offset, e.g., t2 in figure1. In this case, the value of time offset reported via UAI is longer than or same as the value reported by UE capability.

- Case#3: MR wakes up later than the configured time offset, e.g., t3 in figure1. In this case, the value of time offset reported via UAI is shorter/longer than or same as the value reported by UE capability.

Figure 1: Time offset between Option 1-2 LP-WUS and its corresponding timer

Case#2 is the ideal scenario which reached the best balance on PDCCH monitoring latency and power saving gain. For Case#1, the power saving gain is not as good as Case#2, however considering the whole time offset between LP-WUS and PDCCH monitoring will not be long, the gain difference is marginal between the two cases. Thus Case#1 and Case#2 are both acceptable for LP-WUS.

Observation 3: When LP-WUS triggers PDCCH monitoring, the scenario in which MR wakes up earlier than the configured time offset couldn’t reach the best power saving gain than the scenario in which MR wakes up same as the configured time offset.

For case#3, since the start time for PDCCH monitoring is passed for the UE, UE could only monitor PDCCH from t3 in the above example. While for the network, it will consider t2 as the start of PDCCH monitoring time for the UE, thus it will send the PDCCH from t2. In this case, the PDCCH will be missed by the UE, which is not accepted.

Observation 4: When LP-WUS triggers PDCCH monitoring, UE may miss the PDCCH if MR wakes up later than the configured time offset.

With the above, we think case#3 should be avoided as much as possible. Thus, we suggest RAN2 to restrict that the value reported via UAI is longer than UE capability.

Proposal 6: The preferred value of time offset between LP-WUS and PDCCH reported via UAI is expected to be longer than the value for the minimum time gap between LP-WUS and MR to start PDCCH monitoring corresponding to the SCS reported via UE capability.

LP-WUS occasion Collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR

In Rel-16 Power Saving, RAN2 has discussed the collision scenarios for DCP [2]:

- DCP occasion is overlapped with the C-DRX Active Time;

- DCP occasion is overlapped with the measurement gap or within BWP switching interruption length;

- DCP collides with the RAR addressed to C-RNTI, i.e., DCP occasion is during ra-ResponseWindow for BFR via RACH.

When the above collision occurs, UE couldn’t monitor the DCP and will start the corresponding drx-onDurationTimer directly. Similarly, one issue needs to be discussed is for LP-WUS Option 1-1 and Option 1-2, whether a UE is expected or not expected to monitor LP-WUS in C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR.

Figure 2: LP-WUS occasion collides with C-DRX Active Time

This issue is related to whether LR and MR are able to work simultaneously. In our view, it highly depends on UE implementation. In case some hardware e.g., antenna is shared between MR and LR, the UE can only use one receiver at a time. For such UE implementation, the LP-WUR and MR are not able to work simultaneously. Therefore, it may be safer to assume that as a baseline design, LP-WUR is not able to receive signal(s) when MR transmits/receives signals/channels. Thus, the collision issue should be considered for LP-WUS Option 1-1 and Option 1-2.

Proposal 7: For LP-WUS Option 1-1 and Option 1-2, RAN2 should consider the UE behaviour when all LP-WUS occasion collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR.

For LP-WUS Option1-1, we have the following agreement in RAN2#126 meeting [6] ：

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

Thus, if all LO collides with the C-DRX Active Time/measurement gap/BWP switching interruption length/RAR for BFR, UE couldn’t monitor the LP-WUS and UE will start the corresponding drx-onDurationTimer.

For LP-WUS Option 1-2, it is also possible that all LP-WUS occasion collides with the C-DRX Active Time/measurement gap/BWP switching interruption length/RAR for BFR, in this case, since the UE couldn’t monitor the LP-WUS, one issue is whether UE starts the new timer corresponding to the LP-WUS should be considered. In our understanding, a similar mechanism for DCP could be reused, i.e., UE starts the new timer for PDCCH monitoring to avoid missing the potential PDCCH.

Proposal 8: For LP-WUS Option 1-1 and Option 1-2, in case all the LP-WUS occasion of an LP-WUS collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR, UE starts the corresponding drx-onDurationTimer for LP-WUS Option 1-1 and the corresponding new timer for LP-WUS Option 1-2.

  - LP-WUS operation for UE configured with NR-DC/dual DRX

RAN1 discussed the LP-WUS configured in CA and DC cases and had the following agreements [5]:

Agreement in RAN1#118 meeting: LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC Agreement in RAN1#118bis meeting: LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

It could be observed that RAN1 agreed LP-WUS is supported when UE is configured with CA and NR-DC. For NR-DC case, the LP-WUS is configured separately in MCG and SCG, where the LP-WUS related to MCG could only trigger the PDCCH monitoring of the serving cells in MCG and the LP-WUS related to SCG could only trigger the PDCCH monitoring of the serving cells in SCG.

Observation 5: LP-WUS is supported when UE is configured with CA or NR-DC.

Observation 6: For LP-WUS used for NR-DC scenario, LP-WUS only triggers the PDCCH monitoring in the same cell group where LP-WUS is received.

One FFS is for which serving cell(s) of the corresponding cell group the PDCCH monitoring triggered by an LP-WUS is applicable, we think the same mechanism as Rel-16 DCP could be used, i.e., all activated serving cells in the cell group are applicable. Besides, since for both Option 1-1 and Option 1-2 LP-WUS, the LP-WUS could trigger the UE entering the C-DRX Active Time, in this case, the PDCCH monitoring in all activated serving cells could be performed in the C-DRX active time. Thus, it could achieve more power saving gain for LP-WUS to control the PDCCH monitoring in all serving cells.

Proposal 9: For both Option 1-1 and Option 1-2, when UE is configured with CA/DC in RRC CONNECTED mode, the LP-WUS indication is applied to all activated serving cells in the same cell group.

In Release 16, secondary DRX group configuration is introduced for one cell group. And RAN2 concluded that the DCP could not be configured when the secondary DRX group is configured for simplicity since there is not enough time in Release 16 to discuss the coexistence issue [7].

drx-ConfigSecondaryGroup Used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3]. The network does not configure secondary DRX group with DCP simultaneously nor secondary DRX group with a dormant BWP simultaneously.

However, for LP-WUS Option 1-1 and Option 1-2, we think the restriction for DCP is not needed and we have enough time to consider the coexistence between LP-WUS and dual DRX. In this case, except drx-onDurationTimer and drx-InactivityTimer, other DRX parameters are common for two DRX groups and the start of the drx-onDurationTimer is also the same for two DRX groups. So, the Active Time for two DRX groups is overlapped. When there is traffic, the Active Times of the two DRX groups will probably be aligned. In addition, LP-WUS monitoring on both primary and secondary DRX groups in parallel will increase the UE complexity and power consumption. So, we slightly prefer using one set of LP-WUS configuration for dual DRX groups per MAC entity.

Proposal 10: For both Option 1-1 and Option 1-2, LP-WUS could co-exist with dual DRX. And one set of LP-WUS configuration is configured per MAC entity.

  - LP-WUS Option3

RAN1 has agreed to support LP-WUS Option 1-1 and Option 1-2, however, for Option 1-3, RAN1 didn’t have any conclusion on it. Since Option 1-3 is transparent to MAC, which only impacts the physical layers design, we think it is better to wait for RAN1 progress, if any. RAN2 should focus on Option 1-1 and Option 1-2 and won’t discuss Option 1-3 unless RAN1 triggers.

Proposal 11: RAN2 won’t discuss Option 1-3 unless RAN1 triggers.

  - Enable and disable for LP-WUS monitoring

RAN1 has also agreed that gNB RRC signalling should be used to enable/disable the operation of PDCCH monitoring triggered by LP-WUS, but it is still FFS whether to support UE assistance for the enable/disable functionality.

For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance.

We think the enable/disable operation could rely on the configuration of LP-WUS as usual in RAN2. Regarding the additional UE assistance, according to the WID, current RRM/RLM/BFD/CSI/BM measurement and corresponding report are performed by MR as legacy. In this way, network has enough information to enable/disable the LP-WUS functionality. Besides, the enable/disable operation is a long-term configuration. There is no need to dynamically change the configuration. Therefore, we think additional assistance information from UE to gNB for enable/disable operation is not necessary.

Proposal 12: In RRC_CONNECTED mode, PDCCH monitoring triggered by LP-WUS is enabled/disabled by the corresponding configuration of LP-WUS. No additional assistance information from UE is needed.

Regarding the RRC message to carry the LP-WUS configuration, RAN2 agreed the dedicated RRC message is used.

The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

In our understanding, it is straightforward to use the RRCResume and RRCReconfiguration messages as usual.

Proposal 13: LP-WUS configuration is included in RRCResume/RRCReconfiguration message.

  - Activation and deactivation procedure for LP-WUS monitoring

Regarding the activation and deactivation for LP-WUS monitoring, RAN1 has the following options:

- In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

From RAN2 perspective, we want to clarify the following terms firstly:

- Enable for LP-WUS monitoring: which is to enable the LP-WUS functionality while UE couldn’t use LP-WUS if the LP-WUS functionality has some activation mechanisms;

- Activation for LP-WUS monitoring: When LP-WUS function is enabled, and the corresponding activation mechanism is performed, UE could use the LP-WUS functionality;

- Deactivation for LP-WUS monitoring: After LP-WUS monitoring is activated, upon the corresponding deactivation mechanism is performed, UE stops using the LP-WUS functionality.

Considering the LP-WUS configuration overhead is comparable to other dedicated configurations, e.g. L3 measurement configuration, and the LP-WUS monitoring is not an urgent behaviour, there is no motivation to introduce additional activation/deactivation procedure for LP-WUS monitoring. UE could use the LP-WUS when the LP-WUS monitoring is enabled, i.e., when UE receives the RRC dedicated signalling with LP-WUS configuration. And if the network doesn’t expect the UE to use the LP-WUS, it could just send the RRC signalling to disable the LP-WUS monitoring for the UE.

Proposal 14: RAN2 assumes UE could monitor LP-WUS upon LP-WUS is configured, no additional indication/condition is needed for activation/deactivation of LP-WUS functionality. The final decision is up to RAN1.

- Conclusion

In this contribution, we discuss the LP-WUS triggers PDCCH monitoring in RRC_CONNECTED and discuss the issues in RAN1 LS. Based on the discussion, we have the following observations and proposals.

Observations:

Observation 1: Option 1-2 LP-WUS could be used not matter Short DRX cycle or Long DRX cycle is used.

Observation 2: For Option 1-2, within legacy drx-onDurationTimer duration and the MAC entity is not in Active Time, it is up to network configuration to determine whether to report the periodic CSI/L1-RSRP, i.e. same as in Rel-16 DCP.

Observation 3: When LP-WUS triggers PDCCH monitoring, the scenario in which MR wakes up earlier than the configured time offset couldn’t reach the best power saving gain than the scenario in which MR wakes up same as the configured time offset.

Observation 4: When LP-WUS triggers PDCCH monitoring, UE may miss the PDCCH if MR wakes up later than the configured time offset.

Observation 5: LP-WUS is supported when UE is configured with CA or NR-DC.

Observation 6: For LP-WUS used for NR-DC scenario, LP-WUS only triggers the PDCCH monitoring in the same cell group where LP-WUS is received.

Proposals:

RAN1 LS on DRX Command MAC CE and long DRX Command MAC CE

Proposal 1: RAN2 confirms in Option 1-1, LP-WUS is only applied when UE using Long DRX cycle.

Proposal 2: If P1 is agreed, reply RAN1 that for Option 1-1:

- If UE receives DRX Command MAC CE, UE will use Short DRX cycle (if configured) according to current specification, since Option 1-1 is only applied to Long DRX cycle, Option 1-1 won’t be used in this case.

- If UE receives Long DRX Command MAC CE or DRX Command MAC CE in case Short DRX cycle is not configured, UE will use Long DRX cycle according to current specification. When Long DRX Command MAC CE is received, the UE will leave the C-DRX Active Time, and will monitor LP-WUS.

Proposal 3: Reply RAN1 that for Option 1-2, if DRX command MAC CE or Long DRX command MAC CE is received, UE will stop the new timer and drx-InactivityTimer. In this case, the UE will leave C-DRX Active Time and go back to LP-WUS monitoring.

UE behaviour within the Active time

Proposal 4: For LP-WUS Option 1-2, during the timer triggered by LP-WUS is running, periodic CSI/L1-RSRP report and SP CSI report behaviour are the same as legacy drx-onDurationTimer duration without DCP configuration, i.e. always report.

Proposal 5: For Option 1-2, when LP-WUS is used together with PDCCH skipping, the UE stops the PDCCH monitoring upon receiving the PDCCH skipping indication, and starts the LP-WUS monitoring.

UAI report time offset between LP-WUS and PDCCH monitoring

Proposal 6: The preferred value of time offset between LP-WUS and PDCCH reported via UAI is expected to be longer than the value for the minimum time gap between LP-WUS and MR to start PDCCH monitoring corresponding to the SCS reported via UE capability.

LP-WUS occasion Collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR

Proposal 7: For LP-WUS Option 1-1 and Option 1-2, RAN2 should consider the UE behaviour when all LP-WUS occasion collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR.

Proposal 8: For LP-WUS Option 1-1 and Option 1-2, in case all the LP-WUS occasion of an LP-WUS collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR, UE starts the corresponding drx-onDurationTimer for LP-WUS Option 1-1 and the corresponding new timer for LP-WUS Option 1-2.

LP-WUS operation for UE configured with NR-DC/dual DRX

Proposal 9: For both Option 1-1 and Option 1-2, when UE is configured with CA/DC in RRC CONNECTED mode, the LP-WUS indication is applied to all activated serving cells in the same cell group.

Proposal 10: For both Option 1-1 and Option 1-2, LP-WUS could co-exist with dual DRX. And one set of LP-WUS configuration is configured per MAC entity.

LP-WUS Option 3

Proposal 11: RAN2 won’t discuss Option 1-3 unless RAN1 triggers.

Enable and disable for LP-WUS monitoring

Proposal 12: In RRC_CONNECTED mode, PDCCH monitoring triggered by LP-WUS is enabled/disabled by the corresponding configuration of LP-WUS. No additional assistance information from UE is needed.

Proposal 13: LP-WUS configuration is included in RRCResume/RRCReconfiguration message.

Activation and deactivation procedure for LP-WUS monitoring

Proposal 14: RAN2 assumes UE could monitor LP-WUS upon LP-WUS is configured, no additional indication/condition is needed for activation/deactivation of LP-WUS functionality. The final decision is up to RAN1.

- References

- R2-2500012, LS on LP-WUS operation in CONNECTED mode

- 3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".

- RAN2#128_ChairNotes.

- RAN2#127bis_ChairNotes.

- 3GPP RAN1#118bis meeting, Chairman notes

- RAN2#126_ChairNotes.

- 3GPP TS 38.331:"NR; Radio Resource Control (RRC) protocol specification".

Annex

Title:	[Draft] reply LS on LP-WUS operation in CONNECTED mode

Response to:	R2-2500012 / R1-2410909

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	RAN2

To:	RAN1

Contact Person:

Name:	              TBD

E-mail Address:	 TBD

1. Overall Description:

RAN2 would like to thank RAN1 for the LS on LP-WUS operation in CONNECTED mode [R2-2500012/R1-2410909].

TS 38.321 has captured the following description on DRX Command MAC CE and Long DRX Command MAC CE to stop drx-onDurationTimer and drx-InactivityTimer:

1> if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission or a Long DRX Command MAC CE is received: 2> stop drx-onDurationTimer for each DRX group; 2> stop drx-InactivityTimer for each DRX group.

For Option 1-1 of LP-WUS CONNECTED mode operation, RAN2 has the following agreement in RAN2#126 meeting:

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

Following the DCP mechanism, LP-WUS in Option 1-1 could only be monitored when UE uses Long DRX cycle and is not in C-DRX Active Time. When DRX Command MAC CE and Long DRX Command MAC CE is received to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group, the UE behaviour should be:

- If UE receives DRX Command MAC CE, UE will use Short DRX cycle (if configured) according to current specification, since Option 1-1 is only applied to Long DRX cycle, Option 1-1 won’t be used in this case.

- If UE receives Long DRX Command MAC CE or DRX Command MAC CE in case Short DRX cycle is not configured, UE will use Long DRX cycle according to current specification. When Long DRX Command MAC CE is received, the UE will leave the C-DRX Active time, and will monitor LP-WUS.

For Option 1-2 of LP-WUS CONNECTED mode operation, RAN2 has the following agreement in RAN2#128 meeting:

The drx-onDurationTimer is not started with Option 1-2 LP-WUS.

It could be observed that drx-onDurationTimer is not started when LP-WUS is used, no matter Long DRX cycle or Short DRX cycle is used, which means there is no restriction on Long DRX cycle and Short DRX cycle for the use of LP-WUS. Besides, RAN2 has the following agreement in RAN2#127bis and RAN2#128 meeting:

In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS.

Thus, if DRX command MAC CE or Long DRX command MAC CE is received, UE will stop the new timer and drx-InactivityTimer. In this case UE will leave C-DRX Active Time and go back to LPWUS monitoring.

2. Actions:

To RAN1:

RAN2 kindly request RAN1 to take the above information into account and provide feedback, if any.

3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #129bis		7th Apr. – 11th Apr. 2025		             Wuhan, China

TSG RAN WG2 Meeting #130		19th May. – 23rd May. 2025		St Julian’s, Malta