3GPP TSG-RAN WG2 Meeting #129	R2-2500351

Athens, Greece, 17th – 21st Feb. 2025

Source:	vivo

Title:	Remaining issues on DSR enhancements for XR

Agenda Item:	8.7.4.2

Document for:	Discussion and Decision

- Introduction

In RAN2#127bis [1], there were following agreements regarding DSR enhancements:

Agreements on DSR enhancements - We do not change the definition of delay-critical data - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

In this contribution, the following remaining issues on DSR are discussed:

- New DSR format definition for enhanced DSR

- Enhanced DSR to legacy DSR dependence

- DSR format selection

- Discussion

  - Enhanced DSR format definition

In RAN2#127bis, there were the following agreements regarding enhanced DSR format:

- A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

It is not clear yet how the one-bit indication is placed in the enhanced DSR format. There are two options:

- Option A: one bitmap per LCG to indicate the presence of the pairs/portions

For this option, for an LCG configured with multiple reporting thresholds, a pair bitmap is used to indicate the presence of pairs when any pair of this LCG is reported, wherein the pair bitmap length is equal to the maximum number of pairs configured for this LCG and each bit of the pair bitmap corresponds to one pair. For an LCG configured with single reporting threshold or only configured with legacy DSR, no pair bitmap is needed. Figure 1 shows one example for enhanced DSR format of Option A, wherein LCG 7, 5 and 4 are configured with multiple reporting thresholds and each of them can have more than one pair to be reported. Please be noted that pair bitmap of LCGi does not present if LCGi has no delay status to be reported.

Figure 1 Example of enhanced DSR format with one pair presence bitmap per LCG

- Option B: One extension bit per pair to indicate the presence of next pair of one LCG.

For this option, the R bit in each pair of remaining time/buffer size in legacy DSR format (see Figure 6.1.3.72-1 below) is redefined as extension bit to indicate if the next pair corresponds to the same LCG as the current pair or not. Figure 2 shows one example for enhanced DSR format of Option B, wherein the extension bit value 1 or 0 indicates the next pair and the current pair belongs to the same or different LCG respectively. In this example, LCG 7, 5 and 4 are configured with multiplereporting thresholds and respectively have 2 pairs, 2 pairs and 1 pair of remaining time/buffer sizes to be reported. By redefining the R bits, enhanced DSR format according to Option B has a relatively smaller size than that according to Option A as Option A requires a dedicated pair bitmap per LCG.

Figure 6.1.3.72-1: DSR MAC CE

Figure 2 Example enhanced DSR format with one extension bit per pair/portion

- There are the following two options for enhanced DSR format:

Option A: one bitmap per LCG to indicate the presence of the pairs/portions

Option B: One extension bit per pair to indicate the presence of next pair of one LCG

- Option B has smaller overhead than Option A as Option B can reuse the “R” bit of legacy DSR format.

- RAN2 to support Option B for enhanced DSR format, i.e. enhanced DSR format based on one extension bit (e.g. redefining R bit in legacy DSR) per pair to indicate the next pair presence of the same LCG.

  - Enhanced DSR to legacy DSR dependence

In RAN2#127bis[1], there were the following agreements regarding the interaction between legacy DSR and enhanced DSR:

- FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

Then there comes the following question:

Question: For a UE supporting enhanced DSR, whether the UE should also support legacy DSR.

For an LCG which only needs DSR of single pair of remaining time/buffer size, legacy DSR may be configured in principle. If the enhanced DSR can also support configuration of DSR with single pair of remaining time/buffer size per LCG, the gNB can configure enhanced DSR for an LCG which only needs DSR of single pair of remaining time/buffer size. In such sense, an UE which support enhanced DSR does not need to support legacy DSR at the same time. In another aspect, if the enhanced DSR format according to Proposal 1 is adopted, for single pair of remaining time and buffer size for an LCG, the DSR with enhanced DSR format is of the same size as  the DSR with the legacy DSR format for the LCG. This means the legacy DSR function can be fully taken over by enhanced DSR without extra overhead. In such sense, a UE supporting enhanced DSR does not need to also support legacy DSR. To avoid the situation that any UE supporting enhanced DSR should also support legacy DSR, we propose:

- RAN2 agree that enhanced DSR can also be used to configure single pair of remaining time/buffer size per LCG.

In case a UE supports both legacy DSR and enhanced DSR, the gNB can determine whether to configure legacy DSR or enhanced DSR for an LCG, or the gNB can configure legacy DSR for one LCG while enhanced DSR for another LCG for one UE in case the UE has more than one LCG simultaneouly, which is totally up to gNB implementation.

- If a UE supports both legacy DSR and enhanced DSR, the LCGs of the UE can be respectively configured with either legacy DSR or R19 enhanced DSR, which is up to gNB implementation.

  - DSR format selection

For a UE configured with enhanced DSR for at least one LCG, two DSR formats are available: one is R18 legacy DSR format and the other is R19 enhanced DSR format. There is an issue of DSR format selection. In RAN2#127bis[1], there are the following related agreements:

- FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

There is no doubt that enhanced DSR format should be adopted when at least one LCG has more than one pair of remaining time/buffer size to be reported, regardless whether option A or B discussed in In Clause 2.2 is adopted as legacy DSR cannot accommodate  multiple pairs of delay information for one LCG:

- When any LCG configured with multiple reporting thresholds has multiple pairs of remaining time/buffer sizes to be reported, enhanced DSR format must be used.

One possible question can be:

Question: If some LCG is configured with multiple reporting thresholds but all LCGs triggered DSR has just one respective pair of remaining time/buffer sizes to be reported, whether legacy DSR format should be selected.

In this case, whether the legacy format or enhanced DSR format is selected depends on which one of them generates smaller overhead. When a LCG configured with multiple reporting thresholds actually has single pair to be reported upon DSR generation, and enhanced DSR format based on Option A (i.e. introduce pair bitmap) is adopted, the corresponding pair bitmap corresponding to the LCG should be included in the enhanced DSR, which results in the DSR size based on enhanced DSR format is relatively larger than that based on legacy DSR. However, in this case, if enhanced DSR format based on Option B (i.e. one extension bit per pair by redefining R bit) is adopted, the DSR generated according to enhanced DSR format has the same size as the one generated according to the legacy DSR format. In such sense, there is no extra overhead if a UE always selects the enhanced DSR format based on pair extension bit if any LCG of the UE is configured with enhanced DSR.

- The DSR generated according to the enhanced DSR format based on Option B above ( i.e. one extension bit per pair by redefining R bit) has the same size as that the one according to the legacy DSR format when all LCGs with triggered DSR has single pair of remaining time/buffer size.

As no additional benfit to conditionally select between legacy DSR format and enhanced DSR format, to avoid the additional complexity for DSR generation, we should keep the DSR format selection procedure as simple as possible. Hence, we propose:

- Enhanced DSR format is always selected when at least one LCG is configured with enhanced DSR.

- Conclusion

In this contribution, we have discussed the DSR enhancements and potential solutions. Based on the discussions, there are the following observations:

Observation 1	There are the following two options for enhanced DSR format:

Option A: one bitmap per LCG to indicate the presence of the pairs/portions

Option B: One extension bit per pair to indicate the presence of next pair of one LCG

Observation 2	Option B has smaller overhead than Option A as Option B can reuse the “R” bit of legacy DSR format.

Observation 3	When any LCG configured with multiple reporting thresholds has multiple pairs of remaining time/buffer sizes to be reported, enhanced DSR format must be used.

Observation 4	The DSR generated according to the enhanced DSR format based on Option B above ( i.e. one extension bit per pair by redefining R bit) has the same size as that the one according to the legacy DSR format when all LCGs with triggered DSR has single pair of remaining time/buffer size.

Based on the above discussions and the observations, we have the following proposals:

Proposal 1	RAN2 to support Option B for enhanced DSR format, i.e. enhanced DSR format based on one extension bit (e.g. redefining R bit in legacy DSR) per pair to indicate the next pair presence of the same LCG.

Proposal 2	RAN2 agree that enhanced DSR can also be used to configure single pair of remaining time/buffer size per LCG.

Proposal 3	If a UE supports both legacy DSR and enhanced DSR, the LCGs of the UE can be respectively configured with either legacy DSR or R19 enhanced DSR, which is up to gNB implementation.

Proposal 4	Enhanced DSR format is always selected when at least one LCG is configured with enhanced DSR.

- References

- RAN2-127bis, Chair notes

- RAN2-128, Chair notes