3GPP TSG-RAN WG2 Meeting #129		R2-2500352

Athens, Greece, 17th – 21st Feb. 2025

Source:	vivo

Title:	Discussion on RLC enhancement for XR

Agenda Item:	8.7.5

Document for:	Discussion and Decision

- Introduction

In RAN2#128, the following agreements were made for RLC enhancement [1]:

Agreements on RLC timely retransmissions - Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them. Agreements on avoiding unnecessary retransmissions - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced - Special handling to avoid PDCP control PDU discard is not needed. - A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. - The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

In this paper, we will discuss the details of solutions for ensuring timely RLC retransmission and avoiding unnecessary retransmission.

- Discussion

  - Enhancements to avoid unnecessary retransmissions

The following issues need to be discussed regarding the combined approach:

Issue #1: How the Tx side determines which SDUs are outdated

The first issue concerns how the Tx side decides which RLC SDUs are outdated and should no longer be retransmitted. According to TS 38.323, PDCP first performs SDU discard based on whether the discardTimer or discardTimerForLowImportance has expired for a PDCP SDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, then the transmitting PDCP entity will send a discard indication to RLC.

TS 38.323

5.3	SDU discard

When the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU.

When the discardTimer or discardTimerForLowImportance expires for a PDCP SDU, the transmitting PDCP entity shall:

-	if pdu-SetDiscard is configured:

-	discard all PDCP SDUs belonging to the PDU Set to which the PDCP SDU belongs along with the corresponding PDCP Data PDUs;

NOTE 1:	PDCP SDUs subsequently received from upper layers are also discarded if they belong to the PDU Set.

-	else:

-	discard the PDCP SDU along with the corresponding PDCP Data PDU.

If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

TS 38.323

5.3	SDU discard

When the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU.

When the discardTimer or discardTimerForLowImportance expires for a PDCP SDU, the transmitting PDCP entity shall:

-	if pdu-SetDiscard is configured:

-	discard all PDCP SDUs belonging to the PDU Set to which the PDCP SDU belongs along with the corresponding PDCP Data PDUs;

NOTE 1:	PDCP SDUs subsequently received from upper layers are also discarded if they belong to the PDU Set.

-	else:

-	discard the PDCP SDU along with the corresponding PDCP Data PDU.

If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

In our view, the discard indication from PDCP to RLC can be reused by the RLC Tx side to identify outdated SDUs.

Proposal 1: The Tx side of RLC determines outdated SDU(s) based on the discard indication from the Tx side of PDCP.

Issue #2: How the local timer at the Rx side functions

The second issue concerns the operation of the local timer on the Rx side. In the last meeting, it was agreed that a new RLC timer be introduced and the timer starts when a gap is detected at the RLC layer. However, the details of the timer need to be discussed.

In our view, there can be two approaches for the new RLC timer on the Rx side:

- Approach 1: per RLC SDU timer

- Approach 2: per RLC entity timer, similar to the t-Reassembly timer

For approach 1, each RLC SDU will maintain its timer, which may start under the following two cases as illustrated in Figure 1:

- Case 1:  A segment of the corresponding RLC SDU is received, but not all the bytes of the RLC SDU have been received.

- Case 2: An RLC SDU or an RLC SDU segment with a higher SN than that of the concerned RLC SDU is received.

Figure 1. Per RLC SDU Rx timer starting occasion

When the Rx timer for the corresponding RLC SDU expires, the Rx side of the RLC entity treats the corresponding RLC SDU as ACK and updates the Rx window. Subsequently, the Rx side informs the Tx side by including fake ACK in the STATUS report, in accordance with legacy behavior. The Rx timer for the corresponding RLC SDU stops when the RLC SDU is fully received.

For approach 2, the Rx timer is maintained on a per RLC entity basis, meaning that multiple RLC SDUs may be associated with a single timer, and only one timer is active for the RLC entity at any given time. The conditions for starting and stopping the new Rx timer can be the same as those for t-Reassembly. The only difference between the new Rx timer and t-Reassembly lies in the actions taken when the timer expires. When t-Reassembly expires, it triggers a STATUS report; however, when the new Rx timer expires, it results in the abandonment of SDUs and an update of the Rx window and then triggers a STATUS report.

In our view, both options can work well. However, approach 1 may yield better performance than approach 2 in some cases. For approach 2, the timer is maintained on a per-RLC entity basis, meaning that only one timer can be running at any given time. This limitation prevents the initiation of a new timer even if another SN gap appears. Consequently, Approach 1 may allow for the local timer to start earlier than Approach 2 in certain scenarios.

The differences between Approaches 1 and 2 are illustrated in the figure below. As shown, in Approach 1, the timer for SN5 can run concurrently with the timer for SN3. In contrast, in Approach 2, the timer for SN5 can only start after the timer for SN3 has expired or been stopped. With the earlier initiation of the local timer, Approach 1 can report the status of SN5 more quickly than Approach 2.

Figure 2. Comparison of per RLC SDU timer and per RLC entity timer

Although Approach 1 may provide better performance than Approach 2, it is important to note that the current RLC specification does not include a per-RLC SDU timer. Introducing a per-SDU timer could significantly increase the complexity of the specification and the implementation for UE.

We present the pros and cons of the two approaches in the following table. RAN2 should discuss these approaches and select the most suitable method for determining which SDUs should be abandoned on the Rx side.

Table 1: Pros and Cons of different approaches for the local RLC timer

| Approach description | Pros | Cons |
| --- | --- | --- |
| Per RLC SDU timer | - Detection of obsolete SDU(s) quickly in some cases and can provide faster status report | - More UE implementation complexity and specification impact |
| Per RLC entity timer | - Less UE implementation complexity and specification impact | - May delay the detection of obsolete SDU(s) in some cases |

Proposal 2: RAN2 discusses the following two approaches for local timer at the Rx side to determine which SDUs should be abandoned.

- Approach 1: Per RLC SDU timer

- Approach 2: Per RLC entity timer, similar to the t-Reassembly timer

Issue #3: Whether indication is sent from Tx to Rx when the Tx decides the outdated SDU(s)

In the last meeting, the Tx to Rx indicator for the combined approach was discussed, and the following agreements were made.

RAN2#128 Agreement

- There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer

- Unless critical issue is identified, no Tx to Rx indication will be introduced

RAN2#128 Agreement

- There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer

- Unless critical issue is identified, no Tx to Rx indication will be introduced

In our view, the Tx to Rx indication is critical for keeping the timely synchronization between the Tx window and the Rx window. Without the indication, stalling of the Tx and Rx window may occur in some situations.

For example, when PDU set discard is enabled, all PDUs in the entire PDU set will be discarded if any one of the PDUs expires. At this time, the Tx RLC entity may have already stopped (re)transmission for amounts of obsolete RLC SDUs but has not received the STATUS report from the Rx side for a long time. This is because the local timer on the Rx side has not started due to no gap detected (refer to Figure 3 below). In this scenario, both the Tx window and the Rx window may stall and delay the transmission of the subsequent packets. With the indication from Tx to Rx, we can avoid this window stalling.

Figure 3. Window stalling due to without Tx to Rx indicator

Observation 1: Without Tx to Rx indication, stalling of the Tx and Rx window may occur, particularly in cases where PDU set discard is enabled.

Additionally, when the Tx side decides to stop (re)transmission of the RLC SDUs, the Rx side may still send STATUS reports with NACK for the corresponding RLC SDUs before the local timer on the Rx side expires. In practice, the Tx side will ignore these NACK indications for those RLC SDUs and will not perform (re)transmission. Therefore, these STATUs reports waste radio resources and should be avoided. With the Tx to Rx indication, the Rx can promptly identify the outdated SDUs based on the indication and send STATUS report with positive ACK without waiting for the local timer on the Rx side to expire.

Observation 2: Without the Tx to Rx indication, unnecessary STATUS reports with NACK indication for obsolete RLC SDUs will be sent to the Tx side, leading to resource wastage.

Proposal 3: Indication about obsolete SDUs from Tx side to Rx side is needed for the combined approach.

As for the format of the indication from the Tx to the Rx side, we believe a simple RLC control PDU carrying the sets of SNs or SN range of the outdated RLC SDUs is enough.

Proposal 4: The indication from the Tx side to the Rx side should include the sets of SNs or the SN range associated with the outdated RLC SDUs

  - Enhancements to ensure timely RLC retransmissions

2.2.1. Autonomous retransmission

In the RAN2#128 meeting, it was agreed to support autonomous retransmission to ensure timely retransmission. We will discuss the procedures for how autonomous retransmission works in the following sections.

Issue #1: When to trigger autonomous retransmission

In previous meetings, the trigger conditions for autonomous retransmission were discussed, and two main approaches were proposed:

- Approach 1: Autonomous retransmission is triggered for an RLC SDU if the remaining time falls below a specified threshold.

- Approach 2: Autonomous retransmission is triggered for an RLC SDU based on HARQ feedback (e.g., when the number of HARQ failures exceeds a certain threshold).

From our perspective, Approach 1 is preferred because the primary motivation for autonomous retransmission is to ensure timely retransmission. Thus, triggering autonomous retransmission based on the remaining time is a straightforward solution. Additionally, UL autonomous retransmission based on HARQ feedback is not feasible according to the current RAN1 specification, as there is no HARQ feedback in the uplink. The UE cannot determine whether the HARQ transmission was successful or if the network simply abandoned the HARQ retransmission when the NDI is toggled. Therefore, implementing autonomous retransmission in this manner would impact the RAN1 specification.

Observation 3: Autonomous retransmission based on UL HARQ feedback is not feasible according to the existing HARQ mechanism.

Proposal 5: Autonomous retransmission should be triggered when the remaining time of an RLC SDU falls below a specified threshold.

Issue #2: How the autonomous retransmission procedure works

Similar to the existing maxRetxThreshold in the ARQ procedure, a new RRC parameter for the maximum number of autonomous retransmissions is required. This parameter will be related to different QoS requirements and can be configured per RLC bearer by the network.

Proposal 6: The maximum number of autonomous retransmissions should be configured per RLC bearer by the network.

Additionally, a new counter is needed for autonomous retransmission to track the number of retransmissions. It is essential that each instance of autonomous retransmission ensures that all bytes of the associated RLC SDU that have not been positively acknowledged are delivered to the lower layer.

Proposal 7: The autonomous retransmission counter should increment by 1 each time all bytes of the associated RLC SDU that have not been positively acknowledged are delivered to the lower layer.

When the RLC SDU is positively acknowledged or when the maximum number of autonomous retransmissions configured by the network is reached, the RLC entity should stop autonomous retransmissions for the corresponding RLC SDU.

Proposal 8: The transmitting RLC entity should stop autonomous retransmission for the associated RLC SDU when it is positively acknowledged in the STATUS report or when the maximum number of autonomous retransmissions is reached.

Issue #3: Autonomous retransmission activation/deactivation

In XR, one of the common cases of late PDUs is congestion. Thus, the congestion scenario is a primary scenario that requires optimization. In a congested scenario, autonomous retransmission may have a negative impact on the system and worsen the congestion, as the transmitting RLC entity may perform unnecessary RLC retransmission, leading to the wastage of radio resources, even if the packet has already been successfully transmitted.

Observation 4: Autonomous retransmission may delay the transmission of the subsequent packets, and may potentially lead to more packets as delay-critical data from non-delay-critical data especially in congestion scenario.

Therefore, an activation/deactivation mechanism for autonomous retransmission should be supported to ensure that it can be deactivated during periods of congestion.

Proposal 9: A new MAC CE is introduced to activate/deactivate the autonomous retransmission mechanism.

2.2.2. Polling enhancement

Polling enhancement is also agreed to be a direction for ensuring timely retransmission. As for the details of polling enhancement, the triggering of polling mainly includes the following four cases in the current RLC specification [2] below:

- Case 1: PDU_WITHOUT_POLL >= pollPDU or BYTE_WITHOUT_POLL >= pollByte

- Case 2: Buffers become empty

- Case 3: No data to transmit due to window stalling

Case 4: t-PollRetransmit expires

TS 38.322

5.3.3.2	Transmission of a AMD PDU

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall:

-	increment PDU_WITHOUT_POLL by one;

-	increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU;

-	if PDU_WITHOUT_POLL >= pollPDU; or

-	if BYTE_WITHOUT_POLL >= pollByte:

-	include a poll in the AMD PDU as described below.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or

-	if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

-	include a poll in the AMD PDU as described below.

…

5.3.3.4	Expiry of t-PollRetransmit

Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or

-	if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling):

-	consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or

-	consider any RLC SDU which has not been positively acknowledged for retransmission.

-	include a poll in an AMD PDU as described in clause 5.3.3.2.

TS 38.322

5.3.3.2	Transmission of a AMD PDU

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall:

-	increment PDU_WITHOUT_POLL by one;

-	increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU;

-	if PDU_WITHOUT_POLL >= pollPDU; or

-	if BYTE_WITHOUT_POLL >= pollByte:

-	include a poll in the AMD PDU as described below.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or

-	if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

-	include a poll in the AMD PDU as described below.

…

5.3.3.4	Expiry of t-PollRetransmit

Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or

-	if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling):

-	consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or

-	consider any RLC SDU which has not been positively acknowledged for retransmission.

-	include a poll in an AMD PDU as described in clause 5.3.3.2.

In our view, there are mainly two alternatives for enhancing polling to ensure timely retransmission of delay-critical SDUs.

- Alt 1: The UE triggers polling when the remaining time of RLC SDU(s) falls below a threshold

- Alt 2: The network configures dedicated polling thresholds and polling retransmitting timer for delay-critical RLC SDUs

For Alt 1, a new type of polling trigger mechanism is introduced. When the remaining time of an RLC SDU falls below a threshold configured by the network, the UE will trigger polling to obtain a fast status report from the network regarding the status of the delay-critical RLC SDU. Additionally, for Alt 1, it may be necessary to introduce a prohibit timer for this polling trigger to avoid excessively frequent polling. This is particularly important in cases where there are many delay-critical RLC SDUs or when the UE does not receive timely status feedback from the network. Otherwise, the UE will continue to trigger polling until there are no more delay-critical RLC SDUs.

For Alt 2, the legacy polling mechanism is reused as much as possible (i.e., case 1 and case 4). In Alt 2, dedicated polling PDU/BYTE thresholds and polling retransmission timer for RLC SDUs with remaining time below a threshold are introduced. When there are RLC SDUs with remaining time below a threshold, the UE will utilize these dedicated polling thresholds and retransmission timer for making polling triggering decisions. Once all the RLC SDUs with remaining time below a threshold have been transmitted or discarded, the UE will revert to using the legacy polling thresholds and retransmission timer.

From our perspective, Alt 2 reuses the legacy polling mechanism as much as possible and is more flexible, since the threshold and timer values configured by the network can ensure timely retransmission for RLC SDUs with remaining time below a threshold while also avoiding excessive signaling overhead due to polling.

Proposal 10: Dedicated polling PDU/BYTE thresholds and polling retransmission timer are introduced for RLC SDUs with remaining time below a threshold.

2.2.3. Coexistence of autonomous retransmission and polling enhancement

In the last meeting, two types of solutions for the coexistence of autonomous retransmission and polling enhancement were discussed.

Compromise solutions discussed in RAN2#128:

- If the remaining time drops below a threshold, poll is triggered. If SR is not received within a time, then Tx can autonomously retransmit the delay critical packet.

- Have both autonomous and polling enhancement.

Compromise solutions discussed in RAN2#128:

- If the remaining time drops below a threshold, poll is triggered. If SR is not received within a time, then Tx can autonomously retransmit the delay critical packet.

- Have both autonomous and polling enhancement.

It was finally agreed that the network can configure either or both of the two solutions (i.e., solution 2).

RAN2#128 Agreement

- Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

RAN2#128 Agreement

- Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

According to the agreement, two separate UE capabilities should be defined for autonomous retransmission and polling enhancement.

Proposal 11: Separate UE capabilities are introduced for autonomous retransmission and polling enhancement.

Since both autonomous retransmission and polling enhancement are triggered when the remaining time of RLC SDU(s) is lower than a threshold as above, it needs to be discussed whether to introduce one common threshold for both solutions or two separate thresholds for each solution.

Proposal 12: RAN2 to discuss whether to introduce one common threshold or two separate thresholds for polling enhancement and autonomous retransmission.

Case 1: one common threshold is introduced for both polling enhancement and autonomous retransmission

If one common threshold is introduced for both polling enhancement and autonomous retransmission, we believe that the current remainingTimeThreshold for R18 DSR can be reused, and there would be no need to introduce a new threshold.

Proposal 13a: If one common threshold for triggering polling enhancement and autonomous retransmission is agreed in Proposal 12, the remainingTimeThreshold for R18 DSR can be reused.

If one common threshold is introduced for both polling enhancement and autonomous retransmission and the network configures both polling enhancement and autonomous retransmission for the UE, there are two potential approaches for UE to how to work with these two mechanisms:

- Approach 1: It is left to UE implementation to choose whether to use polling enhancement or autonomous retransmission.

- Approach 2: A combined solution is supported (e.g., including a polling bit in the autonomous retransmission RLC SDU for fast positive acknowledgment feedback).

Proposal 13b: If one common threshold for triggering polling enhancement and autonomous retransmission is agreed in Proposal 12 and both solutions are configured for the UE, RAN2 to discuss the following two approaches of working procedure:

- Approach 1: It is up to UE implementation to choose whether to use polling enhancement or autonomous retransmission.

- Approach 2: A combined solution (e.g., polling bit is included in the autonomous retransmission RLC SDU for fast positive ack feedback).

Case 2: two separate thresholds are introduced for polling enhancement and autonomous retransmission

If two separate thresholds are introduced for polling enhancement and autonomous retransmission, in reasonable network configuration, the threshold for enabling autonomous retransmission is expected to be set lower than the threshold for enabling polling enhancement. Otherwise, the threshold for enabling polling enhancement is useless. Autonomous retransmission can lead to excessive retransmissions, which may reduce overall system capacity. Therefore, it should only be used when the RLC SDU(s) are particularly delay-critical. If the RLC SUD(s) is not particularly delay-critical, polling enhancement should be preferred to avoid resource waste caused by autonomous retransmission.

Consequently, when both autonomous retransmission and polling enhancement are configured by the network, the threshold for enabling autonomous retransmission should be lower than the threshold for enabling polling enhancement (refer to Figure 4 below). Which threshold (i.e., the threshold for polling enhancement or the threshold for autonomous retransmission) will reuse the remainingTimeThreshold in R18 DSR is FFS.

Figure 4. Different thresholds for enabling polling enhancement and autonomous retransmission

Proposal 14: If two thresholds are agreed in Proposal 12, the threshold for enabling autonomous retransmission is expected to be lower than the threshold for enabling polling enhancement when both solutions are configured for the UE. Autonomous retransmission and polling enhancement are triggered when the remaining time of RLC SDU is below the corresponding threshold, respectively. Which solution will reuse the remainingTimeThreshold in R18 DSR is FFS.

- Conclusion

In this contribution, we discuss the two directions of RLC AM enhancements, and we have the following observations and proposals:

Enhancements to avoid unnecessary retransmissions

Observation 1: Without Tx to Rx indication, stalling of the Tx and Rx window may occur, particularly in cases where PDU set discard is enabled.

Observation 2: Without the Tx to Rx indication, unnecessary STATUS reports with NACK indication for obsolete RLC SDUs will be sent to the Tx side, leading to resource wastage.

Proposal 1: The Tx side of RLC determines outdated SDU(s) based on the discard indication from the Tx side of PDCP.

Proposal 2: RAN2 discusses the following two approaches for local timer at the Rx side to determine which SDUs should be abandoned.

- Approach 1: Per RLC SDU timer

- Approach 2: Per RLC entity timer, similar to the t-Reassembly timer

Proposal 3: Indication about obsolete SDUs from Tx side to Rx side is needed for the combined approach.

Proposal 4: The indication from the Tx side to the Rx side should include the sets of SNs or the SN range associated with the outdated RLC SDUs

Enhancements to ensure timely RLC retransmission(s)

Observation 3: Autonomous retransmission based on UL HARQ feedback is not feasible according to the existing HARQ mechanism.

Observation 4: Autonomous retransmission may delay the transmission of the subsequent packets, and may potentially lead to more packets as delay-critical data from non-delay-critical data especially in congestion scenario.

Proposal 5: Autonomous retransmission should be triggered when the remaining time of an RLC SDU falls below a specified threshold.

Proposal 6: The maximum number of autonomous retransmissions should be configured per RLC bearer by the network.

Proposal 7: The autonomous retransmission counter should increment by 1 each time all bytes of the associated RLC SDU that have not been positively acknowledged are delivered to the lower layer.

Proposal 8: The transmitting RLC entity should stop autonomous retransmission for the associated RLC SDU when it is positively acknowledged in the STATUS report or when the maximum number of autonomous retransmissions is reached.

Proposal 9: A new MAC CE is introduced to activate/deactivate the autonomous retransmission mechanism.

Proposal 10: Dedicated polling PDU/BYTE thresholds and polling retransmission timer are introduced for RLC SDUs with remaining time below a threshold.

Proposal 11: Separate UE capabilities are introduced for autonomous retransmission and polling enhancement.

Proposal 12: RAN2 to discuss whether to introduce one common threshold or two separate thresholds for polling enhancement and autonomous retransmission.

Proposal 13a: If one common threshold for triggering polling enhancement and autonomous retransmission is agreed in Proposal 12, the remainingTimeThreshold for R18 DSR can be reused.

Proposal 13b: If one common threshold for triggering polling enhancement and autonomous retransmission is agreed in Proposal 12 and both solutions are configured for the UE, RAN2 to discuss the following two approaches of working procedure:

- Approach 1: It is up to UE implementation to choose whether to use polling enhancement or autonomous retransmission.

- Approach 2: A combined solution (e.g., polling bit is included in the autonomous retransmission RLC SDU for fast positive ack feedback).

Proposal 14: If two thresholds are agreed in Proposal 12, the threshold for enabling autonomous retransmission is expected to be lower than the threshold for enabling polling enhancement when both solutions are configured for the UE. Autonomous retransmission and polling enhancement are triggered when the remaining time of RLC SDU is below the corresponding threshold, respectively. Which solution will reuse the remainingTimeThreshold in R18 DSR is FFS.

- References

- R2_128bis_ChairNotes.

- TS 38.322 v18.1.0, Radio Link Control (RLC) protocol specification (Release 18).