3GPP TSG-RAN WG2 Meeting #129		R2-2500354

Athens, Greece, 17th – 21st Feb. 2025

Source:	vivo

Title:	Discussion on MAC CE impact for asymmetric DL sTRP/UL mTRP scenarios

Agenda Item:	8.12.2

Document for:	Discussion and Decision

- Introduction

In RAN2#127bis meeting, an LS on RRC and MAC CE impacts for MIMO from RAN1 was received in [1]. In the LS, RAN1 informs RAN2 the MAC CE impact for pathloss offset update and the corresponding RAN1 agreements:

Agreement (RAN1#117) For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). “MAC-CE impact: MAC-CE updates the PL offset value(s) configured in one or more joint TCI states or UL TCI states”

In RAN2#128 meeting, following agreements has been made for asymmetric DL sTRP/UL mTRP:

=> New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID. => Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB. => In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

Besides, in RAN1#118 [2] and RAN1#118bis [3] meeting, RAN1 has concluded value range of PL offset which is related to MAC CE design:

Agreement (RAN1#118) For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB. - FFS: Extending the range to lower than -10dB - Step size is 4dB Agreement (RAN1#118bis) - The lower limit of the value range of PL offset is extended to -12 dB.

Besides the PL offset update MAC CE, TCI state activation MAC CE design should also be discussed considering that RAN1 has concluded new TCI state activation behavior：

Agreement (RAN1#116) For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate DL/UL TCI state mode of Rel-17/18 unified TCI framework can be configured for both FR1 and FR2. - Joint TCI state mode can be configured at least for FR1 Conclusion (RAN1#117) For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

In this contribution, we will discuss the RAN2 aspects related to these two MAC CE impacts to enable asymmetric DL sTRP/UL mTRP scenarios.

- Discussion

  - Pathloss offset update MAC CE

In RAN2#128 meeting, it was agreed to introduce a new MAC for PL offset update for asymmetric DL sTRP/UL mTRP. Besides, some details were also agreed: absolute value of PL offset is indicated in the new MAC CE; PL offset value can be updated for any configured TCI states with RRC configured PL offset. Before giving a detailed design on MAC CE format, following remaining issues should be discussed:

- A. Whether the PL offset update MAC CE is applied to a single serving cell (i.e. each MAC CE for one serving cell) or multiple serving cells? Whether need to include the DL/UL BWP ID in the MAC CE?

- B. Whether one MAC CE is for only one joint/UL TCI-state, or one MAC CE could be for multiple Joint/UL TCI-states.

- C. The TCI state in the MAC CE is applicable to which channel/signaling.

Considering that TCI states are configured and activated per serving cell per BWP, a PL offset update MAC CE which is used to update PL offset for TCI states should also be applied for only one BWP of a serving cell. And PL offset update for more than one serving cells can be achieved by separate MAC CEs indicating different serving cell IDs and DL/UL BWP IDs. Furthermore, a 2-bit BWP indication is enough for the UL/DL BWP ID. For joint TCI state indication mode, since the TCI states are configured only in DL BWP, the 2-bit BWP indication indicates one DL BWP with the configured TCI states for PL offset update. When separate TCI state mode is configured, UL TCI states consisting the PL offset value and DL TCI states are configured respectively in UL BWP and DL BWP, so the 2-bit BWP indication indicates the UL BWP with the configured UL TCI states for PL offset update.

- One serving cell ID is indicated in the PL offset update MAC CE.

- A 2-bit BWP indication field is needed to indicate DL BWP for joint TCI state mode or UL BWP for separate TCI state mode in the PL offset update MAC CE.

In RAN2#128 meeting, it was agreed that absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB. Thus, a 5-bit indication is needed to indicate the absolute PL offset value.

- A 5-bit absolute PL offset value field is indicated for each TCI state as below in the PL offset update MAC CE:

| PL offset value indication | PL offset value |
| --- | --- |
| 0 | -12 |
| 1 | -8 |
| 2 | -4 |
| … | … |
| 18 | 60 |
| 19~32 | Reserved |

Regarding the MAC CE design, to update PL offset values of any configured TCI states, the MAC CE should indicate the target TCI states and associated PL offset values together to inform UE the PL offset value of each target TCI state, i.e. the MAC CE indicates N set(s) of {TCI state ID, PL offset value}, where N is larger than one, and up to the number of configured TCI states.

- The PL offset update MAC CE indicates N set of {TCI state ID, PL offset value}, where N is larger than one, and up to the number of configured TCI states.

Besides MAC CE design, whether there is any impact on PHR triggering/reporting should also be discussed. PL offset value is pathloss offset to determine total pathloss for the UL-only TRP, which is part of pathloss. In TS 38.321, PHR is triggered when path loss exceeds one threshold. Since PL offset value is consisted in the pathloss, no further enhancement is needed for PHR triggering. Similarly, PHR reporting also includes PL offset value naturally because it is consisted in pathloss calculation part.

- RAN2 assumes there is no impact on PHR triggering and reporting considering the PL offset.

  - TCI state activation/deactivation MAC CE

Based on the conclusion drawn in RAN1 as below,

- When Rel-17 unified TCI/ICBM is configured:

  - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied.

  - For FR2: {one DL TCI state + one UL TCI state} can be applied.

- When Rel-18 unified TCI is configured:

  - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied.

  - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

The existing “Unified TCI state activation/deactivation MAC CE” introduced in Rel-17 can be directly reused for asymmetric DL sTRP/UL mTRP deployment scenario when Rel-17 unified TCI/ICBM is configured, because it does activate codepoint(s) each mapping to one joint TCI state for joint TCI state mode or a full set or sub-set of {one DL TCI state, one UL TCI state} for sperate TCI state mode.

For asymmetric DL sTRP/UL mTRP deployment scenario when Rel-18 unified TCI is configured, however, “Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States” and “Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate TCI States” introduced in Rel-18 cannot directly achieve the required asymmetric number of DL TCI state and UL TCI states, i.e., the TCI state codepoint mapping to the full set or subset of the following cases:

- For FR1,

  - {one joint TCI state for DL, two joint TCI states for UL} for joint TCI state mode

  - {one DL TCI state, two UL TCI state} for separate TCI state mode

- For FR2,

  - {one DL TCI state, two UL TCI state} for separate TCI state mode

This is because {two joint TCI states} or {two DL TCI states, two UL TCI states} are always kept if the Rel-18 MAC CE is used, according to the following cited paragraph in TS 38.214:

“When a UE is configured with dl-OrJointTCI-StateList and is having two indicated TCI-states, if the UE receives a TCI codepoint mapped with a sub-set of first and second TCI-State(s) and/or a sub-set of first and second TCI-UL-State(s), the UE shall update the first/second TCI-State(s) and/or first/second TCI-UL-State(s) mapped to the TCI codepoint, when applicable, and keep the previously indicated first/second TCI-State(s) and/or first/second TCI-UL-State(s) that is/are not updated by the TCI codepoint.”

To support single joint/DL TCI state for DL, further clarification is required because it is different from Rel-18 TCI state framework.

Whether to reuse two indicated joint/DL TCI states framework in Rel-18 for the asymmetric DL sTRP/UL mTRP deployment scenario should be further discussed because it may go beyond the UE capability, even though in some cases single joint/DL TCI state can be applied for DL transmission by following configurations or indications, for example:

- For PDCCH, apply the first or the second TCI state of the two indicated joint/DL TCI states according to applyIndicatedTCI-State configured in ControlResourceSet IE

- For PDSCH scheduled or activated by DCI format 1_0,

  - apply the first or the second TCI state of the two indicated joint/DL TCI states according to applyIndicatedTCI-StateDCI-1-0 configured in BWP-DownlinkDedicated IE;

  - If the UE is not configured with applyIndicatedTCI-StateDCI-1-0, apply the first indicated joint/DL TCI state.

- For PDSCH scheduled or activated by DCI format 1_1/1_2,

  - When the UE is configured with tciSelection-PresentInDCI, apply the first or the second TCI state of the two indicated joint/DL TCI states according to the indication of TCI selection field in DCI format 1_1/1_2;

  - If the UE is not configured with tciSelection-PresentInDCI, apply both indicated joint/DL TCI states;

  - Note that support of DCI format 1_1 and if supported 1_2 configured with TCI selection field is optional with capability signaling.

The drawbacks of directly reusing Rel-18 Unified TCI States Activation/Deactivation MAC CE without any revision for Rel-19 asymmetric DL sTRP/UL mTRP deployment scenario are:

- Additional signaling is needed to distinguish between Rel-18 sDCI based mTRP and Rel-19 asymmetric DL sTRP/UL mTRP

- For joint TCI state mode, indicating two same joint TCI states does not work because same joint TCI states have to be applied to UL transmissions for different TRPs.

- For both joint TCI state mode and separate TCI state mode, indicating two different joint/DL TCI states,

  - Does not work for one or multiple PDSCH reception scheduled/activated by DCI format 1_1/1_2 without TCI selection field, e.g., when the UE does not support the capability of TCI selection field, because both different indicated joint TCI states should be applied for the PDSCH.

  - TCI selection field in DCI format 1_1/1_2 is a must and which is redundant even if the UE supports the capability of TCI selection field since there is only one indicated DL TCI state.

- For separate TCI state mode, indicating two same DL TCI states leads to some restricted or even undefined DL transmission schemes for UEs not supporting the capability of TCI selection field, e.g.,

  - When two DL TCI states are indicated and DMRS ports within two CDM groups are indicated in the DCI, which is the SDM scheme of Rel-16 sDCI-based mTRP, only single CW can be scheduled with transmission layers limited to one of 1+1, 1+2, 2+1, 2+2; in addition, DMRS ports within two CDM groups occupy more REs compared to DMRS ports within one CDM group; what’s more, according to RAN1 spec, PDSCH repetition is not allowed.

  - RAN1 does not specify the transmission scheme when two DL TCI states are indicated and DMRS ports within one CDM groups are indicated in the DCI.

To achieve a single joint/DL TCI state for DL and two joint/UL TCI states for UL, following options can be considered:

- Option 1: Introduce a new MAC CE with a full set or subset of {two joint TCI states} mapped to all codepoints for joint TCI state mode, where one of the two joint TCI states can be applied to DL, and both joint TCI states can be applied to UL; and introduce a new MAC CE with a full set or subset of {one DL TCI state, two UL TCI states} mapped to all codepoints for separate TCI state mode.

- Option 2: Revise Rel-18 “Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States” and “Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate TCI States” with a signaling to indicate that a full set or subset of {two joint TCI states} are mapped to all codepoints for joint TCI state mode, where one of the two joint TCI states can be applied to DL, and both joint TCI states can be applied to UL; or a full set or subset of {one DL TCI state, two UL TCI states} mapped to all codepoints for separate TCI state mode.

- Option 3: Revise Rel-18 “Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States” with an indication per codepoint to indicate whether the codepoint maps to a full set or subset of {two joint TCI states}, where one of the two joint TCI states can be applied to DL, and both joint TCI states can be applied to UL, or the codepoint maps to a full set or subset of {two joint TCI states} where both joint TCI states can be applied to DL and UL; and Revise Rel-18 “Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate TCI States” with an indication per codepoint to indicate whether the codepoint maps to a full set or subset of {one DL TCI state, two UL TCI states}, or the codepoint maps to a full set or subset of {two DL TCI states, two UL TCI states}.

Send an LS to RAN1 to check whether to design new MAC CE(s) for the TCI States Activation/Deactivation or to extend Rel-18 Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States and Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate TCI States for the asymmetric DL sTRP/UL mTRP scenarios.

  - Questions to RAN1 on MAC CE for asymmetric DL sTRP/UL mTRP

According to the above discussion, we think the below questions need to be checked with RAN1 on asymmetric DL sTRP/UL mTRP:

Send an LS to RAN1 to check the below questions:

For PL offset update MAC CE:

- A. Whether one MAC CE is for only one joint/UL TCI-state, or one MAC CE could be for multiple Joint/UL TCI-states.

- B. The TCI state in the MAC CE is applicable to which channel/signaling.

For TCI state activation/deactivation MAC CE:

- C. Whether introducing a new MAC CE or reusing the legacy TCI state activation/deactivation MAC CE.

General:

- D. Whether there is any impact on PHR triggering/reporting.

- Conclusion

In this contribution, we discuss the RAN2 aspects on MAC CE impact for asymmetric DL sTRP/UL mTRP scenarios. Based on the discussion, we have the following observations and proposals.

- One serving cell ID is indicated in the PL offset update MAC CE.

- A 2-bit BWP indication field is needed to indicate DL BWP for joint TCI state mode or UL BWP for separate TCI state mode in the PL offset update MAC CE.

- A 5-bit absolute PL offset value field is indicated for each TCI state as below in the PL offset update MAC CE:

| PL offset value indication | PL offset value |
| --- | --- |
| 0 | -12 |
| 1 | -8 |
| 2 | -4 |
| … | … |
| 18 | 60 |
| 19~32 | Reserved |

- The PL offset update MAC CE indicates N set of {TCI state ID, PL offset value}, where N is larger than one, and up to the number of configured TCI states.

- RAN2 assumes there is no impact on PHR triggering and reporting considering the PL offset.

Send an LS to RAN1 to check whether to design new MAC CE(s) for the TCI States Activation/Deactivation or to extend Rel-18 Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States and Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate TCI States for the asymmetric DL sTRP/UL mTRP scenarios.

Send an LS to RAN1 to check the below questions:

For PL offset update MAC CE:

- A. Whether one MAC CE is for only one joint/UL TCI-state, or one MAC CE could be for multiple Joint/UL TCI-states.

- B. The TCI state in the MAC CE is applicable to which channel/signaling.

For TCI state activation/deactivation MAC CE:

- C. Whether introducing a new MAC CE or reusing the legacy TCI state activation/deactivation MAC CE.

General:

- D. Whether there is any impact on PHR triggering/reporting.

- References

- R2-2407906, LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5 (R1-2407285; contact: Samsung)

- Draft_Minutes_report_RAN1#118

- Draft_Minutes_report_RAN1#118bis