3GPP TSG-RAN WG2 Meeting #129	R2-2500364

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.10.2.2

Source:	Nokia

Title:	MRO for CHO with candidate SCG

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core – Rel-19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were reached on the topic of MRO for CHO with candidate SCG with regard to information to be logged in various SON reports:

RAN2#126 - RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

RAN2#127 - UE includes following information in RLF report:

b.        Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

c.        Measurement results of PCells and PSCells.

RAN2#127bis

UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS.

Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

RAN2#128

RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements.

Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and 	SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may 		already make the UE log this.

This paper discusses further details on information logging in SON reports for MRO for CHO w/ candidate SCG and also provides some comments to the LS RAN3 sent to RAN2.

# 2	Enhancements of MRO reports

The agreements reached by RAN2 so far cover all SON reports: RLF report, SHR and SCGFailureInformation. We summarize below the information agreed to be logged in each report:

RLF report

- Time regarding condition fulfillment

- Measurement result of PCells and PSCells

- time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition

- elapsed time between the point in time of the first fulfilled condition and RLF

- first met execution condition

SHR

- time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell).

- Measurement result of PCells and PSCells

SCGFailureInformation

- time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell).

- Measurement result of PCells and PSCells

Some information seems to still be missing from SHR/SPR and SCGFailureInformation messages in order to cover all the possible failure and near failure scenarios, e.g. failure on MCG or SCG before/during/shortly after CHO with candidate SCG execution and successful execution.

As SCG failure may also happen in a similar way as MCG failure during CHO with candidate SCG, we propose to align the content of SCGFailureInformation with that of RLF report by adding an indication of the first fulfilled execution condition (CHO or CPAC), if any.

Proposal 1: In case RLF is detected on the SCG before, during or shortly after successful completion of CHO with candidate SCG, the SCGFailureInformation message should be enhanced with an indication of the first fulfilled execution condition (CHO or CPAC), if any.

Because no near failure scenarios were defined in RAN3 until now, RAN2 also did not clearly decide on how near failure related information is to be conveyed, i.e. SHR, SPR or both. Irrespective of this, it may be useful to inform the network also in these case about the first fulfilled execution condition. Knowing the time gap alone is insufficient for the network to decide on any optimization actions. This is especially important if only one message is used (e.g. SHR) irrespective of which execution condition was fulfilled first, as the network has no way of knowing this information. Of course, another option would be to allow the usage of both SPR and SHR, possibly with a new trigger, depending on which execution condition was lagging in fulfillment. I.e. if the CHO execution condition was fulfilled first and the CPAC execution condition was fulfilled later, SPR would be generated, and vice versa. In this case, the existence of the report itself would indicate the first fulfilled execution condition.

Proposal 2: RAN2 to discuss and decide on how the indication of the first fulfilled execution condition (CHO or CPAC) should be conveyed to the network for near failure scenarios:

- Explicitly: SHR is enhanced with indication of the first fulfilled execution condition or

- Implicitly: SHR or SPR is generated depending on which execution condition was fulfilled first (SPR is generated if CHO is fulfilled first and CPAC was fulfilled later).

# 3	Comments on LS from RAN3 (R2-2500026/R3-247908)

During the last meeting, an LS was sent from RAN3 to RAN2 [R2-2500026/R3-247908] containing the following wrt to CHO with candidate SCG:

(1) Regarding MRO for CHO with candidate SCG(s):

RAN3 concluded that the following information can be reported by the UE in RLF Report for failure reason detection and optimization:

- The type of the first fulfilled execution condition (e.g. CPAC or CHO). Fulfilment means all events of the type are met.

- Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

- Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

- The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

ACTION: 	RAN3 kindly asks RAN2 to take the above into consideration and update their specifications as needed.

After examining the list proposed by RAN3 in their LS, we had faced some difficulties in understanding both the exact meaning and the usefulness of the items as discussed below.

Regarding ‘The type of the first fulfilled execution condition (e.g. CPAC or CHO). Fulfilment means all events of the type are met.’: It is unclear whether all failure and near failure scenarios are included here, e.g. all SON reports are impacted. Our understanding is that the intention is to cover all failure and near failure scenarios.

Proposal 3: RAN2 understanding is that RAN3 intention is to cover all failure and near failure scenarios in the bullet point “The type of the first fulfilled execution condition (e.g. CPAC or CHO). Fulfilment means all events of the type are met.”

The second and third bullets (“Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered” and “Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered”) seem to point to the same items as listed in the fourth bullet (“The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.”) despite the slightly different wording. Based on the latest agreement made by RAN2#128 to log PCell and PSCell measurements in all SON reports, no further RAN2 enhancements are needed as this information can be easily deduced from the PCell and PSCell measurements that are to be included in all reports (RLF report, SHR, SCGFailureInformation).

Proposal 4: There is no need to log the identifies of the candidate cells that fulfilled the execution conditions as they can be deduced from the PCell and PSCell measurements that has been agreed to be included in all SON reports.

Proposal 5: We propose to send a reply LS to RAN3 with RAN2’s understanding based on proposal 3 and 4 for confirmation (draft LS is in the Annex).

# 4	Conclusion

This document has made the following proposals:

Proposal 1: In case RLF is detected on the SCG before, during or shortly after successful completion of CHO with candidate SCG, the SCGFailureInformation message should be enhanced with an indication of the first fulfilled execution condition (CHO or CPAC), if any.

Proposal 2: RAN2 to discuss and decide on how the indication of the first fulfilled execution condition (CHO or CPAC) should be conveyed to the network for near failure scenarios:

- Explicitly: SHR is enhanced with indication of the first fulfilled execution condition or

- Implicitly: SHR or SPR is generated depending on which execution condition was fulfilled first (SPR is generated if CHO is fulfilled first and CPAC was fulfilled later).

Proposal 3: RAN2 understanding is that RAN3 intention is to cover all failure and near failure scenarios in the bullet point “The type of the first fulfilled execution condition (e.g. CPAC or CHO). Fulfilment means all events of the type are met.”

Proposal 4: There is no need to log the identifies of the candidate cells that fulfilled the execution conditions as they can be deduced from the PCell and PSCell measurements that has been agreed to be included in all SON reports.

Proposal 5: We propose to send a reply LS to RAN3 with RAN2’s understanding based on proposal 3 and 4 for confirmation (draft LS is in the Annex).

# Annex

3GPP TSG-RAN WG2 Meeting #129	R2-250xxxx

Athens, Greece, 17 – 21 February 2025

Title:	[DRAFT] Reply LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT

Response to:	R2-2500026/R3-247908

Release:	Release 19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core – Rel-19

Source:	Nokia [RAN WG2]

To:	RAN WG3

Cc:

Contact Person:

#### Name:	First Last

E-mail Address:	first.last@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 thank RAN3 for sending the LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT in R2-2500026/R3-247908.

RAN2 would like to inform RAN3 that it has concidered the items proposed by RAN3 to be logged in case of MRO for CHO with candidate SCG and has concluded the following:

- RAN2’s understanding is that RAN3 intention is to cover all failure and near failure scenarios in the bullet point ‘The type of the first fulfilled execution condition (e.g. CPAC or CHO). Fulfilment means all events of the type are met’.

- RAN2 has agreed that the PCell and PSCell measurements are included in all SON reports. As the identifies of the candidate cells can be deducted from these measurements, there is no need to additionally log the identifies of the candidate cells that fulfilled the execution conditions.

2. Actions:

To RAN3 group.

ACTION: 	RAN2 respectfully asks RAN3 to take the above RAN2’s conclusion into account.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#129-bis	from 2025-04-07	to 2025-04-11		China, CN

RAN2#130	from 2025-05-19	to 2025-05-23		Malta, MT

RAN2#131	from 2025-08-25	to 2025-08-29		India (TBC), IN