3GPP TSG-RAN WG2 Meeting #129	R2-2500366

Athens, Greece, 17th – 21st Feb, 2025

Agenda Item:	8.9.2

Source:	ZTE Corporation, Sanechips

Title:	Further consideration on S&F operation in IoT NTN

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed. The WID is further refined and the latest version is in [1]. The main objectives related to support of Store&Forward (S&F) satellite operation are as following:

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In RAN2#125bis meeting, RAN2 has had some high level discussion on this topic and made the following assumptions:

RAN2 assumptions: - S&F implies that at least the full eNB will be onboard - An IoT NTN network shall be able to inform UE(s) whether S&F Satellite operation is applied, either via NAS or AS (wait for SA2 progress on this) - The S&F satellite operation is common for NB-IoT and eMTC. - The S&F satellite operation is applied to both CP solution and UP solution (for the UP solution pending on SA2 conclusions on the architecture)

In RAN2#126 meeting, RAN2 has made more agreements for this objective as following:

- For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite): - The UE sends uplink data signalling to eNB when service link is available and the eNB stores it. - When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN. - The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available). - The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again - S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed - RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

In RAN2#127 meeting, based on the latest conclusions in SA2, RAN2 focus the discussion on RAN2 impacts of full CN and split-MME payload architecture options and achieved the following agreements:

- RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options) - RAN2 will consider both single satellite pass and multiple satellite pass scenarios - RAN2 will consider both MO and MT data within scope - UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature) - RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected. - RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In RAN2#127bis meeting, RAN2 focus the discussion about S&F operation indication and the cell bar mechanism, and achieved the following agreements:

- The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed) - RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided. - An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed - If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure - When present, the “S&F operation” indication has two possible settings: ‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell) ‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

In RAN2#128 meeting, RAN2 had discussion about timing information about S&F, RRC procedure and the information provided by NAS, and achieved the following agreements:

- Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start) - It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable - “S&F operation” indication is provided in SIB1 - RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN). - RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation - RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite - RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

In this paper, we will further discuss the remaining issues for supporting S&F operation and give our proposals.

Discussion

According to SA2’s progress, SA2 has specified the following general process to support of Store and Forward Satellite operation [2]:

TS 23.401 v19.0.0 When an MME is operating in S&F Mode: - If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, if the UE supports Store and Forward Satellite operation, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation. - If a UE indicates Store and Forward capability, the MME may provide to the UE a S&F Wait Timer when accepting or rejecting a NAS procedure. NOTE 3: How the MME determines the S&F Wait Timer is up to MME implementation, e.g. based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility etc. When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause. If the UE did not indicate Store and Forward capability in NAS signalling, if the network determines to reject the UE, the network shall reject the UE request with a cause non-specific to S&F Mode.

It can been seen that if the satellite is operating in S&F mode, the network is still able to provide MO or MT service at least for R19 UE supporting S&F, as long as the network has store the UE context or data.

RAN2 has an open issue on which switching timing information about S&F is introduced. We will give our discussion for the following two scenarios:

Scenario1: the serving satellite operating in S&F mode is going to switch to normal mode

Fig.1

As illustrated in Fig.1, for quasi-earth fixed cell, the serving satellite 1 currently works in S&F mode at T1 and is going to switch to normal mode from S&F mode at T2. According to the highlight yellow description in SA2 specification as above, MME onboard may reject the NAS procedure at T1. In order to avoid this failure, in last meeting, RAN2 has assumed that UE could delay some NAS procedures till after T2. This behavior is helpful for both of R19 UE supporting S&F and not supporting S&F mode if they can recognize the switching time information about S&F mode.

Moreover, considering that this behavior is merely a performance optimization, it should not be mandated by specification. Hence, for R19 UEs supporting and not supporting S&F mode, it is up to UE’s implementation on whether to delay some NAS procedures until the feeder link is resumed.

Proposal 1: For R19 UEs supporting and not supporting S&F mode, it is up to UE’s implementation to delay some NAS procedures until the feeder link is resumed.

At the same time, according to the highlight green description in SA2 spec as above, for a R19 UE supporting S&F mode, a wait timer can be provided by NAS for it, and it could delay the NAS procedures until the wait timer expires. However, according to Proposal 1, a R19 UE not supporting S&F mode also needs to delay NAS procedure until the feeder link is resumed. Hence, it’s more useful to provide a start time of normal mode to assist R19 UEs with different capabilities to delay NAS procedure.

In addition, a duration of normal mode could be provided to indicate when the serving satellite is going to switch to S&F mode next time. Moreover, for the serving cell, if S&F indication is present in SIB1, this means this cell is operating in S&F mode, thus, a start time and a duration of normal mode could be broadcast to provide the switching time of normal mode in further.

Proposal 2: For the serving cell, if S&F indication is present in SIB1, a start time and a duration of normal mode could be broadcast to R19 UE.

In previous meeting, a S&F indication is introduced to indicate the cell is operating in S&F mode. Switching time information together with S&F indication could indicate more information about S&F mode. Specifically, if S&F indication is present, and a start time and a duration of normal mode are absent, this means this cell always works in S&F mode. If S&F indication is absent in SIB1, this means the serving cell is operating in normal mode, thus, a start time and a duration of normal mode don’t need to be broadcast.

Proposal 2a: For the serving cell, if S&F indication is present, and a start time and a duration of normal mode are absent, this means this cell always works in S&F mode.

Proposal 2b: For the serving cell, if S&F indication is absent, a start time and a duration of normal mode don’t need to be broadcast.

Moreover, for providing a start time of normal mode, in order to avoid the frequent update of this parameter, an absolute time, e.g., with type of TimeOffsetUTC, can be used. And considering that the start time and duration of normal mode correspond to a satellite not a cell, they can be broadcast in SystemInformationBlockType31(-NB).

Proposal 3: For the serving cell, a start time and a duration of normal mode can be broadcast in SystemInformationBlockType31(-NB). For definition of the start time of normal mode, an absolute time, e.g., with type of TimeOffsetUTC, can be used.

Also as illustrated in the example in Fig.1, the serving satellite 1 is going to stop serving the area at T3. And the next neighbor satellite 2 is going to start serving the area at T4 without the available feeder link, and switch to normal mode at T5. In this case, the t-Service in SystemInformationBlockType3(-NB) of the serving satellite 1 would be set to T3, e.g., the time when the serving satellite is going to stop serving the area.

- Under the continuous coverage scenario, it’s possible that, before the serving satellite stops serving the area, the neighbor satellite will be able to switch to normal mode, so the timing relationship is that T3 > T5 > T4 (“>” means later than). In this case, for quasi-earth fixed cell, there may be two options for setting the legacy parameter t-ServiceStartNeigh in SystemInformationBlockType33(-NB) of neighbor satellite 2 (this parameter will be used by the UE to decide when to start measurements on this satellite 2):

  - If the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) of satellite 2 is set as the time when the satellite 2 is going to start serving the area, e.g., T4, for a legacy UE that is still under the coverage of satellite 1, it will start measurement satellite 2 at T4 and maybe switch to satellite 2 quickly after T4. However, during time period between T4 and T5, although satellite 2 can provide coverage to this area, it has no available feeder link. Therefore the legacy UE cannot trigger normal connection to the satellite 2. We think this is too early switching to satellite 2 due to the unsuitable setting of t-ServiceStartNeigh of satellite 2. Instead, if the legacy UE can stay at satellite 1 till T5, it’s still possible for the legacy UE to trigger normal connection (the only issue may be that the radio quality may be a bit worse than camping at satellite 2). For R19 UE supporting S&F mode, the drawbacks of too early switching to neighbor satellite 2 may not be as big as they are for legacy UE, but we still think it is better to keep it in the current serving satellite operating in normal mode.

  - If the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) of satellite 2 is set as the time when the satellite 2 starts to operate in normal mode, e.g., T5, this would help to delay the switching to satellite 2 of the legacy UE and keep it in the current serving satellite with normal mode for as long as possible.

  - Hence, it’s preferred that the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) of neighbor satellite could be set as the time when the neighbor satellite starts to operate in normal mode.

- Under the continuous coverage scenario, it’s also possible that the neighbor satellite will switch to normal mode after the serving satellite stops serving the area, so the timing relationship is that T5 > T3 > T4. With similar analysis as above, in this case, for quasi-earth fixed cell, if the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) of satellite 2 is set as the time when the neighbor satellite starts to operate in normal mode, e.g., T5, for legacy UE, it may lead to the misunderstanding of the discontinuous coverage. We think this is kind of too late switching to satellite 2. Hence, the more suitable process may be that, the t-ServiceStartNeigh should be set as the time when the neighbor satellite starts serving the area, e.g., T4.

  - For R19 UE, an enhancement can be further considered, that is, a time when the neighbor satellite starts to operate in normal mode could be provided to help UE determine the suitable time when it can establishment of real-time service or some NAS processes.

- Under the discontinuous coverage scenario, the only case is that the neighbor satellite switches to normal mode after the serving satellite stops serving the area, so the timing relationship is that T5 > T4> T3. After the discontinuous coverage, it is likely that the registration or TAU process is triggered by UE. In order to ensure NAS process can be completed successfully at the first time, it is beneficial to set the t-ServiceStart in SystemInformationBlockType32(-NB) as the time when the neighbor satellite starts to operate in normal mode, e.g., T5, not the T4 when the neighbor satellite just starts to serve the area.

In summary, for quasi-earth fixed cell, rather than always setting the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) or t-ServiceStart in SystemInformationBlockType32(-NB) as the time when the neighbor satellite starts serving the area, it can be up to network’s implementation to set the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) or t-ServiceStart in SystemInformationBlockType32(-NB) as the time when the neighbor satellite starts to operate in normal mode or the time when the neighbor satellite starts serving the area. The different setting may bring benefits in different cases.

The earth moving cell case can be further discussed based on the progress for quasi-earth fixed cell case.

Proposal 4: For quasi-earth fixed cell, the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) or t-ServiceStart in SystemInformationBlockType32(-NB) can be set as the time when the neighbor satellite starts to operate in normal mode or the time when the neighbor satellite starts serving the area.

In additional, a time when the neighbor satellite starts to operate in normal mode could be provided. If this time is present, for R19 UE, it is up to R19 UE’s implementation to use this time to start the neighbor cell measurement.

Proposal 5: It can be further considered to provide a time when the neighbor satellite starts to operate in normal mode in SystemInformationBlockType33(-NB).

Proposal 5a: If this time is present, it is up to R19 UE’s implementation to use this time for triggering the neighbor cell measurement.

Proposal 5b: If this time is absent, it is up to R19 UE’s implementation to use t-ServiceStartNeigh or t-ServiceStart for triggering the neighbor cell measurement.

Scenario2: the serving satellite operating in normal mode is going to switch to S&F mode

Fig.2

As illustrated in Fig.2, for quasi-earth fixed cell, the serving satellite 1 serves the area and works at normal mode at T1. It is going to switch to S&F mode at T2 and stop serving the area at T3. And the next neighbor satellite 2 is going to start serving the area at T4 with available feeder link.

- Under the continuous coverage scenario, for quasi-earth fixed cell, the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) of satellite 2 is generally set as the time when the neighbor satellite is going to start serving the area, e.g., T4. After the serving satellite switches to S&F mode, the neighbor satellite starts serving the area with available feeder link, so the timing relationship is that T3 > T4 > T2. In this case, for quasi-earth fixed cell, if the t-Service in SystemInformationBlockType3(-NB) that can be used by UE to decide when to start measurements on neighbor satellite is set as the time when the serving satellite starts to operate in S&F mode, e.g., T2, instead of T3, for legacy UE, it is helpful to trigger the UE to early reselect to the neighbor satellite in normal mode. Hence, in this case, it’s preferred that the t-Service should be set as the time when the serving satellite starts to operate in S&F mode.

- Under the discontinuous coverage scenario, timing relationship can be that T4 > T3 > T2. The t-ServiceStart in SystemInformationBlockType32(-NB) is generally set as the time when the neighbor satellite is going to start serving the area, e.g., T4. Meanwhile, in order to keep the UE in the coverage of satellite as much as possible, instead of setting the t-Service of servicing satellite as the time when the serving satellite starts to operate in S&F mode, e.g., T2, the more suitable way is to still set the t-Service in SystemInformationBlockType3(-NB) as the time when the serving satellite stops serving the area, e.g., T3.

In summary, for quasi-earth fixed cell, instead of always setting the t-Service in SystemInformationBlockType3(-NB) as the time when the serving satellite stops serving the area, the t-Service in SystemInformationBlockType3(-NB) can be set as the time when the serving satellite starts to operate in S&F mode or the time when the serving satellite stops serving the area. The different setting may bring benefits in different cases.

The earth moving cell case can be further discussed based on the progress for quasi-earth fixed cell case.

Proposal 6: For quasi-earth fixed cell, the t-Service in SystemInformationBlockType3(-NB) can be set as the time when the serving satellite starts to operate in S&F mode or the time when the serving satellite stops serving the area.

In additional, a time when the serving satellite starts to operate in S&F mode may be also needed. But according to Proposal 2, the time when the serving satellite starts to operate in S&F mode could be deduced.

Conclusion

Based on the above discussion, the following proposals are given:

Proposal 1: For R19 UEs supporting and not supporting S&F mode, it is up to UE’s implementation to delay some NAS procedures until the feeder link is resumed.

Proposal 2: For the serving cell, if S&F indication is present in SIB1, a start time and a duration of normal mode could be broadcast to R19 UE.

Proposal 2a: For the serving cell, if S&F indication is present, and a start time and a duration of normal mode are absent, this means this cell always works in S&F mode.

Proposal 2b: For the serving cell, if S&F indication is absent, a start time and a duration of normal mode don’t need to be broadcast.

Proposal 3: For the serving cell, a start time and a duration of normal mode can be broadcast in SystemInformationBlockType31(-NB). For definition of the start time of normal mode, an absolute time, e.g., with type of TimeOffsetUTC, can be used.

Proposal 4: For quasi-earth fixed cell, the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) or t-ServiceStart in SystemInformationBlockType32(-NB) can be set as the time when the neighbor satellite starts to operate in normal mode or the time when the neighbor satellite starts serving the area.

Proposal 5: It can be further considered to provide a time when the neighbor satellite starts to operate in normal mode in SystemInformationBlockType33(-NB).

Proposal 5a: If this time is present, it is up to R19 UE’s implementation to use this time for triggering the neighbor cell measurement.

Proposal 5b: If this time is absent, it is up to R19 UE’s implementation to use t-ServiceStartNeigh or t-ServiceStart for triggering the neighbor cell measurement.

Proposal 6: For quasi-earth fixed cell, the t-Service in SystemInformationBlockType3(-NB) can be set as the time when the serving satellite starts to operate in S&F mode or the time when the serving satellite stops serving the area.

Reference

[1] RP-243278, Revised WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc., RAN#106, December 2024

[2] S2-2412903 Introduction of Store and Forward feature in EPC Nokia, Sateliot, Qualcomm, LGE, Intel, OPPO, SHARP, Google, CATT, Novamint, NEC, Tencent, Vivo