3GPP TSG-RAN WG2 Meeting #129                                        R2-2500367

Athens, Greece, 17th – 21st Feb, 2025

Source:	ZTE Corporation, Sanechips

Title:	Further consideration on UL capacity enhancements in IoT NTN

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed. The WID is further refined and the latest one is approved in [1]. The main objectives related to support of capacity enhancements for uplink are as following:

The aim of this WI is enhancement of IoT-NTN with the following objectives: <<skip the unrelated parts>> - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In previous RAN2 meetings, RAN2 has discussed some general aspects for this objective and the feasibility of contention-based Msg3 transmission, and also gave comparison among different anti-collision schemes. The related RAN2 agreements are as below:

RAN2#125bis agreements: - Both NB-IoT and eMTC are within scope of uplink capacity enhancements - Both C-plane and U-plane solutions are within scope of uplink capacity enhancements. - Only CIoT EPS is within scope of uplink capacity enhancements RAN2#126 agreements: - RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). - RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) - If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3. RAN2#127 agreements: - RAN2 will continue working on a CB-msg3 EDT-like mechanism. - RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts - In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1. - For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition RAN2#127b agreements: - RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19. - RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed. - At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details. - CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA) - RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA) - At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission - The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA) - CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN. RAN2#128 agreements: - Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B) - Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels - The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels) - There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC) - The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific - There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts) - For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time - For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs) - The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource. - RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

In this document, we will firstly give analysis on the remaining issues of CB-Msg3 and Msg4 transmission, e.g., the resources configuration, and further give analysis on remaining issues for supporting DSA schemes. Based on the analysis, some proposals are given.

Discussion

The configuration for CB-Msg3 transmission

The resources configuration

Considering the resource configured for EDT transmission is a shared resource, it can be taken as reference when we discuss the shared resources configuration for CB-Msg3 transmission. For example, the NPRACH configuration for EDT is as below:

NPRACH-ConfigSIB-NB information elements

-- ASN1START

NPRACH-ConfigSIB-NB-r13 ::=			SEQUENCE {

nprach-CP-Length-r13				ENUMERATED {us66dot7, us266dot7},

rsrp-ThresholdsPrachInfoList-r13	RSRP-ThresholdsNPRACH-InfoList-NB-r13	OPTIONAL,	-- Need OR

nprach-ParametersList-r13		NPRACH-ParametersList-NB-r13

}

NPRACH-ConfigSIB-NB-v1330 ::=		SEQUENCE {

nprach-ParametersList-v1330			NPRACH-ParametersList-NB-v1330

}

NPRACH-ConfigSIB-NB-v1450 ::=		SEQUENCE {

maxNumPreambleAttemptCE-r14			ENUMERATED {n3, n4, n5, n6, n7, n8, n10, spare1}

}

NPRACH-ConfigSIB-NB-v1530 ::=		SEQUENCE {

tdd-Parameters-r15					SEQUENCE {

nprach-PreambleFormat-r15			ENUMERATED {

fmt0, fmt1, fmt2, fmt0-a, fmt1-a},

dummy								ENUMERATED {

n1, n2, n4, n8, n16, n32, n64, n128,

n256, n512, n1024},

nprach-ParametersListTDD-r15		NPRACH-ParametersListTDD-NB-r15

}	OPTIONAL,		-- Cond TDD

fmt2-Parameters-r15					SEQUENCE {

nprach-ParametersListFmt2-r15		NPRACH-ParametersListFmt2-NB-r15 OPTIONAL,	-- Need OR

nprach-ParametersListFmt2EDT-r15	NPRACH-ParametersListFmt2-NB-r15 OPTIONAL	-- Cond EDT2

}	OPTIONAL,		-- Need OR

edt-Parameters-r15					SEQUENCE {

edt-SmallTBS-Subset-r15				ENUMERATED {true}				OPTIONAL,	-- Need OR

edt-TBS-InfoList-r15				EDT-TBS-InfoList-NB-r15,

nprach-ParametersListEDT-r15		NPRACH-ParametersList-NB-r14	OPTIONAL	-- Need OR

}	OPTIONAL		-- Cond EDT1

}

//skip unrelated parts

NPRACH-ParametersList-NB-r14 ::=		SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF

NPRACH-Parameters-NB-r14

NPRACH-Parameters-NB-r14 ::=			SEQUENCE {

nprach-Parameters-r14					SEQUENCE {

nprach-Periodicity-r14					ENUMERATED {ms40, ms80, ms160, ms240,

ms320, ms640, ms1280, ms2560}

OPTIONAL,	-- NEED OP

nprach-StartTime-r14					ENUMERATED {ms8, ms16, ms32, ms64,

ms128, ms256, ms512, ms1024}

OPTIONAL,	-- NEED OP

nprach-SubcarrierOffset-r14				ENUMERATED {n0, n12, n24, n36, n2, n18, n34, spare1}

OPTIONAL,	-- NEED OP

nprach-NumSubcarriers-r14				ENUMERATED {n12, n24, n36, n48}

OPTIONAL,	-- NEED OP

nprach-SubcarrierMSG3-RangeStart-r14	ENUMERATED {zero, oneThird, twoThird, one}

OPTIONAL,	-- NEED OP

npdcch-NumRepetitions-RA-r14			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1}

OPTIONAL,	-- NEED OP

npdcch-StartSF-CSS-RA-r14				ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64}

OPTIONAL,	-- NEED OP

npdcch-Offset-RA-r14					ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

OPTIONAL,	-- NEED OP

nprach-NumCBRA-StartSubcarriers-r14		ENUMERATED {n8, n10, n11, n12, n20, n22, n23, n24,

n32, n34, n35, n36, n40, n44, n46, n48}

OPTIONAL,	-- NEED OP

npdcch-CarrierIndex-r14					INTEGER (1..maxNonAnchorCarriers-NB-r14)

OPTIONAL,	-- Need OP

...

}	OPTIONAL	-- Need OR

}

//skip unrelated parts

-- ASN1STOP

- For time domain resources configuration, the start time and periodicity are used to represent the resource occasion:

  - As cited above, the value range for periodicity of NPRACH for EDT is {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560}. Considering that the conditions for using CB-msg3 transmission will be stricter than those for using EDT, we can reasonably assume that the number of UEs using CB-msg3 will generally be less than the number of UEs using the EDT, thus, it can be assumed that the periodicity in the time domain resources for CB-msg3 is sparer than NPRACH for EDT. Hence, the periodicity of the resource for CB-msg3 could be in the unit of tens of, or hundreds of, even thousands of milliseconds. So the radio frame, e.g., SFN (e.g., 10~10240 ms) as unit may be enough for the definition of periodicity. Meanwhile, subframe as unit of periodicity means more dense time resource, and seems not so necessity. And H-SFN (i.e., larger than 10.24s) as unit of periodicity may leading to increased latency, also isn’t suggested. In final, only SFN is suggested to be used as unit for the periodicity of CB-msg3. Furthermore, based on the considerations of feasibility and for reducing the complexity, we think such definition way for the periodicity of the shared time domain resources for CB-msg3 can be applicable to both eMTC NTN and NB-IoT NTN.

  - For start time, the start subframe and start SFN are generally needed to represent a specific time occasion within a periodicity.

- For frequency domain resources configuration, multiple occasions could create more transmission opportunities and reduce conflicts, thus, multiple PRB/subcarrier resource can be considered.

  - In order to minimize the impacts on RAN1 specification, the current resource configuration way for PUR can be referred as much as possible.

  - However, for dedicated PUR resources configuration for eMTC, before the eNB configures the sub-PRB resource, eNB needs to know UE capability on whether the UE supports sub-PRB resource allocation for PUSCH when operating in coverage enhancement mode A or B, hence, the sub-PRB resource allocation isn’t applicable to the cell-specific CB-msg3 frequency domain resources configuration.

Observation 1a: For eMTC, the sub-PRB resource allocation isn’t applicable to the cell-specific CB-msg3 frequency domain resources configuration.

  - Moreover, for dedicated PUR resources configuration for eMTC, before the eNB configures a flexible start PRB configuration, eNB also needs a UE capability indicating whether UE supports flexible starting PRB for PDSCH/PUSCH when operating in coverage enhancement mode A/B, hence, the flexible start PRB resource allocation also isn’t applicable to the cell-specific CB-msg3 resources configuration.

Observation 1b: For eMTC, the flexible start PRB resource allocation isn’t applicable to the cell-specific CB-msg3 frequency domain resources configuration.

  - For eMTC, in PUR configuration for CE mode A, a starting RB and a length in terms of contiguously allocated RB (indicated in prb-AllocationInfo-r16 BIT STRING (SIZE(10))) are used for resource configuration using UL resource allocation type 0. Referring to such PUR configuration way, for CB-msg3 for CE mode A, a starting RB and a length can be reused to indicate multiple contiguous PRBs. In order to indicate multiple frequency occasions with different starting RB, a list of combination of a starting RB and a length can be introduced, and each combination of a starting RB and a length can be used to indicate a frequency occasion. In PUR configuration for CE mode B, a narrowband index and PRBs within the indicated narrowband (indicated in prb-AllocationInfo-r16 BIT STRING (SIZE(8))) are used for resource configuration using UL resource allocation type 2. Referring to such PUR configuration way, for CB-msg3 for CE mode B, a narrowband index and PRBs within the indicated narrowband can be reused to indicate multiple contiguous PRB starting from RB 0 in a narrowband. In order to indicate multiple frequency occasions within different narrowband, a list of combination of a narrowband index and PRBs within the indicated narrowband can be introduced, and each combination of a narrowband index and PRBs within the indicated narrowband can be used to indicate a frequency occasion.

TS 36.331 PUR-PUSCH-Config-r16 ::= SEQUENCE { pur-GrantInfo-r16 CHOICE { ce-ModeA SEQUENCE { numRUs-r16 BIT STRING (SIZE(2)), prb-AllocationInfo-r16 BIT STRING (SIZE(10)), mcs-r16 BIT STRING (SIZE(4)), numRepetitions-r16 BIT STRING (SIZE(3)) }, ce-ModeB SEQUENCE { subPRB-Allocation-r16 BOOLEAN, numRUs-r16 BOOLEAN, prb-AllocationInfo-r16 BIT STRING (SIZE(8)), mcs-r16 BIT STRING (SIZE(4)), numRepetitions-r16 BIT STRING (SIZE(3)) } } OPTIONAL, -- Need ON pur-PUSCH-FreqHopping-r16 BOOLEAN, p0-UE-PUSCH-r16 INTEGER (-8..7), alpha-r16 Alpha-r12, pusch-CyclicShift-r16 ENUMERATED {n0, n6}, pusch-NB-MaxTBS-r16 BOOLEAN, locationCE-ModeB-r16 INTEGER (0..5) OPTIONAL -- Cond SubPRB } pur-GrantInfo Indicates UL grant for transmission using PUR. Field set to ce-ModeA indicates the PUR grant is for CE Mode A and the field set to ce-ModeB indicates the PUR grant is for CE Mode B. numRUs indicates DCI field for PUSCH number of resource units, see TS 36.213 [23] clause 8.1.6. prbAllocationInfo indicates DCI field for PUSCH resource block assignment, see TS 36.212 [22], clause 5.3.3.1.10 (CE Mode A) and clause 5.3.3.1.11 (CE Mode B). mcs indicates DCI field for PUSCH modulation and coding scheme, see TS 36.213 [23] clause 8.6. numRepetitions indicates DCI field for PUSCH repetition number, see TS 36.213 [23] clause 8.0. For CE Mode A, numRUs set to '00' indicates use of full-PRB resource allocation, otherwise sub-PRB resource allocation as defined in TS 36.213 [23], clause 8.1.6. For CE Mode B, subPRB-Allocation indicates whether sub-PRB resource allocation is used. ----

  - For NB-IoT, in PUR for single-tone or multi-tone, a number of resource units (such as npusch-NumRUsIndex-r16) and a set of contiguously allocated subcarriers (such as npusch-SubCarrierSetIndex-r16) are used for resource configuration (see below definition in RAN2 and RAN1 spec for reference). Referring to such PUR configuration way, for CB-msg3, multiple sets of subcarriers can be configured as a resource pool. Moreover, in order to support both single-tone and multi-tone transmissions, two separate lists of sets of subcarriers can be configured, one for single-tone transmission, the other for multi-tone transmission.

TS 36.331 PUR-Config-NB-r16 ::= SEQUENCE { //skip unrelated parts pur-PhysicalConfig-r16 SEQUENCE { carrierConfig-r16 CarrierConfigDedicated-NB-r13, npusch-NumRUsIndex-r16 INTEGER (0..7), npusch-NumRepetitionsIndex-r16 INTEGER (0..7), npusch-SubCarrierSetIndex-r16 CHOICE { khz15 INTEGER (0..18), khz3dot75 INTEGER (0..47) }, npusch-MCS-r16 CHOICE { singleTone INTEGER (0..10), multiTone INTEGER (0..13) }, p0-UE-NPUSCH-r16 INTEGER (-8..7), alpha-r16 ENUMERATED {al0, al04, al05, al06, al07, al08, al09, al1}, npusch-CyclicShift-r16 ENUMERATED {n0, n6}, npdcch-Config-r16 NPDCCH-ConfigDedicated-NB-r13 } OPTIONAL, -- Need ON ..., //skip unrelated parts } TS 36.213 16.5.1.1 Resource allocation The resource allocation information in uplink DCI format N0 for NPUSCH transmission or configured by higher layers for NPUSCH transmission using preconfigured uplink resource indicates to a scheduled UE - a set of contiguously allocated subcarriers () of a resource unit determined by the Subcarrier indication field, or by the higher layer parameter npusch-SubCarrierSetIndex in PUR-Config-NB - a number of resource units () determined by the resource assignment field according to Table 16.5.1.1-2, or by the higher layer parameter npusch-NumRUsIndex in PUR-Config-NB - a repetition number () determined by the repetition number field according to Table 16.5.1.1-3, and for a NPUSCH transmission using preconfigured uplink resource, the UE shall use the repetition number configured by higher layers; except for NPUSCH with 16QAM where NRep=1. For NPUSCH transmission with subcarrier spacing, where is the subcarrier indication field and is reserved, or nsc is configured by higher layers parameter npusch-SubCarrierSetIndex in PUR-Config-NB for NPUSCH transmissions using preconfigured uplink resources. For NPUSCH transmission with subcarrier spacing, the subcarrier indication field () in the DCI or npusch-SubCarrierSetIndex in PUR-Config-NB for NPUSCH transmissions using preconfigured uplink resources determines the set of contiguously allocated subcarriers () according to Table 16.5.1.1-1. Table 16.5.1.1-1: Allocated subcarriers for NPUSCH with . Subcarrier indication field (); Set of Allocated subcarriers (). 0 – 11. 12-15. 16-17. 18. 19-63; Reserved. Table 16.5.1.1-2: Number of resource units () for NPUSCH. 0; 1. 1; 2. 2; 3. 3; 4. 4; 5. 5; 6. 6; 8. 7; 10. ---

- For code domain resource configuration, according to companies’ simulations in previous meeting (e.g., in [2]), it can be seen that OCC-based SA scheme can achieve the best performance for some cases. Therefore, RAN2 have made a general assumption that OCC (orthogonal cover codes) sequence can be further applied to (N)PUSCH (Msg3) transmission to reduce the collision caused by using shared time/frequency domain resources and facilitate more UEs multiplexing on a same set time/frequency domain resources. In previous RAN2 meeting, RAN2 has sent LS to RAN1 to check the feasibility of applying OCC to CB-Msg3 NPUSCH. In RAN1#119 meeting, RAN1 also confirms in the response LS [3] that, OCC solution that RAN1 is working on may be applied to CB-Msg3 NPUSCH format 1 single tone at least for OCC length 2, for the subcarrier spacing(s) for which CB-Msg3 is supported, when the power imbalance is small. So from RAN2 perspective, the OCC sequences can be taken as a new resource type which needs to be introduced in the shared (N)PUSCH resources configuration for CB-msg3 transmission.

- For CE level aspect, RAN2 has agreed that the (N)PUSCH resources for CB-msg3 transmission are provided per CE level in previous meeting. Thus, a repetition number needs to be configured and taken as the baseline dimension for configuring other time domain/frequency domain/code domain resources. Furthermore, it needs to be noted that, in legacy system, for a certain CE level, the repetition number for (N)PRACH and (N)PUSCH can be different. Per our understanding, the repetition number configuration of PUR is more suitable as a reference for the relevant configuration for CB-msg3.

- Besides the above discussion on (N)PUSCH resources configuration, (N)PDCCH resource configuration is also needed for UE to monitor the response for CB-msg3 transmission. That can also take PUR configuration as reference.

Based on the discussion above, the proposal about the necessary contents for shared resources configuration for CB-msg3 transmission is given as below:

Proposal 1a: RAN2 can discuss the following resources types for shared resources configuration for CB-msg3:

- Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN)

- Frequency domain resources for (N)PUSCH occasions:

  - for eMTC, to configure a list of combination of a starting RB and a length in terms of contiguously allocated RB for CE mode A; to configure a list of combination of a narrowband index and PRBs within the indicated narrowband for CE mode B;

  - for NB-IoT, to configure carrier index, a RU number and a list of set of subcarriers for single-tone case; to configure a RU number and a list of set of subcarriers for multi-tone case.

- OCC resources

- repetition number

- (N)PDCCH resource

Proposal 1b: The periodicity of the time domain resources for (N)PUSCH occasions could be with the unit of radio frames, the detailed value range can be FFS.

CE level configuration

In RAN2#128 meeting, RAN2 has agreed to reuse the existing CE level selection procedure for CB-Msg3 at least for the initial resources selection. It’s still FFS on the number of levels and whether we can reuse the same thresholds.

In current enhanced coverage, up to 4 CE levels for eMTC and up to 3 CE levels can be supported. The RSRP threshold configuration for achieving such CE levels is as below for reference:

For eMTC: RSRP-ThresholdsPrachInfoList-r13 ::= SEQUENCE (SIZE(1..3)) OF RSRP-Range For NB-IoT: RSRP-ThresholdsNPRACH-InfoList-NB-r13 ::= SEQUENCE (SIZE(1..2)) OF RSRP-Range

Considering the high SINR requirement for receiving CB-msg3, its reliability can be enhanced through repetition transmissions. In other words, a suitable number of repetition transmission could ensure the successful reception of CB-msg3 at any CE level. Therefore, the CB-msg3 transmission can be supported across all 4 CE levels.

The RSRP thresholds for CE levelsis generally determined by the requirements of received SINR. It is obvious that the reception of CB-msg3 requires a higher received SINR compared to reception on (N)PRACH. So we suggest to configure same number of RSRP thresholds as that for EDT for CE level selection for CB-msg3.

Furthermore, considering it’s possible that UE falls back to EDT procedure if CB-msg3 fails, a suitable process should be that the UE prioritizes fallback to EDT or legacy RACH procedure using the same CE level. Therefore, it may need to ensure that the resources configuration of CB-msg3 and the resources configuration of EDT have a consistent CE level configuration. One possible way to achieve aligned configuration between CB-msg3 and EDT is that, a delta RSRP can be configured relative to a certain RSRP threshold of EDT resources configuration.

Additionally, for CB-msg3 with OCC, we understand the similar SINR is required compared to CB-msg3 without OCC. Hence, CB-msg3 with OCC can share the same RSRP threshold configuration as that for CB-msg3 without OCC.

For different CE levels, the required SINR varies, leading to different MCS in each CE level. And different MCS values result in different TBS values. Therefore, the maximum TBS also needs to be configured for each CE level and maybe with different values. Furthermore, due to the same SINR for a certain CE level and same MCS value, CB-msg3 with OCC can share the same maximum TBS configuration as that for CB-msg3 without OCC.

Proposal 2: For CB-msg3 transmission, for eMTC NTN, up to three RSRP thresholds can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two RSRP thresholds can be supported for achieving at most 3 CE levels.

Proposal 3a: For CB-msg3 without OCC, a delta RSRP relative to a certain RSRP threshold of EDT can be configured to achieve corresponding RSRP threshold for CE level selection. And CB-msg3 with OCC can share the same RSRP threshold configuration as that for CB-msg3 without OCC.

Proposal 3b: For CB-msg3 without OCC, a maximum TBS needs to be configured for each CE level. And CB-msg3 with OCC can share the same maximum TBS as that for CB-msg3 without OCC.

Efficient delivery of Msg4

Contention resolution window

Taking legacy contention-based RA procedure or contention-based EDT procedure as reference, in order to guarantee the reception of Msg4, a contention resolution window is necessary.

Per our understanding, CB-msg3 will use similar Msg3 on (N)PUSCH and Msg4 transmission on (N)PDSCH (without Msg1 and Msg2) as that in PUR and the main difference is that CB-msg3 needs an additional contention procedure, so we assume the definition of Msg3 response window in PUR can be referred when defining the contention resolution window for CB-msg3.

The start time of the Msg3 response window in PUR is determined based on some factors, e.g., the RTT, the processing time of Msg3. Taking the following descriptions for NB-IoT NTN from RAN1 and RAN2 specs as reference, it’s suggested the contention resolution window of CB-msg3 in NB-IoT NTN can be also started at the end of Msg3 transmission plus 4 subframes plus UE-eNB RTT.

TS 36.213: - 9.1.5 MPDCCH assignment procedure //skip unrelated parts If the UE has initiated a PUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the MPDCCH UE-specific search space in a search space window starting in subframe n+4+Kmac with duration given by higher layer parameter pur-MPDCCH-SS-window-duration where is provided by higher layer parameter K-mac, otherwise Kmac=0. Upon detection of a MPDCCH with DCI format 6-0A/6-0B with CRC scrambled by PUR-RNTI intended for the UE within the search space window and the corresponding DCI is for PUR ACK/fallback indication (as defined in [4]), the UE is not required to monitor the MPDCCH UE-specific search space for the remaining search space window duration. //skip unrelated parts - 16.6 Narrowband physical downlink control channel related procedures //skip unrelated parts If the UE has initiated a NPUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the NPDCCH UE-specific search space in a search space window starting in subframe n+4+Kmac with duration given by higher layer parameter pur-SS-window-duration. Upon detection of a NPDCCH with DCI format N0 with CRC scrambled by PUR-RNTI intended for the UE within the search space window and the value of "modulation and coding scheme" field () in the corresponding DCI is set to '14', the UE is not required to monitor the NPDCCH UE-specific search space for the remaining search space window duration. //skip unrelated parts TS 36.321: 5.4.7.1 Transmission using PUR //skip unrelated parts After transmission using PUR, the MAC entity shall monitor PDCCH identified by PUR-RNTI in the PUR response window using timer pur-ResponseWindowTimer: - if PUR was transmitted in a non-terrestrial network and UE supports delaying the start of the pur-ResponseWindowTimer: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes plus UE-eNB RTT. - else: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes. //skip unrelated parts

Proposal 4a: For CB-msg3 procedure, a contention resolution window starts at the end of the Msg3 transmission plus 4 subframes plus UE-eNB RTT.

For the length of contention resolution window, we assume the length of Msg3 response window in PUR can be referred. The value of pur-ResponseWindowTimer are as below and seems long enough:

| for eMTC | pur-ResponseWindowTimer-r16 ENUMERATED {sf240, sf480, sf960, sf1920, sf3840, sf5760, sf7680, sf10240} |
| --- | --- |
| For NB-IoT | pur-ResponseWindowTimer-r16 ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64} |

Hence, we suggest the length of contention resolution window CB-msg3 could reuse that of the pur-ResponseWindowTimer.

Proposal 4b: The same value range as pur-ResponseWindowTimer can be reused for the length of contention resolution window for CB-msg3 procedure.

RNTI calculation

In previous meeting, RAN2 has agreed that RNTI for scheduling Msg4 transmission is derived based on the resource associated to the (N)PUSCH occasion used for CB-msg3 transmission.

According to our analysis in 2.1, the resource associated to the (N)PUSCH occasion can include the time domain resource, frequency domain resource and OCC resource. In below, we will discuss one by one whether and how these resource information should be used in the RNTI calculation:

- Regarding the time domain resource, the calculation of RNTI has a close relationship with the length of contention resolution window. If UEs select different transmission occasions in time domain but use the same RNTI, and monitor the same RNTI within the overlapped contention resolution window, they cannot correctly distinguish the Msg4 via RNTI. Therefore, it should ensure that, within the length of a contention resolution window, for two UEs who select two transmission occasions in different periodicity, RNTI calculated by the UEs should be different. According to our Proposal 1b, due to the periodicity will be with the unit of tens of or hundreds of milliseconds, the subframe will keep unchanged in different periodicities and just SFN is used to distinguish the transmission occasion. Hence, SFN index should be used as a factor in the formula of RNTI, meanwhile, subframe index is not needed.

  - Besides, according to some previous discussion for RA-RNTI, to avoid a too large RNTI, it can be considered to compress the part where SFN index is involved. On one hand, since resources are configured according to periodicity, not all SFN indices will be used; that is, only SFN indices at certain intervals will be considered valid resource positions during resource selection. Hence, SFN index can be divided by a minimum periodicity in the formula. On the other hand, since each UE only monitors the DL for a limited time period, e.g., during the contention resolution window, to receive Msg4, the RNTIs of two UEs need only be different during this period. Hence, it also can be considered to modulo the SFN index by the length of contention resolution window.

- Regarding the frequency resource, according to our Proposal 1a, for eMTC mode A, a UL resource allocation includes the start PRB index, PRB number. As long as UEs select the different start PRB index, Msg3 from UEs will not conflict. Hence, the selected start PRB index could be used as a parameter in the formula of RNTI. For eMTC mode B, a UL resource allocation includes a narrowband index and PRBs within the indicated narrowband. As long as UEs select the different narrowband index, Msg3 from UE with mode B will not conflict. However, the value of selected start PRB index for UE with eMTC mode A may be same as the value of selected narrowband index for UE with eMTC mode B. for example, for eMTC mode A, a UE selects a start PRB index 2. For eMTC mode B, a UE selects a start narrowband index 2 (corresponding to PRB index 12). In order to avoid the collision of RNTI between UE with eMTC mode A and UE with eMTC mode B, for eMTC mode B, the selected start PRB index (converted from narrowband index) could also be used as a parameter in the formula of RNTI. Meanwhile, for NB-IoT, a UL resource allocation includes subcarrier set index, RU number and carrier index. As long as UEs select the different subcarrier set and carrier, Msg3 from UEs will not conflict. Hence, the subcarrier set index and carrier index could also be considered in the formula of RNTI.

- Regarding the code domain resource, the OCC index could be used to differentiate the resources in code domain. In order to further differentiate UEs to avoid conflicts, the OCC index could also be considered in the formula of RNTI.

Based on the above discussion, the following proposals are given for RNTI calculation:

Proposal 5a: The following parameters can be taken into account in the RNTI calculation for CB-msg3:

- Time domain resource: SFN index, no need of subframe index

- Frequency domain resource: start PRB index for eMTC; subcarrier set index and carrier index for NB-IoT

- Code domain resource: OCC index

Proposal 5b: For the RNTI calculation for CB-msg3, to avoid a too large RNTI, it can be considered to compress the part where SFN index is involved, for example, the SFN index can be divided by the minimum periodicity, or modulo by the length of contention resolution window.

CB-Msg4 transmission

Similar as that in legacy contention-based procedures, the collision caused by using shared resources for Msg3 transmission cannot be completely resolved by RNTI design. The additional contention resolution scheme is still needed.

The straightforward way is to reuse the contention resolution scheme in legacy random access procedure. That is, after reception of Msg3, eNB can send a DL MAC PDU in which a Contention Resolution Identity is included. For UE side, it needs to check whether the UE Contention Resolution Identity included in the DL MAC PDU matches the 48 first bits of the CCCH SDU transmitted in Msg3.

Proposal 6a: The contention resolution scheme in legacy random access procedure can be reused for CB-msg3 procedure, e.g., a UE Contention Resolution Identity can be included in the DL MAC PDU and UE checks whether the UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in Msg3.

In the procedure for transmission using PUR for CP solution, L1 ACK and L2 ACK (e.g., TA command MAC CE) can be used to early terminate the procedure if there is no data to be sent to the UE. Similarly as PUR, for CB-msg3 for CP solution, upon the reception of the CB-msg3, the network is able to determine that there is no DL data for UE, thereby it can choose to terminate the procedure efficiently. Moreover, considering that the UE Contention Resolution Identity is essential to resolve the collision for msg4, a UE Contention Resolution Identity can be used as L2 ACK to terminate the CB-msg3 procedure.

Proposal 6b: For CB-msg3 for CP solution, a UE Contention Resolution Identity can be used to early terminate the procedure.

Moreover, in legacy RACH procedure, after the reception of msg3, the network could send UE Contention Resolution Identity and RRC message together or separately. This allows the network to resolve the contention and complete the RRC procedure in two steps. The advantage of doing so is that the network can flexibly decide when to send the subsequent RRC message based on the arrival of data packets. For the same reason, the similar procedure could be adopted for CB-msg3 procedure. As a result, for CB-msg3 procedure, according to the information that needs to be carried in msg4, the potential procedures could be categorized into three cases as below:

Figure. 1 the procedures for CB-msg3/msg4 procedure

After transmission of CB-msg3 carrying a RRC message, such as RRCEarlyDataRequest message, the following cases may occur:

- For case 1, upon the reception of a matching UE Contention Resolution Identity, UE could terminate the RRC procedure. This case is applicable to the scenario that there is no DL data.

- For case 2, upon the reception of a matching UE Contention Resolution Identity, UE could stop the contention resolution window, consider this contention resolution successful, wait for the subsequent RRC message and then complete the RRC procedure according to the received RRC message, such as RRCEarlyDataComplete, RRCConnectionReject, or RRCConnectionSetup.

- For case 3, upon the reception of a UE Contention Resolution Identity and RRC message (such as RRCEarlyDataComplete, RRCConnectionReject, RRCConnectionSetup), UE could stop the contention resolution window, consider this contention resolution successful and complete the RRC procedure according to the RRC message.

From the perspective of the network, it is flexible to support those cases.

Proposal 6c: For msg4 of CB-msg3 procedure, UE Contention Resolution Identity and RRC message could be send together or separately.

However, UE couldn’t distinguish the case 1 and case 2 when it receives the UE Contention Resolution Identity as it has no idea whether there will be following RRC message. Since for case2, a C-RNTI is needed to monitor the PDCCH for the following RRC message, it can be considered to let eNB provide a C-RNTI to the UE. This assigned C-RNTI can serve as an indication to let UE know that RRC procedure cannot be terminated and needs to continue. For example, for CB-msg3 procedure, if the msg4 includes a matching Contention Resolution Identity and a C-RNTI, UE could stops the contention resolution window, and further monitors the PDCCH for the following RRC message by using the assigned C-RNTI. In some other cases, if the msg4 includes a matching Contention Resolution Identity but without a C-RNTI, UE could stops the contention resolution window, and terminates the RRC procedure.

By this way, it can also address the issue of which RNTI to be used for monitoring the PDCCH after successful contention resolution for CB-msg3. In legacy RA procedure, upon successful contention resolution, UE will set the C-RNTI to the value of the Temporary C-RNTI which is assigned in Random Access Response message by the eNB. This C-RNTI will be used after successful contention resolution. However, for CB-msg3, there is no assigned Temporary C-RNTI as no Msg1 and Msg2 exchanging. There may be thinking that the RNTI derived based on the resource associated to the (N)PUSCH occasion for CB-msg3 transmission can be taken as C-RNTI. However, we think this is not a general way. By referring to the legacy RA procedure and also fallback case of PUR, we think it’s more suitable to let eNB assign a new C-RNTI to the UE if there will be further RRC signaling to be addressed.

Proposal 7a: For CB-msg3 procedure, if the msg4 includes a matching Contention Resolution Identity and also a C-RNTI, UE could stops the contention resolution window, and further monitors the PDCCH for the following RRC message by using the assigned C-RNTI.

Proposal 7b: For CB-msg3 procedure, if the msg4 includes a matching Contention Resolution Identity but without a C-RNTI, UE could stops the contention resolution window, and terminate the RRC procedure.

Backoff/Fallback schemes

Due to skipping Msg1/Msg2 transmission, if the channel quality is not stable enough, the Msg3/Msg4 transmission may face more failures. In UE side, such failure in RRC layer would be informed to NAS and NAS may trigger another access after a while. In order to avoid triggering higher layers to perform retransmissions which may increase the delay, it can be considered to prioritize remediation at the AS layer.

Generally, two schemes for improving the performance robustness in AS layer, e.g., Backoff and Fallback, can be considered.

Regarding Backoff, it mainly refers to, upon failure occurrence, retriggering the same procedure after a certain time interval. Therefore, for CB-msg3, when the collision or congestion is detected by eNB on a certain resources occasion, the eNB can provide kind of backoff indication via the Msg4 to require backoff.

By using backoff scheme:

- On one hand, the UE can have additional opportunity to retransmit another CB-msg3, e.g, direct Msg3/Msg4 transmission without Msg1/Msg2, rather than immediately falling back to the 4-step access procedure. This is friendly and reasonable for the UE, as the previous failure was likely due to the selected resources by this UE is conflicted with another UE's choice. If given another chance to select resources, it would increase the likelihood of successful transmission.

- On the other hand, even UE is provided with another transmitting opportunity, UE still needs to delay a backoff time, then can choose another time domain resources within the shared resources for Msg3 transmission. This is also be helpful to avoid collisions possibly caused by another direct Msg3/Msg4 transmission. Here we think the range of backoff time given/configured by the network would be important as network can suitably provide this based on the real collision situation.

- Furthermore, for the OCC-based SA scheme, with combined with Backoff scheme, it may have similar benefit as DSA that is brought by more transmission opportunities. But OCC-based SA + Backoff may be better, as in DSA, multiple replicas are always needed. Meanwhile, for OCC-based SA + Backoff, the additional transmission only occurs (may be seldom) when the previous transmission fails. So OCC-based SA + Backoff scheme can achieve a better balance between more transmission opportunities and reduced resource occupation.

Observation 1: OCC-based SA can be combined with Backoff scheme and it may have similar benefit as DSA or may be better than it as the additional transmission is also allowed but only occurs (may be seldom) when the previous transmission fails.

Proposal 8a: RAN2 is suggested to support backoff scheme for CB-msg3. That is, when CB-msg3 fails, after a backoff time indicated by eNB, UE can choose an occasion to perform another CB-Msg3 transmission. FFS details.

If multiple attempts of the CB-msg3 also fail, it’s highly possible due to the bad channel quality of UE or the serious congestion on the current CB-msg3 resources. In order to ensure the successful access, the straightforward way is to select another resource to access. The candidate access resource could be the following:

- The resource of CB-msg3 in other carrier. The reselection leads to the carrier selection. For NB-IoT UE, the carrier selection needs a long switching time. Hence, it is not recommended to select to other carrier during access procedure.

- The resource of CB-msg3 in higher CE level. For the higher CE level, the requirement of received SINR decreases, thereby increasing the probability of the successful CB-msg3 reception. Moreover, due to the possibly larger configured TBS for higher CE level, the condition that the size of the MAC PDU of msg3 is smaller than or equal to the configured TBS is easier to be satisfied. Hence, fallback to higher CE level is feasible.

- The resource of 4-step PRACH. In fact, the decoding requirements of CB-msg3 are more stringent than PRACH. Even the deterioration of channel quality of the UE leads to the failed reception of CB-msg3, the reception condition of PRACH may still be satisfied. Hence, switching to RACH based procedure could increase the successful access probability. Hence, fallback to RACH based EDT procedure is feasible.

In a summary, if the CB-msg3 transmissions fail, fallback to higher CE level, or RACH based EDT procedure could be considered. The former has the advantage of shorter access time of CB-msg3. In contrast, the latter has the advantage of higher probability of successful access. In order to achieve balance between the access delay and access success probability, a stepped fallback mechanism can be considered. For example, if multiple transmissions of the CB-msg3 in a given CE level fail, firstly a higher CE level can be selected and CB-msg3 is triggered in this level. Until the CB-msg3 transmissions in highest CE level fail, a RACH based EDT procedure is triggered.

Proposal 8b: For CB-msg3 procedure, a stepped fallback to higher CE level and then to RACH based EDT procedure can be considered.

DSA

Selection time window for DSA

In last meeting, RAN2 has agreed that there will be a configurable time window for DSA CB-Msg3 occasion selection. Also according to the discussion in last meeting, the following alternatives can be considered to implement the replica selection window:

Figure 2

- A sliding replica selection window: The start time of a sliding replica selection window is determined by the first selected transmission occasion, so it’s flexible. As showed in above figure 2, if SFN 4 is selected for first transmission occasion, replica selection window 1 starts from SFN 4. The disadvantage of this solution is that no matter which replica eNB receives, eNB needs to wait for a certain length of replica selection window before discarding it.

- A fix replica selection window: the window is defined as an integer multiple of the configured periodicity of the resources for CB-msg3, its start time is determined by the configured start time of transmission occasion, so it’s fixed. As showed in above figure 2, selection window 2 starts from SFN 1. The disadvantage of this solution is that the collision probability of the transmission occasion at the end of the window may be high.

Both alternatives have their advantages and disadvantages. However, from the perspective of increasing the successful transmission, a sliding replica selection window is more applicable for CB-msg3 procedure.

Proposal 9: For DSA, it’s slightly preferred to support a sliding replica selection window whose start time is determined by the first selected transmission occasion.

RNTI calculation

For DSA, according to the discussion in last meeting, two alternatives as following can be considered for RNTI calculation:

- A specific RNTI for each replica: In DSA, UE could transmit multiple replicas on multiple time occasions. The RNTI can only be derived based on the transmission occasion for the corresponding replica. For example, in below figure 3, for DSA with 2 replicas, if eNB receives replica 1 and response Msg4, the RNTI 1 calculated based on SFN 4 is used for scrambling Msg4. if eNB receives replica 2 and response Msg4, the RNTI 2 calculated based on SFN 10 is used for scrambling Msg4.

- A common RNTI for different replicas: To avoid the complexity of the UE monitoring multiple RNTIs, in previous meeting, companies suggested to design a common RNTI that corresponds to multiple different replicas. The following sections provide a detailed analysis of every potential solutions for common RNTI.

- A specific RNTI for all replica: a RNTI is used for scrambling all Msg4 for all replicas. For example, in below figure 3, for DSA with 2 replicas, if eNB receives replica 1 and response Msg4, a RNTI 1 calculated based on SFN 4 is used for scrambling Msg4. if eNB receives replica 2 and response Msg4, the RNTI 1 is also used for scrambling Msg4. However, when eNB receives only a replica, eNB can’t know whether there are any other replicas belonging to the same user packet, nor can it determine if other replicas received at other transmission occasions belong to the same user packet as this replica. Hence, this specific RNTI for all replica isn’t feasible.

- A predefined common RNTI for all replicas: similarly as P-RNTI, it means all UEs triggering CB-msg3 would use the same RNTI for monitoring Msg4. On the one hand, if multiple UEs monitors msg4 simultaneously, they have to decode every MAC PDU within the contention resolution window until they identity a matching UE Contention Resolution Identity. This exhaustive decoding process significantly increases UE power consumption. On the other hand, if eNB schedules retransmission by PDCCH, since the UEs cannot distinguish between different PDCCHs, all UEs that receives the PDCCH will retransmit. Due to these issues, a predefined common RNTI isn’t feasible.

- An allocated common RNTI for all replicas: similarly as PUR-RNTI, a dedicated RNTI can be allocated to a UE and applied to all the replicas in CB-msg3. In PUR, the network receives a msg3 on a certain PUR resource and can associate the UE with it to obtain PUR-RNTI. However, due to the shared resource for CB-msg3, the network can’t distinguish the UE via the transmission occasion of CB-msg3 and therefore cannot associate received replicas with a certain common RNTI pre-allocated to a UE.As a result, a retransmission scheduled by PDCCH couldn’t work. Besides, if UE selects to a new cell, DSA function would become disabled. In other word, if dedicated resources can be used, a PUR is preferred rather than CB-msg3 with allocated common RNTI. Due to these issues, an allocated common RNTI also isn’t suitable.

In summary, for DSA, the common RNTI is infeasible. Hence, a specific RNTI for each replica is proposed. For DSA, the RNTI used to schedule Msg4 transmission is derived only based on the resource associated to the transmission occasion used for the corresponding replica.

Proposal 10: For DSA, the RNTI used to schedule Msg4 transmission is derived only based on the resource associated to the transmission occasion used for the corresponding replica.

Contention resolution window

For DSA, when UE transmits multiple replicas, eNB may respond Msg4 for any one of them, hence, UE has to monitor any possible Msg4.

The following alternatives can be considered for UE to apply the contention resolution window:

- Multiple contention resolution windows (Alt1): Each contention resolution window corresponds to a transmitted replica. Similar as that in SA, it is started at the transmission occasion of one replica plus 4 subframes plus UE-eNB RTT. And its length is configured by the network. Within each window, UE monitors a Msg4 addressed by the RNTI derived based on transmission occasion of the corresponding transmitted replica. For example, in figure 3, for DSA with 2 replica, UE starts window 1 at slot 12. The length of window 1 is configured as 8. During window 1, UE monitors RNTI 1 derived based on SFN 4. UE starts window 2 at slot 18. The length of window 2 is configured as 8. During window 2, UE monitors RNTI 2 derived based on SFN 10.

- One contention resolution window (Alt2): It is started at the transmission occasion of first transmitted replica plus 4 subframes plus UE-eNB RTT. Further, there are two schemes

- A long contention resolution window (Alt2-1): Its length is determined by the gap between the transmission occasions of replicas and the configured length. Within this window, in order to save power, UE needs to monitor each RNTI derived based on transmission occasion of each transmitted replica during a segmented time period. For example, in figure 3, for DSA with 2 replica, UE starts window 3 at slot 12. The length of window 3 is configured as 14. During window 3, UE monitors RNTI 1 during a certain time period 1, RNTI 1 and RNTI 2 during a certain time period 2, and RNTI 2 during a certain time period 3. RNTI 1 is derived based on SFN 4 and RNTI 2 is derived based on SFN 10.

- A restarted contention resolution window (Alt2-2): It is restarted at the transmission occasion of subsequent transmitted replica plus 4 subframes plus UE-eNB RTT. Its length is determined by the configured length. Within this restarted window, UE needs to monitor each RNTI derived based on transmission occasion of each transmitted replica during a segmented period. For example, in figure 3, for DSA with 2 replica, UE starts window 4 at slot 12. The length of window 4 is configured as 8. During this window 4, UE monitors RNTI 1. Then, UE restarts window 4 at slot 18. The length of window 4 is configured as 8. During this restarted window 4, UE monitors RNTI 1 and RNTI 2 during a certain time period 1, and RNTI 2 during a certain time period 2. RNTI 1 is derived based on SFN 4 and RNTI 2 is derived based on SFN 10.

Figure 3

From specification perspective, Alt 1 could clearly specify UE behavior. And Alt2 could maintain a timer, but introduce kind of complexity for UE. And If the UE cannot clearly determine the segmented time period to monitor corresponding RNTI, it may miss the response sent by the network. Hence, we suggest to support Alt1.

Proposal 11: For DSA, UE starts multiple contention resolution windows and monitors each RNTI for Msg4 in corresponding contention resolution window.

Conclusion

Based on the above discussion, the following proposals are given:

Proposal 1a: RAN2 can discuss the following resources types for shared resources configuration for CB-msg3:

- Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN)

- Frequency domain resources for (N)PUSCH occasions:

  - for eMTC, to configure a list of combination of a starting RB and a length in terms of contiguously allocated RB for CE mode A; to configure a list of combination of a narrowband index and PRBs within the indicated narrowband for CE mode B;

  - for NB-IoT, to configure carrier index, a RU number and a list of set of subcarriers for single-tone case; to configure a RU number and a list of set of subcarriers for multi-tone case.

- OCC resources

- repetition number

- (N)PDCCH resource

Proposal 1b: The periodicity of the time domain resources for (N)PUSCH occasions could be with the unit of radio frames, the detailed value range can be FFS.

Proposal 2: For CB-msg3 transmission, for eMTC NTN, up to three RSRP thresholds can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two RSRP thresholds can be supported for achieving at most 3 CE levels.

Proposal 3a: For CB-msg3 without OCC, a delta RSRP relative to a certain RSRP threshold of EDT can be configured to achieve corresponding RSRP threshold for CE level selection. And CB-msg3 with OCC can share the same RSRP threshold configuration as that for CB-msg3 without OCC.

Proposal 3b: For CB-msg3 without OCC, a maximum TBS needs to be configured for each CE level. And CB-msg3 with OCC can share the same maximum TBS as that for CB-msg3 without OCC.

Proposal 4a: For CB-msg3 procedure, a contention resolution window starts at the end of the Msg3 transmission plus 4 subframes plus UE-eNB RTT.

Proposal 4b: The same value range as pur-ResponseWindowTimer can be reused for the length of contention resolution window for CB-msg3 procedure.

Proposal 5a: The following parameters can be taken into account in the RNTI calculation for CB-msg3:

- Time domain resource: SFN index, no need of subframe index

- Frequency domain resource: start PRB index for eMTC; subcarrier set index and carrier index for NB-IoT

- Code domain resource: OCC index

Proposal 5b: For the RNTI calculation for CB-msg3, to avoid a too large RNTI, it can be considered to compress the part where SFN index is involved, for example, the SFN index can be divided by the minimum periodicity, or modulo by the length of contention resolution window.

Proposal 6a: The contention resolution scheme in legacy random access procedure can be reused for CB-msg3 procedure, e.g., a UE Contention Resolution Identity can be included in the DL MAC PDU and UE checks whether the UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in Msg3.

Proposal 6b: For CB-msg3 for CP solution, a UE Contention Resolution Identity can be used to early terminate the procedure.

Proposal 6c: For msg4 of CB-msg3 procedure, UE Contention Resolution Identity and RRC message could be send together or separately.

Proposal 7a: For CB-msg3 procedure, if the msg4 includes a matching Contention Resolution Identity and also a C-RNTI, UE could stops the contention resolution window, and further monitors the PDCCH for the following RRC message by using the assigned C-RNTI.

Proposal 7b: For CB-msg3 procedure, if the msg4 includes a matching Contention Resolution Identity but without a C-RNTI, UE could stops the contention resolution window, and terminate the RRC procedure.

Proposal 8a: RAN2 is suggested to support backoff scheme for CB-msg3. That is, when CB-msg3 fails, after a backoff time indicated by eNB, UE can choose an occasion to perform another CB-Msg3 transmission. FFS details.

Proposal 8b: For CB-msg3 procedure, a stepped fallback to higher CE level and then to RACH based EDT procedure can be considered.

Proposal 9: For DSA, it’s slightly preferred to support a sliding replica selection window whose start time is determined by the first selected transmission occasion.

Proposal 10: For DSA, the RNTI used to schedule Msg4 transmission is derived only based on the resource associated to the transmission occasion used for the corresponding replica.

Proposal 11: For DSA, UE starts multiple contention resolution windows and monitors each RNTI for Msg4 in corresponding contention resolution window.

Reference

- RP-243278, Revised WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc., RAN#106, December 2024

- R2-2408334, Comparison of solutions for UL capacity enhancements in IoT NTN, ZTE, RAN2#127b, October, 2024

[3] R1-2410895, Reply LS on OCC for CB-msg3 NPUSCH, RAN1#119, November 2024