3GPP TSG-RAN WG2 Meeting #129	R2-2500368

Athens, Greece, 17th – 21st Feb, 2025

Source:	ZTE Corporation, Sanechips

Title:	Further consideration on PWS support for NB-IoT over NTN

Agenda Item:	8.9.4

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed. In RAN#105, the WID is further refined in [1] and a new objective for PWS support has been added as below:

The aim of this WI is enhancement of IoT-NTN with the following objectives: <<skip the unrelated parts>> - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: <skip the unrelated part> - Support of Capacity enhancements for uplink <skip the unrelated part> - Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In RAN2#127bis meeting, RAN2 focus the discussion about new SIB for PWS and new PWS indication in direct indication information, and achieved the following agreements:

Agreements: - RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS - The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes. - For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields): - SystemInformationBlockType10-NB for primary ETWS notification; - SystemInformationBlockType11-NB for secondary ETWS notification; - SystemInformationBlockType12-NB for CMAS notification. - Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB): - etws-Indication; - cmas-Indication. - Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means) - Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

In RAN2#128 meeting, RAN2 focus the discussion about the remaining issues, and achieved the following agreements:

Agreements: - We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements. - We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed - Add the following PWS indication in Paging-NB: - etws-Indication; - cmas-Indication. - We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed - We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements. - Send a LS to RAN1 asking whether it’s possible to specify support for PWS reception in RRC_CONNECTED for NB-IoT in Rel-19.

In this document, we will give analysis on the remaining issues for broadcasting PWS message for NB-IoT by re-using the LTE mechanisms. Based on the analysis, we also give our proposals.

Discussion

  - PWS acquisition for UE in idle mode

As reference, the IE SystemInformationBlockType10 contains an ETWS primary notification.

SystemInformationBlockType10 information element

-- ASN1START

SystemInformationBlockType10 ::=	SEQUENCE {

messageIdentifier					BIT STRING (SIZE (16)),

serialNumber						BIT STRING (SIZE (16)),

warningType							OCTET STRING (SIZE (2)),

dummy								OCTET STRING (SIZE (50))	OPTIONAL,		-- Need OP

...,

lateNonCriticalExtension			OCTET STRING				OPTIONAL

}

-- ASN1STOP

SystemInformationBlockType10 field descriptions

messageIdentifier Identifies the source and type of ETWS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.44) contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.1, while the trailing bit contains bit 0 of the second octet of the same equivalent IE.

serialNumber Identifies variations of an ETWS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.45), contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.2, while the trailing bit contains bit 0 of the second octet of the same equivalent IE.

dummy This field is not used in the specification. If received it shall be ignored by the UE.

warningType Identifies the warning type of the ETWS primary notification and provides information on emergency user alert and UE popup. The first octet (which is equivalent to the first octet of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.50) contains the first octet of the equivalent IE defined in and encoded according to TS 23.041 [37], clause 9.3.24, and so on.

It can be seen that there is a dummy IE in legacy SIB10 for eMTC. In NR, the dummy IE has been deleted from SIB6 for an ETWS primary notification. There is no use case for keeping the dummy IE. Hence, we suggest to also delete the dummy IE for SystemInformationBlockType10-NB.

Proposal 1: It’s suggested to delete the dummy IE for SystemInformationBlockType10-NB.

In general, when the network changes (some of the) system information, it first notifies the UEs about this change, e.g., via paging message. And then in the next modification period, the network transmits the updated system information. But for ETWS notification and/or CMAS notifications, the network can transmit the SIB10 for ETWS primary notification and/or SIB11 for ETWS secondary notification, and/or SIB12 for one or more CMAS notifications in the nearest scheduled subframes, without waiting for the next modification period.

The corresponding UE behavior in the specification is as below:

TS 36.331 5.3.2.3 Reception of the Paging message by the UE //skip the unrelated part 1> if the etws-Indication is included and the UE is ETWS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary; 2> if the schedulingInfoList indicates that SystemInformationBlockType10 is present: 3> acquire SystemInformationBlockType10; NOTE 2: If the UE is in CE, it is up to UE implementation when to start acquiring SystemInformationBlockType10. 2> if the schedulingInfoList indicates that SystemInformationBlockType11 is present: 3> acquire SystemInformationBlockType11; 1> if the cmas-Indication is included and the UE is CMAS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary as specified in 5.2.1.5; 2> if the schedulingInfoList indicates that SystemInformationBlockType12 is present: 3> acquire SystemInformationBlockType12;

Considering that the scheduling information for SIB10, SIB11 and SIB12 contained in SIB1 (e.g., schedulingInfoList) may change, it’s suggested that, upon receiving of ETWS notification and/or CMAS notifications, ETWS or CMAS capable UE firstly needs to re-acquire SIB1 immediately without waiting until the next system information modification period boundary, and then acquire SIB10, SIB11 and SIB12 according to updated scheduling information in SIB1.

Proposal 2: Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE firstly needs to re-acquire schedulingInfoList contained in SIB1 immediately without waiting until the next system information modification period boundary, and then acquire the corresponding PWS messages.

According to the description in section “10.1.4 Paging and C-plane establishment” in TS 36.300, when extended DRX (eDRX) is used in idle mode, the following exceptional case is mentioned for LTE/eMTC UE regarding PWS function:

- ETWS, CMAS, PWS requirement may not be met when a UE is in eDRX. For EAB, if the UE supports SIB14, when in extended DRX, it acquires SIB14 before establishing the RRC connection;

Also according to TS 22.268, the requirement for ETWS is as below

5.2 Duration of delivery time Duration of the delivery time for PLMN operators is the time from the receipt of the Warning Notification by the PLMN operator, i.e. the edge of the 3GPP network, to the time that the Warning Notification is successfully delivered to the UEs. Provisioning of delivery of Primary and Secondary Notification may be required: - Primary Notification shall be delivered within 4 seconds to the UE in the Notification Area even under congestion situation. - Secondary Notification is delivered to the users in the Notification Area even under congestion situation.

For NB-IoT, the defaultPagingCycle value is {rf128, rf256, rf512, rf1024}. If the defaultPagingCycle is set as rf512, or rf1024, it is longer than 4s. In this case, the requirement of EMTS primary notification can’t be satisfied. Furthermore, if PSM is enabled and UE enters into sleep mode, any notification will be missed. Hence, we suggest to add two above specific cases where ETWS, CMAS, PWS requirement may not be met.

Proposal 3: For NB-IoT UE, the ETWS, CMAS, PWS requirement may not be met in some cases. Besides the case when the UE is in eDRX, there are also other cases, e.g., when the UE is configured with a large DRX cycle (e.g., rf512, rf1024), or when PSM is enabled.

  - PWS acquisition for UE in connected mode

In previous meeting, RAN2 has discussed whether to support the PWS acquisition for UE in connected mode and a LS has been sent to RAN1 regarding this issue.

Considering that PWS notification and acquisition will likely be used by smartphones with IoT NTN capability which may spend a large proportion of its time in the connected state, we think it’s necessary and beneficial to also support PWS acquisition for NB-IoT UE in connected mode. In this section, we will give further analysis on how to support this function from RAN2 perspective.

Taking the relevant discussion for eMTC in previous release as reference, some potential solutions are list as below for further discuss:

- UE is provided ETWS and CMAS notification via dedicated signaling: from network resource standpoint, using dedicated signaling is not efficient. It would be hard for the network to timely schedule all UEs in connected that support the features in case PWS notification needs to be sent. Even such scheduling is possible, the delay may be large which would finally result in that ETWS reception cannot meet the requirement, e.g., within 4 secs. Furthermore, many signaling, e.g., RRC reconfiguration message, cannot be supported by NB-IoT UE in connected mode that also makes this solution difficult to be used.

- UE is released to RRC_IDLE to acquire ETWS/CMAS information: upon reception of ETWS/CMAS notification, eNB could release all capable UEs in connected mode to idle mode to trigger them to acquire SIB messages in idle mode. However, the potential issue that could arise subsequently is that, those UEs may trigger access to the network later almost at the same time, causing serious signalling storm problem.

- UE monitors NPDCCH corresponding to CSS to receive ETWS and/or CMAS notification in the same narrowband where unicast transmission can be received: this is a good option to ensure that UE receives EWTS and/or CMAS notification quickly. Considering that the arrival of PWS notification is sporadic rather than occurring at regular intervals, the UE needs to monitor this CSS continuously while also monitoring USS simultaneously. This may be the main issue for this solution that needs additional UE capability and mainly needs RAN1 check. But once the UE gets the PWS notification, it can shortly stop the connected mode tasks to receive SIB10-NB~SIB12-NB, similar to how UE acquires SIB31 or GNSS in connected mode. With this scheme, the UE is allowed to keep staying in connected mode, thereby avoiding the issues caused by the above second solution. Since acquiring SIB10-NB~SIB12-NB in connected mode will be a very rare case, this will have very small performance impact on the connected mode. Moreover, from specification perspective, based on previous enhancements for acquiring SIB31 or GNSS in connected mode for IoT NTN, we think it might be easier to support SIB10-NB~SIB12-NB acquisition in connected mode over NTN compared to doing this thing in TN network. Again, the only challenge likely lies in whether the UE supporting NB-IoT NTN can receive PWS notification and USS service concurrently in connected mode.

Based on the above analysis, the following proposal is given:

Proposal 4: From RAN2 perspective, it’s suggested that NB-IoT UE in connected mode could concurrently monitor NPDCCH CSS to receive PWS notification, e.g., etws-indication and/or cmas-indication.

Proposal 4a: Once the UE gets PWS notification, it can shortly stop the connected mode tasks (no need to release the connection) to receive SIB10-NB~SIB12-NB.

Conclusion

Based on the above analysis, the following proposals are given:

Proposal 1: It’s suggested to delete the dummy IE for SystemInformationBlockType10-NB.

Proposal 2: Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE firstly needs to re-acquire schedulingInfoList contained in SIB1 immediately without waiting until the next system information modification period boundary, and then acquire the corresponding PWS messages.

Proposal 3: For NB-IoT UE, the ETWS, CMAS, PWS requirement may not be met in some cases. Besides the case when the UE is in eDRX, there are also other cases, e.g., when the UE is configured with a large DRX cycle (e.g., rf512, rf1024), or when PSM is enabled.

Proposal 4: From RAN2 perspective, it’s suggested that NB-IoT UE in connected mode could concurrently monitor NPDCCH CSS to receive PWS notification, e.g., etws-indication and/or cmas-indication.

Proposal 4a: Once the UE gets PWS notification, it can shortly stop the connected mode tasks (no need to release the connection) to receive SIB10-NB~SIB12-NB.

Reference

[1] RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc., RAN#105, September, 2024