3GPP TSG-RAN WG2 Meeting #129												R2-2500377

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.7.3

Source: 	Sharp

Title:  	RAN2 impacts of measurement gap enhancements

Document for: 	Discussion

# Introduction

In RAN2#128 meeting, RAN2 made the following agreements:

Agreements on RRM measurement gaps impacts - No MG-specific enhancements is needed on DSR operation. - RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped - No MG-specific enhancements is needed for DRX operation.

This document discusses remaining open issues on measurement gap enhancements.

# Discussion

## MAC Specification Impact of DCI-based Skipping

DCI-based gap skipping was already agreed in RAN1. The only RAN2 issue is how to capture the DCI-based gap indication. In other words, RAN2 needs to discuss whether and what RAN2 spec impacts are expected. TS 38.321 specifies the MAC behaviour during the activated measurement gap. The legacy measurement gap is configured by RRC. An assumption is that the MAC entity is aware of the time during which the measurement gap is activated. Thus, the MAC specification does not specify how the MAC entity is aware of it.

For DCI indication, RRC signaling does not provide any information on the actual activated time of measurement gap. Thus, MAC entity needs to refer to somewhere in the specification. A simplest way is that the MAC specification specifies that MAC entity considers the corresponding first measurement gap offset after the DCI as not activated. Then, we can just reuse other existing texts without any change.

TS 38.321 specifies: if the explicit indication by DCI to skip a particular gap, the MAC entity considers the corresponding first measurement gap as “not activated”.

## Semi-Static Measurement Gap Skipping

Although RAN1 agreed only DCI-based skipping, RAN2 may need to discuss whether RRC-based semi-static skipping is necessary. RAN1 also considered semi-static skipping but concluded to support only DCI-based skipping. The main reason was that the decision on the gap skipping depends on UE’s instantaneous radio condition. In this scenario, semi-static skipping may not have a great value.

One could argue that semi-static skipping may be helpful for the scenario that consecutive gap skipping is necessary. However, in this case, RRC configuration will be able to re-configure measurement gap. Hence, we do not see the necessity of semi-static measurement gap skipping.

Semi-static measurement gap skipping is not supported.

## UAI on Measurement Occasion

In RAN1#117, RAN1 reached the conclusion on UE assistance information without consensus as follows:

Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.

Instead, RAN1 sent an LS to RAN4 (R1-2405736) to ask both feasibility and usefulness of the UAI on measurement occasion. Triggered by the LS, RAN4 had the relevant discussions last year but they have not reached the conclusion yet. In this situation, RAN2 discussion on UAI may be too early. We see that RAN2 starts the discussion on UAI after RAN4 reaches the conclusion that it is at least feasible.

RAN2 discusses UAI on measurement occasion after RAN4 reaches a conclusion.

# Conclusions

RAN2 is requested to discuss and agree the following proposals:

- TS 38.321 specifies: if the explicit indication by DCI to skip a particular gap, the MAC entity considers the corresponding first measurement gap as “not activated”.

- Semi-static measurement gap skipping is not supported.

- RAN2 discusses UAI on measurement occasion after RAN4 reaches a conclusion.