3GPP TSG-RAN WG2 Meeting #129												R2-2500380

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.7.5

Source: 	Sharp

Title:  	Discussion on RLC Enhancements

Document for: 	Discussion

# Introduction

In RAN2#128 meeting, RAN2 made the following agreements on unnecessary RLC retransmission avoidance:

Agreements on unnecessary RLC retransmissions avoidance - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced Special handling to avoid PDCP control PDU discard is not needed. A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

For the objective of timely RLC retransmissions, RAN2 agreed a combined solution as follows:

Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

This document discusses next-level details of RLC enhancements.

# Discussion

## Avoiding Unnecessary Retransmission

In RAN2#127bit meeting, RAN2 agreed to have a combined approach to avoid unnecessary retransmission, as follows:

- RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

In RAN2#128 meeting, RAN2 agreed not to have a new TX to RX indication. We need to discuss how it would work in the specification. RAN2 agreed that TX stops transmission of an outdated SDU. An issue is how the TX knows whether an SDU is outdated. As a simple solution, we can consider expiry of PDCP discard timer which basically triggers packet discard. PDCP indicates the packet discard to RLC to discard packets stored in RLC. If PDCP discard operation is used to determine the outdatedness of an SDU, no additional RLC timer at the TX is necessary. Since the RX RLC entity is going to abandon the outdated SDU anyway, stopping retransmission does not bring any problem.

When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an obsolete SDU. The TX RLC entity does not perform any transmission and retransmission of the SDU.

A new RLC timer at the TX is not introduced to determine obsolete RLC SDUs.

An issue for RX entity is whether the new timer is per RLC entity or per SDU. Either option would work. RAN2 agreed that the timer starts when an SN gap is detected. This principle is very similar to the start condition of t-reassembly timer. To make the RLC procedural texts consistent throughout the RLC specification, per RLC entity seems better.

The new RLC timer at the RX is per RLC entity.

The new timer operation will be very similar to t-reassembly timer in RLC UM, whose expiry is considered as the permanent outdatedness. This timer operation needs a state variable to indicate the value of the SN which triggered the new timer.

A new RLC RX state variable is introduced to indicate the value of the SN which triggered the new timer.

Typically, the duration of t-reassembly timer is much shorter than the duration that an SDU is considered as obsolete or outdated. Thus, the duration of the new timer should be much larger than that of t-reassembly. One could argue that we can leave it up to NW implementation. However, the RLC operation has similar structure of pseudo codes, which should avoid any error case in actual UE implementation. Hence, it would be good to agree and specify the relationship between t-reassembly and the new timer.

The duration of the new RLC timer is larger than that of t-reassembly.

RAN2 agreed that the timer starts when an SN gap is detected. How the SN gap detection can be specified in the specification is a detailed issue. TS 38.322 subclause 5.2.3.2.3 captures the following start condition of t-reassembly timer in RLC AM, which means an SN gap detection:

- if t-Reassembly is not running (includes the case t-Reassembly is stopped due to actions above): - if RX_Next_Highest> RX_Next +1; or - if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU: - start t-Reassembly; - set RX_Next_Status_Trigger to RX_Next_Highest.

For simplicity, we can reuse this t-reassembly timer start condition in RLC AM:

The new RLC timer at the RX starts if

- - RX_Next_Highest> RX_Next +1 or

- - RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU.

## RLC Timely Retransmission

The conclusion on timely retransmission reached in RAN2#128 is that both autonomous retransmission and fast polling can co-exist as separate configurations. RAN2 should aim to design each solution and avoid conflict between them.

Autonomous retransmission accelerates retransmission of urgent packets without RLC NACK. Whether packet is urgent can be determined by remaining time of PDCP discard timer, similar to Rel-18/19 DSR enhancement or Rel-19 LCP enhancement. The reason for the timely retransmission is also very similar. Hence, we can reuse the similar triggering condition.

An RLC SDU whose remaining time is lower than a threshold triggers autonomous retransmission.

The decision of fast polling should be similar to that of autonomous retransmission. It would be better to unify the triggering condition as much as possible, to make the specification simple and easy to understand.

An RLC SDU whose remaining time is lower than a threshold triggers fast polling.

The next issue is how the TX RLC entity accelerates the polling when the fast polling is triggered. A poll is included when either PDU_WITHOUT_POLL or BYTE_WITHOUT_POLL is greater than its threshold value, as follows:

- increment PDU_WITHOUT_POLL by one; - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte: - include a poll in the AMD PDU as described below.

The thresholds, i.e. pollPDU and pollByte, determine how frequently a poll is transmitted. If pollPDU or pollByte is smaller, a polling is more frequently transmitted to the RX RLC entity. In this context, we can consider make pollPDU or pollByte smaller than usual to accelerate the retransmission. More specifically, an RLC entity can be configure with two sets of pollPDU and pollByte. One default set is used when fast polling is not triggered. The other set is used when fast polling is triggered.

Another set of pollPDU and pollByte is configured and used when fast polling is triggered.

# Conclusions

RAN2 is requested to discuss and agree the following proposals:

< Avoiding Unnecessary Transmission >

- When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an obsolete SDU. The TX RLC entity does not perform any transmission and retransmission of the SDU.

- A new RLC timer at the TX is not introduced to determine obsolete RLC SDUs.

- The new RLC timer at the RX is per RLC entity.

- A new RLC RX state variable is introduced to indicate the value of the SN which triggered the new timer.

- The duration of the new RLC timer is larger than that of t-reassembly.

- The new RLC timer at the RX starts if

- - RX_Next_Highest> RX_Next +1 or

- - RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU.

< RLC Timely Retransmission >

- An RLC SDU whose remaining time is lower than a threshold triggers autonomous retransmission.

- An RLC SDU whose remaining time is lower than a threshold triggers fast polling.

- Another set of pollPDU and pollByte is configured and used when fast polling is triggered.