3GPP TSG-RAN WG2 Meeting #129												R2-2500381

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.7.6

Source: 	Sharp

Title:  	Discussion on XR Rate Control

Document for: 	Discussion

# Introduction

In RAN2#128 meeting, RAN2 made the following agreements on XR rate control:

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

This document discusses remaining open issues.

# Discussion

## New Recommended Bit Rate MAC CE for XR

In RAN2#128 meeting, RAN2 confirmed the feasibility of per-QoS flow congestion estimation and agreed rate indication on a per QoS flow level. The issue is how this QoS flow level indication is supported. Two options are being considered: 1) flow indication by MAC CE and 2) flow indication by RRC with per DRB MAC CE. It seems that either option is feasible.

Figure 1. Recommended bit rate MAC CE in TS 38.321

One potential benefit of per DRB MAC CE could be reuse of existing Rel-15 Recommended Bit Rate MAC CE format. However, a reply LS from SA4 [1] mentions that granularity of uplink bit rate can be as small as 1% depending on algorithms and 1% is generally preferred. As shown in Figure 1, the size of Bit Rate field in the Rel-15 MAC CE is 6 bits which support 56 steps of bit rate. It means that RAN2 needs to define a new format with larger bit rate field anyway. Also, per QoS flow level indication by MAC CE is more flexible and it does not increase excessive RRC reconfiguration message which changes per DRB rate information.

A new Recommended Bit Rate MAC CE is introduced to support bit rate signaling per QoS flow.

To support 1% granularity of bit rate, at least 7 bits are necessary. Looking at the Rel-15 MAC CE, 2 reserved bits can be used to extend the size of bit rate field in the Rel-19 MAC CE. 7 bits or 8 bits will not increase the size of the MAC CE. In our view, 7 bits are sufficient, and we can keep one reserved bit for the future.

A new Recommended Bit Rate MAC CE is introduced to support bit rate field of 7 bits.

Bit rate field supports the value range from 100kbps to 40Mbps.

## Query or Preference Reporting by the UE

The UE usually has better information on application status. When congestion occurs in the communication network, the UE may have preferred bit rate to maintain minimum or acceptable service quality. Hence, it is reasonable to allow UE to provide its preference on the bit rate.

The preference reporting procedure has been supported since Rel-15 recommended bit rate mechanism, which consists of query and indication. Query is triggered with UE’s preferred bit rate by the UE. The query message uses the same MAC CE format as downlink bit rate indication. We see that the sample principle can be applicable for XR. There is no strong reason why a different signaling, e.g. UAI, is needed.

In Rel-19 XR, Recommended Bit Rate Query is supported.

# Conclusions

RAN2 is requested to discuss and agree the following proposals:

- A new Recommended Bit Rate MAC CE is introduced to support bit rate signaling per QoS flow.

- A new Recommended Bit Rate MAC CE is introduced to support bit rate field of 7 bits.

- Bit rate field supports the value range from 100kbps to 40Mbps.

- In Rel-19 XR, Recommended Bit Rate Query is supported.

# Reference

[1] S4-242182, Reply to LS on appropriate range and granularity of bit rate adaptation for XR applications, SA4