3GPP TSG-RAN WG2 Meeting #129	R2-2500398

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.5.2

Source:	Sharp

Title:	Discussion on on-demand SSB SCell operation

Document for:	Discussion and Decision

# Introduction

In RP#102 meeting, the following objective was approved as part of the WID [1].

- Specify procedures and signalling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, SCell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

Moreover, the following agreements were made by RAN2#127 meeting [2].

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

In this contribution, we are going to discuss on-demand SSB (OD-SSB) based L3 measurement.

# Discussions

In RAN2#127bis meeting, as shown below, whether always-on SSB and OD-SSB are measured in Case 2 was discussed. To answer this question, we may first clarify whether OD-SSB and always-on SSB are on the same frequency location or different frequency locations. If OD-SSB and always-on SSB are on different frequency locations, Option 1 should be supported considering it may not be possible for UE to perform measurement on different frequency locations. If OD-SSB and always-on SSB are on the same frequency location, it would be beneficial to support Option 2.

P1 in R2-2408295 (Samsung) - Proposal 1: For case #2, RAN2 to discuss which option is followed by UE for RRM measurements of SCell configured with on demand SSB. - Option1: UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB - Option2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

In addition, as shown below, RAN1 agreed the frequency location of on-demand SSB is the same as the frequency location of always-on SSB for some deployement scenarios [3]. Thus, in our view, we think at least Option 2 shall be supported. Whether to support Option 1 can wait for RAN1’s progress.

Agreement@RAN1#119 Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1: - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB.

Proposal 1: For Case 2, at least Option 2 is supported, i.e., when always-on SSB and on-demand SSB on the cell have the same SSB frequency location, UE uses on-demand SSB for RRM measurements when they are transmitted in addition to always-on SSB for RRM measurements.

In the current specifications, the measurement configuration (i.e., measConfig) may include measurement objects, reporting configurations, and measurement identities, where each measurement identity associates one measurement object with one reporting configuration. For serving cell measurement purpose, when a cell is configured, the UE may be further configured with a measurement object indication (i.e., servingCellMO) indicating the cell is associated with the measurement object indicated by servingCellMO. Then the UE may perform serving cell measurement based on the parameters (e.g., smtc1, ssb-ToMeasure) provided in the associated measurement object, derive measurement results based on the L3 filtering parameters provided in the associated quantity configuration as well as perform measurement reporting based on the parameters (e.g., reportInterval, reportAmount, timeToTrigger) provided in the associated reporting configuration(s).

For Case #2, always-on SSB is SSB supported in Rel-18 specifications, and the UE may perform always-on SSB based measurement as mentioned above. However, after on-demand SSB is activated, the UE may incorrectly perform on-demand SSB based measurement since the legacy parameters (e.g., smtc1, ssb-ToMeasure) provided in the associated measurement object are configured for always-on SSB instead of on-demand SSB. To resolve this, one option is to configure additional corresponding parameters (e.g., smtc1, ssb-ToMeasure) which can be enabled when on-demand SSB is activated. Thus, we suggest RAN2 WG discusses what additional parameter(s) needs to be provided in the measurement object for on-demand SSB and how UE adapts the additional parameter(s) when on-demand SSB is activated/deactivated. Similarly, whether the additional parameter(s) (e.g., reportInterval, reportAmount, timeToTrigger) provided in the associated reporting configuration(s) and additional L3 filtering parameters provided in the associated quantity configuration are needed for on-demand SSB can be further studied.

Proposal 2: For Case 2, RAN2 WG to discuss what additional parameter(s) needs to be provided in the measurement object/report configuration/quantity configuration for on-demand SSB and how UE adapts the additional parameter(s) when on-demand SSB is activated/deactivated.

According to Section 5.5.2.1 of TS 38.331 [4], for L3 measurement, the network shall configure separate measurement objects to be associated with different SSB frequency locations within a cell group. Therefore, for serving cell measurement purpose, when always-on SSB and OD-SSB on the cell have the same SSB frequency location, the cell shall be configured with one measurement object that is associated with both always-on SSB and OD-SSB. Moreover, if always-on SSB and on-demand SSB on the cell have different SSB frequency locations, separate measurement objects shall be configured to be associated with the cell, where one measurement object is for always-on SSB based measurement and another measurement object is for OD-SSB based measurement. For example, to achieve this, the cell can be configured with separate servingCellMO IEs (e.g., legacy servingCellMO and servingCellMO-r19) that indicate different measurement objects.

TS 38.331 Section 5.5.2.1 The network applies the procedure as follows: <Text omitted> - to ensure that, in the measConfig associated with a CG: - for all SSB based measurements there is at most one measurement object with the same ssbFrequency; <Text omitted>

Proposal 3: For Case 2, when always-on SSB and on-demand SSB on the cell have the same SSB frequency location, the cell shall be configured to be associated with one measurement object that is associated with both always-on SSB and on-demand SSB.

Proposal 4: For Case 2, when always-on SSB and on-demand SSB on the cell have different SSB frequency locations, the cell shall be configured to be associated with separate measurement objects, where one measurement object is for always-on SSB based measurement and another measurement object is for OD-SSB based measurement.

# Conclusions

Based on the discussion above, we have the following proposals:

Proposal 1: For Case 2, at least Option 2 is supported, i.e., when always-on SSB and on-demand SSB on the cell have the same SSB frequency location, UE uses on-demand SSB for RRM measurements when they are transmitted in addition to always-on SSB for RRM measurements.

Proposal 2: For Case 2, RAN2 WG to discuss what additional parameter(s) needs to be provided in the measurement object/report configuration/quantity configuration for on-demand SSB and how UE adapts the additional parameter(s) when on-demand SSB is activated/deactivated.

Proposal 3: For Case 2, when always-on SSB and on-demand SSB on the cell have the same SSB frequency location, the cell shall be configured to be associated with one measurement object that is associated with both always-on SSB and on-demand SSB.

Proposal 4: For Case 2, when always-on SSB and on-demand SSB on the cell have different SSB frequency locations, the cell shall be configured to be associated with separate measurement objects, where one measurement object is for always-on SSB based measurement and another measurement object is for OD-SSB based measurement.

# References

- RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson (Moderator).

- R2-2409210, “Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands, 19 - 23 August, 2024”, Chairman (InterDigital).

- “Draft_Minutes_report_RAN1#119_v030”, Chairman (Samsung).

- TS 38.331 V18.2.0, Radio Resource Control (RRC) protocol specification, 2024-06.