3GPP TSG-RAN WG2 Meeting #129	R2-2500401

# Athens, Greece, 17 – 21 February 2025

Agenda item:		8.7.5

Source:	Xiaomi

Title:	RLC AM retransmission enhancements

Document for:		Discussion and Decision

Introduction

In RAN2#128 meeting, following was agreed regarding RLC retransmission enhancements [1]:

Agreements on unnecessary RLC retransmissions avoidance - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced - Special handling to avoid PDCP control PDU discard is not needed. - A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. - The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report. Agreements on Timely RLC retransmissions – autonomous versus polling - Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

In this contribution, we discus RLC AM retransmission enhancements.

Discussion

Avoid unnecessary RLC retransmissions

RAN2 agreed to adopt a “combined” approach for avoiding unnecessary RLC retransmissions. There are several open issues for further discussion.

Tx side operation

Currently RLC SDU discard procedure is specified in TS 38.322, clause 5.4, as below:

When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

For “combined” approach, when indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity shall discard the indicated RLC SDU, regardless of whether the RLC SDU or a segment has been submitted to the lower layers or not. When discarding an RLC SDU, an RLC SN gap can be introduced. Since Rx side uses status report to inform Tx side, there is no issue of discarding the RLC SDU which has already been submitted to MAC, nor introducing an RLC SN gap.

Proposal 1: Tx side discards the RLC SDUs indicated by PDCP layer for discarding, regardless of whether the RLC SDU or a segment has been submitted to the lower layers or not. An RLC SN gap can be allowed during the discard procedure.

Rx timer operation

As agreed in RAN2#128 meeting, “A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer.” One aspect is the criterion of how the gap is detected at RLC layer.

Currently RLC state variables RX_Next and RX_Next_Highest indicate the lower edge of RLC receiving window and highest received state variable, respectively. As shown in Figure 1 below, gap is detected at RLC layer when RX_Next_Highest > RX_Next + 1 (as in case a), or RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU (as in case b). There is no gap in cases (c) and (d).

Figure 1: Gap detection at RLC receiver

Proposal 2: RLC Rx timer is started (i.e. when the gap is detected) if RX_Next_Highest > RX_Next + 1, or RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU.

It is obvious that when the start condition is not satisfied (e.g. due to the update of RX_Next or RX_Next_Highest), RLC Rx timer should be stopped.

Proposal 3: RLC Rx timer is stopped when the start condition is not satisfied.

The purpose of RLC Rx timer is to move the lower edge of the RLC receiving window (i.e. RX_Next) so that RLC Rx won’t wait for the obsolete SDUs. Therefore upon the expiry of the RLC Rx timer, RX_Next should be advanced. RX_Next can be advanced to e.g. the SN of the first RLC SDU with SN > RX_Next for which not all bytes have been received.

Proposal 4: Upon RLC Rx timer expiry, RX_Next is advanced. FFS to which value RX_Next is advanced to.

Rx side informs Tx side about the abandoned SDUs

Following was agreed in RAN2#128 meeting, “The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.”

There are two aspects related to RLC status report:

a) Whether additional triggers are needed for RLC status report;

b) Whether status report content should be enhanced to indicate the abandoned RLC SDUs determined by the RLC Rx timer.

As for the additional trigger for RLC status report, from above discussion, upon RLC Rx timer expiry, the lower edge of RLC receiving window (i.e. RX_Next) is advanced, and this should be informed to the RLC Tx.

Proposal 5: Upon RLC Rx timer expiry, RLC status report is triggered.

Regarding whether status report content should be enhanced to indicate the abandoned RLC SDUs, it should be noted that RLC status report contains the information for ACK/NACK starting from RX_Next, as from TS 38.322 clause 5.3.4:

When constructing a STATUS PDU, the AM RLC entity shall:

-	for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer:

…

In addition, the update of RX_Next already implies that the abandoned RLC SDUs will be considered as received by the Tx side, according to TS 38.322, clause 6.2.3.10:

When the transmitting side of an AM RLC entity receives a STATUS PDU, it interprets that all RLC SDUs up to but not including the RLC SDU with SN = ACK_SN have been received by its peer AM RLC entity, excluding those RLC SDUs indicated in the STATUS PDU with NACK_SN, portions of RLC SDUs indicated in the STATUS PDU with NACK_SN, SOstart and SOend, RLC SDUs indicated in the STATUS PDU with NACK_SN and NACK_range, and portions of RLC SDUs indicated in the STATUS PDU with NACK_SN, NACK range, SOstart and SOend.

Therefore a legacy status report can be used to indicate the abandoned RLC SDUs. With status report informing the Tx side about the update of RLC receiving window, the Tx side can advance the transmitting window accordingly. This is aligned with RAN2 assumption that “the RLC receiving window always advances to any given RLC SN before the transmitting window does”.

There was proposal to have explicit RLC indication of RLC SDUs abandoned by the receiver. However, with proper timer configuration, RLC SDUs abandoned by the receiver have already been discarded by the transmitter anyway, so there is no need to explicitly indicate those abandoned SDUs to the transmitter.

Proposal 6: Legacy status report can be reused without any enhancement. There is no need to introduce explicit RLC indication of RLC SDUs abandoned by the receiver.

Timely RLC retransmission

In RAN2#128 meeting, following was agreed “Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them”.

Polling enhancements

One potential polling enhancement is to trigger polling when there are delay critical RLC SDU(s). Since the main point of polling is to inform the receiver to provide status report, the condition of having delay critical SDU alone is not sufficient for polling. For example, if a delay-critical SDU has not been transmitted yet, it is not useful to include a poll bit. Considering that HARQ retransmission is in general helpful to improve spectrum efficiency, it is helpful to only trigger polling when the time duration of submitting a delay critical RLC SDU to MAC layer exceeds a configured threshold. An example is shown in Figure 2 below. Polling is triggered when both conditions are satisfied: 1) A RLC SDU is delay critical (as the remaining time until expiry of discardTimer is less than remaininingTimeThreshold); and 2) time duration of submitting the RLC SDU to MAC layer (Δt) exceeds a configured threshold.

Figure 2: Polling enhancement considering transmission status

Proposal 7: The Tx RLC entity includes a poll when the time duration of submitting a delay critical RLC SDU to MAC layer exceeds a configured threshold.

Autonomous retransmission

In current RLC AM, retransmissions are prioritized over initial transmissions, as from TS 38.322 clause 5.2.3.1.1, as copied below:

The transmitting side of an AM RLC entity shall prioritize transmission of AMD PDUs containing previously transmitted RLC SDUs or RLC SDU segments over transmission of AMD PDUs containing not previously transmitted RLC SDUs or RLC SDU segments.

Autonomous retransmissions are based on conditions like timer / counter, instead of actual RLC status report. Therefore prioritizing autonomous retransmission over initial transmission might impact system performance (e.g. capacity, packet latency). It is suggested to differentiate autonomous retransmission when considering prioritization in RLC transmitter side.

Proposal 8: Autonomous retransmission should be differentiated from normal RLC retransmission base on RLC status report when considering prioritization in RLC transmitter side.

Conclusion

In this contribution, we discuss RLC AM retransmission enhancements, and propose the following:

Proposal 1: Tx side discards the RLC SDUs indicated by PDCP layer for discarding, regardless of whether the RLC SDU or a segment has been submitted to the lower layers or not. An RLC SN gap can be allowed during the discard procedure.

Proposal 2: RLC Rx timer is started (i.e. when the gap is detected) if RX_Next_Highest > RX_Next + 1, or RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU.

Proposal 3: RLC Rx timer is stopped when the start condition is not satisfied.

Proposal 4: Upon RLC Rx timer expiry, RX_Next is advanced. FFS to which value RX_Next is advanced to.

Proposal 5: Upon RLC Rx timer expiry, RLC status report is triggered.

Proposal 6: Legacy status report can be reused without any enhancement. There is no need to introduce explicit RLC indication of RLC SDUs abandoned by the receiver.

Proposal 7: The Tx RLC entity includes a poll when the time duration of submitting a delay critical RLC SDU to MAC layer exceeds a configured threshold.

Proposal 8: Autonomous retransmission should be differentiated from normal RLC retransmission base on RLC status report when considering prioritization in RLC transmitter side.

- References

[1] R2-2410915, Session chair (Huawei), “Report from session on R18 MBS, R18 QoE and R19 XR”