3GPP TSG-RAN WG2 Meeting #129	R2-2500402

# Athens, Greece, 17 – 21 February 2025

Agenda item:		8.7.6

Source:	Xiaomi

Title:	XR rate control

Document for:		Discussion and Decision

Introduction

In RAN2#128 meeting, following was agreed on XR rate control [1]:

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

In RAN#106 meeting, downlink congestion indication (in addition of the uplink) was discussed, and slide#3 of RP-243305 [3] is endorsed, with related content as below:

RAN2 to continue working on the uplink solution and to liaise with SA2/SA4 if there are issues if the same approach would be extended for the DL as well, assuming the solution extension for DL would not cause issues in RAN2 side (not more work than what would be needed to prevent use of the solution for downlink direction). RAN3 to be kept informed of the progress.

In this contribution, we discus XR rate control.

Discussion

Rate indication

In RAN2#128 meeting, RAN2 agreed that “Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.”

Regarding how per QoS flow level is signalled, there are mainly two alternatives:

- Option a: Indicate QoS flow IDs directly in MAC CE

- Option b: Indicate QoS flows IDs to be rate controlled in RRC signalling, and indicate the DRB ID / LCID in MAC CE.

In Option a, signalling QFI alone is not sufficient since QFI is only unique with a PDU session. In NR SDAP header, PDU Session ID is not signalled since one DRB can only contain data from one PDU Session, as from ASN.1 below. However in MAC CE, indication of PDU Session ID / DRB ID / LCID is needed to uniquely identify a QoS flow. Considering QFI has 6 bits, the signalling overhead to indicate QoS flow is 14 / 12 / 12 bits when using PDU Session ID / DRB ID / LCID, respectively. For comparison, in Option b, only 6 bits are needed to signal DRB ID / LCID.

SDAP-Config ::=                     SEQUENCE {

pdu-Session                         PDU-SessionID,

sdap-HeaderDL                       ENUMERATED {present, absent},

sdap-HeaderUL                       ENUMERATED {present, absent},

defaultDRB                          BOOLEAN,

mappedQoS-FlowsToAdd                SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI OPTIONAL, -- Need N

mappedQoS-FlowsToRelease            SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI OPTIONAL, -- Need N

...

}

PDU-SessionID ::=   INTEGER (0..255)

The other difference between Option a and b is mainly related to the rate control of multiple QoS flows within a DRB. In Option a, the rate of each QoS flow is controlled explicitly by gNB. In Option b, if gNB configures in RRC signalling that multiple QoS flows within a DRB are rate controlled, then after receiving the MAC CE of rate control, UE implementation can decide the rate distribution among different QoS flows. From previous discussion, companies have different views on which way is preferred. Given that Option b has clear benefit of less overhead, it is proposed to agree Option b.

Table 1: Comparison of rate indication options

|  | Option a | Option b |
| --- | --- | --- |
| Signalling overhead to indicate QoS flow | 14 / 12 / 12 bits when using PDU Session ID / DRB ID / LCID + QFI | 6 bits when using DRB ID / LCID |
| Rate control of multiple QoS flows within a DRB | Rate of each QoS flow is controlled explicitly by gNB | UE implementation can decide the rate distribution among different QoS flows. |

Proposal 1: QoS flows to be rate controlled are configured in RRC signalling, and DRB ID / LCID is signalled in MAC CE.

Range / granularity of bit rate

In LS R2-2409273, RAN2 asked SA4 about the appropriate range and granularity of uplink bit rates of XR applications. SA4 reply LS S4-242182 [2] indicates the following:

The range varies from 100kbps to 40Mbps. The numbers are based on the SA4 traffic characteristics study for XR services documented in 3GPP TR 26.926. Specifically, for XR split rendering and cloud gaming, a typical uplink data rate is 200 kbps. For AR conversational, a typical uplink data rate is 20Mbps.

The granularity varies among existing data rate adaptation algorithms, and it can be as small as 1% for some algorithms (e.g., transport-wide congestion control as documented in 3GPP TR 26.822), yet also coarser for other algorithms (e.g., video codecs that can adapt to a given target bit rate constraint). From an application perspective, finer granularity, e.g., 1%, is generally preferred, if possible, but coarser granularity may also be sufficient, e.g., 10%.

In summary, SA4 replies that the bit rate range is from 100 kbps to 40 Mbps, and preferred granularity is 1%. Assuming same exponential formula used for BSR as below is used for rate control:

Bk=Bmin×1+pk, with p=Bmax/Bmin1/N-1-1

We can derive that the number of code points

N=1+lnBmax/Bminln(1+p)

We can derive that N = 604, with p = 0.01, Bmax = 40000, Bmin = 100. This implies that 10 bits are needed to signal the bit rate, with granularity p = 0.587% if all 210 code points are used. If using 9 bits to signal the bit rate, the granularity p = 1.18%.

Observation 1: Assuming bit rate range of 100 kbps to 40 Mbps, 9 and 10 bits of bit rate signalling have the granularity of 1.18% and 0.587%, respectively.

There are two issues of existing Recommended bit rate MAC CE due to the usage of bitRateMultiplier,.

- Duplicated code points: An example is that 100 times of 40 kbps is 4000 kbps, which is another code point when multiplier is not used. There are 56 codes points used in the MAC CE, for data rate in the range of 0~8000 kbps. Considering 4 multipliers, there are 56 * 5 = 280 code points. However, there are only 196 unique values among the code points, which means that (280 – 196) / 280 = 30% code points are wasted.

- The code points are not evenly distributed in linear or exponential scale. For example, following are a subset of codepoints in ascending order. It can be seen that there are code points close to each other, e.g. 450000 and 455000, and code points with larger distance like 800000, 900000, and 1000000. This is not an efficient way to utilize the available code points.

| Code point value | Ratio relative to previous value |
| --- | --- |
| 450000 | 1.071 |
| 455000 | 1.011 |
| 490000 | 1.077 |
| 500000 | 1.021 |
| 525000 | 1.050 |
| 550000 | 1.048 |
| 560000 | 1.018 |
| 600000 | 1.071 |
| 650000 | 1.083 |
| 700000 | 1.077 |
| 750000 | 1.071 |
| 800000 | 1.067 |
| 900000 | 1.125 |
| 1000000 | 1.111 |

Observation 2: Current Recommended bit rate MAC CE has the following issues regarding index to bit rate mapping:

1) There are 30% wasted code points.

2) The code points are not evenly distributed in linear or exponential scale.

From above observations, a new table is needed to address the drawbacks of existing Recommended bit rate MAC CE.

Proposal 2: A new table with improved index to bit rate mapping is needed for rate control.

MAC CE content

Existing Recommended bit rate MAC CE format is specified in TS 38.321, as below. It contains 4 fields: LCID, UL/DL direction, Bit Rate, X (Bit Rate Multiplier).

Figure 6.1.3.20-1: Recommended bit rate MAC CE

Given Proposal 2 in above discussion, a new MAC CE is needed to support new table with improved index to bit rate mapping. From Proposal 1 in above discussion, DRB ID / LCID should be included in MAC CE to identify QoS flows for rate control. X (multiplier) field might not be needed if bit rate table can cover the whole range of bit rate provided by SA4.

Proposal 3: A new MAC CE is introduced for Rel-19 rate control, and it includes at least the following fields: Bit Rate, DRB ID or LCID.

In contributions submitted to RAN2#127bis meeting, there were proposals to introduce a congestion relief MAC CE to signal that there is no more congestion. It should be noted that with the bit rate indication, gNB can increase the bit rate if there is no more congestion. Hence there is no need to explicitly introduce a congestion relief MAC CE.

Proposal 4: A single MAC CE is used for both cases: 1) reduce the data rate when congestion happens; 2) increase the data rate when congestion is relieved.

UL MAC CE query

Legacy Recommended bit rate MAC CE is applicable for both DL and UL data, and can be sent from gNB to UE, or from UE to gNB (as “query”).

In RAN2#128 meeting, one FFS is “whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.” This is also related to the DL rate control aspect from end-to-end perspective. Considering the use case that gNB indicates to the UE to increase data rate when congestion is relieved. In RAN-assisted codec adaptation, the corresponding signalling flow in LTE is documented in TS 26.114 Figure 10.7-2, as copied below:

Figure 10.7-2 Uplink bitrate increase based on ANBR

In step 4, AN-1 (Access Network) sends to UE-1 ANBR (Access Network Bitrate Recommendation, i.e. Recommended bit rate MAC CE) to increase data rate. With the increase of DL data rate, the communication peer (UE-2) needs to increase the DL data rate accordingly, therefore UE-2 sends ANBRQ (Access Network Bitrate Recommendation Query, i.e. Recommended bit rate query MAC CE) to AN-2 in step 7, and AN-2 response with ANBR to configure DL data rate in step 8.

From above signalling flow, the bit rate query message from UE to gNB (in step 7 above) is helpful from end-to-end perspective. Given that the query MAC CE is related to the rate control MAC CE e.g. in terms of bit rate, it is helpful to reuse the rate control MAC CE introduced in Rel-19, instead of legacy query MAC CE.

Triggering of legacy query MAC CE is based on upper layer, as in TS 38.321 (copied below). The same principle can be applied for Rel-19 rate control mechanism.

If the MAC entity is requested by upper layers to query the gNB for the recommended bit rate for a logical channel and for a direction (i.e. for uplink or downlink), the MAC entity shall:

1>	if a Recommended bit rate query for this logical channel and this direction has not been triggered:

2>	trigger a Recommended bit rate query for this logical channel, direction, and desired bit rate.

Proposal 5: Query indication (similar to Recommended bit rate query) from UE to gNB is supported by reusing the rate control MAC CE introduced in Rel-19. Triggering of the query message is from upper layers, as in legacy Recommended bit rate query.

Split bearer in DC

In DC scenario, split bearer can be used for XR services to boost the throughput. If both MN and SN send MAC CEs for rate control of the same split bearer or same QoS flow(s) mapped to split bearer, some discussion might be needed on UE behavior regarding which data rate to be applied for the split bearer or DRB as indicated in the MAC CEs, e.g. whether UE follows latest MAC CE from MN or SN, or UE should consider the MAC CEs jointly.

Proposal 6: RAN2 to discuss whether split bearer should be considered for rate control.

Conclusion

In this contribution, we discuss XR rate control, and have the following observations:

Observation 1: Assuming bit rate range of 100 kbps to 40 Mbps, 9 and 10 bits of bit rate signalling have the granularity of 1.18% and 0.587%, respectively.

Observation 2: Current Recommended bit rate MAC CE has the following issues regarding index to bit rate mapping:

1) There are 30% wasted code points.

2) The code points are not evenly distributed in linear or exponential scale.

We propose the following:

Proposal 1: QoS flows to be rate controlled are configured in RRC signalling, and DRB ID / LCID is signalled in MAC CE.

Proposal 2: A new table with improved index to bit rate mapping is needed for rate control

Proposal 3: A new MAC CE is introduced for Rel-19 rate control, and it includes at least the following fields: Bit Rate, DRB ID or LCID.

Proposal 4: A single MAC CE is used for both cases: 1) reduce the data rate when congestion happens; 2) increase the data rate when congestion is relieved.

Proposal 5: Query indication (similar to Recommended bit rate query) from UE to gNB is supported by reusing the rate control MAC CE introduced in Rel-19. Triggering of the query message is from upper layers, as in legacy Recommended bit rate query.

Proposal 6: RAN2 to discuss whether split bearer should be considered for rate control.

- References

[1] R2-2410915, Session chair (Huawei), “Report from session on R18 MBS, R18 QoE and R19 XR”

[2] S4-242182, SA4 LS, “Reply to LS on appropriate range and granularity of bit rate adaptation for XR applications”

[3] RP-243305, Nokia (moderator), “Release 19 XR enhancements - Way forward on WID scope revision”