3GPP TSG-RAN WG2 Meeting #129	R2-2500403

# Athens, Greece, 17 – 21 February 2025

Agenda item:		8.11.3

Source:	Xiaomi

Title:	Other aspects of SBFD

Document for:		Discussion and Decision

Introduction

In RAN2#128 meeting, following was agreed regarding cell reselection enhancements and inter-cell CSI-RS measurement [1]:

Prioritization of SBFD cells / frequencies during cell reselection is not considered.

RAN2 wait for input from the other WGs regarding whether for inter-cell CSI-RS measurements, UE needs to be provided with information of the SBFD configuration of neighbouring cells.

In this contribution, we discuss RAN2 aspects of SBFD other than RACH.

Discussion

General working procedure in RAN2

In RAN2#127bis, RAN2 confirms RAN1 agreements regarding the RRC configuration of UL resource muting for PUSCH and L1 based UE-to-UE CLI reporting. SFBD is a RAN1-led WI. Typically before the end of WI completion, RAN1 provides RAN2 with a RRC parameter list for related RRC configuration, as well as UE feature list to define related UE capabilities. RAN2 can then prepare related running CRs to TS 38.331 and 38.306 based on RRC parameter list and UE feature list. Unless RAN2 identifies some issues with RAN1 decision, or wants to influence RAN1 discussion of RRC parameter list and UE feature list, there is no need for RAN2 to simply confirm what RAN1 has agreed. RAN2 just needs to focus on RAN2 impacts based on RAN1 agreements.

Proposal 1: In general, RAN2 postpones the discussion on details for RRC configuration and UE capability before receiving relevant RRC parameter list and UE feature list from RAN1, at least for the following aspects: SBFD resource indication, UL resource muting for PUSCH, L1 based UE-to-UE CLI measurement and reporting.

gNB-to-gNB CLI measurement

In RAN1 LS R2-2407917, RAN1 indicates the following agreement:

From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported.

In RAN2#128 meeting, there was proposal (e.g. in R2-2409572) that the set of one or more periodic NZP CSI-RS resource configuration is signalled in RAN2 inter-node RRC message MeasurementTimingConfiguration, with the argument that measurement resource related configuration (e.g. CSI-RS resource for mobility, SSB resource configuration) is already included in MeasurementTimingConfiguration.

It should be noted that in RAN3#125bis meeting,

RAN3 agrees to Exchange the following information over Xn interface and F1 interface:

- Semi-static cell-specific SBFD time and frequency location configuration with cell granularity (FFS on container or explicit IEs over XnAP and F1AP)

- Measurement resource configuration

- Strongest DL beam information (FFS on the details)

- CLI-mitigation request (FFS on the details)

There were already extensive contributions in RAN3#126 meeting regarding how measurement resource configuration is signalled, and MeasurementTimingConfiguration aspect was also mentioned in RAN3 contributions. It is proposed that the issue is left to RAN3.

Proposal 2: RAN2 wait for RAN3’s decision regarding signaling of measurement resource configuration (including NZP CSI-RS resource configuration) for gNB-to-gNB CLI measurement.

Interference issue

In Rel-19 SBFD, gNB can receive in UL and transmit in DL simultaneously. An example is shown in Figure 1 below. In Case 1, the DL reception may cause interference to the PUSCH reception in network side. In Case 2, neighbor cell’s DL transmission may also cause interference to the PUSCH reception in network side, i.e., CLI. Although the interference issue above could be mitigated to certain extent by implementation as well as CLI handling techniques, the PUSCH transmitted in SBFD symbols still suffers more interference than PUSCH transmitted in non-SBFD symbols. RAN2 needs to discuss whether such PUSCH quality difference between SBFD and non-SBFD symbols should be addressed or not.

Figure 1: Interference issue of PUSCH in SBFD symbols

Note that quality difference between SBFD and non-SBFD symbols for RACH was already identified by RAN1, which agreed the following in RAN1#118bis meeting:

For RACH configuration Option 1, support separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols

Proposal 3: RAN2 to discuss whether PUSCH quality difference between SBFD and non-SBFD symbols should be addressed or not.

Conclusion

In this contribution, we discuss RAN2 aspects of SBFD other than RACH, and propose the following:

Proposal 1: In general, RAN2 postpones the discussion on details for RRC configuration and UE capability before receiving relevant RRC parameter list and UE feature list from RAN1, at least for the following aspects: SBFD resource indication, UL resource muting for PUSCH, L1 based UE-to-UE CLI measurement and reporting.

Proposal 2: RAN2 wait for RAN3’s decision regarding signaling of measurement resource configuration (including NZP CSI-RS resource configuration)

Proposal 3: RAN2 to discuss whether PUSCH quality difference between SBFD and non-SBFD symbols should be addressed or not.

- References

[1] R2-2410912, RAN2 Vice Chairman (CATT), "Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD"