3GPP TSG-RAN WG2 Meeting #129		       R2-2500413

Athens, Greece: 17th February – 21th February 2025

Title:	Discussion on LCM for UE-sided models

Source:	ASUSTeK

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

- Introduction

The WI NR_AIML_air agreed to work on the signalling mechanism of applicable functionalities /models. In the previous meeting, RAN2 sent an LS to RAN1 asking several questions regarding applicable functionalities. In this contribution, we would like to discuss some issues based on RAN1 response and RAN2 progress.

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models

- Discussion

In the LS to RAN2 (R1-2410898), RAN1 provided some information regarding the terminologies, what may be the content provided in each step of the applicability reporting procedure, and provided 2 potential directions A and B.

From RAN1 LS response:

- The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs

- The concept/terminology “functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters

- The Activated functionalities may be enabled based on CSI framework.

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig Any specification impact is a separate discussion

For Direction A, signalling overhead may be reduced (i.e., step 5) with the reliability as tradeoff (e.g., the configured functionality in step 3 is non-applicable). The benefit of direction A is that an applicable configuration could be activated directly after step 4, and step 5 could be omitted.

For Direction B, the reliability may be guaranteed with increased signalling (e.g., step 5, repeatedly provided parameters in step 3 and step 5). The benefit of direction B is that it is more flexible to provide the parameters for applicability report (e.g., for training and data collection purposes).

From our perspective, both directions have their pros and cons, which one is better may depend on the use case. But to make progress, we would like to propose RAN2 to first decide which direction to go, or support both and give the flexibility to the NW, in order to form the basis for the discussion.

Proposal 1: RAN2 should support both configuration options of Direction A and B.

If direction A is supported, an issue for direction A is what to do when the configuration received in step 3 is non-applicable at the UE. From the RAN1 agreements, the configurations are activated when the applicability is reported to the NW. It seems to imply that the non-applicable configurations are considered deactivated, which may cause ambiguity (e.g., for the case of periodic CSI reporting). FFS how can the configuration be reactivated (e.g., when the configuration is modified/first released then added).

Proposal 2: For Direction A, RAN2 discuss whether the non-applicable configurations provided in step 3 are discarded or kept as deactivated (for future applicability change).

For Direction B, a container including inference related parameters is provided in step 3, and the inference configuration is provided in step 5. The inference related parameters may be part of the inference configuration.  To prevent the NW providing duplicated parameters in step 3 and step 5, a container ID different from CSI-ReportConfigId may be used for identification of the provided functionality (i.e., container/sets of parameters) in step 3, and the NW may provide a complementary configuration in step 5. For example, when the NW provides three containers with IDs 1, 2, and 3 in step 3. The UE may indicate the container with ID 2 to be applicable in step 4. The NW may provide an inference configuration complement to the container with ID 2 in step 5 (i.e., the inference configuration includes parameters additional to the parameters in the container).

Proposal 3: For Direction B, a container ID is used to identify the container provided in step 3 and report to the NW.

Proposal 4: For Direction B, the NW provides additional parameters in step 5 complement to the parameters provided in step 3 (i.e., the inference configuration is complement to a container).

There is another issue that exists in both directions we think is worth discussion. To configure and/or request for the applicability of configurations, the NW may have to provide excessive configurations/containers that are in step 3, due to the multiple potential combinations of AI/ML settings (not indicated by associated ID) (e.g., Set A and Set B combinations). These configurations/containers may be for a same purpose and therefore share some parameters. For example, when the NW wants to receive measurement results for a Set A, it may not matter which Set B is used as long as the prediction can be done. To achieve this, the NW may have to provide configurations/containers for all possible Set B configurations because it is unclear to the NW which Set B configurations may be applicable by the UE, while other parameters except Set B between multiple provided configurations may be identical. However, when received the applicability of the multiple configurations/containers, the NW may only configure one of them in response, which is not efficient.

Observation 1: The NW may provide multiple configurations/containers for a same purpose in step 3, but only one of them may be eventually configured.

Proposal 5: For one or more inference parameters within a configuration/container provided in step 3, multiple options (e.g., candidate values) may be provided to the UE. The UE may indicate in step 4 that a configuration/container is applicable when configured with the selected options.

- Conclusion

Proposal 1: RAN2 should support both configuration options of Direction A and B.

Proposal 2: For Direction A, RAN2 discuss whether the non-applicable configurations provided in step 3 are discarded or kept as deactivated (for future applicability change).

Proposal 3: For Direction B, a container ID is used to identify the container provided in step 3 and report to the NW.

Proposal 4: For Direction B, the NW provides additional parameters in step 5 complement to the parameters provided in step 3 (i.e., the inference configuration is complement to a container).

Observation 1: The NW may provide multiple configurations/containers for a same purpose in step 3, but only one of them may be eventually configured.

Proposal 5: For one or more inference parameters within a configuration/container provided in step 3, multiple options (e.g., candidate values) may be provided to the UE. The UE may indicate in step 4 that a configuration/container is applicable when configured with the selected options.

References

[1] RP-240774, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”

[2] R1-2410898 Reply LS on applicable functionality reporting for beam management UE-sided model

Annex