3GPP TSG RAN WG2 #129				R2-2500414

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.2.2

Source:		ASUSTeK

Title:				Discussion on Ambient IoT paging message design

Document for: Discussion and Decision

- Introduction

In the WID agreed in RAN#106 meeting [1], the following objectives on Ambient IoT RAN2 scope were set:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

In this contribution, we further discuss the detailed design of A-IoT paging message.

- Discussion

2.1  Contents of paging message

2.1.1  Service information

Firstly, the paging message should include “service information”. The service information here includes service type, command type and/or device type information, which could be received by the reader from CN request for the service. And this information could be used to avoid duplicated response.

The service type (e.g. inventory, command or “inventory+command”) can be indicated to the device, since the device could respond properly. For example, the device knows whether there is subsequent transmission if the service type is known to the device. Similarly, the command type (e.g. read, write) is also beneficial to the device monitoring.

The device type includes device 1, device 2a and device 2b. Although only device 1 is supported in this release, it is still necessary to include device type information for forward compatibility purposes. In future releases, the reader may page devices separately based on their device type due to different device capabilities.

Proposal 1:  A paging message includes service related information, e.g. service type, command type and/or device type information.

2.1.2  D2R resource information

In RAN2 #126 meeting, it was agreed that AIoT paging message indicates information from which the device can determine resources to be used for response. In TR 38.769 [2] as below, how to provide D2R resource for Msg1 still needs further study by RAN1. Design of this part could be postponed till RAN1 agree to use higher-layer signalling to carry D2R resource.

#### 6.1.2.8 Scheduling of D2R For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via the corresponding PRDCH: - Time domain resources - Frequency domain resources - MCS-like information - Chip duration - ID associated with device(s) - Repetitions - Midamble (if supported) related information For each information, it is for further study whether higher-layer signalling and/or L1 R2D control signalling is used - see Clause 6.1.1.10.

Observation 1:  The design of D2R resource information included in a paging message could wait for RAN1’s progress.

2.1.3  ID

For this release, there may be one ID in a paging message. According to the previous agreement, the one ID could be ID for a device or ID for a group.

Proposal 2:  Confirm that the ID included in a paging message could be for a device or a group of devices.

For this release, it is also possible that there is no ID in a paging message. No ID implies that the paging message is a broadcast message for all devices. And, there are two alternatives to implement “no ID”:

Option 1:	The ID field in the paging message is optional. If the paging message is to be broadcasted to all devices, the ID field is absent. An additional field to indicate whether the ID field is present or absent may be required.

Option 2:	The ID field in the paging message is mandatory. If the paging message is to be broadcasted to all devices, the ID field is set to a specific value, e.g. 0.

To reduce device decoding complexity, a fixed format for the paging message is slightly preferred, i.e., option 2.

Proposal 3:  The ID in a paging message set to a specific value implies “a paging message contains no identifier.”

2.1.4  Multiple IDs for forward compatibility

To achieve such purpose, one way is to include a field in a paging message to indicate a number of IDs included in the paging message. Each ID may have its corresponding service information or all IDs share the same service information. And the field is always set to 1 in this release.

Proposal 4:  A paging message includes a field to indicate the number of IDs included in the paging message.

2.2  Avoiding duplicated response to paging

Consider the occurrence of transmission failure, the reader could send subsequent paging for the same service. For a device that has responded to the paging message successfully, it should not trigger the random access procedure again in response to receiving the subsequent paging for the same service. The service information included in the paging message should be stored and used to avoid redundant paging response.

In addition, a timer could be introduced to avoid the device prohibiting to respond the service forever. The device could start the timer when responding the paging message successfully. Besides, the mechanism of timer could work with a session ID. For example, when the timer is running, the device may consider that subsequent paging for the same service is for re-sending purpose and the device should not respond. After the timer expiry, the device would respond to a paging for the same service (e.g. with the same session ID). Introducing the timer is beneficial for the case that network pages the devices mostly for the same service, e.g., inventory.

Proposal 5:  In addition to service information in a paging message, a timer is introduced to avoid redundant response to subsequent paging for the same service.

- Conclusion

Proposal 1:  A paging message includes service related information, e.g. service type, command type and/or device type information.

Observation 1:  The design of D2R resource information included in a paging message could wait for RAN1’s progress.

Proposal 2:  Confirm that the ID included in a paging message could be for a device or a group of devices.

Proposal 3:  The ID in a paging message set to a specific value implies “a paging message contains no identifier.”

Proposal 4:  A paging message includes a field to indicate the number of IDs included in the paging message.

Proposal 5:  In addition to service information in a paging message, a timer is introduced to avoid redundant response to subsequent paging for the same service.

- Reference

[1] RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”

[2] TR 38.769, “Study on solutions for ambient IoT (Internet of Things)”