3GPP TSG RAN WG2 #129				R2-2500416

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.5.4

Source:		ASUSTeK

Title:				Discussion on PRACH adaptation

Document for: Discussion and Decision

- Introduction

In RAN1#119 meeting [1], the following agreements on PRACH adaptation were made:

Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives: - Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: Maximum number of subsets of the additional PRACH resources= [2 or 3 or 4 or 16] - FFS: whether the subset of the additional PRACH resources is in - Alt 2-1: RO level per SSB - Alt 2-2: SSB-to-RO mapping cycle level - Alt 2-3: PRACH association period level - Alt 2-4: PRACH association pattern period level - Alt 2-5: SFN level - Alt 2-6: Network configured time period Agreement At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH.

In this document, we would like to discuss PRACH configuration from RAN2 perspective.

- Discussion

2.1  Feature-specific RA partition

In NR, RA partition is introduced for different features (e.g. (e)RedCap, SDT, NSAG) to ensure access. An issue needs to be clarified is that whether the PRACH resource adaptation is supported for RA partition or not. In our understanding, there is no necessity to perform resource slicing for specific features in for cells in NES mode. Therefore, it is suggested not to support feature-specific RA partition for simplicity from RAN2 perspective.

Proposal 1: RAN2 assumes that PRACH adaptation for feature-specific RA partition is not supported.

2.2  RA type

- Simple to support 2-step if 4-step is supported

Another issue on additional PRACH adaptation is that whether PRACH adaptation is supported for 2-step RA, while 4-step RA is considered as the baseline for PRACH time domain adaptation. Under great channel condition, 2-step RA reduces latency and provides flexibility in network scheduling. In our understanding, these advantage of 2-step RA still hold in additional PRACH resources. Also, it would not be too difficult to support PRACH adaptation for 2-step RA from RAN2 point of view.

Proposal 2: RAN2 confirms that PRACH adaptation is supported in both 4-step RA and 2-step RA from RAN2 perspective.

- Conclusion

Proposal 1: RAN2 assumes that PRACH adaptation for feature-specific RA partition is not supported.

Proposal 2: RAN2 confirms that PRACH adaptation is supported in both 4-step RA and 2-step RA from RAN2 perspective.

Reference

[1] Draft Report of 3GPP TSG RAN WG1 #119 v0.3.0 (Orlando, USA, November 18th – 22nd, 2024)

Annex