3GPP TSG-RAN WG2 Meeting #129	R2-2500418

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.9.2

Source:			ASUSTeK

Title:	Discussion on time information for S&F

Document for: 	Discussion and Decision

- Introduction

In RAN2 #128 meeting, following agreements are made for Store & Forward operation. In this contribution, we would like to further discuss the time information for S&F mode.

Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

3.	“S&F operation” indication is provided in SIB1

4.	RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

5.	RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

6.	RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

7.	RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

- Discussion

The Store and Forward (S&F) operation in IoT NTN is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity for delay-tolerant communication service. In previous meetings, it was agreed that the network could broadcast a “S&F operation indication” via SIB1. The indication would indicate that the NTN cell is operating in a S&F mode. If the S&F operation indication is not provided in an NTN cell, the UE considers that the NTN cell is operating in a real-time mode (or called default/normal/not-S&F mode). For example, when feeder link is unavailable for a satellite, the IoT NTN network would operate in S&F mode as shown in Figure 1. And when feeder link becomes available, the IoT NTN network would switch to real-time mode as shown in Figure 2.

Figure 1: An example for S&F mode in TR 22.865 [1]

Figure 2: An example for real-time mode in TR 22.865 [1]

In last meeting, it was agreed that a time information could be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation mode” to “real-time mode”. For the opposite transition, it would also be beneficial to provide the time information about when to transit from “real-time mode” to “S&F operation mode” to the UE. Since the UE behaviour when the UE is in S&F operation mode should be different from UE behaviour when the UE is in real-time mode, the UE needs to know whether the current serving network is operating in which mode. With the time information on transit from/in S&F operation mode, the UE could have the knowledge about when the S&F operation would be started and stopped on a cell.

For example, without the time information about when to switch to S&F mode, when there is UL data arrival that cannot be operated in S&F mode at the very end of real-time mode, the UE would initiate a scheduling request upon UL data arrival, which may cause useless signaling overhead and power consumption. It is likely that the UE receives a UL grant after switching to S&F mode, and the UL grant cannot be used due to the UL data cannot be operated in S&F mode.

Proposal 1: In addition to time information about when to switch to real-time mode, the network could also provide the UE time information about when to switch to S&F mode.

Based on the time information, the UE could determine when to switch between real-time mode and S&F mode. As agreed in last meeting, the UE could delay some NAS procedures in the S&F mode. Moreover, the UE could determine whether to perform non-delay-tolerant transmission, which is not suitable in S&F mode, based on current satellite operation mode. The UE could also use the time information to determine whether to establish a RRC connection. For example, the UE with non-delay-tolerant data could prevent accessing or camping on a cell which is going to operate in S&F mode. In other hand, the UE with non-delay-tolerant data could access a cell which is going to operate in real-time mode, e.g., after the time that the cell switches to real-time mode.

Proposal 2: The UE could determine whether to request uplink resources and whether to perform RRC connection (re-)establishment/resume procedure based on the time information.

- Conclusion

We have the following proposals for S&F time indication:

Proposal 1: In addition to time information about when to switch to real-time mode, the network could also provide the UE time information about when to switch to S&F mode.

Proposal 2: The UE could determine whether to request uplink resources and whether to perform RRC connection (re-)establishment/resume procedure based on the time information.

- Reference

- 3GPP TR 22.865 v19.2.0, “Study on satellite access Phase 3.”