3GPP TSG-RAN2 Meeting #129	R2-2500420

# Athens, Greece, 17-21 February, 2025

Agenda Item:	8.13.2

Source:	ASUSTeK

Title:	One remaining issue on multi-hop U2N Relay Discovery message forwarding

Document for:	Discussion and Decision

1	Introduction

RAN2#127 made the following agreements on multi-hop U2N Relay Discovery:

A candidate last relay UE for discovery or (re)selection can be in any RRC state, but not outside network coverage.  FFS if acting as a relay requires it to transition to RRC_CONNECTED.

Some or all of the intermediate relay UEs on a multi-hop indirect path can be inside or outside the network coverage.

RAN2#128 made the following agreements on multi-hop U2N Relay Discovery:

A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE.

The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.

This document further discusses the following issue on Relay Discovery message forwarding by a UE being authorized to act as an Intermediate Relay UE:

    - Whether a UE which is authorized to act as an Intermediate Relay UE but cannot connect to the network directly or indirectly needs to forward Relay Discovery Solicitation messages received from other UEs.

2	Discussion

2.1	Whether a UE which is authorized to act as an Intermediate Relay UE but cannot connect to the network directly or indirectly needs to forward Relay Discovery Solicitation messages received from other UEs

RAN2#127 agreed that some of the intermediate relay UEs on a multi-hop indirect path can be outside the network coverage. And, RAN2#128 further agreed that a lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE.

In our understanding, after a UE is powered on, the UE would try continuously or periodically to connect to the network so as to initiate a registration procedure and then listen to paging from the network. If the UE initially cannot connect to the network directly (i.e. outside the network coverage), it would try to connect to the network indirectly via the discovered U2N Relay UE(s) if the UE is capable of U2N Relay operation.

Observation 1: After a UE is powered on, the UE would try continuously or periodically to connect to the network directly or indirectly (if the UE is capable of U2N Relay operation).

If the UE cannot connect to the network directly or indirectly like UE6 and UE7 in Figure 1 and the UE may stay out-of-coverage (OOC) for some time after power-on. In this situation, the UE is not capable of acting as an Intermediate U2N Relay UE because this UE cannot provide indirect connectivity to the network for any Remote UE in proximity. In other words, this UE is not eligible for acting as an Intermediate Relay UE. And, it just wastes the device battery power unnecessarily for this UE to forward the received Relay Discovery Solicitation messages because no other relay UE in proximity of this UE could respond to this solicitation. Thus, in our opinion, there is no need for the UE (e.g. UE6/UE7 in Figure 1) to forward the Relay Discovery Solicitation message received from other remote UEs.

Cell

Lower bound

Higher bound

UE1

(Last Relay)

UE3

(Intermediate

Relay)

Cell coverage

UE5

(Remote UE)

UE6

UE7

Figure 1. Potential relay paths with multiple hops

Observation 2: A UE which is authorized to act as an Intermediate Relay UE but cannot connect to the network directly or indirectly is not eligible for acting as an Intermediate Relay UE because this UE cannot provide indirect connectivity to the network for any Remote UE in proximity. And, it just wastes the device battery power unnecessarily for this UE to forward the Relay Discovery Solicitation messages received from other remote UEs.

Proposal 1:  A UE which is authorized to act as an Intermediate U2N Relay UE but cannot connect to the network directly or indirectly shall not forward Relay Discovery Solicitation messages received from other remote UEs.

3 	Conclusion

In this contribution, we raise the following observations and proposals:

Observation 1:  After a UE is powered on, the UE would try continuously or periodically to connect to the network directly or indirectly (if the UE is capable of U2N Relay operation).

Observation 2: A UE which is authorized to act as an Intermediate Relay UE but cannot connect to the network directly or indirectly is not eligible for acting as an Intermediate Relay UE because this UE cannot provide indirect connectivity to the network for any Remote UE in proximity. And, it just wastes the device battery power unnecessarily for this UE to forward the Relay Discovery Solicitation messages received from other remote UEs.

Proposal 1:  A UE which is authorized to act as an Intermediate U2N Relay UE but cannot connect to the network directly or indirectly shall not forward Relay Discovery Solicitation messages received from other remote UEs.