3GPP TSG-RAN WG2 Meeting #129	R2-2500423

Athens, Greece, Feb. 17th – 21st, 2025

Source: 	Xiaomi

Title:  	Discussion on inter-CU LTM

Agenda Item:	8.6.2

Document for:	Discussion and Decision

# Introduction

A Revised WID on NR mobility enhancements Phase 4 [1] has been approved. In the WID, one of the objectives is to specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM), as shown below.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

In this contribution, we provide our understandings on inter-CU LTM.

# Discussion

  - UE-based TA measurement for Inter-CU LTM

RAN2 has discussed the early sync phase in previous meetings, and early TA acquisition has been supported for inter-CU LTM but whether to support UE based TA measurement is FFS. From RAN2 perspective, supporting UE based TA measurement is beneficial for inter-CU LTM, which can reduce the cell switch interruption when the UE doesn’t perform early TA acquisition. To support UE based TA measurement may need to meet some synchronization requirement. Although RAN4 already defined UE requirement for UE based TA measurement for intra-CU LTM, whether the same requirements are applied for Rel-19 inter-CU LTM needs to be confirmed by RAN4. Hence, from RAN2 perspective, UE-based TA measurement for inter-CU LTM could be considered, and it can be finally decided by RAN4.

Proposal 1: From RAN2 perspective, supporting UE-based TA measurement is beneficial for inter-CU LTM.

  - MCG LTM in DC

RAN2 confirmed that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. And RAN2 has supported intra-CU MCG LTM with SCG configuration. To align with inter-CU LTM, intra-CU MCG LTM with SCG addition is supported.

Proposal 2: RAN2 confirms that intra-CU MCG LTM with SCG addition is supported.

  - SCG LTM

Issue 1: how to configure the coexistence of intra-CU and inter-CU SCG LTM.

From RAN2 perspective, the following coexistence cases in NR-DC can be supported:

- Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM

- Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM

The coexistence case of intra-CU SCG LTM and inter-CU SCG LTM has been supported and RAN2 to support intra-CU SCG LTM in MN RRC message (i.e., MN RRCReconfiguration message), in addition to SN RRC message. Based on current progress, one open issue is whether SCG LTM in MN RRC message and SCG LTM in SN RRC message can be configured to UE simultaneously.

If SCG LTM in MN RRC message and SCG LTM in SN RRC message are configured to UE at the same time, there might be two different LTM candidate ID pools, which can cause ambiguity regarding LTM configuration ID. Because SCG LTM can only be triggered by the Cell Switch Command MAC CE from the serving SN, the UE cannot determine whether the LTM configuration ID indicated in the SN Cell Switch Command MAC CE is from the MN RRC message ID pool or the SN RRC message ID pool. Therefore, the common LTM candidate ID pool is needed for all SCG LTM configurations. In Rel-18 SCPAC, for one UE, only either MN format or SN format (only intra-SN case is possible) is used. The similar restriction, that the mixture of intra-CU and inter-CU SCG LTM is configured to the UE only in MN format (i.e., MN RRCReconfiguration message), can address the common LTM candidate ID pool issue for all SCG LTM configuration.

Proposal 3: The mixture of intra-CU and inter-CU SCG LTM is configured to the UE only in MN RRC message.

Issue 2: whether to reuse Rel-18 ltm-Config for inter-CU SCG LTM or introduce a new ltm-Config.

In Rel-18 LTM, ASN.1 field ltm-Config-r18 can be used to configure both MCG LTM and SCG LTM. To follow Rel-18 principle is a better method. Reusing Rel-18 ltm-Config can reduce the complexity, and the UE doesn’t need to maintain two independent LTM configuration from MN. In the L3 conditional reconfiguration, both CHO and inter-SN CPC are configured by one conditionalReconfiguration associated with MCG. Similar to the coexistence of CHO and inter-SN CPC, a simple method is to use one ltm-Config associated with MCG to configure both intra-MN MCG LTM and inter-SN SCG LTM, and the unified LTM candidate ID pool is used for both.

Proposal 4: Rel-18 ltm-Config can be reused to configure inter-CU SCG LTM.

Because the coexistence of intra-CU MCG LTM and inter-CU SCG LTM is supported, the main concern about reusing Rel-18 ltm-config is that UE needs to differentiate whether a certain candidate configuration in ltm-CandidateToAddModList-r18 received from MCG is for intra-CU MCG LTM or for inter-CU SCG LTM.

For the coexistence of intra-MN MCG LTM and inter-SN SCG LTM, the unified LTM candidate ID pool is used for both intra-CU MCG LTM or for inter-CU SCG LTM when reusing Rel-18 ltm-config. Upon receiving an LTM Cell Switch Command MAC CE including the Target Configuration ID from PCell or PSCell, the UE can know the LTM candidate configuration associated with the Target Configuration ID and apply the candidate configuration for MCG or SCG LTM. Hence, for MAC CE triggered LTM, before LTM execution, UE doesn’t need to know whether a certain candidate is configured for MCG LTM or SCG LTM. Upon LTM execution, UE can know it based on the source of LTM Cell Switch Command.

Otherwise, for LTM based failure recovery, UE needs to know whether the selected cell is one MCG LTM candidate cell. In L3 conditional reconfiguration, the conditionalReconfiguration can be used to configure both CHO and CPAC and the coexistence of CHO and inter-SN CPC is supported in Rel-17. For CHO/CPAC evaluation and CHO based recovery, UE also needs to know whether a certain L3 candidate configuration received from MCG conditionalReconfiguration is for CHO or for inter-SN CPC. For L3 mobility, based on the following highlighted procedures, UE knows whether the candidate is for CHO or CPAC:

In TS 38.331 V18.4.0 ##### 5.3.5.13.4 Conditional reconfiguration evaluation The UE shall: 1> for each condReconfigId within the VarConditionalReconfig: 2> if the RRCReconfiguration within condRRCReconfig includes the masterCellGroup including the reconfigurationWithSync: 3> if the associated condExecutionCondPSCell is configured: 4> consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the masterCellGroup in the received condRRCReconfig to be applicable cell; and 4> consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the secondaryCellGroup within the nr-SCG within the received condRRCReconfig to be applicable cell; 3> else: 4> consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the masterCellGroup in the received condRRCReconfig to be applicable cell; 2> else if the RRCReconfiguration within condRRCReconfig includes the secondaryCellGroup including the reconfigurationWithSync: 3> if the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the secondaryCellGroup within the received condRRCReconfig is not the PSCell: 4> if subsequentCondReconfig is not included for the condReconfigId; or 4> if subsequentCondReconfig is not included for the PSCell; or 4> if subsequentCondReconfig is included for the condReconfigId and there is a subsequentCondReconfig for the PSCell with a matching condReconfigId value in condExecutionCondToAddModList: 5> consider the cell to be applicable cell; …… #### 5.3.7.3 Actions following cell selection while T311 is running Upon selecting a suitable NR cell, the UE shall: …… 1> if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG, except for an LTM cell switch procedure following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or mobility from NR failure, and 1> if attemptCondReconfig is configured; and 1> if the selected cell is not configured with CondEventT1, or the selected cell is configured with CondEventT1 and leaving condition has not been fulfilled; and 1> if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig and the condExecutionCondPSCell is not configured for the corresponding condReconfigId in the MCG VarConditionalReconfig: 2> if the UE supports RLF-Report for conditional handover, set the choCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell; 2> apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3; ……

RAN2 has agreed that upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed. Therefore, the SCG LTM candidate configuration cannot include the masterCellGroup including the reconfigurationWithSync, but the MCG LTM candidate configuration always includes the masterCellGroup including the reconfigurationWithSync.

According to the above analysis, if reusing Rel-18 ltm-Config, UE can distinguish between MCG LTM candidate and SCG LTM candidate based on whether the RRCReconfiguration within ltm-CandidateConfig includes the masterCellGroup including the reconfigurationWithSync.

Proposal 5: If reusing Rel-18 ltm-Config to configure inter-CU SCG LTM, the UE can distinguish between MCG LTM candidate and SCG LTM candidate based on whether the RRCReconfiguration within ltm-CandidateConfig includes the masterCellGroup including the reconfigurationWithSync (Similar to CHO/CPC):

- If the RRCReconfiguration within ltm-CandidateConfig includes the masterCellGroup including the reconfigurationWithSync, the ltm-CandidateConfig is for MCG LTM;

- else (if the RRCReconfiguration within ltm-CandidateConfig includes the secondaryCellGroup including the reconfigurationWithSync), the ltm-CandidateConfig is for SCG LTM.

Issue 3: How to configure LTM CSI resource configuration for SCG LTM.

For SN initiated inter-CU SCG LTM, the source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells. Hence, whether to reuse Rel-18 ltm-Config or not, how to configure the LTM CSI resource configuration for SCG LTM need to be discussed. The inter-CU SCG LTM is configured in MN RRC message, which doesn’t mean the LTM CSI resource configuration for SCG LTM must be configured in MN RRC message.

Because inter-CU SCG LTM candidates are configured by MN and the SCG LTM Cell Switch Command including candidate configuration ID is transmitted by SN. MN and source SN always need to negotiate the candidate configuration ID, and the source SN knows which LTM candidate configuration ID is configured for SCG LTM and associated LTM candidate PSCell. Therefore, the LTM CSI resource configuration for SCG LTM can also be configured by SN in SN RRC message.

Hence, there are two options can be considered to configure LTM CSI resource configuration for SN initiated inter-CU SCG LTM:

- Option 1: the LTM CSI resource configuration for inter-CU SCG LTM is provided to UE in MN RRC message

- Option 2: the LTM CSI resource configuration for inter-CU SCG LTM is provided to UE in SN RRC message

For option 1, the source SN shall send the LTM CSI resource configuration to MN and then MN provides it to UE.

For option 2, the LTM CSI resource configuration provided in SN RRC message shall indicate the LTM candidate ID configured in MN RRC message.

Proposal 6: The following two options can be considered to configure LTM CSI resource configuration for SN initiated inter-CU SCG LTM:

- Option 1: the LTM CSI resource configuration for inter-CU SCG LTM is provided to UE in MN RRC message.

- Option 2: the LTM CSI resource configuration for inter-CU SCG LTM is provided to UE in SN RRC message.

  - Reference configuration for LTM in DC case

For the reference configuration of inter-CU SCG LTM, RAN2 has reached the following agreements:

4. Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline: - The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message. - The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. - The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part. - The MN can request an SCG reference configuration from any of the involved SNs. It's up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

In Rel-19, the following LTM configuration can be modelled as an MN RRCReconfiguration in DC case

- intra-CU MCG LTM with SCG configuration

- inter-CU MCG LTM with SCG configuration

- intra-CU SCG LTM

- inter-CU SCG LTM

Based on above agreements, the reference configuration of inter-CU SCG LTM is modelled as an MN RRCReconfiguration message and can include the MCG part based on the NW implementation. The intra-CU SCG LTM modelled as an MN RRCReconfiguration shall also follow the same principle.

In Rel-19, intra-CU/inter-CU MCG LTM with SCG configuration has been supported, so the reference configuration of intra-CU/inter-CU MCG LTM in DC case is modelled as an MN RRCReconfiguration message which can also include both MCG and SCG part.

For the reference configuration including both MCG part and SCG part, MN generates the MCG part of reference configuration and SN generates the SCG part of reference configuration for both MCG LTM and SCG LTM.

Proposal 7: For the following LTM configuration modelled as MN RRCReconfiguration message in DC case, the reference configuration can include both MCG part and SCG part. And MN generates the MCG part of reference configuration and SN generates the SCG part of reference configuration for both MCG LTM and SCG LTM.

- intra-CU MCG LTM with SCG configuration

- inter-CU MCG LTM with SCG configuration

- intra-CU SCG LTM

- inter-CU SCG LTM

If P7 is agreed, the legacy field ltm-ReferenceConfiguration can be used to configure the reference configuration for both SCG LTM and MCG LTM. If reusing Rel-18 ltm-Config for SCG LTM is supported, one reference configuration is sufficient to support the coexistence of the intra-CU MCG LTM and inter-CU SCG LTM.

# Conclusions

According to the analysis given above, we have the following observations and proposals:

2.1	UE-based TA measurement for Inter-CU LTM

Proposal 1: From RAN2 perspective, supporting UE-based TA measurement is beneficial for inter-CU LTM.

2.2 MCG LTM in DC

Proposal 2: RAN2 confirms that intra-CU MCG LTM with SCG addition is supported.

2.3	SCG LTM

Proposal 3: The mixture of intra-CU and inter-CU SCG LTM is configured to the UE only in MN RRC message.

Proposal 4: Rel-18 ltm-Config can be reused to configure inter-CU SCG LTM.

Proposal 5: If reusing Rel-18 ltm-Config to configure inter-CU SCG LTM, the UE can distinguish between MCG LTM candidate and SCG LTM candidate based on whether the RRCReconfiguration within ltm-CandidateConfig includes the masterCellGroup including the reconfigurationWithSync (Similar to CHO/CPC):

- If the RRCReconfiguration within ltm-CandidateConfig includes the masterCellGroup including the reconfigurationWithSync, the ltm-CandidateConfig is for MCG LTM;

- else (if the RRCReconfiguration within ltm-CandidateConfig includes the secondaryCellGroup including the reconfigurationWithSync), the ltm-CandidateConfig is for SCG LTM.

Proposal 6: The following two options can be considered to configure LTM CSI resource configuration for SN initiated inter-CU SCG LTM:

- Option 1: the LTM CSI resource configuration for inter-CU SCG LTM is provided to UE in MN RRC message.

- Option 2: the LTM CSI resource configuration for inter-CU SCG LTM is provided to UE in SN RRC message.

2.4	Reference configuration for LTM in DC case

Proposal 7: For the following LTM configuration modelled as MN RRCReconfiguration message in DC case, the reference configuration can include both MCG part and SCG part. And MN generates the MCG part of reference configuration and SN generates the SCG part of reference configuration for both MCG LTM and SCG LTM.

- intra-CU MCG LTM with SCG configuration

- inter-CU MCG LTM with SCG configuration

- intra-CU SCG LTM

- inter-CU SCG LTM

# Reference

[1] RP-242356, “Revised WID: NR mobility enhancements Phase 4”, Apple Inc, China Telecom

[2] Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA, 18 - 22 November, 2024