3GPP TSG-RAN WG2 Meeting #129	R2-2500424

Athens, Greece, Feb. 17th – 21st, 2025

Source: 	Xiaomi

Title:  	Discussion on conditional LTM

Agenda Item:	8.6.4

Document for:	Discussion and Decision

# Introduction

A Revised WID on NR mobility enhancements Phase 4 [1] has been approved. In the WID, one of the objectives is to specify support for conditional Intra-CU LTM, as shown below.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In previous meetings, RAN2 has reached the following agreements for conditional LTM [2-3].

RAN2#127bis Agreements on C-LTM 1. Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. 2. Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. 3. Source cell and each candidate cell provides its own execution condition for conditional LTM. 4. It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. 5. RACH-less Conditional intra-CU LTM is supported. 6. RACH based conditional intra-CU LTM is supported. 7. UE based TA measurement mechanism is supported for conditional intra-CU LTM. 8. PDCCH ordered early TA acquisition is supported for conditional LTM. 9. Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. 10. For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach. 11. The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM. RAN2#128 Agreements on C-LTM: 1. The triggering condition of conditional LTM can be based on L3 measurement. 2. CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution. 3. The L1 execution condition of a candidate cell is associated to only one triggering event. 4. For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell. 5. To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition: - The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell. - The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell. 6. The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH. 7. For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration. 8. The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. 9. The network can inform the candidate cell's TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. 10. Candidate cell TA is maintained by a new timer. 11. For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In this contribution, we provide our understandings on conditional intra-CU LTM.

# Discussion

  - The coexistence of LTM and CLTM

For network triggered LTM, the IE LTM-Candidate concerns a LTM candidate configuration to add or modify. Early UL and DL sync have been supported for CLTM and all parameters in LTM-Candidate-r18 can also be reused to configure CLTM. Naturally, Rel-18 IE (e.g., ltm-Config-r18 and LTM-Candidate-r18) can be used to configure CLTM candidate configuration. Compared with the candidate configuration of Rel-18 LTM, the difference is that each CLTM candidate needs to be associated with the CLTM execution condition, as specified in the RRC running CR [4]:

#### – LTM-Config The IE LTM-Config is used to provide LTM configurations. LTM-Config information element LTM-Config-r18 ::= SEQUENCE { ltm-ReferenceConfiguration-r18 SetupRelease {ReferenceConfiguration-r18} OPTIONAL, -- Need M ltm-CandidateToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18 OPTIONAL, -- Need N ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 OPTIONAL, -- Need N ltm-ServingCellNoResetID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need N ltm-CSI-ResourceConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18 OPTIONAL, -- Need N ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18 OPTIONAL, -- Need N attemptLTM-Switch-r18 ENUMERATED {true} OPTIONAL, -- Cond LTM-MCG ltm-ServingCellUE-MeasuredTA-ID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need N ..., [[ ltm-ServingCellNoSecurityChangeID-r19 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need N ltm-SK-Counters-r19 SetupRelease {SK-CounterList-r19} OPTIONAL, -- Need M cltm-ServingCellExecutionConditions-r19 SetupRelease {CLTM-ExecutionConditions-r19} OPTIONAL -- Need M ]] } #### – LTM-Candidate The IE LTM-Candidate concerns a LTM candidate configuration to add or modify. LTM-Candidate information element LTM-Candidate-r18 ::= SEQUENCE { ltm-CandidateId-r18 LTM-CandidateId-r18, ltm-CandidatePCI-r18 PhysCellId OPTIONAL, -- Need M ltm-SSB-Config-r18 LTM-SSB-Config-r18 OPTIONAL, -- Need M ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) OPTIONAL, -- Need M ltm-ConfigComplete-r18 ENUMERATED {true} OPTIONAL, -- Need R ltm-EarlyUL-SyncConfig-r18 OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18) OPTIONAL, -- Need R ltm-EarlyUL-SyncConfigSUL-r18 OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18) OPTIONAL, -- Need R ltm-TCI-Info-r18 LTM-TCI-Info-r18 OPTIONAL, -- Need M ltm-NoResetID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M ltm-UE-MeasuredTA-ID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M ..., [[ ltm-NoSecurityChangeID-r19 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M cltm-ExecutionConditions-r19 SetupRelease {CLTM-ExecutionConditions-r19} OPTIONAL -- Need M ]] }

Observation 1: In RRC running CR, Rel-18 IE (e.g., LTM-Config-r18 and LTM-Candidate-r18) is used configure conditional LTM. The LTM candidate configuration associated with the execution condition is used to configure the CLTM candidate.

For L3 mobility, the coexistence of L3 HO and CHO is supported. During CHO condition evaluation, the serving gNB can send the L3 HO command to UE. Similarly, for the flexibility and controllability of mobility, the coexistence of LTM and CLTM can be supported.

Different from L3 mobility, for both MAC CE tiggered LTM and conditional LTM, it is needed to pre-configure the LTM candidate configuration. As mentioned in observation 1, the LTM-Candidate can be reused to configure CLTM. To avoid unnecessary signalling overhead, one LTM candidate configuration can be configured for both MAC CE tiggered LTM and CLTM. That is, one LTM candidate configured with the CLTM execution condition, the network can also trigger cell switch to the LTM candidate via LTM cell switch Command MAC CE.

Proposal 1: To support the coexistence of LTM and CLTM.

Proposal 2: Even if the CLTM condition is configured for the LTM candidate, the network can also trigger LTM cell switch to the LTM candidate via LTM cell switch Command MAC CE.

  - Early sync

Issue: whether the candidate cell TA is per candidate cell or per TAG.

RAN2 has agreed the candidate cell's TA information is provided to UE via new MAC CE, and candidate cell TA is maintained by a new timer. However, whether the candidate cell TA is per cell or per TAG is an open issue.

In legacy procedure, the TA value of serving cell(s) is maintained per TAG and the maximum number of serving TAG is 4. Whereas, the maximum number of LTM candidates is 8. If the candidate TA is per candidate cell, the UE needs to maintain a large number of TATs (e.g., up to 12 TATs). Some LTM candidate cells may have the same TA value and network can know which candidate cells share the same TA. If TA is per candidate cell, the UE maintains the candidate TA for each candidate cell with the same TA value, which causes unnecessary UE complexity. Hence, we prefer to follow the legacy principle, the candidate TA is per TAG, and one or more candidates can associate with the same TAG.

Proposal 3: The candidate TA is maintained per TAG. One or more CLTM candidate cells can associate with the same TAG.

If the candidate TA is maintained per TAG, whether to use the unified or independent TAGs for serving cells and LTM candidate cells can be discussed. The candidate cell may share the same TA value of source serving cell and the UE can apply the same TA value as the source for RACH-less LTM in Rel-18. So, to use the unified TAGs for serving cells and LTM candidate cells can further reduce the number of TAGs that the UE needs to maintain.

Proposal 4: The legacy TAG configuration can be used to configure TAGs for LTM candidate cells. LTM candidate cells and serving cells can associate with the same TAG.

  - CLTM evaluation

Issue 1: whether we re-use events specified for the event-triggered L1 measurements or we specify events specifically for CLTM.

In RAN2#127bis meeting, RAN2 has agreed event LTM3-like and LTM5-like are used as the conditional LTM execution condition. How to configure LTM3/5-like events associated with C-LTM conditions is an open issue.

In L3 RRM measurement, L3 CHO events (condEventA3/A5) cannot be used for the L3 measurement report, and L3 MR events and L3 CHO events are configured by different parameters (like eventA3/A5 for MR and condEventA3/A5 for CHO evaluation). The restriction is also applicable for the LTM event if it is associated with the CLTM candidate configuration. To distinguish L1 CLTM events from L1 MR events, a separate parameter (e.g., condEventLTM3/LTM5) is defined to configure L1 CLTM events.

Proposal 5: To define a separate parameter (e.g., condEventLTM3/LTM5) for the event associated with CLTM condition to distinguish from the L1 MR event. L1 events associated with the conditional LTM configuration as CLTM condition cannot be used to trigger the L1 measurement report.

Issue 2: how many beams shall be used to evaluate the L1 condition for CLTM.

In previous meeting, RAN2 has discussed how many beams shall be used to evaluate the L1 condition for CLTM. There are two options:

- Option 1: UE triggers the LTM cell switch procedure when at least one beam of the candidate cell fulfils the associated L1 execution condition.

- Option 2: UE triggers the LTM cell switch procedure when at least N beams of the candidate cell satisfy the CLTM condition respectively. The value range of N is {1, 2, 3, 4}.

For L1 event triggered measurement reporting, RAN2 has agreed any beam in candidate RS configuration can be used for LTM event evaluation. Only one beam fulfilling the L1 event can also trigger the measurement report. The minimum number of beams is not introduced for L1 measurement report. The L1 event evaluation for CLTM shall align with the evaluation of L1 MR events. And using one beam to trigger CLTM can achieve a fast handover with the good beam.

The motivation to use more than one beam to trigger CLTM is to avoid the Ping-Pong of CLTM (due to a limited number of good beams). L3 conditions based on cell-level measurement results have been supported for the conditional LTM execution. Therefore, it is not needed to introduce the minimum number of beams for CLTM.

Proposal 6: For CLTM, UE triggers the LTM cell switch procedure when at least one beam of the candidate cell fulfils the associated L1 execution condition.

Issue 3: whether L1 and L3 execution conditions can be configured simultaneously or not.

Independent configurations for the L1 condition and the L3 condition are already supported. Based on the L1 condition, the UE can find the best beam for LTM cell switch. To support the L3 condition for CLTM can increase robustness. For better mobility performance, L1 and L3 execution conditions can be configured simultaneously for one CLTM candidate. And when both L1 and L3 execution conditions are fulfilled, the CLTM is triggered.

Proposal 7: L1 and L3 execution conditions can be configured simultaneously for one CLTM candidate.

  - CLTM execution

Issue 1: how to determine the target cell and target beam for CLTM.

In the network triggered LTM, the network sends cell switch command MAC CE to trigger cell switch. The cell switch command indicates the target cell (via target configuration ID) and the target beam (via the TCI state). However, for conditional LTM, there is no LTM cell switch command MAC CE to indicate the target cell and target beam for the CLTM execution and the UE triggers conditional LTM cell switch autonomously when the CLTM condition is fulfilled.

To support CLTM, the target LTM configuration ID (the target cell) and the target beam can be determined by UE based on the condition evaluation and the UE selects the candidate ID/the candidate beam which fulfils the execution condition as target for CLTM execution.

In some cases, more than one LTM candidate cell/beam fulfil the LTM cell switch execution, how to select the target cell/beam for CLTM execution can be discussed.

Issue1-1: how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution.

For CLTM, RACH-less is supported. To reduce the handover interruption, the UE shall preferentially select the LTM candidate cell with valid TA values for RACH-less CLTM.

Proposal 8: If more than one LTM candidate cell fulfil the LTM cell switch execution, the UE shall preferentially select the LTM candidate cell with a valid TA value.

Issue 1-2: if multiple beams satisfy the event for CLTM, how to select target beam for RACH-less C-LTM.

To support the RACH-less conditional LTM, the UE shall determine the target beam for cell switch. As mentioned above, any beam that satisfies the L1 condition and any beam of candidate cells which fulfil L3 conditions can be considered as the selected beam for RACH-less CLTM. RAN2 has agreed the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration. For RACH-less CLTM and reduce handover interruption, the UE shall select the activated TCI states associated beams as the target beam.

Proposal 9: When multiple beams satisfy the execution condition of CLTM, the UE shall select the activated TCI states associated beams as the target beam.

Issue 2: how to select the CG resource for RACH-less CLTM.

For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. But how to select the configured grant occasion for conditional LTM is an open issue. In the Rel-18 RACH-less LTM, the UE selects the configured grant occasion associated with the beam indicated in the cell switch command. Similarly, The UE uses the CG resource associated with the UE-selected beam for first UL transmission for CG-based RACH-less conditional LTM.

Proposal 10: The UE uses the CG resource associated with the UE-selected beam for CG-based RACH-less conditional LTM.

Issue 3: whether to support RACH-less CLTM with DG.

Whether to support the DG-based RACH-less is FFS for conditional LTM. Compared with CG-based CLTM, the network doesn’t need to reserve resource during CLTM evaluation for DC-based RACH-less. One concern in supporting the DG-based RACH-less is how the network can schedule a dynamic grant to UE for conditional cell switch execution, because the network cannot know when the UE performs the cell switch and which target cell and target beam are selected by the UE to transmit first UL data.

Nevertheless, the issue can be solved by the network. Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM, and UE selects the activated beam as the target beam (if P9 is agreed). Hence, the network can only send the dynamic scheduling information on the activated beam, which associated with the activated TCI State. DG-based RACH-less conditional LTM can be considered without too much specification effort.

Proposal 11: If no much specification effort is required, DG-based RACH-less conditional LTM can be considered.

Issue 4: what information should be provided to upper layers from MAC for CLTM execution.

For MAC CE triggered LTM, when the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell, the MAC entity shall indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the LTM Cell Switch Command MAC CE and the upper layer (RRC) performs the LTM cell switch execution procedure.

For CLTM with L1 execution condition, the condition evaluation is at MAC level. When L1 execution condition is fulfilled, the MAC layer also needs to indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID associated with the fulfilled L1 condition.

Proposal 12: When the L1 execution condition is satisfied at MAC entity, the MAC entity shall indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID corresponding to the satisfied L1 execution condition.

# Conclusions

According to the analysis given above, we have the following proposals:

2.1 The coexistence of LTM and CLTM

Observation 1: In RRC running CR, Rel-18 IE (e.g., LTM-Config-r18 and LTM-Candidate-r18) is used configure conditional LTM. The LTM candidate configuration associated with the execution condition is used to configure the CLTM candidate.

Proposal 1: To support the coexistence of LTM and CLTM.

Proposal 2: Even if the CLTM condition is configured for the LTM candidate, the network can also trigger LTM cell switch to the LTM candidate via LTM cell switch Command MAC CE.

2.2	Early sync

Proposal 3: The candidate TA is maintained per TAG. One or more CLTM candidate cells can associate with the same TAG.

Proposal 4: The legacy TAG configuration can be used to configure TAGs for LTM candidate cells. LTM candidate cells and serving cells can associate with the same TAG.

2.3	CLTM evaluation

Proposal 5: To define a separate parameter (e.g., condEventLTM3/LTM5) for the event associated with CLTM condition to distinguish from the L1 MR event. L1 events associated with the conditional LTM configuration as CLTM condition cannot be used to trigger the L1 measurement report.

Proposal 6: For CLTM, UE triggers the LTM cell switch procedure when at least one beam of the candidate cell fulfils the associated L1 execution condition.

Proposal 7: L1 and L3 execution conditions can be configured simultaneously for one CLTM candidate.

2.4	CLTM execution

Proposal 8: If more than one LTM candidate cell fulfil the LTM cell switch execution, the UE shall preferentially select the LTM candidate cell with a valid TA value.

Proposal 9: When multiple beams satisfy the execution condition of CLTM, the UE shall select the activated TCI states associated beams as the target beam.

Proposal 10: The UE uses the CG resource associated with the UE-selected beam for CG-based RACH-less conditional LTM.

Proposal 11: If no much specification effort is required, DG-based RACH-less conditional LTM can be considered.

Proposal 12: When the L1 execution condition is satisfied at MAC entity, the MAC entity shall indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID corresponding to the satisfied L1 execution condition.

# Reference

[1] RP-242356, “Revised WID: NR mobility enhancements Phase 4”, Apple Inc, China Telecom

[2] Report of 3GPP TSG RAN WG2 meeting #127bis

[3] Report of 3GPP TSG RAN WG2 meeting #128

[4] Rapporteur RRC for inter-CU and conditional LTM