3GPP TSG-RAN WG2 Meeting #129                                     R2-2500453

Athens, Greece, Feb. 17th – 21st, 2025

Source: 			OPPO

Title: 	Discussion on providing MBS service area in NTN network

Agenda item:		8.8.4

Document for: 	Discussion and Decision

- Introduction

Agreements:

1.	The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle.

2.	The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon.

3.	The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed)

4.	A signaled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s).

5.	RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates.

6.	Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way)

7.	The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

In the last RAN2#128 meeting, agreements on the support of broadcast service in the NTN network have been achieved as follows:

In this paper, we would like to further present our opinions on the topic.

- Discussion

- 2.1 how to link the service area for each MBS session

In the last RAN2#128 meeting, we have agreed that a new SIB is introduced to include a list of intended service areas and related points. FFS if we point to the intended service area via the index in the list or with an ID or another way.

Currently, for the MBS broadcast, the list of ongoing MBS broadcast sessions transmitted via broadcast MRB is provided via the MCCH message. To let the UE further recognize the area restricted for such ongoing MBS broadcast session, in the RAN2#127bis meeting, we agreed that for each MBS service we include one or more intended service area IDs in MCCH. Naturally, such ID should be also included in the new SIB, in order to link each MBS session in the MCCH to intended service area(s) in the new SIB, as indicated as follows:

Figure 1:  missing of the intended service area ID in the new SIB

Proposal 1: in the new SIB, attach intended service area ID to each intended service area, in order to link each MBS session in the MCCH to a defined intended service area.

Note that also in the last RAN2#128 meeting, we also made an agreement that a signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbour cell(s). That means that in the new SIB message, not only the intended service area of the current serving cell, but also those of neighbour cells are needed to be included. In our opinion, there are two possible operations. The first one is to isolate the intended service area of the current cell from the service areas of neighbour cells belonging to each MBS session and attach different IDs for these service areas. Subsequently, only the ID of the service area of the current cell is included in the mbs-SessionInfoList-r17 IE included in the MBSBroadcastConfiguration message, and the IDs of the services areas of neighbour cells belonging to the same MBS session are included in other IEs, e.g., MBS-NeighbourCellList. Another choice is to view the intended service area of one particular MBS session as a whole, and hence capture the shape of the area in the new SIB, and attach the ID of the area to the mbs-SessionInfo IE.

Generally, the first option may result in overwhelmingly large signalling overhead for updating the service area range periodically in case of earth-moving cell, since the location co-ordinates of service areas are always changing. Hence, we suggest RAN2 to agree on the second option, i.e., to attach the ID of the whole intended service area belonging to one particular MBS session to the mbs-SessionInfo IE

Observation 2: for one particular MBS session, isolating the intended service area restricted in current cell range from the ones restricted in other neighbour cells range may result in overwhelmingly large signalling overhead for updating the service area range periodically in case of earth-moving cell.

Proposal 2: RAN2 to agree to capture the intended service area of both of serving cell and neighbour cells as a whole in the new SIB.

- 2.2 is it necessary for MCCH acquisition relaxation?

In the last RAN2#128 meeting, we discussed whether or not the mapping between the intended service area ID and the MBS broadcast service is also needed to be included in the Service Announcement message, and no conclusion was made, and we agreed to further discuss this problem in this RAN2 meeting.

Firstly, note that the mapping information is already agreed to be included in the MCCH in the RAN2#127bis meeting, as follows:

For each MBS service we include one or more intended service area IDs into MCCH.

So the advantage of inclusion of the mapping information in the Service Announcement message is just letting the UE acquire the mapping information when the broadcast session is established, i.e., before the MCCH acquisition. But according to the annex of [1], the main purpose of receiving the Service Announcement is just to let the UE may become interested in an MBS service in the first place, introduce of the mapping information cannot help for this purpose. Also, since in the last RAN2 #128 meeting we have already agreed that the geographic area information for the intended service areas can be semi-static and does not cause frequent updates, from our point of view, the UE becoming interested in any MBS service can read MCCH once and only once for obtaining the mapping relationship between MBS session and intended service ID in a long-time duration, which does not suffer too much from power wasting problem. Also, considering that cross working group coordination is needed to introduce the mapping information of the MBS and intended service ID to the Service Announcement message which implies more standardization effort, we suggest to keep the current status.

Observation 3: according to [1], the main purpose of receiving the Service Announcement is just to let the UE be aware of the existence of the MBS service and become interested in an MBS service in the first place, introduce of the mapping information in it cannot help for this purpose.

Observation 4: since the geographic area information for the intended service area can be semi-static and does not cause frequent updates, the UE becoming interested in any MBS service can read MCCH once and only once for obtaining the mapping relationship between MBS session and intended service ID in a long-time duration, which does not bring much power wasting problem.

Proposal 3: does not pursue inclusion of the mapping relationship between MBS session and the intended service area ID in the Service Announcement message.

- 2.3 MBS interest indication

According to TS 38.331, the purpose of this procedure is to inform the network that the UE in RRC_Connected is receiving or is interested to receive MBS broadcast service(s) and/or to inform the network about the priority of MBS broadcast versus unicast and multicast MRB reception. Even if the UE is not entering any intended service area, the UE could still perform the MBS interest indication procedure towards the network. Subsequently, if the network would like to provide the MBS service to the UE, no matter where the UE is, the network could just send the MBS content towards the UE via unicast. As a result, we think the legacy mechanism of MBS interest indication could be followed by the NTN cell, and no other additional enhancement such as UE providing the MBS interest indication only when the UE enters one intended service area is needed.

Observation 5: after the UE sends the MBS interest indication, if the network would like to provide the MBS service towards the UE no matter where the UE is, the network could just send the MBS content towards the UE via unicast.

Proposal 4: does not introduce any additional restriction on when to initialize the MBS interest indication, on the top of the legacy UE behaviour.

- Conclusion and proposals

In this paper, following observations and proposals have been made by us:

how to link the service area for each MBS session:

Proposal 1: in the new SIB, attach intended service area ID to each intended service area, in order to link each MBS session in the MCCH to a defined intended service area.

Observation 1: defining service area for each cell separately for a particular MBS session and include them all in the MBSBroadcastConfiguration IE allows the UE to identify clearly whether or not it is still in the current cell service area and therefore can make decision if it is necessary to perform reselection to other cell for service continuity.

Observation 2: for one particular MBS session, isolating the intended service area restricted in current cell range from the ones restricted in other neighbour cells range may result in overwhelmingly large signalling overhead for updating the service area range periodically in case of earth-moving cell.

Proposal 2: RAN2 to agree to capture the intended service area of both of serving cell and neighbour cells as a whole in the new SIB.

is it necessary for MCCH acquisition relaxation?:

Observation 3: according to [1], the main purpose of receiving the Service Announcement is just to let the UE be aware of the existence of the MBS service and become interested in an MBS service in the first place, introduce of the mapping information in it cannot help for this purpose.

Observation 4: since the geographic area information for the intended service area can be semi-static and does not cause frequent updates, the UE becoming interested in any MBS service can read MCCH once and only once for obtaining the mapping relationship between MBS session and intended service ID in a long-time duration, which does not bring much power wasting problem.

Proposal 3: does not pursue inclusion of the mapping relationship between MBS session and the intended service area ID in the Service Announcement message.

MBS interest indication:

Observation 5: after the UE sends the MBS interest indication, if the network would like to provide the MBS service towards the UE no matter where the UE is, the network could just send the MBS content towards the UE via unicast.

Proposal 4: does not introduce any additional restriction on when to initialize the MBS interest indication, on the top of the legacy UE behaviour.

Reference:

[1] R2-2410862, Support for broadcast services in NR NTN, Ericsson