3GPP TSG RAN WG2#128                                                                                   R2-2500454

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.2.4

Source:	OPPO

Title:	Discussion on A-IoT data transmission related functionalities

Document for:	Discussion, Agreement

Introduction

In the last RAN2#128 meeting, following agreements regarding A-IOT functionalities have been made:

- RAN2 assumes that there is no other L2 AS layer (i.e., A-IOT MAC layer only). There is no CP/UP protocol stack differentiation on AIOT interface.

- Capture the option that it is up to the reader whether to use the random ID as AS ID or assigns a new AS ID. FFS what message is used.

- Capture in the TR that to down-select we need to consider which message need to consider whether we need the reader to handle the collision.

- Segmentation indication will be discussed in WI phase.

- If segmentation is supported, it is beneficial for the reader to be able to trigger a re-transmission of a segment. FFS how this is done.

- The reader needs to know whether a response is expected and expected D2R message size. FFS how the reader gets this information. Wait for SA2 LS response on expected D2R message size to resolve the FFS. FFS if command type is explicit, if it can be inferred from expected D2R message size if available.

- Ask SA2 if they can provide information that RAN2 agreed on some information that are useful for ther reader ask whether they can be provided (e.g., service type). Ask SA2 if there are requirements from SA2 perspective (e.g., on latency of completion time of procedure)

- We will not capture a new use case in the TR. RAN2 confirms that this use case can be done via reader implementation, as per previous agreement that it is up to the reader whether MSG2 is transmitted. FFS if the device needs to be aware whether msg2 will be transmitted.

- RAN2 did not study this scenario in the study item

In this contribution, we aim to address our view on the aspects e.g., segmentation for D2R, AS ID, message size information, etc.

Discussion

Issue-1: segmentation

In RAN2#127bis meeting, RAN2 has agreed to study segmentation. The related agreements are given below.

- Agreements on segmentations

- RAN2 will study segmentation and RAN2 will need to give some guidance once there is more clarity on solution, on the minimum TB size.

- Study the functionality of segmentation in the MAC.   We focus on D2R and can consider the support for R2D later depending on SA2 and RAN1.   The solution should aim to be applicable to both direction (i.e. unified solution), if we decide to support on both directions.

- Do not support sequence number for A-IoT segmentation functionalities.

- Do not support segment number and the number of segments for A-IoT segmentation functionalities.

- Capture the following option in TR: An indication is used to indicate to reader whether the data is segmented and whether it is last segment.    FFS the details on how this is designed and number of bits (one or two).

- RAN2 assumes that for the device we will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s).

As captured in the TR 38.769, a reader could know the expected D2R message size from either the CN or the device. If the time-frequency resource is not sufficient, the reader has to issue a D2R transmission resource accommodating less payload than the expected D2R message size. In such case, segmentation of the D2R message before transmission of it on the Au interface is necessary.

Noted that in the RAN2#127bis meeting, we have agreed that sequence number for A-IOT segmentation is not supported. So only the stop & wait mechanism could be applied, i.e., the A-IOT device will not be able to transmit subsequent segments before the successful transmission of a particular segment. Naturally, re-transmission of the failed segment is a feasible approach to ensure the success of the transmission of a segment before moving to the transmission of the next segment. In the last RAN2#128 meeting, a FFS regarding how to support re-transmission of a segment in case of transmission failure is left.

In our opinion, the basic operation procedure is follows:

- The reader sends the D2R transmission resource to the A-IOT device in the R2D message

- The A-IOT device identifies if the D2R message could be accommodated in the provided transmission resource

- If the answer is yes, then the A-IOT device will not do segmentation and transmit the whole D2R message to the network. Otherwise, the A-IOT device will do segmentation on the D2R message and only transmits the first segment to the Reader

- If a whole D2R message received, the reader will forward it to the CN node. Otherwise, if it is a segment received, the reader will send the ACK/NACK to the A-IOT device to trigger transmission of a next segment or re-transmission of the current segment. If it is the last segment successfully received, then the reader may abort the communication if no further R2D message is required to be sent to the A-IOT device

- In the case of segmentation, the A-IOT device will perform transmission of the next segment or re-transmission of the current segment according to the reader’s instruction

Obviously, ACK/NACK is needed to trigger the A-IOT device to perform transmission of the next segment/re-transmission of the current segment. Also, the reader needs to identify if the currently received segment is the last one segment to decide whether or not to abort the current access process if no further R2D message is required to be sent to the A-IOT device. In our opinion, one-bit indication is sufficient for this purpose. In addition, another bit indicating if the message is a segment or an integrated complete D2R message is also needed.

Introduce ACK/NACK feedback for D2R data transmission.

Introduce one-bit flag for identification of not last segment/last segment in the A-IOT D2R MAC PDU.

Introduce one-bit flag for indication of segment or an integrated complete D2R message in the A-IOT D2R MAC PDU.

Issue-2: AS ID

As mentioned in the introduction part, in the last meeting, RAN2 agreed that it is up to the reader whether to use the random ID as AS ID or assigns a new AS ID. Since the A-IOT paging message could allocated access occasions or set a parameter for randomizing access occasions for targeted A-IOT devices, there is no need to allocate an AS ID for A-IOT msg1 transmission. For the A-IOT msg2 reception, the RN16 included in the A-IOT msg1 could be used as the AS ID, since the number of A-IOT devices performing the A-IOT msg1 transmission in a given access occasion is limited, and therefore the probability of them transmitting the same RN16 in the msg1 is quite low. However, the RN16 included in the msg1 is not proper for scheduling of subsequent D2R transmission and R2D reception, since the contention is no longer only limited to the devices performing A-IOT msg1 transmission in a given access occasion, but all devices waiting for the subsequent message scheduling/arrival. Hence, in the msg2, it is necessary for the reader to issue AS ID towards targeted A-IOT devices.

A-IOT device uses the RN16 included in the A-IOT msg1 for msg2 reception.

In the msg2, the reader can include an AS ID for subsequent R2D reception and D2R scheduling.

Issue-3: D2R response message indication and size

After the last RAN2#128 meeting, we sent a LS to SA2 asking them if the CN can indicate to the reader whether a message is expected in D2R direction in response to an R2D message for command service type. Later in SA2#166 ad-hoc meeting this year, they send the reply LS back to us in [1] that D2R response is always expected for a R2D message regardless the service type, as indicated as follows:

Q2: Can there be a case where a D2R response is not expected for a R2D message for command service type? If so, can the CN indicate to the reader whether a message is expected in D2R direction in response to an R2D message for command service type (e.g., either explicitly or implicitly by indicating the response message size if available)?

SA2 answer: No, a D2R response is always expected for a R2D message for command service type (e.g., the service data response for 'Read' service operation, and the operation result indication response for 'Write' or 'Disable' service operation).

In addition, in the SA2#166 meeting, SA2 indicates that AF can provides the information of the approximate D2R message size via AIOTF based on the AF request to the reader, but FFS whether the CN can determine such information if without AF’s provision, as indicated as follows:

Q1: Can the CN provide, to the reader, an estimate of the expected size of the following D2R message(s) in response to the service request?

SA2 answer: If provided by AF, AIOTF provides the reader with the approximate D2R message size based on AF request. Whether the CN can determine the approximate D2R message size if not provided by AF is under discussion in SA2.

In this regard, we think that RAN2 can confirm that D2R response is always expected for a R2D message, and wait for further conclusion of SA2 on whether or not the CN could determine and provide the reader with the information of the expected D2R message size regardless if it could be provided by the AF, before we make any further agreement on whom, the A-IOT device or the CN node, should provide the expected D2R message size information to the reader.

Confirm that D2R response is always expected for a R2D message.

Wait for further conclusion of SA2 on whether or not the CN could determine and provide the reader with the information of the expected D2R message if without AF’s provision, before we make any further agreement on who should provide the expected D2R message size information to the reader.

Issue-4: QoS requirement

In the reply LS from SA2 [1], they also indicate that no consensus on whether there will be any latency related requirements associated with completion time of a given A-IOT service to be provided/visible to the reader has been reached so far. As a result, we propose RAN2 to wait for further progress to be made, before making any agreement on introduction of QoS requirement in the Au interface.

Wait for SA2 further progress on whether or not there is latency related requirement associated with completion time of a given A-IOT service, before making any agreement on introduction of QoS requirement in the Au interface.

- Conclusion

We have the following proposals:

- Introduce ACK/NACK feedback for D2R data transmission.

Introduce one-bit flag for identification of not last segment/last segment in the A-IOT D2R MAC PDU.

Introduce one-bit flag for indication of segment or an integrated complete D2R message in the A-IOT D2R MAC PDU.

A-IOT device uses the RN16 included in the A-IOT msg1 for msg2 reception.

In the msg2, the reader can include an AS ID for subsequent R2D reception and D2R scheduling.

Confirm that D2R response is always expected for a R2D message.

Wait for further conclusion of SA2 on whether or not the CN could determine and provide the reader with the information of the expected D2R message if without AF’s provision, before we make any further agreement on who should provide the expected D2R message size information to the reader.

Wait for SA2 further progress on whether or not there is latency related requirement associated with completion time of a given A-IOT service, before making any agreement on introduction of QoS requirement in the Au interface.

Reference:

[1] S2-2501241, Reply LS on Assistance Information from CN to the Reader, SA2