3GPP TSG-RAN WG2 Meeting#129                               	R2-2500462

Athens, Greece, February 17th – 21st, 2025

Agenda item:	4.1

Source: 	Google

Title: 	Detection of consecutive HARQ feedback failures in NB-IoT NTN

Document for:	Discussion and Decision

# Introduction

In the live NB-IoT NTN network, the NAS attach procedures can stall due to asymmetric uplink/downlink conditions at the user equipment (UE). This occurs when the UE can receive downlink transmissions (including RS) but its uplink transmissions are blocked, for example, by temporary obstacles or inappropriate device orientation. The long attach delay is unacceptable if the user is trying to attach the network for sending an emergency/SOS message.

In this paper, the problem is analyzed in detailed with several observations being made, and a way forward that allows UE to escape from the prolonged NAS attach failure is proposed.

# Discussion

Figure 1 below illustrates a NAS ATTACH scenario where the UE reissues the attach request message after failing to receive the response (e.g., attach accept) of the first attach request the UE has sent. Initially, the UE NAS transmits an attach request to the network. The network responds with an authentication request, which the UE acknowledges successfully. Upon receiving the authentication response, the network transmits a security mode command, which the UE tries to acknowledge. After the UE transmits the first HARQ feedback (i.e., HARQ ACK), the UE monitors the PDCCH and receives a New Data Indicator (NDI) that is not toggled, implying that the HARQ feedback is not properly received by the network. In response, the UE transmits again the HARQ feedback that again is not received by the network. Consecutive uplink HARQ feedback failures often result from temporary obstacle (e.g., cloud) between the user equipment (UE) and the satellite, or improper device handling (e.g., incorrect orientation). After several unsuccessful HARQ feedback transmissions, the UE no longer receives network signals and enters a silent period, indicating the network has released the RRC connection. Unaware of this release, the UE must wait for a timeout (typically the T3410 timer or a data inactivity timer) before initiating a new NAS attach procedure. One particular situation allowing UE to abort a pending NAS attach procedure is the detection of a lower layer failure, as indicated below in TS 24.301 [1]. However, as in this scenario the DL reception does not encounter any problem, there will be no “out-of-sync” indication and hence no RLF caused by the DL reception problem will be triggered.

b) Lower layer failure not covered in case aa) or release of the NAS signalling connection without "Extended wait time" and without "Extended wait time CP data" received from lower layers before the ATTACH ACCEPT or ATTACH REJECT message is received The attach procedure shall be aborted, and the UE shall proceed as described below.

Figure 1. A NAS attach scenario where the UE waits for a long time before reissuing the attach request message, after consecutive HARQ feedback failures.

Consecutive uplink transmission problems can stall a NAS attach procedure.

UE has to wait until the data inactivity timer or T3412 expires before it can restart a stranded NAS attach procedure, if RLF is not triggered by the UE.

If the UE is not provided with the SR resource, consecutive RACH failures (due to temporary UL problems) triggered by a pending UL transmission may also trigger an LTE device to declare RLF. However, for an NB-IoT device, the UE wouldn’t trigger a RACH procedure if there is a UL resource (i.e., PUSCH) provided to the UE for transmitting the HARQ feedback, as indicated below in TS 36.321 [2].

- For NB-IoT: - if the MAC entity has no valid resource for SR together with acknowledgement of the data in this TTI and no valid PRACH resource for SR configured in any TTI: - initiate a Random Access Procedure (see clause 5.1), and cancel all pending SRs in the first subframe containing PRACH for preamble transmission. - else: - if the MAC entity has valid resource for SR together with acknowledgement of the data in this TTI: - instruct the physical layer to signal the SR together with acknowledgement of the data. - cancel, if any, initiated Random Access Procedure for SR.

A pending UL transmission does not trigger a RACH procedure if the UE is provided with a UL resource for transmitting HARQ feedback.

As the current conditions specified in TS 36.331 (section 5.3.11.3) [3] do not trigger the NB-IoT device encountering consecutive UL transmission problems to declare RLF, the NB-IoT device is not able to abort and restart the stranded NAS attach procedure, at least not before the timer T3410 or the data inactivity timer expires. The long waiting time before reissuing a NAS attach request is unacceptable if the user is trying to attach an IoT-NTN network for sending an emergency/SOS message, which is nowadays the most common use case in the market. To make it possible for the NB-IoT device to abort and restart the stranded NAS attach procedure (and also other stranded NAS procedures) earlier, we respectfully ask RAN2 to consider adding a mechanism in TS 36.321 that allows an NB-IoT device to detect consecutive HARQ feedback failures, and use the detection of the consecutive HARQ feedback failures to trigger RLF.

RAN2 to adopt a mechanism (in TS 36.321 and TS 36.331) allowing an NB-IoT device to detect consecutive HARQ feedback failures and to trigger RLF upon the detection.

If Propose 1 can be agreed, the TP attached at the end of this paper (in the Annex) can be taken as the baseline for further discussion. Figure 2 illustrates the solution based on the attached TP for TS 36.321, where the UE starts a timer counting the silent period upon detecting consecutive HARQ feedback A2N (ACK-to-NACK) problems, and sends an indication to upper layer upon the expiry of the timer.

Figure 2. An improved procedure that allows UE to abort the stranded NAS attach procedure upon detecting consecutive HARQ feedback failures.

Consider the TP in the Annex as the baseline for further discussion.

# Conclusion

In this paper, we discussed the issue of the prolonged NAS attach failure in NB-IoT NTN, and have the following observations.

Observation 1	Consecutive uplink transmission problems can stall a NAS attach procedure.

Observation 2	UE has to wait until the data inactivity timer or T3412 expires before it can restart a stranded NAS attach procedure, if RLF is not triggered by the UE.

Observation 3	A pending UL transmission does not trigger a RACH procedure if the UE is provided with a UL resource for transmitting HARQ feedback.

Based on the observation and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposal.

Proposal 1	RAN2 to adopt a mechanism (in TS 36.321 and TS 36.331) allowing an NB-IoT device to detect consecutive HARQ feedback failures and to trigger RLF upon the detection.

Proposal 2	Consider the TP in the Annex as the baseline for further discussion.

# Reference

3GPP TS 24.301 v17.12.0, Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS).

3GPP TS 36.321 v17.7.0, Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification.

3GPP TS 36.331 v17.11.0, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.

# Annex (TP for TS 36.321 and TS 36.331)

TP for TS 36.321

#### 5.3.2.2	HARQ process

For each TTI where a transmission takes place for the HARQ process, one or two (in case of downlink spatial multiplexing) TBs and the associated HARQ information are received from the HARQ entity.

For each received TB and associated HARQ information, the HARQ process shall:

-	if the NDI, when provided, has been toggled compared to the value of the previous received transmission corresponding to this TB; or

-	if the HARQ process is equal to the broadcast process and if this is the first received transmission for the TB according to the system information schedule indicated by RRC; or

-	if this is the very first received transmission for this TB (i.e. there is no previous NDI for this TB):

-	consider this transmission to be a new transmission.

-	if UE is an NB-IoT UE and operating in a non-terrestrial network:

-	set NTN_ACK_COUNT = 0.

-	else:

-	consider this transmission to be a retransmission.

-	if ntn_SilentTimer is running:

-	restart ntn_SilentTimer.

The MAC entity then shall:

-	if this is a new transmission:

-	attempt to decode the received data.

-	else if this is a retransmission:

-	if the data for this TB has not yet been successfully decoded:

-	combine the received data with the data currently in the soft buffer for this TB and attempt to decode the combined data.

-	if the data which the MAC entity attempted to decode was successfully decoded for this TB; or

-	if the data for this TB was successfully decoded before:

-	if the HARQ process is equal to the broadcast process:

-	deliver the decoded MAC PDU to upper layers.

-	else if this is the first successful decoding of the data for this TB:

-	deliver the decoded MAC PDU to the disassembly and demultiplexing entity.

-	if UE is an NB-IoT UE and operating in a non-terrestrial network:

-	increment NTN_ACK_COUNT by 1.

-	if NTN_ACK_COUNT >= NTN_NUM_A2N:

-	if ntn_SilentTimer is not running:

-	start ntn_SilentTimer:

-	generate a positive acknowledgement (ACK) of the data in this TB.

-	else:

-	replace the data in the soft buffer for this TB with the data which the MAC entity attempted to decode.

-	generate a negative acknowledgement (NACK) of the data in this TB.

-	if ntn_SilentTimer expires:

-	indicate a consecutive HARQ A2N problem to upper layers.

-	if the HARQ process is associated with a transmission indicated with a Temporary C-RNTI and the Contention Resolution is not yet successful (see clause 5.1.5); or

-	if the HARQ process is equal to the broadcast process; or

-	if the HARQ process is not associated with a transmission indicated with a PUR-RNTI and the timeAlignmentTimer, associated with the TAG containing the serving cell on which the HARQ feedback is to be transmitted, is stopped or expired:

-	do not indicate the generated positive or negative acknowledgement to the physical layer.

-	else:

-	indicate the generated positive or negative acknowledgement for this TB to the physical layer.

The MAC entity shall ignore NDI received in all downlink assignments on PDCCH for its Temporary C-RNTI when determining if NDI on PDCCH for its C-RNTI has been toggled compared to the value in the previous transmission.

NOTE 1:	When the MAC entity is configured with more than one serving cell, UE behaviors for storing data to the soft buffer is specified in TS 36.213 [2].

NOTE 2:	If the MAC entity receives a retransmission with a TB size different from the last valid TB size signalled for this TB, the UE behavior is left up to UE implementation.

End of TP for TS 36.321

TP for TS 36.331

#### 5.3.11.3	Detection of radio link failure

The UE shall:

1>	in case any DAPS bearer is configured, only the target PCell is considered in the following;

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon T318 expiry; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or

1> upon consecutive HARQ A2N problem indication from lower layers, if UE is an NB-IoT UE and operating in a non-terrestrial network; or

1>	upon indication from MCG RLC, which is allowed to be send on PCell, that the maximum number of retransmissions has been reached for an SRB or DRB:

2>	consider radio link failure to be detected for the MCG i.e. RLF;

2>	discard any segments of segmented RRC messages received;

2>	store the following radio link failure information in the VarRLF-Report (VarRLF-Report-NB in NB-IoT) by setting its fields as follows:

3>	clear the information included in VarRLF-Report (VarRLF-Report-NB in NB-IoT), if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	except for NB-IoT, set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	if the UE was configured to perform measurement reporting, not related to NR sidelink communication, for one or more neighbouring NR frequencies, include the measResultListNR;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	except for NB-IoT, if available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

3>	except for NB-IoT, if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise , except for NB-IoT, to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	except for NB-IoT, set the tac-FailedPCell to the tracking area code, if available, of the PCell where radio link failure is detected;

3>	except for NB-IoT, if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned an intra E-UTRA handover:

5>	include the previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO:

5>	include the previousUTRA-CellId and set it to the physical cell identity, the carrier frequency and the global cell identity, if available, of the UTRA Cell in which the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from NR and if the UE supports Radio Link Failure Report for Inter-RAT MRO NR:

5>	include the previousNR-PCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3.

3>	except for NB-IoT, if the UE supports QCI1 indication in Radio Link Failure Report and has a DRB for which QCI is 1:

4>	include the drb-EstablishedWithQCI-1;

3>	except for NB-IoT, set the connectionFailureType to rlf;

3>	except for NB-IoT, set the c-RNTI to the C-RNTI used in the PCell;

3>	except for NB-IoT, set the rlf-Cause to the trigger for detecting radio link failure;

2>	if the UE is configured with (NG)EN-DC; and

2>	if T316 is configured; and

2>	if SCG transmission is not suspended; and

2>	if the SCG is not deactivated; and

2>	if neither NR PSCell change nor NR PSCell addition is ongoing (i.e. T304 for the NR PSCell is not running as specified in TS 38.331 [82], clause 5.3.5.5.2, in (NG)EN-DC):

3>	initiate the MCG failure information procedure as specified in 5.6.26 to report MCG radio link failure;

2>	else:

3>	if AS security has not been activated:

4>	if the UE is a NB-IoT UE:

5>	if the UE is connected to EPC and the UE supports RRC connection re-establishment for the Control Plane CIoT EPS optimisation; or

5>	if the UE is connected to 5GC, the UE supports RRC connection re-establishment for the Control Plane CIoT 5GS optimisation and the UE is configured with a truncated 5G-S-TMSI:

6>	initiate the RRC connection re-establishment procedure as specified in 5.3.7;

5>	else:

6>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

4>	else:

5>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7;

In case of DC or NE-DC, the UE shall:

1>	upon T313 expiry; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC, which is allowed to be sent on PSCell, that the maximum number of retransmissions has been reached for an SCG, for a split DRB or for a split SRB:

2>	consider radio link failure to be detected for the SCG i.e. SCG-RLF;

2>	if the UE is configured with DC; or

2>	if the UE is configured with NE-DC and MCG transmission is not suspended:

3>	initiate the SCG failure information procedure as specified in 5.6.13 to report SCG radio link failure;

2>	else:

3>	initiate the connection re-establishment procedure as specified in TS 38.331 [82], clause 5.3.7.

In case of CA PDCP duplication, the UE shall:

1>	upon indication from an RLC entity, which is restricted to be sent on SCell only, that the maximum number of retransmissions has been reached:

2>	initiate the failure information procedure as specified in 5.6.21 to report RLC failure of type duplication;

If any DAPS bearer is configured and T304 is running, the UE shall:

1>	upon T310 expiry for the source PCell; or

1>	upon random access problem indication from source MCG MAC; or

1>	upon indication from source MCG RLC, which is allowed to be sent on source PCell, that the maximum number of retransmissions has been reached for an DRB:

2>	consider radio link failure to be detected for the source MCG;

2>	suspend the transmission of all DRBs in the source MCG;

2>	reset MAC for the source MCG;

2>	release the source connection;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report (VarRLF-Report-NB in NB-IoT), 48 hours after the radio link failure is detected, upon power off or upon detach, and for NB-IoT, upon entering another RAT.

End of TP for TS 36.331