3GPP TSG-RAN WG2 Meeting#129                               	R2-2500464

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.9.4

Source: 	Google

Title: 	Discussion on the support for PWS in NB-IoT

Document for:	Discussion and Decision

# Introduction

The support for the broadcast of PWS messages in NB-IoT was discussed in the RAN2#128 meeting [1], with the following agreements being made.

Agreements: - We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements. - We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed - Add the following PWS indication in Paging-NB: - etws-Indication; - cmas-Indication.

Based on these agreements, several important aspects on the support of PWS in NB-IoT are further explored in this paper.

# Discussion

## PWS message segmentation

It has been agreed that the LTE SIBs for PWS are used as a baseline for designing NB-IoT SIBs for PWS. As shown below, the maximum number of segments for a PWS message transmitted by LTE SIBs is 64.

#### – SystemInformationBlockType12 The IE SystemInformationBlockType12 contains a CMAS notification. SystemInformationBlockType12 information element -- ASN1START SystemInformationBlockType12-r9 ::= SEQUENCE { messageIdentifier-r9 BIT STRING (SIZE (16)), serialNumber-r9 BIT STRING (SIZE (16)), warningMessageSegmentType-r9 ENUMERATED {notLastSegment, lastSegment}, warningMessageSegmentNumber-r9 INTEGER (0..63), warningMessageSegment-r9 OCTET STRING, dataCodingScheme-r9 OCTET STRING (SIZE (1)) OPTIONAL, -- Cond Segment1 lateNonCriticalExtension OCTET STRING OPTIONAL, ..., [[ warningAreaCoordinatesSegment-r15 OCTET STRING OPTIONAL -- Need OR ]] } -- ASN1STOP

As shown below, 3GPP TS 36.331 [2] specifiies that the maximum number of bits for a LTE SIB is 2216 bits.  Thus, the maximum PWS message size supported by LTE is 64 * 2216 bits = 141,824 bits.

## 5.2 System information ### 5.2.1 Introduction #### 5.2.1.1 General <unrelated part omitted> NOTE 1: The physical layer imposes a limit to the maximum size a SIB can take. When DCI format 1C is used the maximum allowed by the physical layer is 1736 bits (217 bytes) while for format 1A the limit is 2216 bits (277 bytes), see TS 36.212 [22] and TS 36.213 [23]. For BL UEs and UEs in CE, the maximum SIB and SI message size is 936 bits, see TS 36.213 [23]. For NB-IoT, the maximum SIB and SI message size is 680 bits, see TS 36.213 [23].

Because each NB-IoT SIB cannot exceed 680 bits, if only up to 64 segments are allowed for transmitting a PWS message (same as in LTE),  the maximum PWS message size is reduced to 43,520 bits (64 segments * 680 bits). This is approximately one-third of the maximum PWS message size supported by LTE (141,824 bits).  A government disseminates PWS messages across multiple operators’ networks. If NB-IoT uses the same segmentation limit as LTE, PWS messages exceeding 43,520 bits cannot be transmitted via an NB-IoT network, potentially leaving users on NB-IoT networks uninformed.  To address this, RAN2 should support a similar maximum PWS message size for NB-IoT as for LTE. This would ensure consistent message delivery across different network technologies.

If NB-IoT uses the same segmentation limit as LTE, PWS messages exceeding 43,520 bits cannot be transmitted via an NB-IoT network.

RAN2 to discuss whether to increase the maximum number of segments for PWS messages beyond 64, for example, to 192. This is to enable NB-IoT to support PWS messages of comparable size to LTE.

## Handling of PWS segments after switching to a new cell

In LTE, if a UE (in the idle or connected mode) switching to a new cell has not fully received a segmented PWS message in the original cell, it discards any previously buffered segments, as specified below in 3GPP TS 36.331[2]. This is because the segmentation of the same PWS message can vary across base stations.

- 5.2.2.4 System information acquisition by the UE <unrelated part omitted> 1> if the UE is ETWS capable: 2> upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> discard any previously buffered warningMessageSegment; 3> clear, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType11; <unrelated part omitted> 1> if the UE is CMAS capable: 2> upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> discard any previously buffered warningMessageSegment; 3> clear, if any, stored values of messageIdentifier and serialNumber for SystemInformationBlockType12 associated with the discarded warningMessageSegment;

An LTE UE switching to a new cell would discard any previously buffered PWS segments, if it has not fully received a segmented PWS message in the original cell.

However, in NB-IoT NTN, it is very likely the segmentation of the PWS message in the original cell and in the new cell are performed by the same base station, especially in the transparent satellite payload architecture and when the cells are earth-moving cells. Therefore, discarding previously buffered PWS segments is not always a preferable approach, as it would prolong the PWS reception time significantly, resulting in greater UE power consumption. A more reasonable handling is to allow UE to keep the previously buffered PWS segments and resume the PWS reception after switching to a new cell in some circumstances. For instance, the UE can resume the reception of a segmented PWS message in the new cell when the new cell and the original cell share the same tracking area identity, or when the new cell indicates that the resumption of the PWS reception is supported.

Under certain circumstances (e.g., operating in NTN mode), the NB-IoT UE may resume the reception of a segmented PWS message after switching to a new cell.

## Inclusion of the PWS area information in Paging

The Cell Broadcast Entity (CBE) first takes care of all the aspects of formatting alerts messages, including the splitting of an alter message into a number of pages. The alerts messages are then passed all the way to the MME, and then eventually reach the eNB operating an NTN cell. The UE, under the coverage of the NTN cell and upon receiving the paging message including the CMAS indication, proceeds to acquire SIB12 and obtains the information in SIB12. If the geo-fencing information (i.e., warningAreaCoordinatesSegment) is also included in SIB12, the UE is able to determine whether to take actions (e.g., display the alert messages) based on whether the UE is within the alert area(s) or not. Since the cell coverage of the NTN cell is much larger than the alert area, most of the UEs within the NTN cell do not need to respond to the CMAS message, due to out of the alert area. However, as these UEs (within the NTN cell) still receive the paging message including a CMAS notification, they need to further acquire and store SIB1/SIB12 to determine whether to take further actions, which is a significant power waste for these UEs.

The likelihood of paging an irrelevant UE in an NTN cell for a PWS alert is very high.

To alleviate the aforesaid UE power waste, the simplest way is to duplicate the geo-fencing information (i.e., warningAreaCoordinatesSegment) in SIB12 and put the same geo-fencing information also in the paging message. However, as the geo-fencing information can include the coordinates of up to 100 vertices, embedding such detailed geo-fencing information in the paging message could bring significant signalling overheads. To achieve good trade-off between the UE power saving and the signalling overheads, the paging message may include the coarse information regarding the affected area of the PWS, where the coarse information is able to indicate affected area in a coarse level, but requires much less bits. For example, as illustrated in Figure 1, an NTN cell can be divided equally into four quadrants in accordance with the reference location of the NTN cell. The paging message includes the coordinate of the reference location and also a 4-bit long bitmap indicating whether there is any alert area within each quadrant, if the CMAS indication (i.e., cmas_Indication) is also present. The four quadrants are mapped to the 1st bit, 2nd bit, 3rd bit, and 4th bit in the bitmap, respectively, and the value ‘1’ in each bit indicates there is at least one alert area while the value ‘0’ indicates there is no alert area in the corresponding quadrant. The reference location can be even omitted in the paging message, and in such case, the UE will acquire and use the reference location in SIB31 as the basis to divide the NTN cell. In the example illustrated in Figure 1, as the alert area falls within the 3rd quadrant, the values of the bitmap are {0, 0, 1, 0}.

PWS area information in coarse level can be signaled together with a PWS indication in Paging-NB. The UE not within the area determined by the PWS area information can skip acquiring the system information relevant to the PWS.

Figure 1.The coarse geographical information in the Paging-NB divides an NTN cell into four quadrants.

## PWS reception in RRC_CONNECTED

Unlike LTE, NB-IoT UEs cannot receive broadcast SIBs while operating in the RRC_CONNECTED state. Consequently, to ensure the successful reception of a broadcast PWS message, the network may force NB-IoT devices into the RRC_IDLE state. This transition, however, disrupts ongoing services, impacting user experience.

To ensure seamless PWS reception without service interruption, NB-IoT should support the delivery of PWS messages through dedicated signaling.

Support transmission of PWS messages via dedicated signaling for NB-IoT UEs operating in the RRC_CONNECTED state.

To simplify the implementation, we propose reusing an existing DL RRC message for transmitting PWS messages to NB-IoT UEs in the RRC_CONNECTED state. Given that the PWS message is an upper layer message, the DLInformationTransfer-NB message is a suitable candidate for this purpose.

Utilize the DLInformationTransfer-NB message to transmit PWS messages to NB-IoT UEs operating in the RRC_CONNECTED state.

# Conclusion

In this paper, we discuss several extended issues for supporting the broadcast of PWS messages in NB-IoT, based on the current progress in RAN2. Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

PWS message segmentation

Observation 1	If NB-IoT uses the same segmentation limit as LTE, PWS messages exceeding 43,520 bits cannot be transmitted via an NB-IoT network.

Proposal 1	RAN2 to discuss whether to increase the maximum number of segments for PWS messages beyond 64, for example, to 192. This is to enable NB-IoT to support PWS messages of comparable size to LTE.

Handling of PWS segments after switching to a new cell

Observation 2	An LTE UE switching to a new cell would discard any previously buffered PWS segments, if it has not fully received a segmented PWS message in the original cell.

Proposal 2	Under certain circumstances (e.g., operating in NTN mode), the NB-IoT UE may resume the reception of a segmented PWS message after switching to a new cell.

Inclusion of the PWS area information in Paging

Observation 3	The likelihood of paging an irrelevant UE in an NTN cell for a PWS alert is very high.

Proposal 3	PWS area information in coarse level can be signaled together with a PWS indication in Paging-NB. The UE not within the area determined by the PWS area information can skip acquiring the system information relevant to the PWS.

PWS reception in RRC_CONNECTED

Proposal 4	Support transmission of PWS messages via dedicated signaling for NB-IoT UEs operating in the RRC_CONNECTED state.

Proposal 5	Utilize the DLInformationTransfer-NB message to transmit PWS messages to NB-IoT UEs operating in the RRC_CONNECTED state.

# Reference

Draft Report of 3GPP TSG RAN WG2 meeting #128, ETSI MCC.

3GPP TS 36.331 v18.4.0, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.