3GPP TSG-RAN WG2 Meeting#129                               	R2-2500467

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.9.2

Source: 	Google

Title: 	Open issues on the S&F operation

Document for:	Discussion and Decision

# Introduction

In Rel-19, the benefits of enabling the Store & Forward operation for IoT-NTN are identified and the following scopes are included in the revised WID of Rel-19 IoT-NTN [1].

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

The Rel-19 IoT-NTN discussion was kicked off in the RAN2#125bits meeting, and some progress was made in RAN2#127bis meeting, with regard to the S&F indication, the time information associated with the S&F status, and the list of satellite IDs provided by MME [2].

RAN2 Agreements in RAN2#128: - Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start) - It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable - “S&F operation” indication is provided in SIB1 - RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN) - RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation - RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite - RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list

In this paper, we discuss several FFS issues highlighted in the above RAN2#128 agreements, and also the impact of the list of satellite IDs provided by MME on RAN. Based on the discussion, several way forwards are proposed in this paper.

# Discussion

## Time information associated with the S&F mode transition

It was agreed in RAN2#128 [2] that the time information can be broadcasted to indicate when the current satellite operation mode will transit from S&F mode to normal mode (i.e., non-S&F mode), and the usage of such time information is to allow UE to at least delay some NAS procedures until the feeder link is resumed. The time information associated with the opposite transition (from normal mode to S&F mode) is still FFS. We believe the time information indicating when the “S&F mode” will start is beneficial to at least the Rel-19 UEs not supporting the S&F operation, as these UEs can determine whether to initiate/trigger an access attempt based on whether there is sufficient time to fulfill the access purpose. Without this information, the UEs not supporting the S&F operation may initiate an access attempt right before the satellite turns into the S&F mode, and then will be released by the network soon after the satellite turns into the S&F mode, which consumes considerable radio resources and UE energy.

In addition, for the UEs not supporting the S&F operation, the principle is to make these UEs stay in the cells operating in the normal mode for as long as possible, even if they are allowed to camp on the cell in the S&F mode. Therefore, while camping on the cell operating in the normal mode and indicating when S&F mode will start, the Rel-19 UEs not supporting the S&F operation should trigger the intra-/inter-frequency measurement before the S&F mode starts. Similar to the case of t-service in Rel-18, these UEs should trigger the intra-/inter-frequency measurement regardless of other triggering conditions (e.g., the serving cell quality, the distance to the reference location).

It is beneficial for the Rel-19 UE not supporting S&F operation to know when the S&F mode will start, as based on the time information the UE can determine: 1) whether to initiate an access attempt, and 2) when to trigger the neighbor cell measurement.

Therefore, to provide better access control and to facilitate the cell reselection for the Rel-19 UEs not supporting the S&F operation, the time information indicating when the “S&F mode” will start should be provided in the system information as well.

The time information indicating when the current satellite operation mode will transit from the normal mode to the S&F mode, is provided in the system information.

If proposal 1 can be agreed, there is no need to introduce a new IE in addition to the agreed IE indicating when the normal mode will start. Both time information can share the same IE and be differentiated based on the presence or absence of the S&F operation indication. That is, if the time information is signaled together with the S&F operation indication, the time information indicates when the normal mode will start; if the time information is signaled but the S&F operation indication is absent, the time information indicates when the S&F mode will start.

When the S&F operation indication is present, the time information associated with the S&F mode transition indicates when the normal mode will start.

When the S&F operation indication is absent, the time information associated with the S&F mode transition indicates when the S&F mode will start.

As the interpretation of the time information associated with the S&F mode transition depends on the S&F operation indication signaled in SIB1, we think it is better to also signal the time information in SIB1. This is also to avoid the ambiguity in interpreting the time information in the gap time between SIB1 and other SIB (e.g., SIB31). We think it is better to use absolute time in the time information as relative time would change (i.e., decrease) every time when it appears in the system information, which might bring another discussion related to whether to trigger the system information change notification or the modification of systemInfoValueTag in SIB1.

The time information associated with the S&F mode transition is an absolute time signaled in SIB1.

## RRC and NAS suspension for the S&F operation

It was briefly discussed in the RAN2#128 meeting that whether a new release cause is needed when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable. In our view, as most likely the UE would not complete a NAS procedure within the coverage time of a single satellite operating in the S&F mode, the UE may have moved between different RRC states multiple times during the lifetime of a NAS procedure. As these transitions between different RRC states may clear/reset the NAS progress especially if the UE has moved to RRC_IDLE once, we should avoid UE having an on-going NAS procedure moving to a “pure” idle state (RRC_IDLE as well as ECM_IDLE) whenever the feeder link or the service link becomes unavailable. For this purpose, when the feeder becomes not available, eNB can suspend UE’s RRC connection by sending an RRC Connection Release message to the UE supporting the S&F operation, where the RRC Connection Release message may include an rrc-InactiveConfig IE or set the releaseCause as ‘rrc-Suspend’. Upon receiving the RRC Connection Release message, the UE RRC indicates the suspension of the RRC to the NAS layer, which may result in the suspension to the NAS layer as well. However, since the existing RRC_INACTIVE state and RRC_IDLE state with RRC suspension is only applicable to the UE with AS security being established, a new RRC suspension state/mode is needed (i.e., a new release cause is needed) to support the initial ATTACH scenario and the CIoT-CP solution.

When the feeder-link becomes unavailable, eNB can release a R19 UE to RRC_IDLE with a new release cause. The new release cause will trigger the suspension of RRC (similar to the rrc-Suspend) even without the AS security being established.

## The list of satellite IDs provided by MME

SA2 has discussed [3] a list of satellite IDs over which the UE may re-attempt a NAS procedure if the procedure cannot be completed within the coverage time of a single satellite due to the S&F operation, where the list of satellite IDs can be provided during the Attach or TAU procedure. In the normative phase, the agreed CR S2-2412903 [4] considered the list of satellite IDs provided by MME as an FFS and captured it in the Editor’s Note.

The MME may adapt periodic update timer, mobile reachable timer and Implicit Detach timer to the fact that the UE is served in S&F mode. Editor’s Note: How to handle the specification of the “List of satelliteID(s)” and “S&F Monitoring List” as per conclusions in TR 23.700-29 is FFS. When an MME is operating in S&F Mode: - If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, if the UE supports Store and Forward Satellite operation, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation. - If a UE indicates Store and Forward capability, the MME may provide to the UE a S&F Wait Timer when accepting or rejecting a NAS procedure.

RAN2 also discussed the impact of the list of satellite IDs in the RAN2#128 meeting, and concluded that the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list. RAN2 also sent an LS R2-2411197 [5] to communicate with SA2 regarding the RAN2 understand. In our understanding, the satellite ID list provided by MME is ineffective unless used during AS cell selection and reselection procedure, as the list's purpose is to prioritize UE camping on the specified satellites.

The list of satellite IDs provided by MME cannot serve for the intended purpose (to prioritize camping on the satellites in the list) without influencing the cell selection/reselection procedure specified in TS 36.304.

While we are still waiting for the response from SA2, we may start looking into the impact of the MME’s satellite list on AS, especially on the cell selection/reselection procedure. Of course, we should follow the RAN2 conclusion that any change shall not prevent the UE from camping on a satellite that is not included in the MME’s satellite list. One possible change we can consider in RAN2 is to re-define the “suitable cell” and take the satellite ID (acquired from SIB31) of a cell as one factor while determining whether the cell is a suitable cell or not. This would not prevent the UE from camping on a satellite not included in the MME’s satellite list, as such a satellite can still be an “acceptable cell” after all. Another possible change is to increase the frequency priority of the carrier frequency associated with the satellite ID in the MME’s satellite list, or to increase the cell ranking of the cell associated with the satellite ID in the MME’s satellite list.

If needed, RAN2 to discuss the possible impact of the MME’s satellite list on the cell selection/reselection procedure, including but not limited to 1) re-defining the suitable cell taking into account the satellite ID, and 2) prioritizing the frequencies or cells associated with the MME’s satellite IDs during cell reselection.

# Conclusion

In this paper, we discuss the time information associated with the S&F mode transition, the new RRC release cause, and the impact of the list of satellite IDs provided by MME on RAN, with the following observation being made.

Observation 1	It is beneficial for the Rel-19 UE not supporting S&F operation to know when the S&F mode will start, as based on the time information the UE can determine: 1) whether to initiate an access attempt, and 2) when to trigger the neighbor cell measurement.

Observation 2	The list of satellite IDs provided by MME cannot serve for the intended purpose (to prioritize camping on the satellites in the list) without influencing the cell selection/reselection procedure specified in TS 36.304.

Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

Proposal 1	The time information indicating when the current satellite operation mode will transit from the normal mode to the S&F mode, is provided in the system information.

Proposal 2	When the S&F operation indication is present, the time information associated with the S&F mode transition indicates when the normal mode will start.

Proposal 3	When the S&F operation indication is absent, the time information associated with the S&F mode transition indicates when the S&F mode will start.

Proposal 4	The time information associated with the S&F mode transition is an absolute time signaled in SIB1.

Proposal 5	When the feeder-link becomes unavailable, eNB can release a R19 UE to RRC_IDLE with a new release cause. The new release cause will trigger the suspension of RRC (similar to the rrc-Suspend) even without the AS security being established.

Proposal 6	If needed, RAN2 to discuss the possible impact of the MME’s satellite list on the cell selection/reselection procedure, including but not limited to 1) re-defining the suitable cell taking into account the satellite ID, and 2) prioritizing the frequencies or cells associated with the MME’s satellite IDs during cell reselection.

# Reference

RP-243278, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.

Draft Report of 3GPP TSG RAN WG2 meeting #128, ETSI MCC.

3GPP TR 23.700-29 v19.0.0, Study on integration of satellite components in the 5G architecture.

S2-2412903, Introduction of Store and Forward feature in EPC, Nokia et al.

R2-2411197, LS on Satellite IDs for store-and-forward operation, CATT.